3GPP TSG-CT WG4 Meeting #105-e	C4-214521

# E-Meeting, 17th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0721 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SDM Shared Data Subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hewlett Packard Enterprise |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When subscribing to be notified of shared data, the NF Consumer indicates the specifc sharedData resource URI to be subscribed in the monitoredResourceUri attribute, e.g.: {apiRoot}/nudm-sdm/v2/shared-data/{sharedDataId}. Figure 6.1.3.1-1 however, does not show the sharedDataId as a resource below the sharedData anymore as it was incorrectly removed in C4-204327. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify shared data URI for SharedDataSubscriptions in Resource URI structure of the nudm-sdm API, as is already the case in 29.505. Typos in 5.2.2.3.3 are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification with respect to how an NF Consumer can subscribe to shared data. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.3, 6.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

##### 5.2.2.3.3	Subscription to notifications of shared data change

Figure 5.2.2.3.3-1 shows a scenario where the NF service consumer sends a request to the UDM to subscribe to notifications of shared data change. The request contains a callback URI and the URI of the monitored resource.

Figure 5.2.2.3.3-1: NF service consumer subscribes to notifications of shared data change

1.	The NF service consumer sends a POST request to the parent resource (collection of subscriptions) (.../shared-data-subscriptions), to create a subscription as present in message body. The payload body of the POST request shall contain a representation of the shared data individual subscription resource to be created. There shall be only one shared data individual subscription per NF service consumer identified by NfInstanceId in SdmSubscription.

2.	On success, the UDM responds with "201 Created" with the message body containing a representation of the created subscription. The Location HTTP header shall contain the URI of the created subscription.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

* * * Next Change * * * *

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the nudm-sdm API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name (Archetype) |  |  | Resource URI |  |  | HTTP method or custom operation |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supi (Document) |  |  | /{supi} |  |  | GET |  |  | Retrieve UE's subscription data |  |  |
| Nssai (Document) |  |  | /{supi}/nssai |  |  | GET |  |  | Retrieve the UE's subscribed Network Slice Selection Assistance Information |  |  |
| UeContextInAmfData (Document) | /{supi}/ue-context-in-amf-data |  |  | GET |  |  | Retrieve the UE's Context in AMF Data |  |  |
| AccessAndMobilitySubscriptionData (Document) |  |  | /{supi}/am-data |  |  | GET |  |  | Retrieve the UE's subscribed Access and Mobility Data |  |  |
|  |  |  | /{supi}/am-data/update-sor |  |  | update-sor (POST) |  |  | Trigger the update of Steering of Roaming Information at the UE |  |  |
| SorAck (Document) |  |  | /{supi}/am-data/sor-ack |  |  | PUT |  |  | Providing acknowledgement of Steering of Roaming |  |  |
| UpuAck (Document) |  |  |  |  | /{supi}/am-data/upu-ack |  |  | PUT |  |  | Providing acknowledgement of UE parameters update |  |  |
| CagAck (Document) |  |  |  |  | /{supi}/am-data/cag-ack |  |  | PUT |  |  | Providing acknowledgement of UE CAG configuration update |  |  |
| EnhancedCoverageRestrictionData |  |  |  |  | /{supi}/am-data/ecr-data |  |  | GET |  |  | Retrieve the UE's subscribed Enhance Coverage Restriction Data |  |  |
| SmfSelectionSubscriptionData (Document) |  |  | /{supi}/smf-select-data |  |  | GET |  |  | Retrieve the UE's subscribed SMF Selection Data |  |  |
| UeContextInSmfData (Document) |  |  | /{supi}/ue-context-in-smf-data |  |  | GET |  |  | Retrieve the UE's Context in SMF Data |  |  |
| SessionManagementSubscriptionData (Document) |  |  | /{supi}/sm-data |  |  | GET |  |  | Retrieve the UE's session management subscription data |  |  |
| SMSSubscriptionData (Document) |  |  | /{supi}/sms-data |  |  | GET |  |  | Retrieve the UE's SMS subscription data |  |  |
| SMSManagementSubscriptionData (Document) |  |  | /{supi}/sms-mng-data |  |  | GET |  |  | Retrieve the UE's SMS management subscription data |  |  |
| LcsPrivacySubscriptionData (Document) |  |  | /{ueId}/lcs-privacy-data |  |  | GET |  |  | Retrieve the UE's LCS privacy subscription data |  |  |
| LcsMobileOriginatedSubscriptionData (Document) |  |  | /{supi}/lcs-mo-data |  |  | GET |  |  | Retrieve the UE's LCS Mobile Originated subscription data |  |  |
| LcsBroadcastAssistanceSubscriptionData (Document) |  |  | /{supi}/lcs-bca-data |  |  | GET |  |  | Retrieve the UE's LCS Broadcast Assistance subscription data |  |  |
| V2xSubscriptionData (Document) |  |  | /{supi}/v2x-data |  |  | GET |  |  | Retrieve the UE's V2X subscription data |  |  |
| SdmSubscriptions (Collection) |  |  | /{ueId}/sdm-subscriptions |  |  | POST |  |  | Create a subscription |  |  |
| Individual subscription (Document) |  |  | /{ueId}/sdm-subscriptions/{subscriptionId} |  |  | DELETE |  |  | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |  |  |
|  |  |  |  |  |  | PATCH |  |  | Modify the sdm-subscription identified by {subscriptionId} |  |  |
| IdTranslationResult (Document) |  |  | /{ueId}/id-translation-result |  |  | GET |  |  | Retrieve a UE's SUPI or GPSI |  |  |
| UeContextInSmsfData (Document) |  |  | /{supi}/ue-context-in-smsf-data |  |  | GET |  |  | Retrieve the UE's Context in SMSF Data |  |  |
| TraceData (Document) |  |  | /{supi}/trace-data |  |  | GET |  |  | Retrieve Trace Configuration Data |  |  |
| SharedData (Collection) |  |  | /shared-data |  |  | GET |  |  | Retrieve shared data |  |  |
| SharedDataSubscriptions (Collection) |  |  | /shared-data-subscriptions |  |  | POST |  |  | Create a subscription |  |  |
| SharedDataIndividual subscription (Document) |  |  | /shared-data-subscriptions/{subscriptionId} |  |  | DELETE |  |  | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |  |  |
|  |  |  |  |  |  | PATCH |  |  | Modify the shared data subscription identified by {subscriptionId} |  |  |
| GroupIdentifiers (Document) |  |  | /group-data/group-identifiers |  |  | GET |  |  | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |  |  |
| SnssaisAck (Document) |  |  | /{supi}/am-data/subscribed-snssais-ack |  |  | PUT |  |  | Providing acknowledgement of UE for subscribed S-NSSAIs |  |  |

* * * End of Changes * * * *