3GPP TSG-CT WG4 Meeting #106-e	C4-215122

# E-Meeting, 11th – 15th October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0579 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling and disabling the adjustment of DL traffic steering rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS23.501 CR#3027 (see S2-2106647) and CR#3111 (see S2-2106648) have specified the detail PMF messages (i.e. PMF-UAD, PMF-UAT) exchanged between the UE and the UPF to adjust the DL traffic steering rules as per UE provided assistance data. It is propsed to align with stage 2 specification to clarify the UPF behavior and conditions for enabling/disabling the adjustment of DL traffic steering rules. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the UPF behaviour and conditions for enabling/disabling the adjustment of DL traffic steering rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UPF behaviour and conditions for enabling/disabling the adjustment of DL traffic steering rules is not quite clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

5.20.6	PMFP message handling in UPF

The access performance measurement supports the following procedures, as specified in clause 5.32.5.1 of 3GPP TS 23.501 [28]:

-	RTT measurement procedure initiated by the UE, or the UPF;

-	Packet Loss Rate measurement procedure initiated by the UE, or the UPF;

-	Access available/unavailable report procedure from the UE to the UPF;

-	UE Assistance Data provisioning procedure from the UE to the UPF.

The per QoS flow access performance measurement procedure includes the RTT measurement and the Packet Loss Rate measurement, which may be initiated by the UE or the UPF(PSA).

PMFP messages are used by the PMF functionality in the UE and the PMF functionality in the UPF (PSA) to perform the access performance measurement procedures, as specified in 3GPP TS 24.193 [59].

PMFP messages shall be exchanged between the UE and the UPF via the default QoS flow or a dedicated QoS flow, as specified in 3GPP TS 23.501 [28]. To support this, the UPF shall learn the QFI identifying the QoS flow on which PMFP messages from the UE are transmitted, and shall send downlink PMFP messages on that QoS flow when needed.

Once an uplink PMFP message is detected, the UPF shall internally forward the PMFP message to the PMF functionality in the UPF, for subsequent handling.

The PMF functionality in the UPF shall learn the UE's address information for PMFP from the received PMFP message, e.g. to detect the IP address and UDP port of the PMF in the UE (for IP PDU sessions) or the MAC address of the PMF in the UE (for Ethernet PDU sessions), for the purpose of sending PMFP messages to the UE.

If the Steering mode is set to Load-Balancing and the UE Assistance Indicator is set, when the UPF receives a PMF message with UE Assistance Data (i.e. PMF-UAD message) from the UE, the UPF may take the split percentages in the UE Assistance Data into account and create new DL traffic steering rules to steer the downlink traffic. Meanwhile, the UPF shall store the original DL traffic steering rules provisioned by the SMF, for the purpose of restoring the original DL traffic steering rules when the UE assistance mode is terminated. When the UPF receives a PMF message indicating the termination of UE assistance mode (i.e. PMF-UAT message), the UPF shall remove the DL traffic steering rules created for the UE assistance mode, and continue applying the original DL traffic steering rules received from the SMF.

* * * End of Changes * * * *