3GPP TSG-CT WG4 Meeting #107-bis-e	C4-220197

# E-Meeting, 17th – 21st January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.501 | CR | 0119 | rev | - | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Formatting of Description Fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide guidance on how to format description fields in OpenAPI, considering the Markdown formatting syntax; in particular for multi-line description fields where line-breaks must be kept to convey the intended style. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause describing how to format description fields. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The documentation of 3GPP OpenAPI descriptions, which is frequently generated automatically with tools from the YAML files, is not formatted as intended and gives an image of low quality documentation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.12, 5.3.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.3.12	Enumerations

For enumerations, as defined in clause 5.2.4.3, the OpenAPI Specification [4] file shall contain a definition in the components/schemas clause defining a schema with the name of the enumeration as key.

The schema

-	shall contain the "anyOf" keyword listing as alternatives:

1.	the "type: string" keyword and the "enum" keyword with a list of all defined values for the enumeration; and

2.	the "type: string" keyword and the "description" keyword with a description stating that the string is only provided for forward compatibility with future extensions and is not used to encode contents defined in the present version of the specification. Future extensions may need to be defined in conjunction with the supported feature mechanism as specified in clause 6.6.2 of 3GPP TS 29.500 [2]. and

-	may contain a description listing the defined values of the enumeration together with explanations of those values.

NOTE:	The "enum" keyword restricts the permissible values of the string to the enumerated ones. This can lead to extensibility problems when new values need to be introduced.

Example:

Table 5.3.12-1: Enumeration ExampleEnumeration

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| One | Value One description |  |
| Two | Value Two description |  |

The data structure in table 5.3.12-1 is described in an OpenAPI specification file as follows:

components:

schemas:

ExampleEnumeration:

anyOf:

- type: string

enum:

- One

- Two

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- One: Value One description

- Two: Value Two description

* * * Next Change * * * *

### 5.3.xx	Formatting of description fields

As indicated in OpenAPI Specification [4], "description" fields support the CommonMark Markdown formatting (see https://spec.commonmark.org/0.27). In the 3GPP APIs, when a description field consists of multiple lines of text, they should be documented as follows:

-	If line breaks need to be kept in the output formatting, this shall be indicated by two trailing white spaces (see https://spec.commonmark.org/0.27/#hard-line-breaks). Those are represented in the example below as gray-shaded non-break spaces for illustration purposes, but the YAML file shall contain two normal white spaces:

description: |

Text in the first line.

More text in the second line.

### Additional text in the third line.

-	If the text in the different lines should be formatted as a bulleted HTML list, each line may be prepended by the hyphen "-" character, as follows:

description: |

Text in the first line:

- List item 1

- List item 2

-	If the text should be displayed in the output formatting as a single line, the YAML file may be formatted using one of the following options:

-	All text in a single YAML line, following the "description" keyword, such as:

description: This is a long description text that is formatted as a single YAML line, which is correct, but may not be optimal when the YAML file is displayed on a text editor configured to not automatically wrap long lines.

-	The "description" keyword is followed by the ">" character and the text is placed in different lines (this option should be the preferred approach, rather than having extremely long lines in the YAML file), such as:

description: >

This text is the first part of a single line,

followed by this second line,

where all these 3 lines will be displayed in the same line in the output formatting.

* * * End of Changes * * * *