3GPP TSG-CT WG4 Meeting #108-e	C4-221398

# E-Meeting, 17th – 25th February 2022												Revision of C4-221385

Source:	Huawei

Title:	Pseudo-CR on Correction to StatusSubscribe update

Spec:	3GPP TS 29.532 v1.1.0

Agenda item:	6.1.16

Document for:	Decision

1. Reason for change

StatusSubscribe (update) service operation specifies in 4xx/5xx response, the message body shall contain a ProblemDetails structure. But the respective data structure misses ProblemDetails.

Rev1: changes to OpenAPI are added.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 29.532 v1.1.0.

*******

* * * Background * * * *

##### 5.3.2.6.3	Subscription update

When the StatusSubscribe service operation is used for updating a subscription, the NF Service Consumer (e.g. NEF, MBSF or AF) shall update its subscription to MB-SMF notifications by using the HTTP PATCH method as shown in Figure 5.3.2.6.3-1.

Figure 5.3.2.6.2-1: Updating a subscription to MB-SMF notifications

1.	The NF Service Consumer shall send a PATCH request to update the individual subscription resource at the MB-SMF (/mbs-sessions/subscriptions/{subscriptionId}). The message body contains an array(PatchItem), where each PatchItem type indicates a requested change to the StatusSubscription data (see clause 5.2.4.3 in 3GPP TS 29.571 [18]). The following information may be requested to be modified with array(PatchItem) structure (see Table 6.2.3.5.3.1-2):

-	Notification URI (callback URI), indicating the address where the MB-SMF shall send the notifications;

-	New expiration time;

-	Event ID and optionally also the information on the event.

2a.	On success, the MB-SMF shall return a "200 Ok" response with a representation of the modified subscription (StatusSubscription data structure, see clause 6.2.6.2.9).

2b.	On failure or redirection, one of the HTTP status code listed in the data structures supported by the PATCH Response Body (see Table 6.2.3.5.3.1-3) shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in the same Table 6.2.3.5.3.1-3.

* * * The Change * * * *

##### 6.2.3.5.3	Resource Standard Methods

6.2.3.5.3.1	PATCH

This method updates an individual subscription resource for an MBS session in the MB-SMF with StatusSubscribe service operation for the subscription update (see clause 5.3.2.6.3).

This method shall support the URI query parameters specified in table 6.2.3.5.3.1-1.

Table 6.2.3.5.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.1-2 and the response data structures and response codes specified in table 6.2.3.5.3.1-3.

Table 6.2.3.5.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1 | It shall contain the list of changes to be made to the Status Subscription (i.e. StatusSubscription data type), according to the JSON PATCH format specified in IETF RFC 6902 [16]. |

Table 6.2.3.5.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| StatusSubscription | M | 1 | 200 OK | Upon success, a response body shall be returned containing the updated Status Subscription. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same MB-SMF or MB-SMF (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same MB-SMF or MB-SMF (service) set. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.2.3.5.3.1-4: Headers supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

Table 6.2.3.5.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

Table 6.2.3.5.3.1-6: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same MB-SMF or MB-SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target MB-SMF (service) instance ID towards which the request is redirected |

Table 6.2.3.5.3.1-7: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same MB-SMF or MB-SMF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target MB-SMF (service) instance ID towards which the request is redirected |

* * * 2nd Change * * * *

## A.3	Nmbsmf_MBSSession API

openapi: 3.0.0

info:

title: Nmbsmf-MBSSession

version: 1.0.0-alpha.5

description: |

<API Name> Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.532 V1.2.0; 5G System; 5G Multicast-Broadcast Session Management Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.532/

## Skipped for clarity ##

/mbs-sessions/subscriptions/{subscriptionId}:

patch:

summary:  StatusSubscribe to modify (update or renew) an individual subscription

tags:

- Individual Subscription for an MBS session

operationId: StatusSubscribeMod

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the individual subscription to be modified

schema:

type: string

requestBody:

description: Data to be modified in the StatusSubscription

required: true

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

responses:

'200':

description: >

Successful modification of the individual Status Subscription

content (StatusSubscription).

ProblemDetails are reported at least with '400'.

application/json:

schema:

$ref: '#/components/schemas/StatusSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

## Skipped for clarity ##

* * * End of Changes * * * *