3GPP TSG-CT WG4 Meeting #109-e	C4-222132

# E-Meeting, 06th – 12th April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.527 | CR | 0045 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Failure to restore a PFCP session at UPF restart |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-03-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UPF behavior when it cannot restore a PFCP session with the SMF requested F-TEID and/or UE IP address is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the UPF cannot accept the requested F-TEID and/or UE IP address (e.g. because the IP address is no longer available at the UPF), the UPF shall reject the PFCP Session Establishment Request with a cause indicating so. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of interoperability issues if the SMF and UPF are implemented based on different assumptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2, 4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.3.2	Restoration Procedure for PSA UPF Restart

If F-TEID and/or UE IP address allocation is performed in the UPF, the UPF shall ensure that previously used F-TEID values and/or UE IP addresses are not immediately reused after a UPF restart, in order to avoid inconsistent F-TEID and/or UE IP address allocation throughout the network and to enable the restoration of PFCP sessions affected by the failure. How this is ensured is implementation specific.

The UPF shall not send GTP-U Error indication message for a configurable period after an UPF restart when the UPF receives a G-PDU not matching any PDRs.

During or immediately after an UPF Restart, the UPF shall place a local UPF Recovery Time Stamp value in all Heartbeat Request/Response messages.

Immediately after the re-establishment of a PFCP association between the SMF and the UPF, the SMF may start restoring PFCP sessions in the UPF.

The SMF should prioritize the PFCP sessions to restore based on operator's policy.

The SMF should control the load induced on the UPF when performing the PFCP restoration procedures, the way it is done is implementation specific.

When re-establishing a PFCP session and if F-TEID allocation and/or UE IP address is performed in the PSA UPF by network configuration, the SMF shall include a restoration indication in the PFCP Session Establishment Request message to indicate to the UPF it is for a restoration of an existing PFCP session and the UPF shall accept SMF allocated F-TEID and/or UE IP address if possible. If the UPF cannot accept the requested F-TEID and/or UE IP address (e.g. because the IP address is no longer available at the UPF), the UPF shall reject the PFCP Session Establishment Request with a cause indicating so.

* * * Next Change * * * *

### 4.3.4	Restoration Procedure for Intermediate UPF Restart

The SMF will receive the UPF recovery time stamps in PFCP heartbeat requests/responses.

After an Intermediate UPF restart, the PFCP association between the SMF(s) and the Intermediate UPF has to be re-established.

The restoration of the PFCP sessions may start immediately after the PFCP association setup procedure:

-	if the restoration is supported in the SMF on a proactive basis, the SMF may start re-establishing PFCP sessions matching any PDRs.

-	as defined in clause 4.3.2, the SMF should prioritize the PFCP sessions restoration.

-	if the restoration is supported in the SMF on a reactive basis:

-	the SMF shall establish an PFCP session with a wildcarded PDR to instruct the Intermediate UPF to forward G-PDU packets which are not matching any other PDRs to the SMF (to a F-TEID uniquely assigned in the SMF for this PFCP-u tunnel);

-	upon receipt of G-PDUs from this PFCP-u tunnel, the SMF shall then check if it has an active session for each received G-PDU packet:

-	if so, the SMF shall perform the PFCP Session establishment procedures to re-establish the corresponding PFCP sessions in the Intermediate UPF;

-	otherwise the SMF shall generate a GTP-U Error Indication with a destination address set to the source IP address of the received G-PDU, and send it to the Intermediate UPF. The Intermediate UPF shall forward this GTP-U Error Indication transparently. The SMF shall delete the G-PDU after the check for active sessions.

NOTE 1:	The UPF can filter the G-PDU packets with same target F-TEID and send only one such G-PDU to the Intermediate SMF.

When re-establishing a PFCP session and if F-TEID allocation is performed in the Intermediate UPF by network configuration, the SMF shall include a restoration indication in the PFCP Session Establishment Request message to indicate to the UPF it is for a restoration of an existing PFCP session and the UPF shall accept the SMF allocated F-TEID if possible. If the UPF cannot accept the requested F-TEID (e.g. because the IP address is no longer available at the UPF), the UPF shall reject the PFCP Session Establishment Request with a cause indicating so.

The Intermediate UPF shall not send any Error indication messages for an operator configurable period after an Intermediate UPF restart when the Intermediate UPF receives G-PDU not matching any PDRs.

NOTE 2:	If restoration on a reactive basis is used, the period needs to be longer than the time required by the SMF to detect the UPF restart, to establish the PFCP association and provision the wildcarded PDR. Otherwise, the period needs to be longer than the time required by the SMF to restore all the PFCP sessions on a proactive basis.

* * * End of Changes * * * *