3GPP TSG-CT WG4 Meeting #113	Draft_revision_of_C4-225125

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0766 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IP Ranges and IP Index |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc18 |  |  |  |  |  | Date: |  |  | 2022-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the DnnUpfInfoIntem, it is contains IP ranges which has been limited to be used for a PDU session with a subscribed static IP address, where the SMF need to use it find the right UPF. However, for the scenarios when the SMF or external servers, e.g. AAA/Radius server, performs the UE IP Address allocation, the SMF still needs to select a UPF supporting that UE IP address. So, NOTE 1 in the DnnUpfInfoIntem should be extended, e.g.: NOTE 1:The list of ranges of IPv4/v6 address may be used by the SMF to select a UPF which supports a UE static IP address received in user subscription, or may be used by the SMF to select a UPF when the UE IP address is allocated by the SMF or external server, e.g. AAA/Radius. Available UE IP address(es) space for a DNN/S-NSSAI may be divided by a number of IP Indexes, the SMF may receive such IP Index from the UDM or the PCF, and then the SMF is configured to determine if the UPF, or external server or the SMF shall be requested to allocate UE IP address, the SMF needs to select a UPF to serve the PDU session (with that the allocated UE IP address) regardless which entity performing UE IP address allocation. So, the ipv4IndexList and ipv6IndexList in the DnnUpfInfoIntem shall contains a list of IP Index(es) that the UPF supports regardless of if the UE IP address is allocated by the UPF, or is allocated by the SMF or external server. In addition, the SMF would not receive IP index from the Radius server, as described in clause 5.8.2.2.1 of TS 23.501, the SMF may receive the subscribed IP index from the UDM and the PCF. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the usage of IP range and IP Index. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification may lead interoperability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.15	Type: DnnUpfInfoItem

Table 6.1.6.2.15-1: Definition of type DnnUpfInfoItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnn | Dnn | M | 1 | Supported DNN. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. |
| dnaiList | array(Dnai) | O | 1..N | List of Data network access identifiers supported by the UPF for this DNN. The absence of this attribute indicates that the UPF can be selected for this DNN for any DNAI. |
| pduSessionTypes | array(PduSessionType) | O | 1..N | List of PDU session type(s) supported by the UPF for a specific DNN. The absence of this attribute indicates that the UPF can be selected for this DNN for any PDU session type supported by the UPF (see clause 6.1.6.2.13). |
| ipv4AddressRanges | array(Ipv4AddressRange) | O | 1..N | List of ranges of IPv4 addresses handled by UPF. (NOTE 1) |
| ipv6PrefixRanges | array(Ipv6PrefixRange) | O | 1..N | List of ranges of IPv6 prefixes handled by the UPF. (NOTE 1) |
| ipv4IndexList | array(IpIndex) | O | 1..N | List of Ipv4 Index supported by the UPF. (NOTE 3) |
| ipv6IndexList | array(IpIndex) | O | 1..N | List of Ipv6 Index supported by the UPF. (NOTE 3) |
| dnaiNwInstanceList | map(string) | O | 1..N | Map of a network instance per DNAI for the DNN, where the key of the map is the DNAI. When present, the value of each entry of the map shall contain a N6 network instance that is configured for the DNAI indicated by the key. (NOTE 2) |
| NOTE 1: The list of ranges of IPv4/v6 address may be used by the SMF to select a UPF which supports a UE static IP address received in user subscription, or may also be used by the SMF to select a UPF, e.g. when the UE IP address is to be allocated by the SMF or external server, e.g. AAA/Radius Server. NOTE 2: This IE may be used by the SMF to determine the Network Instance associated to a given S-NSSAI, DNN and DNAI. If this IE is not present, the SMF needs to be configured with corresponding information. NOTE 3: The list of IPv4/v6 Indexes may be used by the SMF to select a UPF which supports a specific IP Index received from the UDM or the PCF for a UE's PDU session. |  |  |  |  |

* * * End of Change * * * *