3GPP TSG-CT WG4 Meeting #113	C4-225559

# Toulouse, France, 14th – 18th November 2022									Revision of C4-225239

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.007 | CR | 0384 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PGW triggered PDN connection restoration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, RPCPSET |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PGW triggered PDN connection restoration procedure is defined in clause 31.4 of 3GPP TS 23.007, the PGW-C/SMF may send an Update Bearer Request (or a Create Bearer Request or a Delete Bearer Request) towards the SGW, either immediately or only when the PGW-C/SMF needs to send signalling (Create, Update or Delete Bearer Request) towards the SGW and MME. Create Bearer Request or a Delete Bearer Request is used to create/delete the bearer of the UE, it cannot be sent by the PGW-C/SMF immediately. It is clarified the update bearer towards the default bearer can be sent immediately by the PGW-C/SMF to notify the PGW Change Info IE and without update any parameters related to the default bearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description that the PGW-C/SMF may send Update bearer request message immediately towards the SGW and MME. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect definition on PGW triggered PDN connection restoration may cause different implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 31.4, 31.4B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

31.4	PGW triggered PDN connection restoration

The PGW-C/SMF currently supporting a PDN connection, or a new PGW-C/SMF from the same PGW-C/SMF set, may trigger a PDN connection restoration to move the PDN connection to a different PGW-C/SMF as defined in this clause.

This may happen e.g. when a PGW-C/SMF is de-instantiated (scale-in operation), or when a new PGW-C/SMF from the same PGW-C/SMF set is reselected for the PDN connection upon a request from the PCF or the PGW-U/UPF and, more generally, when the PGW-C/SMF supporting a PDN connection is no longer available and a new PGW-C/SMF of the same PGW-C/SMF set needs to send signalling to the SGW or MME, before the MME has triggered a PDN connection restoration.

Figure 31.4-1: PGW triggered PDN connection restoration

1.	The PGW-C/SMF currently supporting the PDN connection or another PGW-C/SMF from the same PGW-C/SMF set determines that the PDN connection needs to be moved to a different PGW-C/SMF, e.g. during a PGW-C/SMF set scale-in operation, or e.g. due to PCF or PGW-U/UPF reselecting an alternative PGW-C/SMF from the same PGW-C/SMF set due to the PGW-C/SMF serving the PDN connection being unavailable and this triggers bearer signalling towards the MME.

2 or 3.	The PGW-C/SMF currently supporting the PDN connection or the new PGW-C/SMF taking over the PDN connection may send an Update Bearer Request for the default bearer towards the SGW immediately or send Create Bearer Request (or a Update Bearer Request or a Delete Bearer Request) when the PGW-C/SMF needs to send this signalling towards the SGW and MME, including the PGW Change Info IE. The SGW shall relay transparently the PGW Change Info IE towards the MME. The PGW Change Info IE shall contain the new S5/S8 PGW IP address for control plane.

4.	Same as step 4 of Figure 31.3-1, with the MME including in the Create Session Request the new S5/S8 PGW-C/SMF IP address received in the PGW Change Info.

5.	Same as step 5 of Figure 31.3-1.

6.	Same as step 6 of Figure 31.3-1.

* * * Next Change * * * *

31.4B	PGW triggered PDN connection restoration towards ePDG

The PGW-C/SMF currently supporting a PDN connection, or a new PGW-C/SMF from the same PGW-C/SMF set, may trigger a PDN connection restoration to move the PDN connection to a different PGW-C/SMF as defined in this clause.

This may happen e.g. when a PGW-C/SMF is de-instantiated (scale-in operation), or when a new PGW-C/SMF from the same PGW-C/SMF set is reselected for the PDN connection upon a request from the PCF or the PGW-U/UPF and, more generally, when the PGW-C/SMF supporting a PDN connection is no longer available and a new PGW-C/SMF of the same PGW-C/SMF set needs to send signalling to the ePDG, before the ePDG has triggered a PDN connection restoration.

Figure 31.4B-1: PGW triggered PDN connection restoration towards ePDG

1.	The PGW-C/SMF currently supporting the PDN connection or another PGW-C/SMF from the same PGW-C/SMF set determines that the PDN connection needs to be moved to a different PGW-C/SMF, e.g. during a PGW-C/SMF set scale-in operation, or e.g. due to PCF or PGW-U/UPF reselecting an alternative PGW-C/SMF from the same PGW-C/SMF set due to the PGW-C/SMF serving the PDN connection being unavailable and this triggers bearer signalling towards the ePDG.

2 or 3.	The PGW-C/SMF currently supporting the PDN connection or the new PGW-C/SMF taking over the PDN connection may send an Update Bearer Request for the default bearer towards the ePDG immediately or send Create Bearer Request (or a Update Bearer Request or a Delete Bearer Request) when the PGW-C/SMF needs to send this signalling towards the ePDG, including the PGW Change Info IE. The PGW Change Info IE shall contain the new S2b PGW IP address for control plane.

4.	Same as step 4 of Figure 31.3A-1, with the ePDG including in the Create Session Request the new S5/S8 PGW-C/SMF IP address received in the PGW Change Info.

5.	Same as step 5 of Figure 31.3A-1.

6.	Same as step 6 of Figure 31.3A-1.

* * * End of Changes * * * *