3GPP TSG-CT WG4 Meeting #114	C4-230234

# Athens, Greece, 27th Feb – 03rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0842 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple location report for MT-LR Immediate Location Request for the regulatory service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MLR |  |  |  |  |  | Date: |  |  | 2023.02.01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to S2-2210990, In the case of regulatory services (e.g. emergency and lawful interception) it would be beneficial to receive multiple locations estimates in a single session for a target UE, this would enable the regulatory service to get more UE location information. The Proposed solution introduces new procedures for MT-LR Immediate Location Request for the regulatory service to provide multiple location estimates to LCS Client for target UE during a short period of time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GMLC will send to AMF that the intermediate response is enabled and the duration of time the intermediate response is expected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-compliance with SA2 requirements. AMF will not support Multiple location report for MT-LR Immediate Location Request for the regulatory service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.2.1, 6.4.6.1, 6.4.6.2.2, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backward compatible features to the OpenAPI definition of the Namf_Location Service API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.5.2.2.1	General

The ProvidePositioningInfo service operation is used in the following procedure:

-	5GC-MT-LR Procedure without UDM Query (see 3GPP TS 23.273 [42], clause 6.10.2)

-	5GC-MT-LR Procedure (see 3GPP TS 23.273 [42], clause 6.1)

-	5GC-MT-LR multiple location procedure for the regulatory location service (see 3GPP TS 23.273 [42], clause 6.1.3)

-	Initiation and Reporting of Location Events (see 3GPP TS 23.273 [42], clause 6.3.1)

-	Location Continuity for Handover of an Emergency session from NG-RAN (see 3GPP TS 23.273 [42], clause 6.10.3)

The ProvidePositioningInfo service operation shall be invoked by the NF Service Consumer (e.g. GMLC) to request the current or deferred geodetic and optionally local and/or civic location of the UE. The service operation triggers the AMF to invoke the service towards the LMF.

The NF Service Consumer shall invoke the service operation by sending POST to the URI of the "provide-pos-info" custom operation on the "Individual UE Context" resource (See clause 6.4.3.2.4.2). See also figure 5.5.2.2.1-1.

Figure 5.5.2.2.1-1: NF Service Consumer requests the positioning information of the UE

1.	The NF Service Consumer shall send a POST request to the resource URI of "provide-pos-info" custom operation of the "Individual UE context" resource of the AMF. The payload body of the POST request may contain an indication of a positioning request from emergency services or commercial services client, the required QoS, Supported GAD shapes and scheduled location time, reliable UE Location Request, the indication of acceptance of intermediate response and, if intermediate responses are accepted, the maximum response time and LIR reference number to be sent within intermediate responses.

If the NF service consumer wants

-	the location change information,

-	deferred location information to be notified (e.g. during a handover procedure or for activation or completion of deferred location) or

-	intermediate location responses (during a 5GC-MT-LR multiple location procedure for the regulatory location service),

it also provides a callback URI on which the EventNotify service operation is executed (see clause 5.5.2.3) or towards which intermediate location responses shall be sent.

2a.	On success, "200 OK" shall be returned, the payload body containing the LCS correlation identifier, the location estimate, its age and accuracy and the information about the positioning method. If the request is invoked during a handover the response body shall also include the target AMF node identifier as specified in clause 6.10.3 of 3GPP TS 23.273 [42].

2b.	On accept, "204 No Content" shall be returned to acknowledge that AMF supports a deferred location request, and a deferred location is accepted as specified in step 6 of clause 6.3.1 of 3GPP TS 23.273 [42].

2c.	On failure or redirection, one of the HTTP status codes listed in Table 6.4.3.2.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.4.3.2.4.2.2-2.

* * * Next Change * * * *

#### 6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the Namf_Location service based interface protocol.

[…]

Table 6.4.6.1-2: Namf_Location re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |
| Gpsi | 3GPP TS 29.571 [6] | General Public Subscription Identifier |
| Pei | 3GPP TS 29.571 [6] | Permanent Equipment Identifier |
| ExternalClientType | 3GPP TS 29.572 [25] | LCS Client Type (Emergency, Lawful Interception …) |
| LocationQoS | 3GPP TS 29.572 [25] | LCS QoS (accuracy, response time) |
| SupportedGADShapes | 3GPP TS 29.572 [25] | LCS supported GAD shapes |
| GeographicArea | 3GPP TS 29.572 [25] | Estimate of the location of the UE |
| AccuracyFulfilmentIndicator | 3GPP TS 29.572 [25] | Requested accuracy was fulfilled or not |
| AgeOfLocationEstimate | 3GPP TS 29.572 [25] | Age Of Location Estimate |
| PositioningMethodAndUsage | 3GPP TS 29.572 [25] | Usage of each non-GANSS positioning method |
| VelocityEstimate | 3GPP TS 29.572 [25] | Estimate of the velocity of the target UE |
| VelocityRequested | 3GPP TS 29.572 [25] | Indication of the Velocity requirement |
| LcsPriority | 3GPP TS 29.572 [25] | Priority of the LCS client |
| GnssPositioningMethodAndUsage | 3GPP TS 29.572 [25] | Usage of each GANSS positioning method |
| CivicAddress | 3GPP TS 29.572 [25] | Civic address |
| BarometricPressure | 3GPP TS 29.572 [25] | Barometric Pressure |
| Altitude | 3GPP TS 29.572 [25] | Altitude estimate of the UE |
| LocalArea | 3GPP TS 29.572 [25] | Local area specified by different shape |
| MinorLocationQoS | 3GPP TS 29.572 [25] | Minor Location QoS |
| Ecgi | 3GPP TS 29.571 [6] | UE EUTRAN cell information |
| Ncgi | 3GPP TS 29.571 [6] | UE NR cell information |
| SupportedFeatures | 3GPP TS 29.571 [6] | Supported Features |
| RatType | 3GPP TS 29.571 [6] | RAT type |
| TimeZone | 3GPP TS 29.571 [6] | Time Zone |
| DateTime | 3GPP TS 29.571 [6] | Date and Time |
| UserLocation | 3GPP TS 29.571 [6] | User Location |
| LcsServiceType | 3GPP TS 29.572 [25] | The LCS service type |
| LdrType | 3GPP TS 29.572 [25] | The type of LDR for deferred location |
| Uri | 3GPP TS 29.571 [6] | URI |
| LdrReference | 3GPP TS 29.572 [25] | LDR Reference Number for deferred location |
| PeriodicEventInfo | 3GPP TS 29.572 [25] | Information for periodic event reporting |
| AreaEventInfo | 3GPP TS 29.572 [25] | Information for area event reporting |
| MotionEventInfo | 3GPP TS 29.572 [25] | Information for motion event reporting |
| ExternalClientIdentification | 3GPP TS 29.515 [46] | External LCS client identification |
| NFInstanceId | 3GPP TS 29.571 [6] | Identification of an NF or AF |
| CodeWord | 3GPP TS 29.515 [46] | Codeword for a 5GC-MT-LR or deferred 5GC-MT-LR |
| LMFIdentification | 3GPP TS 29.572 [25] | Identification of a serving LMF for periodic or triggered location |
| TerminationCause | 3GPP TS 29.572 [25] | Termination cause for a deferred location |
| UePrivacyRequirements | 3GPP TS 29.515 [46] | The location related privacy requirements on UE |
| DiameterIdentity | 3GPP TS 29.571 [6] | Diameter Identity |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |
| RedirectResponse | 3GPP TS 29.571 [6] | Response body of the redirect response message. |
| E164Number | 3GPP TS 29.503 [35] | The E.164 number. |
| DurationSec | 3GPP TS 29.571 [6] | Duration in seconds |

* * * Next Change * * * *

##### 6.4.6.2.2	Type: RequestPosInfo

Table 6.4.6.2.2-1: Definition of type RequestPosInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| lcsClientType | ExternalClientType | M | 1 | This IE shall contain the type of LCS client (Emergency, Lawful Interception etc.,.) issuing the location request |
| lcsLocation | LocationType | M | 1 | This IE shall contain the type of location measurement requested, such as current location,current or last known location, deferred location, etc. (NOTE 2) |
| supi | Supi | C | 0..1 | If the SUPI is available, this IE shall be present. |
| gpsi | Gpsi | C | 0..1 | If the GPSI is available, this IE shall be present. |
| priority | LcsPriority | O | 0..1 | If present, this IE shall contain the priority of the LCS client issuing the positioning request. |
| lcsQoS | LocationQoS | O | 0..1 | If present, this IE shall contain the quality of service requested, such as the accuracy of the positioning measurement and the response time of the positioning operation. Multiple QoS Class (lcsQosClass sets to "MULTIPLE_QOS") shall only be used when AMF support MUTIQOS feature. |
| velocityRequested | VelocityRequested | O | 0..1 | If present, this IE shall contain an indication of whether or not the Velocity of the target UE is requested. |
| lcsSupportedGADShapes | SupportedGADShapes | O | 0..1 | If present, this IE shall contain one GAD shape supported by the LCS client. |
| additionalSuppGADShapes | array(SupportedGADShapes) | C | 1..N | Shall be absent if lcsSupportedGADShapes is absent. Shall be present if the LCS client supports more than one GAD shape. |
| locationNotificationUri | Uri | O | 0..1 | The callback URI on which location change event notification or intermediate location responses are reported. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.4.8 is supported. |
| oldGuami | Guami | C | 0..1 | This IE shall be present during an AMF planned removal procedure when the NF Service Consumer initiates a request towards the target AMF, for a UE associated to an AMF that is unavailable (see clause 5.21.2.2 of 3GPP TS 23.501 [2]). |
| pei | Pei | C | 0..1 | This IE shall be present if supi and gpsi are not available. |
| lcsServiceType | LcsServiceType | O | 0..1 | This IE contains the LCS service type for an external client. (NOTE 1) |
| ldrType | LdrType | C | 0..1 | This IE contains the type of LDR for a deferred location request. This IE shall be present when lcsLocation is set to "DEFERRED_LOCATION". |
| hgmlcCallBackURI | Uri | C | 0..1 | This IE contrains the callback URI of the H-GMLC for a deferred location request. This IE shall be present when lcsLocation is set to "DEFERRED_LOCATION". |
| liResponseInd | boolean | O | 0..1 | When present this IE shall indicate whether the NF service consumer accepts intermediate location responses, as specified in 3GPP TS 23.273 [4] clause 6.1.3. - true: intermediate response is accepted. - false (default) intermediate response is not accepted. |
| maxResponseTime | DurationSec | C | 0..1 | This IE shall be included when liResponseInd is present and set to true. When present, maxResponseTime (in seconds) indicates the maximum response time to receive the final response. |
| lirReference | LdrReference | C | 0..1 | When present, this IE shall contain the LIR Reference Number to be sent within intermediate location responses. This IE shall be present if liResponseInd is present and set to true. |
| ldrReference | LdrReference | C | 0..1 | This IE contains the LDR Reference Number for a deferred location request This IE shall be present when lcsLocation is set to "DEFERRED_LOCATION". |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | This IE contains information for periodic event reporting for a deferred location request. This IE shall be present when ldrType is set to "PERIODIC". |
| areaEventInfo | AreaEventInfo | C | 0..1 | This IE contains information for area event reporting for a deferred location request. This IE shall be present when ldrType is set to "ENTERING_INTO_AREA", "LEAVING_FROM_AREA" or "BEING_INSIDE_AREA". |
| motionEventInfo | MotionEventInfo | C | 0..1 | This IE contains information for motion event reporting for a deferred location request. This IE shall be present when ldrType is set to "MOTION". |
| externalClientIdentification | ExternalClientIdentification | O | 0..1 | This IE provides the external LCS client identification (e.g. the name of the LCS client). (NOTE 1) |
| afID | NfInstanceId | O | 0..1 | This IE provides the identification of an AF that initiated the location request. (NOTE 1) |
| codeWord | CodeWord | O | 0..1 | This IE provides a codeword for a location request which is provided by an external Client or AF and is sent to and verified by a target UE as part of privacy verification. (NOTE 1) |
| uePrivacyRequirements | UePrivacyRequirements | O | 0..1 | If present, the IE provides the indication of location related notification or verification for the target UE, the indication of codeword check in UE |
| scheduledLocTime | DateTime | O | 0..1 | If present, the IE provides the scheduled time that the UE needs to be located. |
| reliableLocReq | boolean | C | 0..1 | This IE shall be included with the value "true" to indicate that reliable UE location information is required, as specified in 3GPP TS 33.256 [57] clause 5.3.2. When present, this IE shall be set as following: - true: the reliable UE location information is required - false (default): the reliable UE location information is not required |
| NOTE 1: At least one of these IEs should be present when uePrivacyCallSessionUnrelatedClass indicates notification and/or verification for the target UE. NOTE 2: If the lcsLocation IE is set to value "NOTIFICATION_VERIFICATION_ONLY", then the lcsServiceAuthInfo attribute in the uePrivacyRequirements IE, if present, shall be set to either "NOTIFICATION_ONLY" or "NOTIFICATION_AND_VERIFICATION_ONLY". |  |  |  |  |

* * * Next Change * * * *

# A.5	Namf_Location

openapi: 3.0.0

info:

version: 1.3.0-alpha.1

title: Namf_Location

description: |

AMF Location Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

schemas:

RequestPosInfo:

description: Data within Provide Positioning Information Request

type: object

properties:

lcsClientType:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/ExternalClientType'

lcsLocation:

$ref: '#/components/schemas/LocationType'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

priority:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LcsPriority'

lcsQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

velocityRequested:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/VelocityRequested'

lcsSupportedGADShapes:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/SupportedGADShapes'

additionalLcsSuppGADShapes:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/SupportedGADShapes'

minItems: 1

locationNotificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oldGuami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

lcsServiceType:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LcsServiceType'

ldrType:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

liResponseInd:

type: boolean

default: false

maxResponseTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

lirReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

ldrReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

periodicEventInfo:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/MotionEventInfo'

[…]

* * * End of Changes * * * *