3GPP TSG-CT WG4 Meeting #114	C4-230350

# Athens, Greece, 27th Feb – 03rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0157 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add GNSS integrity requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_ph2 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As defined in TS 23.273, when invoking location procedure, LCS client/AF may provide the GNSS integrity requirement in the request message. When NEF/GMLC receives the parameter, it further delivers the GNSS integrity requirement to the LMF. Similarly, when requsting MO-LR, UE may provide the GNSS integrity requirement in the request message. As the agreed SA2 CR 0282 (S2-2300953), it is proposed to agree three parameters, Time-to-Alert (TTA), Target Integrity Risk (TIR) and Alert Limit(AL) for GNSS integrity requirement. It proposes to add GNSS integrity requirement to align with SA2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to add GNSS integrity requirement to align with SA2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not aligned with Stage2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.2.2.2, 6.1.6.1, 6.1.6.2.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0282 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatibile new features to the OpenAPI files of Nlmf_Location API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Begin of Changes * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 7807: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 29.518: "Access and Mobility Management Services".

[24]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[25]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[26]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[x]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

* * * Next Change * * * *

5.2.2.2	DetermineLocation

5.2.2.2.1	General

The following procedures are defined, using the "DetermineLocation" service operation:

-	Retrieve UE Location

-	Retrieve UE Location for 5G-MO-LR

5.2.2.2.2	Retrieve UE Location

This procedure allows a consumer NF to request the location information (geodetic location and, optionally, local and/or civic location) for a target UE or to activate periodic or triggered deferred location for a target UE.

Figure 5.2.2.2.2-1: DetermineLocation Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the "determine-location" custom operation. The input parameters for the request (external client type, LCS correlation identifier, serving cell identifier, location QoS, supported GAD shapes, LDR Type, H-GMLC address, LDR Reference, UE connectivity state per access type, TNAP identifier, TWAP identifier, scheduled location time, integrity requirement, ….) may be included in the HTTP POST request body;

If UE geographical area identified by the country, area within a country or international area needs to be determined, the NF Service Consumer shall include UE geographical area determination for PLMN selection verification indication in the request;

If UE LCS Capability is received in the request indicating LPP is not supported by the UE, the LMF shall not send LPP messages to the UE in subsequent positioning procedures.

2a.	On success, "200 OK" shall be returned. The response body shall contain the parameters related to the determined position of the UE if any (geodetic position, local location, civic location, positioning methods…);

If the NF service consumer has requested to determine UE country, area within a country or international area, the LMF shall also include ueAreaInd.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.4.2.2-2.

5.2.2.2.3	Retrieve UE Location for 5G-MO-LR

This procedure allows a consumer NF (i.e. an AMF) to request the location information or location assistance data for a target UE which initiates MO-LR procedure (see 3GPP TS 23.273 [19]).

Figure 5.2.2.2.3-1: DetermineLocation Request for 5G-MO-LR

The same requirements in clause 5.2.2.2.2 shall be applied with following modifications:

1.	Same as step 1 of figure 5.2.2.2.2-1, the request body shall include the following additional information:

-	The indication received from UE indicating whether a location estimate or location assistance data is required.

-	The LPP messages if received in MO-LR Request from UE

-	UE's subscribed assistance data for 5GC-MO-LR if received from UDM.

2a.	Same as step 2a of figure 5.2.2.2.2-1 if a consumer NF requests the location information for a target UE. If a NF consumer requests location assistance data for a target UE and LMF has successfully delivered location assistance data to the UE, 204 No Content shall be returned.

2b.	Same as step 2b of figure 5.2.2.2.2-1.

* * * Next Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nlmf_Location service based interface protocol.

Table 6.1.6.1-1: Nlmf_Location specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| InputData | 6.1.6.2.2 | Information within Determine Location Request |
| LocationData | 6.1.6.2.3 | Information within Determine Location Response |
| GeographicalCoordinates | 6.1.6.2.4 | Geographical coordinates |
| GeographicArea | 6.1.6.2.5 | Geographic area specified by different shape |
| Point | 6.1.6.2.6 | Ellipsoid Point |
| PointUncertaintyCircle | 6.1.6.2.7 | Ellipsoid point with uncertainty circle |
| PointUncertaintyEllipse | 6.1.6.2.8 | Ellipsoid point with uncertainty ellipse |
| Polygon | 6.1.6.2.9 | Polygon |
| PointAltitude | 6.1.6.2.10 | Ellipsoid point with altitude |
| PointAltitudeUncertainty | 6.1.6.2.11 | Ellipsoid point with altitude and uncertainty ellipsoid |
| EllipsoidArc | 6.1.6.2.12 | Ellipsoid Arc |
| LocationQoS | 6.1.6.2.13 | QoS of Location request |
| CivicAddress | 6.1.6.2.14 | Indicates a Civic address |
| PositioningMethodAndUsage | 6.1.6.2.15 | Indicates the usage of a positioning method |
| GnssPositioningMethodAndUsage | 6.1.6.2.16 | Indicates the usage of a Global Navigation Satellite System (GNSS) positioning method |
| VelocityEstimate | 6.1.6.2.17 | Velocity estimate |
| HorizontalVelocity | 6.1.6.2.18 | Horizontal velocity |
| HorizontalWithVerticalVelocity | 6.1.6.2.19 | Horizontal and vertical velocity |
| HorizontalVelocityWithUncertainty | 6.1.6.2.20 | Horizontal velocity with speed uncertainty |
| HorizontalWithVerticalVelocityAndUncertainty | 6.1.6.2.21 | Horizontal and vertical velocity with speed uncertainty |
| UncertaintyEllipse | 6.1.6.2.22 | Ellipse with uncertainty |
| UeLcsCapability | 6.1.6.2.23 | Indicates the LCS capability supported by the UE. |
| PeriodicEventInfo | 6.1.6.2.24 | Indicates the information of periodic event reporting |
| AreaEventInfo | 6.1.6.2.25 | Indicates the information of area based event reporting |
| ReportingArea | 6.1.6.2.26 | Indicates an area for event reporting |
| MotionEventInfo | 6.1.6.2.27 | Indicates the information of motion based event reporting |
| CancelLocData | 6.1.6.2.29 | Information within Cancel Location Request |
| LocContextData | 6.1.6.2.30 | Information within Transfer Location Context Request |
| EventReportMessage | 6.1.6.2.31 | Indicates an event report message |
| EventReportingStatus | 6.1.6.2.32 | Indicates the status of event reporting |
| UELocationInfo | 6.1.6.2.33 | Indicates location information of a UE |
| EventNotifyData | 6.1.6.2.34 | Information within Event Notify Request |
| UeConnectivityState | 6.1.6.2.35 | Indicates the connectivity state of a UE |
| LocalOrigin | 6.1.6.2.36 | Indicates a Local origin in a reference system. |
| RelativeCartesianLocation | 6.1.6.2.37 | Relative Cartesian Location |
| Local2dPointUncertaintyEllipse | 6.1.6.2.38 | Local 2D point with uncertainty ellipse |
| Local3dPointUncertaintyEllipsoid | 6.1.6.2.39 | Local 3D point with uncertainty ellipsoid |
| UncertaintyEllipsoid | 6.1.6.2.40 | Ellipsoid with uncertainty |
| LocalArea | 6.1.6.2.41 | Local area specified by different shape |
| UeAreaIndication | 6.1.6.2.42 | UE area Indication |
| MinorLocationQoS | 6.1.6.2.43 | Minor Location QoS |
| Altitude | 6.1.6.3.2 | Indicates value of altitude |
| Angle | 6.1.6.3.2 | Indicates value of angle |
| Uncertainty | 6.1.6.3.2 | Indicates value of uncertainty |
| Orientation | 6.1.6.3.2 | Indicates value of orientation angle |
| Confidence | 6.1.6.3.2 | Indicates value of confidence |
| Accuracy | 6.1.6.3.2 | Indicates value of accuracy |
| InnerRadius | 6.1.6.3.2 | Indicates value of the inner radius |
| CorrelationID | 6.1.6.3.2 | LCS Correlation ID |
| AgeOfLocationEstimate | 6.1.6.3.2 | Indicates value of the age of the location estimate |
| HorizontalSpeed | 6.1.6.3.2 | Indicates value of horizontal speed |
| VerticalSpeed | 6.1.6.3.2 | Indicates value of vertical speed |
| SpeedUncertainty | 6.1.6.3.2 | Indicates value of speed uncertainty |
| BarometricPressure | 6.1.6.3.2 | Specifies the measured uncompensated atmospheric pressure |
| LcsServiceType | 6.1.6.3.2 | LCS service type |
| LdrReference | 6.1.6.3.2 | LDR Reference |
| ReportingAmount | 6.1.6.3.2 | Number of required periodic event reports |
| ReportingInterval | 6.1.6.3.2 | Event reporting periodic interval |
| MinimumInterval | 6.1.6.3.2 | Minimum interval between event reports |
| MaximumInterval | 6.1.6.3.2 | Maximum interval between event reports |
| SamplingInterval | 6.1.6.3.2 | Maximum time interval between consecutive evaluations by a UE of a trigger event |
| ReportingDuration | 6.1.6.3.2 | Maximum duration of event reporting |
| LinearDistance | 6.1.6.3.2 | Minimum straight line distance moved by a UE to trigger a motion event report |
| LMFIdentification | 6.1.6.3.2 | LMF identification |
| EventReportCounter | 6.1.6.3.2 | Number of event reports received from the target UE |
| EventReportDuration | 6.1.6.3.2 | Duration of event reporting |
| UePositioningCapabilities | 6.1.6.3.2 | Indicates the positioning capabilities supported by the UE. |
| ExternalClientType | 6.1.6.3.3 | Indicates types of External Clients |
| SupportedGADShapes | 6.1.6.3.4 | Indicates supported GAD shapes |
| ResponseTime | 6.1.6.3.5 | Indicates acceptable delay of location request |
| PositioningMethod | 6.1.6.3.6 | Indicates supported positioning methods |
| PositioningMode | 6.1.6.3.7 | Indicates supported modes used for positioning method |
| GnssId | 6.1.6.3.8 | Global Navigation Satellite System (GNSS) ID |
| Usage | 6.1.6.3.9 | Indicates usage made of the location measurement |
| LcsPriority | 6.1.6.3.10 | Indicates priority of the LCS client |
| VelocityRequested | 6.1.6.3.11 | Indicates velocity requirement |
| AccuracyFulfilmentIndicator | 6.1.6.3.12 | Indicates fulfilment of requested accuracy |
| VerticalDirection | 6.1.6.3.13 | Indicates direction of vertical speed |
| LdrType | 6.1.6.3.14 | Indicates LDR types |
| ReportingAreaType | 6.1.6.3.15 | Indicates type of event reporting area |
| OccurrenceInfo | 6.1.6.3.16 | Specifies occurrence of event reporting |
| ReportingAccessType | 6.1.6.3.17 | Specifies access types of event reporting |
| EventClass | 6.1.6.3.18 | Specifies event classes |
| ReportedEventType | 6.1.6.3.19 | Specifies type of event reporting |
| TerminationCause | 6.1.6.3.20 | Specifies causes of event reporting termination |
| LcsQosClass | 6.1.6.3.21 | Specifies LCS QoS class |
| UeLocationServiceInd | 6.1.6.3.22 | Specifies location service types requested by UE |

Table 6.1.6.1-2 specifies data types re-used by the Nlmf_Location service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_Location service based interface.

Table 6.1.6.1-2: Nlmf_Location re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Supi | 3GPP TS 29.571 [8] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [8] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [8] | Generic Public Subscription Identifier |
| Ecgi | 3GPP TS 29.571 [8] | E-UTRA Cell Identity |
| Ncgi | 3GPP TS 29.571 [8] | NR Cell Identity |
| NfInstanceId | 3GPP TS 29.571 [8] | Network Function Instance ID |
| Uri | 3GPP TS 29.571 [8] | Uniform Resource Identifier |
| RefToBinaryData | 3GPP TS 29.571 [8] | Reference to binary data |
| AccessType | 3GPP TS 29.571 [8] | Access type |
| CmState | 3GPP TS 29.518 [23] | Connection Management State |
| Guami | 3GPP TS 29.571 [8] | GUAMI |
| SupportedFeatures | 3GPP TS 29.571 [8] | Supported Features |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |
| TwapId | 3GPP TS 29.571 [8] | TWAP identifier |
| TnapId | 3GPP TS 29.571 [8] | TNAP identifier |
| DateTime | 3GPP TS 29.571 [8] | Date and Time |
| IntegrityRequirements | 3GPP TS 29.515 [x] | Integrity Requirements |

* * * Next Change * * * *

6.1.6.2.2	Type: InputData

Table 6.1.6.2.2-1: Definition of type InputData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| externalClientType | ExternalClientType | O | 0..1 | When present, this IE shall carry the external client type of the requester. |  |
| correlationID | CorrelationID | O | 0..1 | When present, this IE shall carry the correlation ID of the request. |  |
| amfId | NfInstanceId | O | 0..1 | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |  |
| locationQoS | LocationQoS | O | 0..1 | When present, this IE shall carry the QoS of the location request. |  |
| supportedGADShapes | array(SupportedGADShapes) | O | 1..N | When present, this IE shall carry the GAD shapes supported by the requester. |  |
| supi | Supi | O | 0..1 | Indicates the SUPI of the target UE. |  |
| pei | Pei | O | 0..1 | Indicates the PEI of the target UE. |  |
| gpsi | Gpsi | O | 0..1 | Indicates the GPSI of the target UE. |  |
| ecgi | Ecgi | O | 0..1 | When present, this IE shall indicate the identifier of the E-UTRAN cell serving the UE or the serving cell identifier of the Primary Cell in the Master RAN Node that is an E-UTRAN node on Dual Connectivity scenarios. (NOTE 2) |  |
| ecgiOnSecondNode | Ecgi | O | 0..1 | When present, the serving cell identifier of the Primary Cell in the Secondary RAN Node that is an E-UTRAN node when available on Dual Connectivity scenarios. (NOTE 3) (NOTE 4) |  |
| ncgi | Ncgi | O | 0..1 | When present, this IE shall indicate the identifier of the NR cell serving the UE or the serving cell identifier of the Primary Cell in the Master RAN Node that is a NR node on Dual Connectivity scenarios. (NOTE 2) |  |
| ncgiOnSecondNode | Ncgi | O | 0..1 | When present, the serving cell identifier of the Primary Cell in the Secondary RAN Node that is a NR node when available on Dual Connectivity scenarios. (NOTE 3) (NOTE 4) |  |
| priority | LcsPriority | O | 0..1 | When present, this IE shall indicate the priority of the location request. |  |
| velocityRequested | VelocityRequested | O | 0..1 | When present, this IE shall indicate whether velocity is requested or not. |  |
| ueLcsCap | UeLcsCapability | O | 0..1 | When present, this IE shall indicate the LCS capability supported by the UE. |  |
| lcsServiceType | LcsServiceType | O | 0..1 | The LCS service type |  |
| ldrType | LdrType | O | 0..1 | The type of LDR |  |
| hgmlcCallBackURI | Uri | C | 0..1 | Callback URI of the H-GMLC It shall be present, if attribute LdrType is present. |  |
| vgmlcAddress | Uri | C | 0..1 | V-GMLC address that corresponds to the V-GMLC that receives Location Request It shall be present, if attribute LdrType is present and the target UE is in roaming case. |  |
| ldrReference | LdrReference | C | 0..1 | LDR Reference Number It shall be present, if attribute LdrType is present. |  |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting |  |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting |  |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting |  |
| reportingAccessTypes | array(ReportingAccessType) | O | 1..N | Allowed access types for event reporting |  |
| ueConnectivityStates | array(UeConnectivityState) | O | 1..N | When present, this IE shall indicate the UE connectivity state per access type |  |
| ueLocationServiceInd | UeLocationServiceInd | C | 0..1 | If UE sends an MO-LR Request message, this IE shall be present and indicate the request type for a 5GC-MO-LR. |  |
| moAssistanceDataTypes | LcsBroadcastAssistanceTypesData | O | 0..1 | When present, this IE shall indicate a list of one or more types of location assistance data that UE subscribed. |  |
| lppMessage | RefToBinaryData | C | 0..1 | If UE includes the first LPP message in MO-LR Request, this IE shall be present and Indicate the binary data of LPP message. (NOTE 5) |  |
| lppMessageExt | array(RefToBinaryData) | C | 1..N | If UE includes the additional LPP messages (maximum 3) in MO-LR Request, this IE shall be present and Indicates the binary data of LPP message. (NOTE 5) |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.9 is supported. |  |
| uePositioningCap | UePositioningCapabilities | O | 0..1 | When present, this IE shall indicate the positioning capabilities supported by the UE. |  |
| tnapId | TnapId | O | 0..1 | When present, this IE shall contain the TNAP Identifier. This IE may be present for non-3GPP access. |  |
| twapId | TwapId | O | 0..1 | When present, This IE shall contain the TWAP Identifier. This IE may be present for non-3GPP access. |  |
| ueCountryDetInd | boolean | O | 0..1 | When present, This IE shall contain an indication of determining the UE geographical area identified by the country, area within a country or international area indication where UE is located for PLMN selection verification. | SAT |
| scheduledLocTime | DateTime | O | 0..1 | When present, this IE shall contain the scheduled time that the UE needs to be located. |  |
| reliableLocReq | boolean | C | 0..1 | This IE shall be included with the value "true" to indicate that reliable UE location information is required, as specified in 3GPP TS 33.256 [26] clause 5.3.2. When present, this IE shall be set as following: - true: the reliable UE location information is required - false (default): the reliable UE location information is not required |  |
| integrityRequirements | IntegrityRequirements | O | 0..1 | When present, this IE shall indicate the integrity requirements. |  |
| NOTE 1: At least one of the attributes defined in this table shall be present in the InputData structure. NOTE 2: Attribute "ecgi" and "ncgi" shall not be present at the same time. NOTE 3: Attribute "ecgiOnSecondNode" and "ncgiOnSecondNode" shall not be present at the same time. NOTE 4: Attribute "ecgiOnSecondNode" or "ncgiOnSecondNode" shall not be present if neither attribute "ecgi" nor "ncgi" is present. NOTE 5: If 3 LPP messages are received, then first LPP message shall be encoded in lppMessage IE and additional 2 LPP messages shall be encoded in lppMessageExt IE. |  |  |  |  |  |

* * * Next Change * * * *

A.2	Nlmf_Location API

openapi: 3.0.0

(... text not shown for clarity ...)

#

# COMPLEX TYPES

#

InputData:

description: Information within Determine Location Request.

type: object

not:

required: [ ecgi, ncgi ]

properties:

externalClientType:

$ref: '#/components/schemas/ExternalClientType'

correlationID:

$ref: '#/components/schemas/CorrelationID'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ecgiOnSecondNode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ncgiOnSecondNode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

priority:

$ref: '#/components/schemas/LcsPriority'

velocityRequested:

$ref: '#/components/schemas/VelocityRequested'

ueLcsCap:

$ref: '#/components/schemas/UeLcsCapability'

lcsServiceType:

$ref: '#/components/schemas/LcsServiceType'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vgmlcAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

reportingAccessTypes:

type: array

items:

$ref: '#/components/schemas/ReportingAccessType'

minItems: 1

ueConnectivityStates:

$ref: '#/components/schemas/UeConnectivityState'

ueLocationServiceInd:

$ref: '#/components/schemas/UeLocationServiceInd'

moAssistanceDataTypes:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsBroadcastAssistanceTypesData'

lppMessage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

lppMessageExt:

description: Indicates the lpp message extension.

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

tnapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TnapId'

twapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TwapId'

ueCountryDetInd:

type: boolean

scheduledLocTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

reliableLocReq:

type: boolean

default: false

integrityRequirements:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/IntegrityRequirements'

(... text not shown for clarity ...)

* * * End of Changes * * * *