3GPP TSG-CT WG4 Meeting #115-e	C4-231038

# E-Meeting, 17th – 21st April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.540 | CR | 0007 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add SBI-Support indication in Routing Info retrieval from HSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSMS_SBI |  |  |  |  |  | Date: |  |  | 2023-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current version of TS 23.540, there is no description how to get SBI-Support indication of SMSF/SMS Router/IP-SM-GW from HSS. This may cause incompatibility poblems, as described in C4-230696: - Incompatibility issues between SBI-based SMS and legacy SMS, e.g. during SM MT delivery, when SMS-GMSC makes use of MAP or Diameter to retrieve RoutingInfoForSM from legacy UDM/HSS (i.e. UDM/HSS do not support SBI-based SMS), the response message is still lack of SBI-support information of SMSF/IP-SM-GW/SMS Router, which may cause incompatibility problems. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SBI-Support indication of SMSF/SMS Router/IP-SM-GW in Routing Info retrieval from HSS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompatibility problems of SM-delivery, when SMS-GMSC retrieve routing info from HSS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

5.1.1	General

The procedure is used to support the Mobile Terminated short message transfer for SBI-based interfaces as defined in clause 4.1, which is based on Short message mobile terminated procedure defined in clause 10.1 of 3GPP TS 23.040 [2] and clauses 4.13.3.6, 4.13.3.7 and 4.13.3.8 of 3GPP TS 23.502 [4].

This new services introduced in this procedure can be registered in NRF, and discovered by NF service consumers.

SMSF/SMS Router/IP-SM-GW should indicate whether it supports SMS_SBI or not. For the case of SMSF/IP-SM-GW, "SBI support indication" should be brought when SMSF/IP-SM-GW registers in UDM/HSS. For the case of SMS Router, "SBI support indication" should be configured locally in UDM/HSS, together with the SMS Router address. UDM/HSS stores the "SBI support indication" indication and provides it to the SMS-GMSC during Routing Info retrieval. SMS-GMSC selects legacy or SBI based protocol based on the indication received during Routing Info retrieval.

* * * End of Change * * * *