3GPP TSG-CT WG4 Meeting #117	C4-233145

# Göteborg, Sweden, 21st– 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0673 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of ABNF rule of 3gpp-Sbi-Origination-Timestamp header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc18 |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ABNF rules of the generic custom headers defined in TS 29.500 have been corrected at the last CT4 meeting and a new normative annex added to TS 29.500 containing all the ABNF definitions. CT4 agreed to do similar corrections to API specific custom headers. The ABNF definition of the 3gpp-Sbi-Origination-Timestamp header defined in TS 29.502 contains the following errors: - Rules must not start with a digit - Header fields are defined generically in RFC 7230, section 3.2, and they should allow leading and trailing white spaces (OWS) The ABNF rule should also be formatted in PL style. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ABNF definition of the 3gpp-Sbi-Origination-Timestamp header is corrected. A new normative Annex is defined containing the ABNF definitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A wrong syntax definition leads to wrong implementation of the header content, which complicates interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3, Annex C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.2.3	HTTP custom headers

##### 6.1.2.3.1	General

For 3GPP specific HTTP custom headers used across all service based interfaces, see clause 5.2.3 of 3GPP TS 29.500 [4].

Additional HTTP custom headers applicable to the Nsmf_PDUSession service are specified in the following clauses.

##### 6.1.2.3.2	3gpp-Sbi-Origination-Timestamp

The header contains the date and time (with a millisecond granularity) when the originating entity initiated the request.

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [31].

Sbi-Origination-Timestamp-Header = "3gpp-Sbi-Origination-Timestamp:" OWS day-name "," SP date1 SP time-of-day "." milliseconds SP "GMT" OWS

milliseconds = 3DIGIT

day-name, date1, time-of-day shall comply with the definition in clause 7.1.1.1 of IETF RFC 7231 [32].

NOTE:	This is the same format as the Date header of clause 7.1.1.2 of IETF RFC 7231 [32], but with the time expressed with a millisecond granularity.

EXAMPLE:	3gpp-Sbi-Origination-Timestamp: Sun, 04 Aug 2019 08:49:37.845 GMT

* * * Next Change * * * *

Annex C (Normative):
ABNF grammar for 3GPP SBI HTTP custom headers

# C.1	General

This Annex contains a self-contained set of ABNF rules, comprising the re-used rules from IETF RFCs, and the rules defined by the 3GPP custom headers defined in this specification (see clause 6.1.2.3).

This grammar may be used as input to existing tools to help implementations to parse 3GPP custom headers.

# C.2	ABNF definitions (Filename: "TS29502_CustomHeaders.abnf")

; ----------------------------------------

;   RFC 5234

; ----------------------------------------

HTAB  = %x09 ; horizontal tab

SP    = %x20

DIGIT = %x30-39 ; 0-9

; ----------------------------------------

;   RFC 7230

; ----------------------------------------

OWS = *( SP / HTAB )

; ----------------------------------------

;   RFC 7231

; ----------------------------------------

date1       = day SP month SP year ; e.g., 02 Jun 1982

day         = 2DIGIT

month       = %x4A.61.6E ; "Jan", case-sensitive

/ %x46.65.62 ; "Feb", case-sensitive

/ %x4D.61.72 ; "Mar", case-sensitive

/ %x41.70.72 ; "Apr", case-sensitive

/ %x4D.61.79 ; "May", case-sensitive

/ %x4A.75.6E ; "Jun", case-sensitive

/ %x4A.75.6C ; "Jul", case-sensitive

/ %x41.75.67 ; "Aug", case-sensitive

/ %x53.65.70 ; "Sep", case-sensitive

/ %x4F.63.74 ; "Oct", case-sensitive

/ %x4E.6F.76 ; "Nov", case-sensitive

/ %x44.65.63 ; "Dec", case-sensitive

year        = 4DIGIT

day-name    = %x4D.6F.6E ; Mon

/ %x54.75.65 ; Tue

/ %x57.65.64 ; Wed

/ %x54.68.75 ; Thu

/ %x46.72.69 ; Fri

/ %x53.61.74 ; Sat

/ %x53.75.6E ; Sun

time-of-day = hour ":" minute ":" second

hour        = 2DIGIT

minute      = 2DIGIT

second      = 2DIGIT

; ----------------------------------------

;   3GPP TS 29.502

;

;   Version: 18.4.0 (September 2023)

;

;   (c) 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

; ----------------------------------------

;

; Header: 3gpp-Sbi-Origination-Timestamp

;

Sbi-Origination-Timestamp-Header = "3gpp-Sbi-Origination-Timestamp:" OWS day-name ","

SP date1 SP time-of-day "." milliseconds SP "GMT" OWS

milliseconds                     = 3DIGIT

* * * End of Changes * * * *