3GPP TSG-CT WG4 Meeting #117	C4-233147

# Göteborg, Sweden, 21st– 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.501 | CR | 0142 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API prefix in Callback URIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc18 |  |  |  |  |  | Date: |  |  | 2023-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 29.501 clause 4.4.3, Callback URI is defined with the following structure: URI = scheme ":" "//" host [ ":" port ] / path where 'path' is a path to an NF consumer resource and does not explicitly support an optional deployment-specific string (a.k.a. API Prefix). In contrary the Resource URI and Custom Operations URI may optionally include an API prefix as per the definition in clause 4.4.1: {apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart> "apiRoot" shall be a concatenation of the following parts: - scheme ("http" or "https") - the fixed string "://" - authority (host and optional port) as defined in IETF RFC 3986 [9]. The host should be represented by the operator specific FQDN (for FQDN examples see clause 28.3.2 in 3GPP TS 23.003 [27]). - an optional deployment-specific string (API prefix) that starts with a "/" character. TS 29.500 CR0287 (C4-216395 “Notifying different NF Service consumer instances sharing same authority“) extended the 3gpp-Sbi-Consumer-Info HTTP header with the "callback-uri-prefix" parameter. A NF service consumer may include this parameter when providing a Callback URI, in order to indicate that the service consumer path includes a prefix. This allows NF Service Producers to distinguish NF Service Consumers that share the same authority. While the aforementioned CR allows NF Service Consumers indicate the presence of a prefix in the Callback URI when subscribing to a NF Producer, this information is not conveyed in Callback/Notification Requests nor is populated in DefaultNotificationSubscriptions attribute in the NFProfile registered in the NRF. Furthermore the handling of this parameter was described only in the context of group bindings when NF Service Consumers share the same authority. These limitations mean that SCP managed reselection cannot work when different NF service consumer instances use different prefixes, since the SCP cannot know whether the Callback URI includes a prefix or not, and thus is unable to perform the proper prefix replacement. Furthermore, support of prefix in Callback URI is restricted only to use cases related to group binding. In order to overcome this limitation: - the definition of the Callback URI in TS 29.501 should be updated to include an optional API Prefix - TS 29.500 should be updated to support the presence of API Prefix in Callback/Notification Requests in case of indirect communication - the API Prefix should be indicated in Callback/Notification Responses in case of Rebinding to a new NF Consumer Service Instance. - NF Service Consumer reselection procedures in TS. 23.501, 23.527 and TS 29.500 should describe the replacement of the API prefix of the Callback URI when appropriate. - the NF Service and DefaultNotificationSubscriptions attribute in the NFProfile in TS 29.500 should be extended to support an optional API prefix in the Callback URI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of the of Callback URI in clause 4.4.3, to include an optional API prefix. |  |  |  |  |  |  |  |  |
| a |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - SCP managed NF service consumer reselection does not work when Service consumers use different API Prefix since the SCP cannot determine whether the original Callback URI included a prefix or not. - The support of prefix in Callback URI restricted only to use cases with group binding where all NF Service Consumer instances share the same authority |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.4.3	Callback URI structure

The purpose of the callback URI is to enable NF service consumer to provide the URI to be used by an NF Service Producer to send notification or callback requests.

The callback URI shall be in the form of an absolute URI as defined in clause 4.3 of IETF RFC 3986 [9], including an authority, and excluding any query component, any fragment component and any userinfo subcomponent.

Therefore, callback URI consists of the following components, specified with ABNF syntax (see IETF RFC 5234 [26]):

URI = {apiRoot} / path

Where "apiRoot" is defined in 3GPP TS 29.501 clause 4.4.1, and it has the optional deployment-specific string, a.k.a., API prefix, that starts with a "/" character.

* * * End of Changes * * * *