3GPP TSG-CT WG4 Meeting #117	C4-233376

# Goteborg, Sweden, 21st – 25th August 2023

Source:	Huawei

Title:	Conclusion for Key Issue #1 of FS_NRFe

Spec:	3GPP TR 29.831 v1.0.0

Agenda item:	6.1.3

Document for:	Approval

1. Introduction

This paper proposes to include Conclusion for KI#1 in TR 29.831.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 29.831 v1.0.0

* * * First Change * * * *

8.1	Conclusion of Solutions for Key Issue#1

To optimize the duplicate configuration, storage and transmission of shareable data, it is proposed to adopt the following interim conclusion principles:

- Shared data for NF Set level shall be enabled, to satisfy the sharing requirements within a specific NF Set.

  - To avoid the complexity in guaranteeing the consistence between shared data in NF(s) and NRF(s), OAM is proposed to configure and maintain the shared data into NRF(s) which serves the NF Set, Solution #X can be adopted as basis in normative phase.

  - To simplify the discovery procedure, one-round-trip is proposed to be adopted by providing the shared data of the NF Set when the discovery result includes the NF instance(s) of the NF Set.

- Shared data for PLMN level can be adopted as optionally, to satisfy the shared data requirements of cross-NF-Set scenario.

  - For shared data requirements of PLMN level, Solution #4 is proposed to be adopted to avoid the provisioning of shared data in all NRF instances, and interaction between NRFs can be involved to enable the transmission of shared data corresponding to shared data ID when needed.

  - Unique shared data ID can be involved to avoid the binding of NRF instance ID and shared data ID, thus the NRF which maintains the shared data can be determined based on the shared data ID. Details for the procedure can be defined during the normative phase.

- Paginated data retrieval mechanism in Solution #3 can be adopted as baseline for large volume data downloading, and can act as complement to other solution for data sharing between NFs.

- Shareable Profile Items listed in Solution #7 can be adopted as baseline for the definition of shareable data.

* * * End of Changes * * * *