3GPP TSG-CT WG4 Meeting #117	C4-233721

# Göteborg, Sweden, 21st– 25th August 2023								    Revision of C4-233365

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0949 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 QoS parameters related to RSPP transport over PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to S2-2307480 which has been approved and implemented in TS 23.586: “When receiving the Ranging/SL Positioning in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters for RSPP transport over PC5 reference point used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [6].” More specifcally, in TS 23.586 in clause 6.3.7 (Delivery of PC5 QoS parameters related to Ranging/SL positioning to NG-RAN), it is described that: the PCF (or V-PCF in case of roaming) sends the “PC5 QoS parameters related to Ranging/SL positioning” to AMF as the N2 container in the Namf_Communication_N1N2MessageTransfer. Therefore, this CR proposes to add RSPP feature to Namf_Communication and transfer the related PC5 policy as an N2 Information content class, and define the related IEs to transfer the PC5 QoS parameters for Ranging/sl positioning in N1N2MessageTransfer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2: TS 23.586 added to the references. 6.1.6.1: new data type for "RSPP" is defined for the Namf_Communication service based interface protocol. 6.1.6.2.15: N2InfoContainer updated to include RSPP information. 6.1.6.2.Y (new clause): new data type RsppInformation defined. 6.1.6.2.27: "ProSe" and "RSPP" are added as possible "ngapIeType" within N2InfoContent. 6.1.6.4.3.2: NGAP IEs for for class RSPP are defined. 6.1.6.3.4: "RSPP" is added to Enumeration of N2InformationClass. A.2: Open API Namf_Communication API is updated to reflect the above changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Delivery of PC5 QoS parameters related to Ranging/SL positioning to NG-RAN as defined in stage 2 TS 23.586 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.1, 6.1.6.2.15, 6.1.6.2.Y (new clause), 6.1.6.2.27, 6.1.6.4.3.2, 6.1.6.3.4, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces new backward compatible features to the OpenAPI Namf_Communication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[8]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[9]	IETF RFC 2387: "The MIME Multipart/Related Content-type".

[10]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) Protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[13]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[14]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[15]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[16]	3GPP TS 29.502: "5G System, Session Management Services; Stage 3".

[17]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[18]	3GPP TS 29.531: "Network Slice Selection Services; Stage 3".

[19]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[20]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[21]	Void.

[22]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[23]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0".

[24]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[25]	3GPP TS 29.572: "5G System, Location Management Services; Stage 3".

[26]	Void.

[27]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[28]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[29]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[30]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[31]	Void.

[32]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3".

[33]	3GPP TS 23.527: "5G System; Restoration Procedures".

[34]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[35]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[36]	IETF RFC 7807: "Problem Details for HTTP APIs".

[37]	3GPP TR 21.900: "Technical Specification Group working methods".

[38]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[39]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[40]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[41]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[44]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[45]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[46]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[47]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[48]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[49]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[50]	3GPP TS 29.010: "Information element mapping between Mobile Station - Base Station System (MS - BSS) and Base Station System - Mobile-services Switching Centre (BSS - MSC); Signalling Procedures and the Mobile Application Part (MAP)".

[51]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[52]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[53]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[54]	3GPP TS 24.554: " Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[55]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[56]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[57]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[xx]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Data within an AMF Status Change Subscription request and response. |
| AmfStatusChangeNotification | 6.1.6.2.3 | Data within an AMF Status Change Notification request. |
| AmfStatusInfo | 6.1.6.2.4 | AMF Status Information |
| AssignEbiData | 6.1.6.2.5 | Data within an EBI assignment request. |
| AssignedEbiData | 6.1.6.2.6 | Data within a successful response to an EBI assignment request. |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |
| UEContextRelease | 6.1.6.2.8 | Data within a Release UE Context request. |
| N2InformationTransferReqData | 6.1.6.2.9 | Data within a N2 Information Transfer request containing the N2 information requested to be transferred to 5G AN. |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Data within a create subscription requestfor non-UE specific N2 information notification. |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | Data for the created subscription for non-UE specific N2 information notification. |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Data within a create subscription request for UE specific N1 and/or N2 information notification. |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | Data for the created subscription for UE specific N1 and/or N2 information notification. |
| N2InformationNotification | 6.1.6.2.14 | Data within a N2 information notification request. |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |
| N1MessageNotification | 6.1.6.2.16 | Data within a N1 message notification request. |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container. |
| N1N2MessageTransferReqData | 6.1.6.2.18 | Data within a N1/N2 message transfer request. |
| N1N2MessageTransferRspData | 6.1.6.2.19 | Data within a N1/N2 message transfer response. |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |
| UeContextTransferReqData | 6.1.6.2.23 | Data within a UE Context Transfer Request to start transferring of an individual ueContext resource from old AMF to new AMF. |
| UeContextTransferRspData | 6.1.6.2.24 | Data within a successful response to the UE Context Transfer request. |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | Data within a N1/N2 Message Transfer Failure Notification request |
| N1N2MessageTransferError | 6.1.6.2.31 | Data within a N1/N2 Message Transfer Error response. |
| N1N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |
| N2InformationTransferRspData | 6.1.6.2.33 | Data within a successful response to the N2 Information Transfer request to transfer N2 Information to the AN. |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |
| NssaiMapping | 6.1.6.2.38 | Represents the mapping between a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Data within a UE registration status update request to indicate a completion of transferring at a target AMF. |
| AssignEbiError | 6.1.6.2.40 | Data within a failure response to the EBI assignment request. |
| UeContextCreateData | 6.1.6.2.41 | Data within a request to create an individual ueContext resource |
| UeContextCreatedData | 6.1.6.2.42 | Data within a successful response for creating an individual ueContext resource |
| UeContextCreateError | 6.1.6.2.43 | Data within a failure response for creating a UE context |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |
| N2InformationTransferError | 6.1.6.2.45 | Data within a failure response for a non-UE related N2 Information Transfer. |
| PWSResponseData | 6.1.6.2.46 | Data related PWS included in a N2 Information Transfer response. |
| PWSErrorData | 6.1.6.2.47 | Data related to PWS error included in a N2 Information Transfer failure response. |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 3GPP TS 33.501 [27]) |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 3GPP TS 33.501 [27]). |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Data within a UE registration status update response to provides the status of UE context transfer status update at a source AMF. |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |
| N2InfoNotificationRspData | 6.1.6.2.54 | Data within a N2 information notification response. |
| SmallDataRateStatusInfo | 6.1.6.2.55 | Represents the small data rate status |
| SmfChangeInfo | 6.1.6.2.56 | SMF change information for PDU session(s) |
| V2xContext | 6.1.6.2.57 | Represents the V2X services related parameters |
| ImmediateMdtConf | 6.1.6.2.58 | Immediate MDT Configuration |
| V2xInformation | 6.1.6.2.59 | V2X related N2 information |
| EpsNasSecurityMode | 6.1.6.2.60 | Indicates the EPS NAS Security Mode |
| UeContextRelocateData | 6.1.6.2.61 | Data within a Relocate UE Context request |
| UeContextRelocatedData | 6.1.6.2.62 | Data within a Relocate UE Context |
| EcRestrictionDataWb | 6.1.6.2.64 | Enhanced Coverage Restriction Data for WB-N1 mode. |
| ExtAmfEventSubscription | 6.1.6.2.65 | AMF event subscription extended with additional information received for the subscription |
| AmfEventSubscriptionAddInfo | 6.1.6.2.66 | Additional information received for an AMF event subscription, e.g. binding indications. |
| UeContextCancelRelocateData | 6.1.6.2.67 | Data structure used for cancellation of UE Context Relocation. |
| UeDifferentiationInfo | 6.1.6.2.68 | Represents the UE Differentiation Information and its validity time. |
| CeModeBInd | 6.1.6.2.69 | CE-mode-B Support Indicator |
| LteMInd | 6.1.6.2.70 | LTE-M Indication |
| NpnAccessInfo | 6.1.6.2.71 | NPN Access Information |
| ProseContext | 6.1.6.2.72 | Represents the ProSE services related parameters |
| AnalyticsSubscription | 6.1.6.2.73 | Analytics subscriptions created in the NWDAF. |
| NwdafSubscription | 6.1.6.2.74 | Individual NWDAF subscription identified by the subscription Id. |
| UpdpSubscriptionData | 6.1.6.2.75 | UE policy delivery related N1 message notification subscription data |
| ProSeInformation | 6.1.6.2.76 | 5G ProSe related N2 information. |
| ReleaseSessionInfo | 6.1.6.2.77 | PDU session Id(s) and the cause for triggering the release |
| AreaOfInterestEventState | 6.1.6.2.78 | Area Of Interest Event State in old AMF |
| TssInformation | 6.1.6.2.79 | TSS related N2 information |
| RsppInformation | 6.1.6.2.Y | Ranging/SL positioning related N2 information |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |
| MSClassmark2 | 6.1.6.3.2 | Indicates the MS Classmark 2 of a 5G SRVCC UE |
| SupportedCodec | 6.1.6.3.2 | Indicates the supported codec of a 5G SRVCC UE |
| StatusChange | 6.1.6.3.3 | Enumeration for AMF status |
| N2InformationClass | 6.1.6.3.4 | Enumeration for N2 Information Class |
| N1MessageClass | 6.1.6.3.5 | Enumeration for N1 Message Class |
| N1N2MessageTransferCause | 6.1.6.3.6 | Enumeration for N1N2Message Transfer Cause |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |
| SmsSupport | 6.1.6.3.11 | Indicates the supported SMS delivery of a UE. |
| ScType | 6.1.6.3.12 | Indicates the security context type. |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |
| PolicyReqTrigger | 6.1.6.3.15 | Policy Request Triggers |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |
| SmfChangeIndication | 6.1.6.3.19 | Indicates the I-SMF or V-SMF change or removal |
| SbiBindingLevel | 6.1.6.3.20 | SBI Binding Level |
| EpsNasCipheringAlgorithm | 6.1.6.3.21 | Indicates the supported EPS NAS Ciphering Algorithm |
| EpsNasIntegrityAlgorithm | 6.1.6.3.22 | Indicates the supported EPS NAS Integrity Algorithm |
| PeriodicCommunicationIndicator | 6.1.6.3.23 | Indicates the Periodic Communication Indicator |
| UuaaMmStatus | 6.1.6.3.24 | Indicates UUAA-MM status |
| ReleaseCause | 6.1.6.3.25 | The cause for triggering the release |

* * * Next Change * * * *

##### 6.1.6.2.15	Type: N2InfoContainer

Table 6.1.6.2.15-1: Definition of type N2InfoContainer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n2InformationClass | N2InformationClass | M | 1 | This IE represents the class of N2 information to be transferred. |  |
| smInfo | N2SmInformation | C | 0..1 | This IE shall be present if session management N2 information is to be transferred. When present, it represents a session management SMF related N2 information data part. |  |
| ranInfo | N2RanInformation | C | 0..1 | This IE shall be present if RAN related N2 information is to be transferred (i.e. n2InformationClass is "RAN"). When present, it shall contain the RAN related N2 information data part. |  |
| nrppaInfo | NrppaInformation | C | 0..1 | This IE shall be present if location service related N2 information is to be transferred. When present, it represents a NRPPa related N2 information data part. |  |
| pwsInfo | PwsInformation | C | 0..1 | This IE shall be present if PWS related N2 information is to be transferred. |  |
| v2xInfo | V2xInformation | C | 0..1 | This IE shall be present if V2X related N2 information is to be transferred. |  |
| proseInfo | ProSeInformation | C | 0..1 | This IE shall be present if 5G ProSe related N2 information is to be transferred. | ProSe |
| tssInfo | TssInformation | C | 0..1 | This IE shall be present if TSS related N2 information is to be transferred. | NTSSM |
| rsppInfo | RsppInformation | C | 0..1 | This IE shall be present if Ranging/SL positioning related N2 information is to be transferred. | RSPP |

* * * Next Change * * * *

##### 6.1.6.2.Y	Type: RsppInformation

Table 6.1.6.2.Y-1: Definition of type RsppInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n2Pc5RsppPol | N2InfoContent | M | 1 | This attribute contains the N2 PC5 policy for Ranging/SL positioning. This IE may contain the NGAP ProSe related IEs specified in clause 9.3.1.234 of 3GPP TS 38.413 [12] or the NGAP V2X related IEs specified in clause 9.3.1.150 of 3GPP TS 38.413 [12]. |

* * * Next Change * * * *

##### 6.1.6.2.27	Type: N2InfoContent

Table 6.1.6.2.27-1: Definition of type N2InfoContent

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ngapMessageType | Uinteger | C | 0..1 | This IE shall be present if PWS related N2 information is to be transferred, or during the AMF planned removal procedure with UDSF deployed procedure to transfer a RAN N2 message. When present, it shall indicate the NGAP Message type of the ngapData as specified in clause 6.1.6.4.3.3. Its value equals the value of the Procedure Code defined in ASN.1 in clause 9.4.7 in 3GPP TS 38.413 [12]. |
| ngapIeType | NgapIeType | C | 0..1 | This IE shall be present if SM, RAN, V2X, TSS, NRPPa or RSPP related N2 information is to be transferred. When present, it shall indicate the NGAP IE type of the ngapData as specified in clause 6.1.6.4.3.2. |
| ngapData | RefToBinaryData | M | 1 | This IE reference the N2 Information binary data corresponding to the N2 information class. See clause 6.1.6.4.3. |

* * * Next Change * * * *

##### 6.1.6.4.3	N2 Information

###### 6.1.6.4.3.1	Introduction

N2 Information shall encode NG Application Protocol (NGAP) IEs, as specified in clause 9.4 of 3GPP TS 38.413 [12] (ASN.1 encoded), using the vnd.3gpp.ngap content-type.

###### 6.1.6.4.3.2	NGAP IEs

For N2 information class SM, N2 Information may encode following NGAP SMF related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-1.

Table 6.1.6.4.3-1: N2 Information content for class SM

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| PDU Session Resource Setup Request Transfer | 9.3.4.1 | PDU SESSION RESOURCE SETUP REQUEST |
| PDU Session Resource Release Command Transfer | 9.3.4.12 | PDU SESSION RESOURCE RELEASE COMMAND |
| PDU Session Resource Modify Request Transfer | 9.3.4.3 | PDU SESSION RESOURCE MODIFY REQUEST |
| Handover Command Transfer | 9.3.4.10 | HANDOVER COMMAND |
| Handover Required Transfer | 9.3.4.14 | HANDOVER REQUIRED |
| Handover Preparation Unsuccessful Transfer | 9.3.4.18 | HANDOVER COMMAND |
| Secondary RAT Data Usage Report Transfer | 9.3.4.23 | SECONDARY RAT DATA USAGE REPORT |

For N2 information class RAN, N2 Information may encode one of the following NGAP Transparent Container IEs specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-2.

Table 6.1.6.4.3-2: N2 Information content for class RAN

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| Source to Target Transparent Container | 9.3.1.20 | HANDOVER REQUIRED, HANDOVER REQUEST |
| Target to Source Transparent Container | 9.3.1.21 | HANDOVER COMMAND, HANDOVER REQUEST ACKNOWLEDGE |
| Target to Source Failure Transparent Container | 9.3.1.186 | HANDOVER FAILURE |
| UE Radio Capability | 9.3.1.74 | UE RADIO CAPABILITY INFO INDICATION. (NOTE 1). |
| UE Radio Capability for Paging | 9.3.1.68 | UE RADIO CAPABILITY INFO INDICATION. (NOTE 1). |
| SON Configuration Transfer | 9.3.3.6 | UPLINK RAN CONFIGURATION TRANSFER, DOWNLINK RAN CONFIGURATION TRANSFER |
| RAN Status Transfer Transparent Container | 9.2.3.13, 9.2.3.14 | UPLINK RAN STATUS TRANSFER, DOWNLINK RAN STATUS TRANSFER |
| Early Status Transfer Transparent Container | 9.2.3.16, 9.2.3.17 | UPLINK RAN EARLY STATUS TRANSFER DOWNLINK RAN EARLY STATUS TRANSFER |
| RIM Information Transfer | 9.3.3.28 | UPLINK RIM INFORMATION TRANSFER, DOWNLINK RIM INFORMATION TRANSFER |
| NOTE 1: The AMF receives the UE Radio Capability and UE Radio Capability for Paging within a UE CAPABILITY INFO INDICATION message and then the AMF shall store the UE Radio Capability information and UE Radio Capability for Paging, and transfer them to the target AMF during an inter AMF mobility procedure. |  |  |

For N2 information class NRPPa, N2 Information may encode the following NGAP NRPPA Transport related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-3

Table 6.1.6.4.3-3: N2 Information content for class NRPPa

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| NRPPa-PDU | 9.3.3.14 | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT UPLINK UE ASSOCIATED NRPPA TRANSPORT DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |

For N2 information class V2X, N2 Information may encode the following V2X related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-4

Table 6.1.6.4.3-4: N2 Information content for class V2X

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| PC5 QoS Parameters | 9.3.1.150 | INITIAL CONTEXT SETUP REQUEST UE CONTEXT MODIFICATION REQUEST HANDOVER REQUEST PATH SWITCH REQUEST ACKNOWLEDGE |

For N2 information class PROSE, N2 Information may encode the following ProSe related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-5

Table 6.1.6.4.3-5: N2 Information content for class PROSE

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| 5G ProSe PC5 QoS Parameters | 9.3.1.234 | INITIAL CONTEXT SETUP REQUEST UE CONTEXT MODIFICATION REQUEST HANDOVER REQUEST PATH SWITCH REQUEST ACKNOWLEDGE |

For N2 information class TSS, N2 Information may encode the following TSS (Timing Synchronisation Status information) related IE specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-6

Table 6.1.6.4.3-6: N2 Information content for class TSS

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| Clock Quality Reporting Control Information | 9.3.1.X | FFS |
| RAN Timing Synchronisation Status Information | 9.3.1.Y | FFS |

For N2 information class RSPP, N2 Information may encode one of the following RSPP related IEs specified in 3GPP TS 38.413 [12], as summarized in Table 6.1.6.4.3-7.

Table 6.1.6.4.3-7: N2 Information content for class RSPP

| NGAP IE | Reference (3GPP TS 38.413 [12]) | Related NGAP message |
| --- | --- | --- |
| PC5 QoS Parameters | 9.3.1.150 | INITIAL CONTEXT SETUP REQUEST UE CONTEXT MODIFICATION REQUEST HANDOVER REQUEST PATH SWITCH REQUEST ACKNOWLEDGE |
| 5G ProSe PC5 QoS Parameters | 9.3.1.234 | INITIAL CONTEXT SETUP REQUEST UE CONTEXT MODIFICATION REQUEST HANDOVER REQUEST PATH SWITCH REQUEST ACKNOWLEDGE |

Editor's note:	Further details on the new NGAP IEs to be aligned with RAN WG3.

* * * Next Change * * * *

##### 6.1.6.3.4	Enumeration: N2InformationClass

Table 6.1.6.3.4-1: Enumeration N2InformationClass

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SM" | N2 SM information. |  |
| "NRPPa" | N2 NRPPa information. |  |
| "PWS" | N2 PWS information of PWS type. |  |
| "PWS-BCAL" | N2 Broadcast Completed Area List or the Broadcast Cancelled Area List. |  |
| "PWS-RF" | N2 Restart Indication or Failure Indication |  |
| "RAN" | N2 RAN related information. |  |
| "V2X" | N2 V2X information |  |
| "PROSE" | N2 5G ProSe information. | ProSe |
| "TSS" | N2 Timing Synchronisation Status information |  |
| "RSPP" | N2 QoS information of RSPP transport over PC5 | RSPP |

* * * Next Change * * * *

### 6.1.8	Feature Negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the AMF and the NF Service Consumer, for the Namf_Communication service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Namf_Communication service, if any, by including the supportedFeatures attribute in payload of the HTTP Request Message for following service operations:

-	N1N2MessgeTransfer, as specified in clause 5.2.2.3.1;

-	N1N2MessageSubscribe, as specified in clause 5.2.2.3.3;

-	NonUeN2InfoSubscribe, as specified in clause 5.2.2.4.2;

-	UeContextTransfer, as specified in clause 5.2.2.2.1;

-	CreateUEContext, as specified in clause 5.2.2.2.3

The AMF shall determine the supported features for the service operations as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in payload of the HTTP response for the service operation.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [6].

The following features are defined for the Namf_Communication service.

Table 6.1.8-1: Features of supportedFeatures attribute used by Namf_Communication service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | DTSSA | O | Deployments Topologies with specific SMF Service Areas. An AMF that supports this feature shall support the procedures specified in clause 5.34 of 3GPP TS 23.501 [2] and in clause 4.23 of 3GPP TS 23.502 [3]. |
| 2 | ENS | O | This feature bit indicates whether the AMF supports procedures related to Network Slicing (see 3GPP TS 23.501 [2] clause 5.15.7). This includes supporting the RelocateUEContext service operation (see clause 5.2.2.2.5). |
| 3 | CIOT | O | Cellular IoT Support of this feature implies the support of all the CIoT features specified in clause 5.31 of 3GPP TS 23.501 [2], including in particular corresponding service's extensions to support: - NB-IoT and LTE-M RAT types; - Control Plane CIoT 5GS Optimisation; - Rate control of user data. |
| 4 | MAPDU | O | This feature bit indicates whether the AMF supports Multi-Access PDU session procedures related to Access Traffic Steering, Switching and Splitting (see clauses 4.2.10 and 5.32 of 3GPP TS 23.501 [2]). |
| 5 | NPN | O | Non-Public Network Support of this feature implies support of NPN information and receipt of a Create UE context error response with a binary part during an Inter-AMF N2 Handover. |
| 6 | ELCS | O | This feature indicates supports of enhanced LCS, including the capability for the AMF to send an LCS message through the target access type requested by the LMF. |
| 7 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. SMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the Namf_Communication service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 8 | EAEA | O | EBI and ARP mapping update in EBIAssignment Support of this feature implies support of updating the mapping of EBI and ARP in EBIAssignment for a QoS flow to which an EBI is already allocated. |
| 9 | ProSe | O | This feature indicates the support of UE policy and N2 information provisioning for 5G ProSe. |
| 10 | UAV | O | Uncrewed Aerial Vehicle This feature indicates the support of UAV feature at AMF. When this feature is supported at the AMF, the AMF shall perform the UUAA-MM procedure defined in 3GPP TS 23.256 [56]. |
| 11 | SPAE | O | SM Policy Association Events This feature bit indicates whether the AMF supports the SM Policy Association establishment and termination event notification information handling, i.e. whereby the PCF for UE subscribes to SM Policy Association events to the PCF for SM Policy via the AMF and SMF, as specified in clause 4.3.2.2.1 and clause 4.3.3.2 of 3GPP TS 23.502 [3]. |
| 12 | eNPN | O | Enhanced support of Non-Public Networks (eNPN) This feature indicates the AMF supports UE registration for onboarding in an SNPN, see clause 5.30.2.10.2.6 and clause 5.30.2.10.2.7 in 3GPP TS 23.501 [2]). |
| 13 | 3GA-N3GA-HO | O | Handover between 3GPP and non-3GPP accesses An AMF and SMF that supports Handover between 3GPP and non-3GPP accesses shall support this feature, i.e. setting the targetAccess IE in N1N2MessageTransfer Request to the old access type when releasing the N2 PDU session resources in the old access (see clauses 5.2.2.3.1.1 and 6.1.6.2.18) |
| 14 | PAR-NS | O | Partially Allowed/Rejected Network Slice This feature indicates the AMF supports the partially allowed network slice and the partially reject Network slice. See 3GPP TS 23.501 [2] clause 5.15.17. |
| 15 | NTSSM |  | Network Timing Synchronization Status Monitoring This feature bit indicates that the AMF supports transferring TSS related information in the N2InformationTransferReqData and a subscription, e.g. created by a TSCTSF, to get notification for Non-UE related N2 Information for the Network Timing Synchronization Status information. |
| 16 | RACS | O | Support of RACS feature as specified in clause 5.4.4.1a in 3GPP TS 23.501 [2]). During an Inter AMF mobility procedure, the source AMF may decide to not include ueRadioCapability and ueRadioCapabilityForPaging in the UeContextTransferRspData or UeContextCreateData if the target AMF supports RACS feature, the target AMF can retrieve the same using UE Radio Capability ID Id(s) included in the MM Context. |
| yy | RSPP | O | This feature indicates the support of UE policy and N2 information provisioning for Ranging/SL positioning as specified in clause 6.3.7 of 3GPP TS 23.586 [xx]. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * * *

# A.2	Namf_Communication API

openapi: 3.0.0

info:

version: 1.3.0-alpha.3

title: Namf_Communication

description: |

AMF Communication Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[non-changing part skipped for clarity]

N2InfoContainer:

description: N2 information container

type: object

properties:

n2InformationClass:

$ref: '#/components/schemas/N2InformationClass'

smInfo:

$ref: '#/components/schemas/N2SmInformation'

ranInfo:

$ref: '#/components/schemas/N2RanInformation'

nrppaInfo:

$ref: '#/components/schemas/NrppaInformation'

pwsInfo:

$ref: '#/components/schemas/PwsInformation'

v2xInfo:

$ref: '#/components/schemas/V2xInformation'

proseInfo:

$ref: '#/components/schemas/ProSeInformation'

tssInfo:

$ref: '#/components/schemas/TssInformation'

rsppInfo:

$ref: '#/components/schemas/RsppInformation'

required:

- n2InformationClass

[non-changing part skipped for clarity]

V2xInformation:

description: V2X related N2 information

type: object

properties:

n2Pc5Pol:

$ref: '#/components/schemas/N2InfoContent'

ProSeInformation:

description: Represents 5G ProSe related N2 information.

type: object

properties:

n2Pc5ProSePol:

$ref: '#/components/schemas/N2InfoContent'

RsppInformation:

description: Represents Ranging/SL positioning related N2 information.

type: object

properties:

n2Pc5RsppPol:

$ref: '#/components/schemas/N2InfoContent'

[non-changing part skipped for clarity]

# ENUMERATIONS

#

StatusChange:

description: Enumeration for AMF status

anyOf:

- type: string

enum:

- AMF_UNAVAILABLE

- AMF_AVAILABLE

- type: string

N2InformationClass:

description: Enumeration for N2 Information Class

anyOf:

- type: string

enum:

- SM

- NRPPa

- PWS

- PWS-BCAL

- PWS-RF

- RAN

- V2X

- PROSE

- TSS

- RSPP

- type: string

[non-changing part skipped for clarity]

* * * End of Changes * * * *