3GPP TSG-CT WG4 Meeting #117	C4-233854

# Göteborg, Sweden, 21st– 25th August 2023                                        (Revision of C4-233642)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.536 | CR | 0067 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource and Data types of NSACF NSAC Quota Update service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per agreed S2-2307494 CR, a new service operation has been added for NSACF which handles the Quota Update for the NSACF at VPLMN with the maximum number of Registered UEs to be enforced and/or the maximum number of LBO PDU sessions to be admitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new service operation of Quota Update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not compliant with stage 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.1, 6.1.3.x, 6.1.6.1, 6.1.6.2.x, 6.1.6.2.y, 6.1.6.3.x, 6.1.9, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR provides new backward compatible features to the definition of Nnsacf_NSAC API OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

The figure 6.1.3.1-1 describes the resource URI structure of the Nnsacf-NSAC API.

Figure 6.1.3.1-1: Resource URI structure of the Nnsacf_NSAC API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Slice Collection Subject to NSAC for UEs (Collection) | /slices/ues | POST | Request the NSACF to perform network slice admission control related to the number of UEs registered to a network slice, or a group of network slices. |
| Slice Collection Subject to NSAC for PDU sessions (Collection) | /slices/pdus | POST | Request the NSACF to perform network slice admission control related to the number of PDU sessions established to a network slice, or a group of network slices. |
| Slice Collection configuration | /slices/configs | POST | Request the NSACF to update the Network slice local configuration for NSAC procedure, i.e., to update the local maximum number of registered UEs and/or the local maximum number of PDU sessions established per Network slice. |
| Slice Collection Roaming Quotas | /slices/roaming-quotas/query | query (POST) | Request the NSACF at HPLMN to update the maximum number of registered UEs and/or number of PDU sessions of the network slice at the NSACF of VPLMN for inbound roamers for VPLMN NSAC admission mode. |

* * * Next Change * * * *

#### 6.1.3.x	Resource: Slice Collection Roaming Quotas

##### 6.1.3.x.1	Description

This resource represents the Slice Collection subject to roaming quotas update, i.e. quota update of the maximum number of registered UEs and/or maximum number of PDU sessions established at a network slice in a primary or central VPLMN NSACF for inbound roamers for VPLMN NSAC admission mode.

##### 6.1.3.x.2	Resource Definition

Resource URI: {apiRoot}/<apiName>/<apiVersion>/slices/roaming-quotas

This resource shall support the resource URI variables defined in table 6.1.3.x.2-1.

Table 6.1.3.x.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.x.3	Resource Standard Methods

None.

##### 6.1.3.x.4	Resource Custom Operations

6.1.3.x.4.1	Overview

Table 6.1.3.x.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| query | /slices/roaming-quotas/query | POST | Request the NSACF at HPLMN to update the maximum number of registered UEs and/or number of PDU sessions of the network slice at the NSACF of VPLMN for inbound roamers for VPLMN NSAC admission mode. |

###### 6.1.3.x.4.2	Operation: query

6.1.3.x.4.2.1	Description

6.1.3.x.4.2.2	Operation Definition

This custom operation is used to request the central or primary NSACF at HPLMN to update the maximum number of registered UEs and/or number of PDU sessions of the network slice at the central or primary NSACF at VPLMN for inbound roamers for VPLMN NSAC admission mode.

This operation shall support the request data structures specified in table 6.1.3.x.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.x.4.2.2-2.

Table 6.1.3.x.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| QuotaUpdateRequestData | M | 1 | Provide the data for NSAC procedure related to quota update of the maximum number of registered UEs and/or number of PDU sessions of the network slice at the primary or central NSACF in a VPLMN for inbound roamers for VPLMN NSAC admission mode. |

Table 6.1.3.x.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| QuotaUpdatetResponseData | M | 1 | 200 OK | Response data for NSAC procedure related to the quota update of the number of UEs and/or number of PDU sessions of the network slice in VPLMN for inbound roamers for VPLMN NSAC admission mode. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during a NSAC procedure. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP or SEPP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.x.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same NSACF or NSACF (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.x.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same NSACF or NSACF (service) set. For the case when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnsacf_NSAC service based interface protocol.

Table 6.1.6.1-1: Nnsacf_NSAC specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UeACRequestData | 6.1.6.2.2 | Input data for NSAC procedure related to the number of UEs per slice. |  |
| UeACResponseData | 6.1.6.2.3 | Response data of NSAC procedure for controlling the number of UEs. |  |
| EACNotification | 6.1.6.2.4 | EAC mode notification |  |
| AcuOperationItem | 6.1.6.2.5 | An operation item for NSAC procedure, indicating an S-NSSAI subject to NSAC and the associated operation. |  |
| AcuFailureItem | 6.1.6.2.6 | A failure item which indicates the failed S-NSSAI and the failure reason. |  |
| PduACRequestData | 6.1.6.2.7 | Input data for NSAC procedure related to the number of PDUs per slice. |  |
| PduACResponseData | 6.1.6.2.8 | Response data of NSAC procedure for controlling the number of PDU sessions. |  |
| EACMode | 6.1.6.3.3 | EAC mode |  |
| AcuFlag | 6.1.6.3.4 | Update Flag for NSAC procedure |  |
| AcuFailureReason | 6.1.6.3.5 | Indicates the failure reason for an S-NSSAI in the NSAC procedure |  |
| UeACRequestInfo | 6.1.6.2.9 | One item of a UE and associated NSAC action. |  |
| PduACRequestInfo | 6.1.6.2.10 | One item of a PDU session and associated NSAC action. |  |
| UeAdmissionValue | 6.1.6.2.11 | Local maximum number of UEs | HNSAC |
| PduAdmissionValue | 6.1.6.2.12 | Local maximum number of PDUs | HNSAC |
| ACUpdateData | 6.1.6.2.13 | Input data for NSAC procedure to update the local maximum number of registered UEs and the local maximum number of PDU sessions. | HNSAC |
| SliceQuotaType | 6.1.6.3.x | Type of quota update for NSAC procedure. |  |
| QuotaUpdateRequestData | 6.1.6.2.x | Input data for NSAC procedure to fetch the maximum number of registered UEs and/or the maximum number of PDU sessions. |  |
| QuotaUpdateResponseData | 6.1.6.2.y | Input data for NSAC procedure to update the maximum number of registered UEs and/or the maximum number of PDU sessions. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nnsacf_NSAC service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnsacf_NSAC service based interface.

Table 6.1.6.1-2: Nnsacf_NSAC re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [16] | Problem Details |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Redirect Response |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Supported Features |  |
| Supi | 3GPP TS 29.571 [16] | Subscription Permanent Identifier |  |
| Snssai | 3GPP TS 29.571 [16] | Single NSSAI |  |
| NfInstanceId | 3GPP TS 29.571 [16] | NF Instance ID |  |
| Uri | 3GPP TS 29.571 [16] | Resource or callback URI |  |
| AccessType | 3GPP TS 29.571 [16] | Access Type |  |
| NFType | 3GPP TS 29.510 [17] | NF Type |  |
| Fqdn | 3GPP TS 29.571 [16] | FQDN |  |
| PduSessionId | 3GPP TS 29.571 [16] | PDU Session Identifier |  |
| PlmnId | 3GPP TS 29.571 [16] | PLMN ID |  |
| Uinteger | 3GPP TS 29.571 [16] | Unsigned Integer |  |

* * * Next Change * * * *

##### 6.1.6.2.x	Type: QuotaUpdateRequestData

Table 6.1.6.2.x-1: Definition of type QuotaUpdateRequestData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | Indicates the Network slice for which the NSACF shall provide the maximum number of registered UEs number and/or the maximum number of PDU sessions. It shall indicate the mapped S-NSSAI in home PLMN. |  |
| plmnId | PlmnId | M | 1 | Indicates the serving PLMN ID of UE. |  |
| quotaType | SliceQuotaType | M | 1 | Indicates the type of the requested quota (i.e., maximum number of registered UEs and/or maximum number of PDU sessions). |  |

##### 6.1.6.2.y	Type: QuotaUpdateResponseData

Table 6.1.6.2.y-1: Definition of type QuotaUpdateResponseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | Indicates the Network slice for which the NSACF shall update the maximum number of registered UEs number and/or the maximum number of PDU sessions. It shall indicate the mapped S-NSSAI in home PLMN. |  |
| maxUesNumber | integer | O | 0..1 | Indicates the updated maximum number of registered UEs of the S-NSSAI. |  |
| maxPdusNumber | integer | O | 0..1 | Indicates the updated maximum number of PDU Sessions of the S-NSSAI |  |

* * * Next Change * * * *

##### 6.1.6.3.x	Enumeration: SliceQuotaType

The enumeration SliceQuotaType indicates the type of the quota (i.e. maximum number of registered UEs and/or maximum number of PDU sessions) for an S-NSSAI for which the fetching of the available quota is performed during the NSAC procedure. It shall comply with the provisions defined in table 6.1.6.3.x-1.

Table 6.1.6.3.x-1: Enumeration SliceQuotaType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "MAX_UE_NUM" | Indicates that quota type for update relates to maximum number of registered UEs. |  |
| "MAX_PDU_NUM" | Indicates that quota type for update relates to maximum number of PDU sessions. |  |
| "BOTH" | Indicates that quota type for update relates to both maximum number of registered UEs and maximum number of PDU sessions. |  |

* * * Next Change * * * *

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nnsacf_NSAC API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nnsacf_NSAC API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nnsacf_NSAC service.

The Nnsacf_NSAC API defines the following scopes for Oauth2 authorization:

Table 6.1.9-1: OAuth2 scopes defined in Nnsacf_NSAC API

| Scope | Description |
| --- | --- |
| "nnsacf-nsac" | Access to the Nnsacf_NSAC API. |
| "nnsacf-nsac:slice-ues" | Access to service operations applying to the collection of slice subject to NSAC for UEs. |
| "nnsacf-nsac:slice-pdus" | Access to service operations applying to the collection of slice subject to NSAC for PDU sessions. |
| "nnsacf-nsac:ac-info-update" | Access (from a primary NSACF) to update the local NSACF configuration i.e., the maximum number of registered UEs and/or the maximum number of PDU sessions of a network slice at an NSACF. |
| "nnsacf-nsac:roaming-quotas-query" | Access (from a primary or central VPLMN NSACF) to request the primary or central HPLMN NSACF the maximum number of registered UEs and/or the maximum number of PDU sessions of a network slice for inbound roamers for VPLMN NSAC admission mode. |

* * * Next Change * * * *

# A.2	Nnsacf_NSAC API

openapi: 3.0.0

info:

title: Nnsacf_NSAC

version: 1.1.0-alpha.2

description: |

Nnsacf_NSAC Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

paths:

[…]

/slices/roaming-quotas/query:

post:

summary: >

Network Slice Admission Control fetching of the maximum number of registered UEs and/or

number of PDU sessions of the network slice at NSACF.

operationId: QuotaUpdate

tags:

- slice collection

security:

- {}

- oAuth2ClientCredentials:

- nnsacf-nsac

- oAuth2ClientCredentials:

- nnsacf-nsac

- nnsacf-nsac: roaming-quotas-query

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/QuotaUpdateRequestData'

required: true

responses:

'200':

description: Successful operation

content:

application/json:

schema:

$ref: '#/components/schemas/QuotaUpdateResponseData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

[…]

ACUpdateData:

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

maxUesNumber:

type: integer

maxPdusNumber:

type: integer

required:

- snssai

QuotaUpdateRequestData:

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

quotaType:

items:

$ref: '#/components/schemas/SliceQuotaType'

required:

- snssai

- plmnId

- quotaType

QuotaUpdateResponseData:

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

maxUesNumber:

type: integer

maxPdusNumber:

type: integer

required:

- snssai

[…]

AcuFailureReason:

anyOf:

- type: string

enum:

- SLICE_NOT_FOUND

- EXCEED_MAX_UE_NUM

- EXCEED_MAX_UE_NUM_3GPP

- EXCEED_MAX_UE_NUM_N3GPP

- EXCEED_MAX_PDU_NUM

- EXCEED_MAX_PDU_NUM_3GPP

- EXCEED_MAX_PDU_NUM_N3GPP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Failure Reason of ACU operation to an S-NSSAI. Possible values are

- SLICE_NOT_FOUND

- EXCEED_MAX_UE_NUM

- EXCEED_MAX_UE_NUM_3GPP

- EXCEED_MAX_UE_NUM_N3GPP

- EXCEED_MAX_PDU_NUM

- EXCEED_MAX_PDU_NUM_3GPP

- EXCEED_MAX_PDU_NUM_N3GPP

SliceQuotaType:

- type: string

enum:

- "MAX_UE_NUM"

- "MAX_PDU_NUM"

- "BOTH"

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

[…]

* * * End of Changes * * * *