3GPP TSG-CT WG4 Meeting #121	C4-240739

# Athens, Greece; 26th Feb – 1st March 2024											was_C4-240401

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.080 | CR | 0113 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN for SL-MO-LR message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following ENs are left for the SL-MO-LR message: “Editor’s Note (CR 0107, Ranging_SL): It is FFS whether slppContainer is needed. Editor’s Note (CR 0107, Ranging_SL): It is FFS whether and how to encapsulate multiple relative information (i.e., between different pairs of UEs) in one response message.” As for the first EN, SLPP container of other UEs (i.e., except UE1) can be included in the SL-MO-LR request /response message to achieve slpp messages communication. But as for the IE structure, SLPP messages can be included as a separate IE, not in the relatedUEInfo. Thus, the EN can be removed and the identifier description related to SLPP container can also be removed. Issue of second EN is whether the network needs to respond to the target UE with the ranging result one by one (e.g., one LCS-SLMOLRRes is the relative location between target UE and located UE, another one is the relative velocity between target UE and reference UE). If so, multiple message will increase the signaling flows. A better design for the LCS-SLMOLRRes is to support the aggregation of the multiple ranging results. Thus, this CR proposes to aggregate multiple relative information in one LCS-SLMOLRRes message. Besides, as per the progress in CT1, the agreed CR (C1-240321) introduces the ueOnlyRSLPosAllowed IE to enable the LMF to permit the UE only operation for a given duration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - slppContainer is not included in the rangingUEInfo identifier; - multiple relative information can be encapsulated in one LCS-SLMOLRRes message. - Add ueOnlyRSLPosAllowed IE in the response message. - As for the data type for the relativeLocation and rangeDirection, we also propose to add a EN for further study. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2, 4.4.3.104, 4.4.3.106, 4.4.3,x1(new), 4.4.3,x2(new), 4.4.3,x3(new), 4.4.3,x4(new), 4.4.3,x5(new), 4.4.3,x6(new), 4.4.3,x7(new), 4.4.3,x8(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.4.2	ASN.1 data types

This clause provides an ASN.1 module defining the abstract data types in operations and errors specification. Only data types which are specific for this specification are defined. All other data types are imported from MAP together with the import of operations and errors.

(… text not shown for clarity …)

SLMOLR-Type::= ENUMERATED {

rangingSidelink (0),

...}

-- exception handling:

-- an unrecognized value shall be rejected by the receiver with a return error cause of

-- unexpected data value.

LCS-SLMOLRArg	::= SEQUENCE {

slmolr-Type	[0]	SLMOLR-Type,

lcs-QoS	[1]	LCS-QoS		OPTIONAL,

lcsClientExternalID	[2] LCSClientExternalID	OPTIONAL,

mlc-Number	[3]	ISDN-AddressString	OPTIONAL,

supportedGADShapes	[4]	SupportedGADShapes	OPTIONAL,

lcsServiceTypeID	[5]	LCSServiceTypeID	OPTIONAL,

pseudonymIndicator	[7]	NULL	OPTIONAL,

h-gmlc-address	[8] GSN-Address	OPTIONAL,

calculationAssistIndicator	[9] BOOLEAN	OPTIONAL,

preferredRangingResult	[10] PreferredRangingResult	OPTIONAL,

relatedUEInfo	[11] RelatedUEInfo	OPTIONAL,

...}

-- The parameter slmolr-Type shall set to value rangingSidelink.

-- Only mlc-number could be used to represent the address of H-GMLC of the UE, h-gmlc-address shall not be used for this purpose.

PreferredRangingResult	::= SEQUENCE {

absoluteLocationIndicator	[0]	BOOLEAN	OPTIONAL,

absoluteVelocityIndicator	[1]	BOOLEAN	OPTIONAL,

relativeLocationIndicator	[2]	BOOLEAN	OPTIONAL,

rangeDirection	[3]	BOOLEAN	OPTIONAL,

relativeVelocityIndicator	[4]	BOOLEAN	OPTIONAL,

... }

RelatedUEInfo	::= SEQUENCE SIZE (1..maxRelatedUE) OF RangingUEInfo

maxRelatedUE	INTEGER ::= 16

RangingUEInfo	::= SEQUENCE {

applicationLayerID	[0]	NULL,

rangingRole	[1]	RangingRole,

slppContainer	[2]	NULL	OPTIONAL,

... }

RangingRole	::= ENUMERATED {

targetUE (0),

locatedUE (1),

slReferenceUE (2),

slServerUE (3),

slClientUE (4),

... }

LCS-SLMOLRRes::= SEQUENCE {

absoluteLocation	[0]	Ext-GeographicalInformation	OPTIONAL,

absoluteVelocity	[1]	VelocityEstimate	OPTIONAL,

relativeResult	[2]	RelativeResult	OPTIONAL,

ueOnlyRSLPosAllowed	[4]	Duration	OPTIONAL,

timestamp	[5]	DateTime	OPTIONAL,

... }

RelativeResult	::= SEQUENCE SIZE (1..maxRelatedUE) OF SingleRelativeResult

SingleRelativeResult	::= SEQUENCE {

relatedUEInfo	[0] RelatedUEInfo	OPTIONAL,

relativeLocation	[1]	Ext-GeographicalInformation	OPTIONAL,

rangeDirection	[2]	Ext-GeographicalInformation	OPTIONAL,

relativeVelocity	[3]	VelocityEstimate	OPTIONAL,

... }

(… text not shown for clarity …)

* * * Next Change * * * *

4.4.3.104	rangingUEInfo

The rangingUEInfo identifier refers to the application ID and the role of related UEs participating the sidelink positioning/ranging location procedure.

* * * Next Change * * * *

4.4.3.106	lcs-SLMOLRRes

The lcs-SLMOLRRes identifier refers to the SL-MO-LR response parameters which are sent to the MS by the network.

* * * Next Change * * * *

4.4.3.x1	absoluteLocation

The absoluteLocation identifier refers to the absolute location information of the target UE.

4.4.3.x2	absoluteVelocity

The absoluteVelocity identifier refers to the absolute velocity information of the MS.

4.4.3.x3	relativeResult

The relativeResult identifier refers to one or more relative information between the MS and other related UEs.

4.4.3.x4	singleRelativeResult

The relativeResult identifier refers to one relative information between the MS and other related UE.

4.4.3.x5	relativeLocation

The relativeLocation identifier refers to the relative location information between the MS and other related UE.

4.4.3.x6	rangeDirection

The rangeDirection identifier refers to the rang and direction information between the MS and other related UE.

Editor’s Note (CR 0113, Ranging_SL):	Whether a new data type or the Ext-GeographicalInformation data type is used for the relativeLocation and rangeDirection is FFS.

4.4.3.x7	relativeVelocity

The relativeVelocity identifier refers to the relative velocity information between the MS and other related UE.

4.4.3.x8	ueOnlyRSLPosAllowed

The ueOnlyRSLPosAllowed identifier refers to the time duration when the MS is permitted to operate for ranging and sidelink positioning.

* * * End of Changes * * * *