3GPP TSG-CT WG4 Meeting #122	C4-241261

Changsha, P.R.China; 15th – 19th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.556 | CR | 0037 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N6 tunneling between V-UPF and V-EASDF for HR-SBO PDU sessions with overlapping IP addresses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_PH2 |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the Home Routed-Session BreakOut scenario, there are use cases where UEs from the same or different HPLMNs may establish HR-PDU sessions with the same Private UE IP address. If these UEs are roaming in the same VPLMN that supports HR-SBO and are allocated to the same V-UPF and the same V-EASDF, it creates problems for the EASDF to distinguish these UEs/PDU sessions, hence it is not possible to apply proper DNS context rules over these HR-SBO PDU Sessions. 3GPP TS 23.548 clause 6.7.2.2 specifies that one solution relies on N6 tunneling between the V-UPF and V-EASDF: NOTE 2: The network needs to ensure that EASDF can disambiguate the DNS traffic of different UEs that would be allocated with same private UE IP address. This can be done by implementation and/or deployment specific means, e.g. tunnelling on N6, network instance or UE source IP address mapping. To disambiguate the DNS traffic of different UEs allocated with same private UE IP address, the V-SMF can update the Local PSA-UPF with N4 PDR and FAR including either N6 traffic routing information or network instance (as described in clause 5.6.12 of 23.501 [2]) to forward the DNS traffic to the V-EASDF. When N6 traffic routing tunnel is used, the V-SMF can configure V-EASDF with the N6 traffic routing information towards the Local PSA-UPF. Alternatively, the V-SMF can update the local UPF to translate the UE source IP address with the replaced IP address in the IP pool of the Local PSA UPF. In TS 29.556, it is specified that the V-EASDF should be provisioned with the N6 traffic routing information (in the DNS context of a particular PDU session): - N6 traffic routing information that may contain the IP address and optional port number of the Local PSA-UPF, for HR-SBO in VPLMN, when using N6 tunneling between the L-PSA and V-EASDF, to disambiguate the DNS traffic using a private UE IP address from a certain HPLMN using a specific L-PSA UPF address of the N6 tunnel between the Local PSA-UPF and the V-EASDF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clairfy the N6 Routing Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous requirements may leads to interoperability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.1, 5.2.3.2.3, 6.1.6.2.23, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backward compatible feature to the OpenAPI definition of the Neasdf_DNSContext API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** First Change ***

##### 5.2.2.2.1	General

The Create service operation shall be used to create an individual DNS context for a given PDU Session in the EASDF.

It is used in the following procedures:

-	EAS Discovery Procedure with EASDF (see clause 6.2.3.2.2 of 3GPP TS 23.548 [14]); and

-	EAS Discovery Procedure with V-EASDF for HR-SBO (see clauses 6.7.2.2 and 6.7.2.3) of 3GPP TS 23.548 [14].

There shall be only one individual DNS context created in an EASDF per PDU session.

The NF Service Consumer (e.g. SMF) shall create a DNS context by using the HTTP POST method as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1: DNS context creation

1.	The NF Service Consumer shall send a POST request to the resource representing the DNS contexts collection resource of the EASDF. The content of the POST request shall contain:

-	the UE IP address, S-NSSAI and the DNN of the related PDU session;

-	a notification URI for receiving DNS context related event notifications, if notifications are requested;

-	one or more DNS rules.

In addition, for a DNS context creation in the V-EASDF for HR-SBO in VPLMN:

-	the UE IP address shall be set to an unspecified address, i.e. to the IPv4 address 0.0.0.0, or the IPv6 address ::/128, if the IP address of the UE's PDU session is not known yet, or to a mapped address (when using UE source IP address mapping at the Local PSA-UPF);

-	the S-NSSAI shall correspond to the VPLMN S-NSSAI value; and

-	the content of the POST request shall contain the HPLMN ID.

The payload body of the POST request may further contain:

-	N6 traffic routing information that may contain the IP address and optional port number of the Local PSA-UPF and the IP address and optional port number of the V-EASDF, for HR-SBO in VPLMN, when using N6 tunneling between the L-PSA and V-EASDF, to disambiguate the DNS traffic using a private UE IP address from a certain HPLMN using a specific L-PSA UPF address and a specific V-EASDF address for the N6 tunnel between the Local PSA-UPF and the V-EASDF.

2a.	On success, a "201 Created" response shall be returned with the "Location" header containing the URI of the created resource.

The POST response body shall include:

-	the IP address of the EASDF (to be sent by the SMF to the UE).

2b.	On failure, or redirection, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned.

# *** Next Change ***

##### 5.2.3.2.3	Processing flow for incoming DNS messages

Upon receipt of a DNS message, the EASDF shall first identify the DNS context corresponding to the DNS message as follows:

-	for DNS Query message: by using the source IP address of the DNS Query message and by matching it with the UE IP address provisioned in the DNS Query MDTs if any or with the UE IP address provisioned in the DNS context; and

-	for a DNS Response message: by matching the DNS response with the DNS Query (either by the EASDF assigning a specific Transaction ID when forwarding the DNS Query message and by matching the Transaction ID in the DNS Query and DNS Response, or by the EASDF using a unique couple of source IP address and UDP port per DNS context when forwarding the DNS Query message and by matching the DNS Response message using the destination IP address and UDP port) and by retrieving the DNS context that is associated with the DNS query.

NOTE 1:	The EASDF has direct user plane connectivity (i.e., without any NAT) with the PSA UPF over N6 for the transmission of DNS signalling exchanged with the UE. The deployment of a NAT between EASDF and PSA UPF is not supported.

The following options may be used to support DNS contexts with the same (private) UE IP address from different S-NSSAI and DNNs:

-	the EASDF may associate a specific EASDF address with a specific S-NSSAI and DNN and provide this address to the SMF during the creation of a DNS context associated with this S-NSSAI and DNN. If so, the EASDF shall also use the destination IP address of DNS Query messages to identify the DNS context matching an incoming DNS request;

-	the SMF may set the UE IP address to a mapped N6 address, when using UE source IP address mapping at the Local PSA-UPF;

-	the SMF may provide N6 traffic routing information (in the DNS context information) that may contain the IP address and optional port number of the Local PSA-UPF and the IP address and optional port number of the V-EASDF, for HR-SBO in VPLMN, when using N6 tunneling between the L-PSA and V-EASDF. If so, the V-EASDF shall also use the N6 traffic routing information of DNS Query messages to identify the DNS context matching an incoming DNS request.

NOTE X:	When using N6 tunneling, the V-EASDF sends the DNS query response message in the same N6 tunnel as the one on which the related DNS Query messages was received.

To support DNS contexts with the same (private) UE IP address from different PLMNs and from different S-NSSAI and DNNs, the V-EASDF may associate a specific V-EASDF address with a specific HPLMN ID, S-NSSAI and DNN and provide this address to the V-SMF during the creation of a DNS context associated with this HPLMN ID, S-NSSAI and DNN. If so, the V-EASDF shall also use the destination IP address of DNS Query messages to identify the DNS context matching an incoming DNS request.

A DNS context containing an unspecified UE IP address shall be considered to never match any DNS Query message.

If there is no DNS context matching a DNS Query or Response message, the EASDF should forward the DNS Query message towards a preconfigured DNS server and the DNS response towards the UE.

After finding the DNS context, the EASDF shall look up for a DNS rule matching the DNS message, among all DNS rules provisioned in the DNS Context, starting with the DNS rules with the highest precedence and continuing then with DNS rules with a lower precedence, in decreasing order of precedence. If there is no DNS rule matching the DNS message, the EASDF should forward the DNS Query message towards a preconfigured DNS server/resolver for resolution.

NOTE Y:	The SMF can provision in the DNS context a DNS rule with the lowest precedence and with a DNS Query MDT or a DNS Response MDT containing a wildcard FQDN, such as to associate a default behavior to all DNS messages not matching any other DNS rule, e.g. forward DNS Query messages to a specific DNS Server.

After having found a matching DNS rule, the EASDF shall stop looking up for other DNS rules and shall apply the list of actions provisioned in the matching DNS rule.

A DNS message matches a DNS rule if it matches at least one MDT of the DNS Rule or one BD MDT referred by the DNS rule.

The DNS message processing models for DNS Query and DNS Response are depicted in Figure 5.2.3.2-2 and 5.2.3.2-3 respectively.

Figure 5.2.3.2-2: DNS Query processing flow in the EASDF

Figure 5.2.3.2-3: DNS Response processing flow in the EASDF

# *** Next Change ***

##### 6.1.6.2.23	Type: N6RoutingInfo

Table 6.1.6.2.23-1: Definition of type N6RoutingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upfIpv4Address | Ipv4Addr | O | 0..1 | IPv4 address of the Local PSA-UPF for the N6 tunnel between the Local PSA-UPF and V-EASDF |
| upfIpv6Address | Ipv6Addr | O | 0..1 | IPv6 address of the Local PSA-UPF for the N6 tunnel between the Local PSA-UPF and V-EASDF |
| upfPortNumber | Uinteger | O | 0..1 | Port number of the Local PSA-UPF for the N6 tunnel between the Local PSA-UPF and V-EASDF. |
| easdfIpv4Address | Ipv4Addr | O | 0..1 | IPv4 address of the V-EASDF for the N6 tunnel between the Local PSA-UPF and V-EASDF |
| easdfIpv6Address | Ipv6Addr | O | 0..1 | IPv6 address of the V-EASDF for the N6 tunnel between the Local PSA-UPF and V-EASDF |
| easdfPortNumber | Uinteger | O | 0..1 | Port number of the V-EASDF for the N6 tunnel between the Local PSA-UPF and V-EASDF. |

# *** Next Change ***

# A.2	Neasdf_DNSContext API

openapi: 3.0.0

info:

version: '1.1.0-alpha.5'

title: 'Neasdf_DNSContext'

description: |

EASDF DNS Context Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.556 V18.5.0; 5G System; Edge Application Server Discovery Services; Stage3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.556/

servers:

- url: '{apiRoot}/neasdf-dnscontext/v1'

variables:

apiRoot:

default: https://example.com

description:  apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

******Skipped for Clarity******

N6RoutingInfo:

description: N6 traffic routing information

type: object

properties:

upfIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

upfIpv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

upfPortNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

easdfIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

easdfIpv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

easdfPortNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

******Skipped for Clarity******

# *** End of Changes ***