3GPP TSG-CT WG4 Meeting #123	C4-242133

# Hyderabad, India; 27th – 31st May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1011 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to SubscrCond data structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2024-04-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a conflict on the definiton of SubscrCond data type, which is defined as a "oneOf" of the following schemas (among others): SubscrCond: oneOf: - $ref: '#/components/schemas/NfTypeCond' - $ref: '#/components/schemas/NfGroupCond' - $ref: '#/components/schemas/NfGroupListCond' where each of those data types is defined as follows (only relevant information shown below): NfTypeCond: type: object required: - nfType not: required: [ nfGroupId ] NfGroupCond: type: object required: - nfType - nfGroupId NfGroupListCond: type: object required: - conditionType - nfType - nfGroupIdList With these definitions, a JSON object instance with type NfGroupListCond will also successfully validate as type NfTypeConf. The issue appeared when NfGroupListCond was introduced, and added to the list of "oneOf" schemas since, prior to that, there was no conflict between NfTypeCond and NfGroupCond, due to the "not" constraint present in NfTypeCond. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add further "not" constraints to NfTypeCond (bound via "allOf") to disambiguate between NfTypeCond and NfGroupListCond. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to create a subscription on the NRF to get updates from all NFs determined from a list of NF Group IDs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections with impacts on the following APIs: - TS29510_Nnrf_NFManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# A.2	Nnrf_NFManagement API

(... text not shown for clarity ...)

NfTypeCond:

description: Subscription to a set of NFs based on their NF Type

type: object

required:

- nfType

allOf:

- not:

required: [ nfGroupId ]

- not:

required: [ conditionType, nfGroupIdList ]

properties:

nfType:

$ref: '#/components/schemas/NFType'

(... text not shown for clarity ...)

* * * End of Changes * * * *