3GPP TSG-CT WG4 Meeting #123	C4-242184

# Hyderabad, India; 27th – 31st May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0266 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remote UE Indication in Location Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 has agreed that a Remote UE include an indication of Remote UE when providing the location information to the LMF, as specified in TS 37.355: -- ASN1START CommonIEsProvideCapabilities ::= SEQUENCE { ..., [[ segmentationInfo-r14 SegmentationInfo-r14 OPTIONAL, -- Cond Segmentation lpp-message-segmentation-r14 BIT STRING { serverToTarget (0), targetToServer (1) } OPTIONAL ]], [[ remoteUE-Indication-r18 BOOLEAN OPTIONAL, -- Cond NR locationEstimateAndMeasurementReporting-r18 ENUMERATED { supported } OPTIONAL ]] } -- ASN1STOP CommonIEsProvideCapabilities field descriptions. remoteUE-Indication This field indicates whether the target device in NR access is configured as a L2 U2N Remote UE. The target device in NR access may transmit a ProvideCapabilities message with an appropriate value of this field when it starts or stops operation as a U2N Remote UE.. The LMF will be aware for this during position calculation that the Cell ID itself and its coverage is not really providing valid location information (rather for Relay UE in this case). As the NR Cell ID (NCGI) is also provided to GMLC, to avoid GMLC incorrectly use the NCGI for further position calculation/validation, the Remote UE information shall also be propagated to GMLC as well together with NCGI to allow the GMLC to make the distinction. |
| Summary of change: |  | Add new Remote UE indication IE in Location Data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the location information is for a Remote UE, the GMLC may incorrectly use NCGI and generate wrong result. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features in Nlmf_Location API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.3	Type: LocationData

Table 6.1.6.2.3-1: Definition of type LocationData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locationEstimate |  | GeographicArea |  | M |  | 1 |  | For a request for triggered location where location estimates are not required, the location estimate can be based on current serving cell. |  |  |  |
| accuracyFulfilmentIndicator |  | AccuracyFulfilmentIndicator |  | O |  | 0..1 |  | When present, this IE shall indicate fulfilment of required accuracy. |  |  |  |
| ageOfLocationEstimate |  | AgeOfLocationEstimate |  | O |  | 0..1 |  | When present, this IE shall indicate age of the location estimate. |  |  |  |
| timestampOfLocationEstimate |  | DateTime |  | O |  | 0..1 |  | When present, this IE shall indicate the estimated UTC time when the location estimate corresponded to the UE location (i.e. when the location estimate and the actual UE location was the same). |  |  |  |
| velocityEstimate |  | VelocityEstimate |  | O |  | 0..1 |  | When present, this IE shall indicate velocity estimate. |  |  |  |
| civicAddress |  | CivicAddress |  | O |  | 0..1 |  | When present, this IE shall indicate a civic address. |  |  |  |
| localLocationEstimate |  | LocalArea |  | O |  | 0..1 |  | When present, this IE shall indicate a local area in reference system. |  |  |  |
| positioningDataList |  | array(PositioningMethodAndUsage) |  | O |  | 1..N |  | When present, this IE shall include a list of data related to positioning methods. |  |  |  |
| gnssPositioningDataList |  | array(GnssPositioningMethodAndUsage) |  | O |  | 1..N |  | When present, this IE shall include a list of data related to GNSS positioning methods. |  |  |  |
| ecgi |  | Ecgi |  | O |  | 0..1 |  | When present, this IE shall indicate the ID of the E-UTRAN cell serving the UE. |  |  |  |
| ncgi |  | Ncgi |  | O |  | 0..1 |  | When present, this IE shall indicate the ID of the NR cell serving the target UE, or the NR cell serving the relay UE if remoteUeInd IE is present with the value true. |  |  |  |
| remoteUeInd |  | boolean |  | C |  | 0..1 |  | This IE shall be present with the value true when the UE is a Remote UE. Presence of this IE with the value false shall be prohibited. |  | Ranging_SL |  |
| altitude |  | Altitude |  | O |  | 0..1 |  | Altitude of the positioning estimate. When the shape used in "locationEstimate" supports conveying the altitude parameter, this IE shall be absent. |  |  |  |
| barometricPressure |  | BarometricPressure |  | O |  | 0..1 |  | If present, this IE contains the barometric pressure measurement as reported by the target UE. |  |  |  |
| servingLMFidentification |  | LMFIdentification |  | O |  | 0..1 |  | When present, this IE shall indicate the identity of the serving LMF |  |  |  |
| uePositioningCap |  | UePositioningCapabilities |  | O |  | 0..1 |  | When present, this IE shall indicate the positioning capabilities supported by the UE. |  |  |  |
| supportedFeatures |  | SupportedFeatures |  | C |  | 0..1 |  | This IE shall be present if at least one optional feature defined in clause 6.1.9 is supported. |  |  |  |
| ueAreaInd |  | UeAreaIndication |  | O |  | 0..1 |  | When present, this IE shall contain a country, area in a country or international area indication where UE is located. If UE is outside of the area of any known country, i.e. international area, then the LMF shall provide the International Area Indication without a country. |  | SAT |  |
| achievedQos |  | MinorLocationQoS |  | O |  | 0..1 |  | When present, this IE shall contain the achieved Location QoS Accuracy of the estimated location. This IE may be present if the LCS QoS Class required in the request message is "MULTIPLE_QOS". If it's absent, LCS QoS Class required in the request message is "MULTIPLE_QOS" and AccuracyFulfilmentIndicator is "REQUESTED_ACCURACY_FULFILLED", it indicates that the location QoS in the highest priority is achieved. |  | MUTIQOS |  |
| directReportInd |  | boolean |  | C |  | 0..1 |  | When present, this IE shall be set for the following value: - true: location determination will be sent by LMF to GMLC directly - false (default): location determination will not be sent by LMF to GMLC directly |  |  |  |
| indoorOutdoorInd |  | IndoorOutdoorInd |  | O |  | 0..1 |  | When present, this IE shall indicate whether the UE is indoor or outdoor. |  |  |  |
| acceptedPeriodicEventInfo |  | PeriodicEventInfo |  | C |  | 0..1 |  | This IE shall be present if PeriodicEventInfo was received in the request which includes reportingAmountInf IE and/or reportingIntervalMs IE. When present, this IE shall provide the accepted periodic event reporting information. |  |  |  |
| haGnssMetrics |  | HighAccuracyGnssMetrics |  | O |  | 0..1 |  | When present, this IE shall indicate the high accuracy GNSS metrics received from the device in the LPP HA-GNSS-Metrics-r17 IE as specified in 3GPP TS 37.355 [21]. |  |  |  |
| losNlosMeasureInd |  | LosNlosMeasureInd |  | O |  | 0..1 |  | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |  |  |  |
| rangingSlCapability |  | RangingSlCapability |  | O |  | 0..1 |  | When present, this IE shall indicate that the UE supports Ranging/Sidelink Positioning Capability. |  |  | Ranging_SL |  |
| relatedApplicationlayerId |  | ApplicationlayerId |  | O |  | 0..1 |  | Identifies the application layer ID of the related UE for ranging and sidelink positioning, such as located UE, reference UE, etc. |  |  | Ranging_SL |  |
| distanceDirection |  | RangeDirection |  | O |  | 0..1 |  | When present, this IE identifies a distance and direction from a point A to a point B, comprising a distance from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B. |  |  | Ranging_SL |  |
| 2dRelativeLocation |  | 2DRelativeLocation |  | O |  | 0..1 |  | When present, this IE identifies a relative 2D location with uncertainty ellipse, characterised by a point described in 2D local co-ordinates with origin corresponding to another known point, distances r1 and r2 and an angle of orientation A. |  |  | Ranging_SL |  |
| 3dRelativeLocation |  | 3DRelativeLocation |  | O |  | 0..1 |  | When present, this IE identifies a relative 3D location with uncertainty ellipsoid, characterised by a point described in 3D local co-ordinates with origin corresponding to another known point, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"). |  |  | Ranging_SL |  |
| relativeVelocity |  | VelocityEstimate |  | O |  | 0..1 |  | When present, this IE identifies UE velocity relative to the UE identified with relatedApplicationlayerId. |  |  | Ranging_SL |  |

* * * Next Change * * * *

# A.2	Nlmf_Location API

********************** Text Skipped for Clarity **********************

LocationData:

description: Information within Determine Location Response.

type: object

required:

- locationEstimate

properties:

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

accuracyFulfilmentIndicator:

$ref: '#/components/schemas/AccuracyFulfilmentIndicator'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

timestampOfLocationEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

civicAddress:

$ref: '#/components/schemas/CivicAddress'

localLocationEstimate:

$ref: '#/components/schemas/LocalArea'

positioningDataList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: '#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

remoteUeInd:

type: boolean

enum:

- true

altitude:

$ref: '#/components/schemas/Altitude'

barometricPressure:

$ref: '#/components/schemas/BarometricPressure'

servingLMFIdentification:

$ref: '#/components/schemas/LMFIdentification'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

ueAreaInd:

$ref: '#/components/schemas/UeAreaIndication'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

achievedQos:

$ref: '#/components/schemas/MinorLocationQoS'

directReportInd:

type: boolean

default: false

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

acceptedPeriodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

haGnssMetrics:

$ref: '#/components/schemas/HighAccuracyGnssMetrics'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

relatedApplicationlayerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationlayerId'

distanceDirection:

$ref: '#/components/schemas/RangeDirection'

2dRelativeLocation:

$ref: '#/components/schemas/2DRelativeLocation'

3dRelativeLocation:

$ref: '#/components/schemas/3DRelativeLocation'

relativeVelocity:

$ref: '#/components/schemas/VelocityEstimate'

rangingSlCapability:

$ref: '#/components/schemas/RangingSlCapability'

********************** Text Skipped for Clarity **********************

* * * End of Changes * * * *