3GPP TSG-CT WG4 Meeting #123	C4-242350

Hyderabad, India; 27th – 31st May 2024	was C4-242069

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.274 | CR | 2107 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the PGW Restart Notification Acknowledge message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, RPCPSET |  |  |  |  |  | Date: |  |  | 2024-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of the PGW Restart Notification Acknowledge message reads as follows: "as a response of PGW Restart Notification to indicate that the MME/S4-SGSN deletes all the relevant PDN connections as specified in 3GPP TS 23.007 [17] if the Cause IE includes an acceptance cause." This is incorrect for the PDN connection restoration for PGW-C/SMF change as specified in clause 31.3 of 3GPP TS 23.007, see below: " When detecting that the PGW-C/SMF has failed, the SGW-C shall send a PGW Restart Notification to the MME as specified in clause 16.1A.2. When detecting that a PGW-C/SMF has failed, the SGW-C and MME shall maintain the contexts of the PDN connections served by that PGW-C/SMF for which this procedure is supported." |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is corrected that for the scenario of PDN connection restoration for PGW-C/SMF change specified in clause 31 of TS 23.007, the MME and the SGW maintain the contexts of the PDN connections served by the PGW-C/SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent requirements lead to incorrect implementation and possible interoperability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: remove the change over change in the new NOTE. |  |  |  |  |  |  |  |  |

# *** First Change ***

### 7.9.6	PGW Restart Notification Acknowledge

The PGW Restart Notification Acknowledge shall be sent as a response of PGW Restart Notification.

When the PGW Restart Notification (Acknowledge) messages are used in the procedures specified in clauses 16.1A.2, 20.2.2.2 and 27.2.2.2 of 3GPP TS 23.007 [17], if the Cause IE includes an acceptance cause, this indicates that the MME/S4-SGSN deletes all the relevant PDN connections .

NOTE:	In the restoration of PDN connections after a PGW-C/SMF change procedure specified in clause 31 of 3GPP TS 23.007 [17], the MME and the SGW maintain the contexts of the PDN connections served by the PGW-C/SMF after the exchange of the PGW Restart Notification (Acknowledge) messages.

Possible Cause values are specified in Table 8.4-1.

Table 7.9.6-1: Information Elements in PGW Restart Notification Acknowledge

| Information elements | P | Condition / Comment | IE Type | Ins. |
| --- | --- | --- | --- | --- |
| Cause | M |  | Cause | 0 |
| Private Extension | O |  | Private Extension | VS |

# *** End of Change ***