- 3GPP TSG-CT WG4 Meeting #28	C4-051176
London, UK. 29th Aug. – 02nd Sept. 2005.

| CR-Form-v7.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.234 | CR | 072 |  rev | - |  Current version: | 6.3.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | IMSI handling at the PDG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Work item code:  | WLAN |  |  |  |  |  |  | Date:  |  | 09/08/2005 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Ph2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) |  |
| Reason for change:  |  | Essential correction. In order to allow implementation of the solution in normative Annex F of 23.234, it is necessary that the PDG hold the user’s IMSI. The IMSI is required in the Create PDP Context request for the first PDP Context Activation procedure. It will be used to identify each PDP Context as can be seen in this text extracted from section 7.3.1 in TS 29.060: “The IMSI information element together with the NSAPI information element uniquely identifies the PDP context to be created.” This fact is also clear in that the IMSI is a Mandatory Information Element in the GGSN in the GPRS data table in section 5.2 in TS 23.008. The IMSI is not mandatory in the Create PDP Context request, but this is only because it will not be present in the Secondary PDP Context Activation procedures. This condition is stated also in 7.3.1 in TS 29.060: “When using the Secondary PDP Context Activation Procedure, the Selection mode, IMSI, MSISDN, End User Address, Access Point Name and APN Restriction information elements shall not be included in the message.” However, at least for the case where the user is Scenario 2 authenticated, the user may not use the permanent identity in the IKE_v2 tunnel set up signalling. Rather it may use fast reauthentication ID or pseudonym. This CR is to add the IMSI in the answer that the 3GPP AAA Server sends back to the PDG if the PDG cannot send the user’s IMSI in the request. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The Permanent User Identity Information Element is added in the Authorization Answer in table 8.3.2.2. There are also some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | It will not be possible to implement the alternative in 23.234. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | X |  | Other core specifications  |  |  | CR 23.008-159 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## >>>>>>>>>>> First modified section <<<<<<<<<<<

# 8	Wm Description

## 8.1	Functionality

The Wm reference point is defined between the 3GPP AAA Server and the PDG. The description of the reference point and its functionality is given in 3GPP TS 23.234 [4].

This clause specifies a Diameter application that supports the functionality of this reference point.

In the roaming case, the 3GPP AAA Proxy shall act as a stateful proxy between the PDG and 3GPP AAA Server.

## 8.2	Protocols

Diameter EAP application is used for authentication of the user. In this case, the PDG shall act as the NAS, as described in 3GPP TS 33.234 [18]. For authorization and other Wm functionalities, NASREQ and base protocol procedures are used.

The Application-Id to be advertised over Wm reference point corresponds to the EAP, NASREQ or Diameter Base Protocol Application-Id, depending on the command sent over Wm.

## 8.3	Procedures Description

### 8.3.1	Authentication Procedures

According to the requirements specified in chapter 10.1, Wm reference point shall enable:

-	Messaging for service authentication between WLAN UE and 3GPP AAA Server/Proxy.

The authentication procedure is used between the PDG and 3GPP AAA Server/Proxy.  It is invoked by the PDG, on receipt from the WLAN-UE of a "tunnel establishment request" message. This takes the form of forwarding an IKE v2 (3GPP TS 33.234 [18]) exchange with the purpose of authenticating in order to set up a Security Association (SA) between the UE and the PDG. Once the SA has been authenticated, more than one tunnel SA can be negotiated inside the IKE v2 SA. Hence additional tunnels between the UE and PDG do not need to trigger further Diameter_EAP authentication messaging to the 3GPP AAA Server.

The Wm reference point performs authentication based on the reuse of the DER/DEA command set defined in Diameter_EAP (3GPP TS 33.234 [18]).

Table 8.3.1.1: Authentication Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity | User-Name | M | This information element contains the identity of the user. |
| EAP payload | EAP payload | M | Encapsulated EAP payload used for UE - 3GPP AAA Server mutual authentication |
| Authentication Request Type | Auth Req Type | M | Defines whether authentication only or authentication and authorization are required. AUTHENTICATION_ONLY is required in this case |
| Visited Network Identifier | Visited-Network-Identifier | C | Identifier that allows the home network to identify the Visited Network. This AVP shall be present if the PDG is not in the WLAN-UE's home network i.e. the WLAN-UE is roaming. |
| EAP Lower Layer | EAP Lower Layer | M | This AVP shall contain the value “3” to indicate IKE_v2 has been used to carry EAP messages to the PDG, according to [27] |

Table 8.3.1.2: Authentication Answer

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| EAP payload | EAP payload | M | Encapsulated EAP payload used for UE - 3GPP AAA Server mutual authentication |
| Master-Session-Key | Master-Session-Key | C | It contains keying material for protecting the communication between the user and the NAS. Present when Result Code is set to "Success". |
| Result code | Result Code / Experimental-Result-Code | M | Result of the operation. Result-Code AVP shall be used for errors defined in the Diameter Base Protocol or as per in NASREQ . 1xxx should be used for multi-round, 2xxx for success. Experimental-Result AVP shall be used for Wm errors. This is a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |

#### 8.3.1.1	3GPP AAA Server Detailed Behaviour

On receipt of the DER message, the 3GPP AAA Server shall check if the Session-ID corresponds to an ongoing session. If it corresponds to an on-going session, the 3GPP AAA Server shall process the DER message according to 3GPP TS 33.234 [18] and no Diameter EAP authentication shall be triggered over the Wm interface.

If the Session-ID does not correspond to an on-going session, the 3GPP AAA Server shall:

1)	Check that the user exists in the 3GPP AAA Server. If not, the 3GPP AAA Server shall use the procedures defined for the Wx interface to authenticate the user.

2)	Check that the user has 3GPP-WLAN subscription. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_NO_WLAN_SUBSCRIPTON.

Otherwise, DIAMETER_SUCCESS shall be returned to indicate successful authentication procedure and authentication information shall be returned.

Exceptions to the cases specified here shall be treated by 3GPP AAA Server as error situations, the Result-Code shall be set to DIAMETER_UNABLE_TO_COMPLY. No authentication information shall be returned.

#### 8.3.1.2	3GPP AAA Proxy Detailed Behaviour

The 3GPP AAA Proxy is required to handle roaming cases in which the PDG is in the VPLMN. The 3GPP AAA Proxy shall act as a stateful proxy.

On receipt of the DEA message, the AAA Proxy shall record the state of the connection (i.e. Authentication Successful).

### 8.3.2	Authorization Procedures

According to the requirements stated in subclause 10.1, Wm reference point shall enable:

-	Carrying messages for service authorization between PDG and 3GPP AAA Server/Proxy.

-	Allow the 3GPP AAA Server/Proxy to retrieve tunnelling attributes and WLAN UE's IP configuration parameters from/via Packet Data Gateway.

This procedure is used between the PDG and 3GPP AAA Server and Proxy. It is invoked by the PDG, on receipt from the WLAN-UE of a "tunnel establishment request" message and subsequent to the success of tunnel authentication i.e. on receipt of a DEA message from the 3GPP AAA Server with Result Code set to "Success".

The Wm reference point performs authorization download based on the reuse of the NASREQ [12] AAR-AAA command set.

Table 8.3.2.1 Wm Authorization Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity | User-Name | M | This information element contains the identity of the user. |
| Request-Type | Session-Request -Type | M | Type of Wm specific Diameter application request. The following values are to be used: AUTHORIZATION REQUEST (0) This value shall indicate the initial request for authorization of the user to the APN. ROUTING POLICY (1) This value shall indicate that routing policy AVP is present. |
| Visited Network Identifier | Visited-Network-Identifier | C | Identifier that allows the home network to identify the Visited Network. This AVP shall be present if the PDG is not in the WLAN-UE's home network, i.e. the WLAN-UE is roaming. |
| W-APN-ID | APN-Id | C | This information element contains the W-APN which the UE is requesting authorization. This AVP is present when Session-Request-Type AVP is set to AUTHORIZATION REQUEST. |
| Routing Policy | Routing-Policy | C | This AVP includes the routing policy of the tunnel set-up. This AVP shall be present when Session-Request-Type AVP is set to ROUTING POLICY. The exact format of this AVP is specified in section 10.1.24. |
| Routing Information | Destination-Host | M | The 3GPP AAA Server name is obtained from the Origin-Host AVP of a previously received message. |

Table 8.3.2.2: AA-Answer

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Registration Result | Result Code/ Experimental Result Code | M | Result of the operation. Result-Code AVP shall be used for errors defined in the Diameter Base Protocol. Experimental-Result AVP shall be used for Wm errors. This is a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP |
| Subscription-ID AVP | Subscription-ID AVP | C | This AVP shall contain the MSISDN of the user. This AVP shall be present is the Diameter Result Code is set to DIAMETER_SUCCESS |
| Max-Subscribed-Bandwidth | Max-Requested-Bandwidth | O | The Max requested bandwidth AVP. Can be sent by the 3GPP AAA Server to the PDG if it is present in the user subscription info held at the 3GPP AAA Server. |
| Charging Data | Charging-Data | C | Charging information for the W-APN for that user. It shall be present when Result-Code is equal to DIAMETER_SUCCESS and when the received Session-Request –Type was set to AUTHORIZATION REQUEST. |
| Framed-IP-Address | Framed-IP-Address | O | This AVP contains the remote IPv4 address of the WLAN UE that the 3GPP AAA Server downloaded from the HSS. This AVP shall not be present when the 3GPP AAA Server received an authorisation request with Session-Request –Type AVP set to ROUTING POLICY. |
| Framed-IP-Prefix | Framed-IP-Prefix | O | This AVP contains the remote IPv6 prefix of the WLAN UE that the 3GPP AAA Server downloaded from the HSS. This AVP shall not be present when the 3GPP AAA Server received an authorisation request with Session-Request –Type AVP set to ROUTING POLICY. |
| Permanent User Identity | User-Name | C | This information element contains the IMSI of the user. This shall be present if Registration Result Code is set to "Success" and the AAR did not contain the IMSI. |

#### 8.3.2.1	3GPP AAA Server Detailed Behaviour

The 3GPP AAA Server shall, in the following order (if there is an error in any of the steps, the 3GPP AAA Server shall stop processing and return the corresponding error code):

1)	Check that the user exists in the 3GPP AAA Server. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

2)	Check the Session-Request-Type AVP:

-	If Request type is set to AUTHORIZATION REQUEST, it indicates that the WLAN-UE is attempting to access the particular W-APN at the PDG and is requesting authorization for such a W-APN access.

- The 3GPP AAA Server shall check that the user has subscription for the W-APN requested. If not, Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_NO_APN_SUBSCRIPTON.

- The 3GPP AAA Server shall check whether the user has access to that W-APN, otherwise Result-Code shall be set to DIAMETER_AUTHORIZATION_REJECTED.

- If the user is roaming (indicated by the presence of the Visited-Network-Identifier AVP), the 3GPP AAA Server shall check if the user is allowed to access the W-APN from a VPLMN. This information is obtained from the HSS within the APN-Authorization AVP. If not, Experimental-Result-Code shall be set to DIAMETER_ERROR _ROAMING_NOT_ALLOWED.

- If the WLAN UE does not already have an active access to this W-APN, the PDG shall initiate an Access-Number counter for that W-APN and set it to one. If the Access-Number counter has already been initiated, the PDG shall increment the counter by one. The PDG shall then check the counter value against the Maximum-Number-Accesses for that W-APN from that user’s data. If the Access-Number exceeds Maximum-Number-Accesses, the 3GPP AAA Server shall use the 3GPP AAA Server initiated disconnection procedures towards the PDG with which the user has the oldest established access in order to initiate the tear down of the SA associated with that access. The 3GPP AAA Server shall update accordingly the information of active accesses for the W-APN and shall store the PDG IP address.

- The 3GPP AAA Server shall download user data relevant to the W-APN,e.g. WLAN UE remote IP address if present and the charging information as received from the HSS. The Result-Code shall be set to DIAMETER_SUCCESS.

-	If Request type is set to ROUTING POLICY, it indicates that the WLAN-UE already has an active tunnel to the given PDG and is informing the 3GPP AAA Server of the routing policy for the tunnel. The 3GPP AAA Server shall store the Routing-Policy AVP and use Wg procedures to install this policy at the WAG. If this is successful, 3GPP AAA Server shall set Result-Code AVP to DIAMETER_SUCCESS in the AAA message.  If not, Result-Code shall be set to DIAMETER_UNABLE_TO COMPLY.

Exceptions to the cases specified here shall be treated by 3GPP AAA Server as error situations, the Result-Code shall be set to DIAMETER_UNABLE_TO_COMPLY. No authorization information shall be returned.

#### 8.3.2.2	AAA Proxy Detailed Behaviour

The 3GPP AAA Proxy is required to handle roaming cases in which the PDG is in the VPLMN. On this interface, it may act to limit policy enforcement by modifying messages. It shall therefore maintain session state. The 3GPP AAA Proxy shall, in the following order (if there is an error in any of the steps, the 3GPP AAA Proxy shall stop processing and return the corresponding error code).

Check the Request Type AVP:

1)	If Request type indicates AUTHORIZATION REQUEST, it indicates that the WLAN-UE does not have a tunnel active to the particular APN at the PDG and is requesting authorization for such an APN.

a)	The 3GPP AAA Proxy shall check locally configured information whether users from the HPLMN are allowed to access to the W-APN requested from this (V)PLMN. If not, Experimental-Result-Code shall be set to DIAMETER_ERROR _ROAMING_NOT_ALLOWED and the AA-A message sent to the PDG. In all other cases, the message shall be forwarded transparently to the 3GPP AAA Server.

2)	If Request-Type indicates ROUTING POLICY:

a)	This indicates that the WLAN-UE already has an active tunnel to the given PDG and is informing the 3GPP AAA Server of the routing policy for the tunnel. The 3GPP AAA Proxy shall store the Routing-Policy AVP and use Wg procedures to download the policy to the WAG. If this is successful, 3GPP AAA Server shall set Result Code to "Success" and send the AAR reply.  If not, Result Code shall be set to DIAMETER_UNABLE_TO COMPLY.

Exceptions to the cases specified here shall be treated by 3GPP AAA Proxy as error situations, the Result-Code shall be set to DIAMETER_UNABLE_TO_COMPLY and AA-A message sent to the PDG.

### 8.3.3	PDG Initiated Session Termination Procedure

This procedure is used between the PDG and the 3GPP AAA Server. It is invoked by the PDG when the user's tunnel associated with the W-APN has been disconnected.

Table 8.3.3.1: Session Termination Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity | User-Name | M | This information element contains the identity of the user. |
| W-APN-ID | APN-Id | M | This information element contains the W-APN which the UE is requesting access. |
| Routing Information | Destination-Host | M | The 3GPP AAA Server name is obtained from the Origin-Host AVP of a previous received message. |

Table 8.3.3.2: Session Termination Answer

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Result | Result-Code / Experimental-Result | M | Result of the operation. Result-Code AVP shall be used for errors defined in the Diameter Base Protocol. Experimental-Result AVP shall be used for Wm errors. |

#### 8.3.3.1	3GPP AAA Server Detailed behaviour

On receipt of the STR, the 3GPP AAA Server shall, in the following order (if there is an error in any of the steps, the 3GPP AAA Server shall stop processing and return the corresponding error code):

a)	Check from the User Name AVP that this corresponds to a user. If not Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

b)	Check that the user has an active session on the received W- APN. If not, Experimental-Result-Code shall be set to DIAMETER_ERROR_W-APN_UNUSED_BY_USER.

c)	If the User is known and the W-APN corresponds to a known session, the 3GPP AAA Server shall remove any PDG specific information connected to that user on that W-APN. and update the status of the subscriber if needed. If the user was a home user, the 3GPP AAA Server shall signal to the WAG to initiate procedures to remove any filtering policy associated with that user's session.  The Result Code shall be set to DIAMETER_SUCCESS.

#### 8.3.3.2	3GPP AAA Proxy Detailed Behaviour

In the roaming case, the 3GPP AAA Proxy shall forward the STR message to the 3GPP AAA Server. On receipt of an STA with Result-Code set to DIAMETER_SUCCESS, the 3GPP AAA Proxy shall remove any session specific information associated with that user at that W-APN. It shall signal to the WAG to initiate procedures to remove any filtering policy associated with that user's session.

### 8.3.4	3GPP AAA Server Initiated Tunnel Disconnect Procedure

This procedure is used between the 3GPP AAA Server and the PDG. It is invoked by the 3GPP AAA Server when the WLAN subscription for the user has been deleted/prohibited in the 3GPP AAA Server or if the particular session must be terminated for any reason and the PDG must be updated with respect to these changes.

The Wm reference point performs the disconnection of user tunnel initiated by the 3GPP AAA Server based on the use of the RFC 3588 [7] Abort-Session-Request / Answer (ASR/ASA) commands.

Table 8.3.4.1: 3GPP AAA Server Initiated Tunnel Disconnection - Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity | User-Name | M | This information element contains the identity of the user. |
| W-APN-Id (see clause 8.5.15) | APN-Id | M | W-APN Identification. |
| Routing Information | Destination-Host | M | The PDG name is obtained from the Origin-Host AVP of a previous message received from the PDG e.g. included in the authentication command. |

Table 8.3.4.2: 3GPP AAA Server Initiated Tunnel Disconnection - Answer

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Result | Result-Code / Experimental-Result | M | Result of the operation. Result-Code AVP shall be used for errors defined in the Diameter Base Protocol. Experimental-Result AVP shall be used for Wm errors. This is a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |

#### 8.3.4.1	Detailed Behaviour

The 3GPP AAA Server shall make use of this procedure to instruct the PDG to disconnect a particular W-APN for a specific user. On receipt of the message, the PDG shall:

1)	Check from the user is known in the PDG. If not, Experimental-Result-Code shall be set to DIAMETER_ERROR_USER_UNKNOWN.

2)	Check that the user has an active session on the received W-APN. If not, Experimental-Result-Code shall be set to DIAMETER_ERROR_W-APN_UNUSED_BY_USER.

3)	If the User is known and the W-APN corresponds to a known session, the PDG shall perform tunnel disconnect procedure of the tunnels associated with that user on that W-APN. The PDG shall further remove any stored user information pertaining to that APN.

4)	The PDG shall set the Result-Code to DIAMETER_SUCCESS and send back the SAA command to the 3GPP AAA Server.

On receipt of the message, the 3GPP AAA Server shall update the related service information and/or status of the subscriber and remove any filtering policy related to the disconnected tunnel from WAG if necessary.

#### 8.3.4.2	3GPP AAA Proxy Behaviour

On receipt of the ASA message with Diameter Result Code set to DIAMETER_SUCCESS, the 3GPP AAA Proxy shall signal to the WAG to initiate procedures to remove any filtering policy associated with that user's session.

### 8.3.5	Access and Service Authorization information Update Procedure

This procedure is used between the 3GPP AAA Server and the PDG and is used to modify the authorization parameters provided to the PDG. This may happen due to a modification of WLAN subscriber profile in the HSS

This procedure is performed in two steps:

-	The 3GPP AAA server issues an unsolicited re-authentication and/or re-authorization request towards the PDG.  Upon receipt of such a request, the PDG shall respond to the request and indicate the disposition of the request. This procedure is mapped to the Diameter command codes Re-Auth-Request and Re-Auth-Answer specified in RFC 3588 [7].  Information element content for these messages are shown in tables 8.3.5.1 and 8.3.x.2.

-	Receiving the re-authentication and re-authorization request, the PDG shall initiate a re-authentication and/or re-authorization procedure towards the WLAN-UE and shall then invoke the authentication and/or authorization procedure as described in the sections 8.2.1 and 8.3.2. Information element content for these messages are shown in tables 8.3.2.1 and 8.3.2.2.

- Table 8.3.5.1: Access and Service Authorization Information Update request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| User Identity | User-Name | M | This information element contains the identity of the user. |
| Re-Auth Request Type | Re-Auth–Request-Type | M | Defines whether the user is to be re-authenticated only, re-authorized only or both. If it indicates AUTHENTICATE_ONLY, the PDG shall just perform an authentication procedure as described in section 8.2.1. If it indicates AUTHORIZE_ONLY, the PDG shall just perform an autorization procedure as described in section 8.2.2. If it indicates AUTHORIZE_AUTHENTICATE, the PDG shall perform both authentication and authorization. |
| Routing Information | Destination-Host | M | This information element is obtained from the Origin-Host AVP, which was included in a previous command received from the PDG. |

- Table 8.3.5.2: Access and Service Authorization information Update response

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Result | Result-Code / Experimental-Result | M | Result of the operation. Result-Code AVP shall be used for errors defined in the Diameter Base Protocol. Experimental-Result AVP shall be used for Wm errors. This is a grouped AVP which contains the 3GPP Vendor ID in the Vendor-Id AVP, and the error code in the Experimental-Result-Code AVP. |
| User Identity | User-Name | M | This information element contains the identity of the user. |

##### 8.3.5.1	Detailed behaviour

The 3GPP AAA server shall make use of this procedure to indicate and update relevant service authorization information in the PDG.

The PDG shall overwrite, for the subscriber identity indicated in the request, current information with the information received from the 3GPP AAA server. A deactivation of service may be initiated if the subscriber lost the authorization of the activated service.

## 8.4 	Information Element Contents

### 8.4.1 	Authentication Request/Answer Messages

ABNF for the Wm Authentication Request and Authentication Answer are given below:

<Diameter-EAP-Request> ::= < Diameter Header: 268, REQ, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Request-Type }

{ EAP-Payload }

[ Destination-Host ]

[ User-Name ]

[ Visited-Network-Identifier ]

[ NAS-IP-Address ]

[ NAS-IPv6-Address ]

[Calling Station-ID]

* [ Proxy-Info ]

* [ Route-Record ]

* [ AVP ]

For the DEA, the following are necessary:

<Diameter-EAP-Answer> ::= < Diameter Header: 268, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Request-Type }

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ Master-Session-Key ]

[ EAP-Payload ]

* [ Proxy-Info ]

* [ AVP ]

### 8.4.2 	Authorization Procedures

The authorization request and response messages are mapped onto the NASREQ AAR/AAA messages. The ABNF are indicated below:

<AA-Request> ::= < Diameter Header: 265, REQ, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Request-Type }

[ Destination-Host ]

[ Session-Request-Type ]

[ Visited-Network-Identifier ]

[ APN-ID]

[ Routing-Policy]

[ NAS-Identifier ]

[ NAS-IP-Address ]

[ NAS-IPv6-Address ]

[ NAS-Port ]

[ NAS-Port-Id ]

[ NAS-Port-Type ]

[ Origin-State-Id ]

[ Port-Limit ]

[ User-Name ]

[ User-Password ]

[ Service-Type ]

[ State ]

[ Authorization-Lifetime ]

[ Auth-Grace-Period ]

[ Auth-Session-State ]

[ Callback-Number ]

[ Called-Station-Id ]

[ Calling-Station-Id ]

[ Originating-Line-Info ]

[ Connect-Info ]

[ CHAP-Auth ]

[ CHAP-Challenge ]

* [ Framed-Compression ]

[ Framed-Interface-Id ]

[ Framed-IP-Address ]

[ Framed-IP-Netmask ]

[ Framed-MTU ]

[ Framed-Protocol ]

[ ARAP-Password ]

[ ARAP-Security ]

* [ ARAP-Security-Data ]

* [ Login-IP-Host ]

* [ Login-IPv6-Host ]

[ Login-LAT-Group ]

[ Login-LAT-Node ]

[ Login-LAT-Port ]

[ Login-LAT-Service ]

* [ Tunneling ]

* [ Proxy-Info ]

* [ Route-Record ]

* [ AVP ]

The ABNF for the AAA is as follows:

<AA-Answer> ::= < Diameter Header: 265, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Request-Type }

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ Subscription-ID-AVP]

[ Max-Subscribed-Bandwidth ]

[ Framed-IP-Address ]

[ Framed-IP-Prefix ]

[ Charging-Data ]

[ Service-Type ]

* [ Class ]

* [ Configuration-Token ]

[ Acct-Interim-Interval ]

[ Error-Message ]

[ Error-Reporting-Host ]

* [ Failed-AVP ]

[ Idle-Timeout ]

[ Authorization-Lifetime ]

[ Auth-Grace-Period ]

[ Auth-Session-State ]

[ Re-Auth-Request-Type ]

[ Session-Timeout ]

[ State ]

* [ Reply-Message ]

[ Origin-State-Id ]

* [ Filter-Id ]

[ Password-Retry ]

[ Port-Limit ]

[ User-Name ]

[ Prompt ]

[ ARAP-Challenge-Response ]

[ ARAP-Features ]

[ ARAP-Security ]

* [ ARAP-Security-Data ]

[ ARAP-Zone-Access ]

[ Callback-Id ]

[ Callback-Number ]

[ Framed-Appletalk-Link ]

* [ Framed-Appletalk-Network ]

[ Framed-Appletalk-Zone ]

* [ Framed-Compression ]

[ Framed-Interface-Id ]

[ Framed-IP-Address ]

* [ Framed-IPv6-Prefix ]

[ Framed-IPv6-Pool ]

* [ Framed-IPv6-Route ]

[ Framed-IP-Netmask ]

* [ Framed-Route ]

[ Framed-Pool ]

[ Framed-IPX-Network ]

[ Framed-MTU ]

[ Framed-Protocol ]

[ Framed-Routing ]

* [ Login-IP-Host ]

* [ Login-IPv6-Host ]

[ Login-LAT-Group ]

[ Login-LAT-Node ]

[ Login-LAT-Port ]

[ Login-LAT-Service ]

[ Login-Service ]

[ Login-TCP-Port ]

* [ NAS-Filter-Rule ]

* [ QoS-Filter-Rule ]

* [ Tunneling ]

* [ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

* [ Proxy-Info ]

* [ AVP ]

### 8.4.3 	PDG Initiated Session Termination Procedure

This procedure is mapped onto the STR/STA procedures. The ABNF are as follows:

<STR> ::= < Diameter Header: 275, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Application-Id }

{ Termination-Cause }

[ User-Name ]

[ APN-Id ]

[ Destination-Host ]

* [ Class ]

[ Origin-State-Id ]

* [ Proxy-Info ]

* [ Route-Record ]

* [ AVP ]

For the response:

<STA>  ::= < Diameter Header: 275, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

* [ Class ]

[ Error-Message ]

[ Error-Reporting-Host ]

* [ Failed-AVP ]

[ Origin-State-Id ]

* [ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

* [ Proxy-Info ]

* [ AVP ]

### 8.4.4 	3GPP AAA Server Initiated Tunnel Disconnect Procedure

ABNF for the 3GPP AAA Server Initiated Tunnel Disconnect Procedure are mapped onto the ASR and ASA commands are as follows:

<ASR>  ::= < Diameter Header: 274, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Destination-Host }

{ Auth-Application-Id }

[ User-Name ]

[ APN-Id ]

[ Origin-State-Id ]

* [ Proxy-Info ]

* [ Route-Record ]

*[ AVP ]

<ASA>  ::= < Diameter Header: 274, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ Origin-State-Id ]

[ Error-Message ]

[ Error-Reporting-Host ]

* [ Failed-AVP ]

* [ Redirected-Host ]

[ Redirected-Host-Usage ]

[ Redirected-Max-Cache-Time ]

* [ Proxy-Info ]

* [ AVP ]

### 8.4.5	Access and Service Authorization Information Update Procedure

ABNF for the RAR/RAA commands on the Wm interface are identical to those described in section 4.4.2.4.

ABNF for the AAR/AAA commands on the Wm interface are identical to those described in section 8.4.2

## >>>>>>>>>>> End of first modified section <<<<<<<<<<<