3GPP TSG CT4 Meeting #75	C4-171140

Dubrovnik, Croatia; 13th – 17th February 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.329 | CR | 0241 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Notif-Eff feature description correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Hewlett Packard Enterprise |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  |  | Date: |  | 2017-03-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | When Notif-Eff feature is supported, combination of multiple DataReferences into a single XML applies as well to PNR message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include this correction in Notif-Eff feature description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential IOT problems |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.1	Version Control

The version control mechanisms specified in 3GPP TS 29.229 [6] clauses 7.1, 7.2 and 7.3 apply to this specification.

The following table of features shall apply to the Sh interface.

Table 7.1.1: Features of feature list 1 used in Sh

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | Notif-Eff | M | This feature is applicable to the UDR/UDA, SNR/SNA and PNR/PNA command pairs. It requires support in both the HSS and the AS. If multiple subscriptions to notifications are associated with a Public User Identity, the HSS may combine the notifications for multiple Data References and/or Service Indications and/or Identity Sets into a single Push-Notification-Request message. The User-Data-Request / Answer and the Subscribe-Notifications-Request / Answer allow multiple Data References, Service Indications and Identity Sets. The User-Data-Answer and Push-Notification-Request allow combining multiple DataReference items resulting in the User Data AVP including a single XML document with the separable XML sections populated. |
| 1 | Update-Eff | O | This feature is applicable to the PUR/PUA commands. If both the HSS and the AS support this feature, a PUR command where the Data reference type is Repository Data, may contain an XML document with several Repository Data instances. |
| 2 | Update-Eff-Enhance | O | This feature is an enhancement of the Update-Eff feature and requires Update-Eff is supported. It is applicable to the PUR/PUA commands. If both the HSS and the AS support this feature, a PUR command may contain an XML document with several Data instances. |
| 3 | Additional-MSISDN | O | This feature is applicable to UDR/UDA commands. It requires support in both HSS and AS. If enabled, it extends the information returned with DataReference=17. Instead of returning only MSISDN, it includes ExtendedMSISDN. See 3GPP TS 29.328 [1] for more information. |
| Feature bit: The order number of the bit within the Supported-Features AVP, e.g. “1”. Feature: A short name that can be used to refer to the bit and to the feature, e.g. “MOM”. M/O: Defines if the implementation of the feature is mandatory (“M”) or optional (“O”). Description: A clear textual description of the feature. |  |  |  |

The following table shall apply to the Sh interface; the column Application identifier lists the used application identifiers on Sh and 3GPP.

Table 7.1.2: Application identifiers used in Sh

| Application identifier | First applied |
| --- | --- |
| 16777217 | 3GPP Rel-5 |

* * * End of changes * * * *