3GPP TSG-CT4 Meeting #86	C4-187092

# Vilnius, Lithuania, 15th Oct 2018 - 19th Oct 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0012 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the entity upon which content encoding is performed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 29.500 stipulates that content coding is applied to the response body whereas RFC 7231 3.1.2.2. Content-Encoding stipulates it is applied to the representation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the entity upon which the content coding is performed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 29.500 is not compliant with RFC 7231 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.2.2.2	Mandatory to support HTTP standard headers

The HTTP request standard headers and the HTTP response standard headers that shall be supported on SBI are defined in Table 5.2.2.2-1 and in Table 5.2.2.2-2 respectively. Mandatory to support HTTP standard headers does not mean all the HTTP requests and responses carry the identified request and response headers respectively. It only means it is mandatory to support the processing of the identified headers in request and response message.

Table 5.2.2.2-1: Mandatory to support HTTP request standard headers

| Name | Reference | Description |
| --- | --- | --- |
| Accept | IETF RFC 7231 [11] | This header is used to specify response media types that are acceptable. |
| Accept-Encoding | IETF RFC 7231 [11] | This header may be used to indicate what response content-encodings (e.g gzip) are acceptable in the response. |
| Content-Length | IETF RFC 7230 [12] | This header is used to provide the anticipated size, as a decimal number of octets, for a potential payload body. |
| Content-Type | IETF RFC 7231 [11] | This header is used to indicate the media type of the associated representation. |
| User-Agent | IETF RFC 7231 [11] | This header shall be mainly used to identify the NF type of the HTTP/2 client. The pattern of the content should start with the value of NF type (e.g. udm, see NOTE 1) and followed by a "-" and any other specific information if needed afterwards. |
| Cache-Control | IETF RFC 7234 [20] | This header may be used in some HTTP/2 requests to provide the HTTP cache-control directives that the client is willing to accept from the server. |
| If-Modified-Since | IETF RFC 7232 [24] | This header may be used in a conditional GET request, for server revalidation. This is used in conjunction with the Last-Modified server response header, to fetch content only if the content has been modified from the cached version. |
| If-None-Match | IETF RFC 7232 [24] | This header may be used in a conditional GET request. This is used in conjunction with the ETag server response header, to fetch content only if the tag value of the resource on the server differs from the tag value in the If-None-Match header. |
| If-Match | IETF RFC 7232 [24] | This header may be used in a conditional POST or PUT or DELETE or PATCH request. This is used in conjunction with the ETag server response header, to update / delete content only if the tag value of the resource on the server matches the tag value in the If-Match header. |
| Via | IETF RFC 7230 [12] | This header shall be inserted by HTTP proxies. |
| Authorization | IETF RFC 7235 [21] | This header shall be used if OAuth 2.0 based access authorization with "Client Credentials" grant type is used as specified in subclause 13.4.1 of 3GPP TS 33.501 [17], clause 7 of IETF RFC 6749 [22] and IETF RFC 6750 [23]. |
| NOTE 1: The value of NF type in the User-Agent header shall comply with the enumeration value of Table 6.1.6.3.3-1 in 3GPP TS 29.510 [8]. |  |  |

Table 5.2.2.2-2: Mandatory to support HTTP response standard headers

| Name | Reference | Description |
| --- | --- | --- |
| Content-Length | IETF RFC 7230 [12] | This header may be used to provide the anticipated size, as a decimal number of octets, for a potential payload body. |
| Content-Type | IETF RFC 7231 [11] | This header shall be used to indicate the media type of the associated representation. |
| Location | IETF RFC 7231 [11] | This header may be used in some responses to refer to a specific resource in relation to the response. |
| Retry-After | IETF RFC 7231 [11] | This header may be used in some responses to indicate how long the user agent ought to wait before making a follow-up request. |
| Content-Encoding | IETF RFC 7231 [11] | This header may be used in some responses to indicate to the HTTP/2 client the content encodings (e.g gzip) applied to the representation beyond those inherent in the media type. |
| Cache-Control | IETF RFC 7234 [20] | This header may be used in some responses (e.g. NRF responses to queries) to provide HTTP response cache control directives. The cache directives "no-cache", "no-store", "max-age" and "must-revalidate" values shall be supported. |
| Age | IETF RFC 7234 [20] | This header may be inserted by HTTP proxies when returning a cached response. The "Age" header field conveys the sender's estimate of the amount of time since the response was generated or successfully validated at the origin server. The presence of an Age header field implies that the response was not generated or validated by the origin server for this request. |
| Last-Modified | IETF RFC 7232 [24] | This header may be sent to allow for conditional GET with the If-Modified-Since header. |
| ETag | IETF RFC 7232 [24] | This header may be sent to allow for conditional GET with the If-If-None-Match header or a conditional POST / PUT / PATCH / DELETE with the If-Match header. |
| Via | IETF RFC 7230 [12] | This header shall be inserted by HTTP proxies. |
| Allow | IETF RFC 7231 [11] | This header field shall be used to indicate methods supported by the target resource. |
| WWW-Authenticate | IETF RFC 7235 [21] | This header field shall be included when an NF service producer rejects a request with a "401 Unauthorized" status code (e.g when a request is sent without an OAuth 2.0 access token or with an invalid OAuth 2.0 access token). |

* * * End of Changes * * * *