3GPP TSG CT WG4 Meeting #87	C4-188034

West Palm Beach, Florida, 26th – 30th November 2018

Source:	Cisco Systems, SoftBank Corp.

Title:	Submode 4 of SRv6 Enhanced Mode

Spec:	3GPP TR 29.892

Agenda item:	6.1.1

Document for:	Approval

1. Introduction

This document introduces a submode of SRv6 Enhanced mode.

2. Reason for Change

This section introduces a new submode for SRv6 Enhanced mode and also discusses about the system impact of this submode.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.892 version 0.3.0.

* * * First  Change * * * *

6.2.x.y.z.4 Submode 4

In this submode the SMF signal the Apply Action to the headend UPF and the headend UPF encodes Apply Action in the SID before forwarding it to next UPF on N9 interface. The section SRv6 SID encoding (section 6.2.2.3) illustrates an example of SID encoding with Apply Action. Following SID encoding structure is used for this submode

Figure 6.2.x.y.z.4-1 SID Encoding for Enhanced submode4

The Apply Action field can be encoded as follows

| Bit 7 Spare | Bit 6 Spare | Bit 5 Spare | Bit 4 DUPL | Bit 3 NOCP | Bit 2 BUFF | Bit 1 FORW | Bit 0 DROP |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 6.2.x.y.z.4-2 Apply Action Encoding Details

| Bit 0 | DROP (Drop): when set to 1, this indicates a request to drop the packets |
| --- | --- |
| Bit 1 | FORW (Forward): when set to 1, this indicates a request to forward the packets. |
| Bit 2 | BUFF (Buffer): when set to 1, this indicates a request to buffer the packets. |
| Bit 3 | NOCP (Notify the CP function): when set to 1, this indicates a request to notify the CP function about the arrival of a first downlink packet being buffered. |
| Bit 4 | DUPL (Duplicate): when set to 1, this indicates a request to duplicate the packets. |
| Bit 5 to Bit 7 | Spare for future use and set to 0. |

One and only one of the DROP, FORW and BUFF flags shall be set to 1.

The NOCP flag may only be set if the BUFF flag is set.

The DUPL flag may be set with any of the DROP, FORW, BUFF and NOCP flags.

Editor's Note:	The exact call flow of packet forwarding, drop, buffering, duplicate etc is FFS.

This submode does require changes in the 3GPP 5GC system architecture, as explained in 23.501 [7]. The SMF and UPF functions requires changes to support this submode.This submode offers benefits of SRv6 advanced feature.

* * * End of Change * * * *