3GPP TSG CT WG4 Meeting #89	C4-190144

Montreal, CANADA; 25th Feb – 01st Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.002 | CR | 1247 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HLR restart impact to SMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2019-01-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the HLR restarts, it may have lost (parts of) its Message Waiting Data. This results in scenarios where SMS Service Centres are not alerted when the HLR detects that an absent subscriber becomes available, and the SMS Service Centre keeps waiting for an alert that will never be received. It is therefore proposed as an option to allow HLRs after restart to alert all Service Centres form a preconfigured list, so that the Service Centres become aware of the situation and retry SMS delivery based on e.g. an internal schedule for the impacted waiting short messages, resulting in either a successful delivery or a recovery of the MWD in the HLR. Impacted short messages are identified based on a Reset-Id. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Reset-Id List to MAP-Alert-Service-Centre Add Reset-Id to MAP-Inform-Service-Centre and to AbsentSubscriberSM-Param Move ASN.1 definition of Reset-Id from MS-DataTypes to CommonDataTypes Enhance HLR fault recovery and Short Message Alert procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SMS Service Centres may never receive an alert for waiting short messages and short messages are not delivered although the subscriber may have become reachable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.5, 12.6, 17.7.1, 17.7.6, 17.7.7, 17.7.8, 19.3.2, 23.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 12.5	MAP-ALERT-SERVICE-CENTRE service

### 12.5.1	Definition

This service is used between the HLR and the interworking MSC. The HLR initiates this service, if the HLR detects that a subscriber, whose MSISDN is in the Message Waiting Data file, is active or the MS has memory available.

This service is also used between an MME (via an IWF), SGSN or an MSC using Deployment Option 2 (see subclause 8.2.4a.1 of 3GPP TS 23.272 [143] and 3GPP TS 23.040 [6]) and the SMS-GMSC (possibly via an SMS Router), to indicate that a MS, for which one or more MT SMS have been requested to be retransmitted at a later time, is now available for MT SMS delivery or has moved under the coverage of another MME, SGSN or MSC. This procedure is used according to the call flows described in subclause 10.1 of 3GPP TS 23.040 [26].

This service may also be used between HLR and interworking MSC after HLR restart when the HLR has lost its Message Waiting Data.

The MAP-ALERT-SERVICE-CENTRE service is a confirmed service using the primitives from table 12.5/1.

### 12.5.2	Service primitives

Table 12.5/1: MAP-ALERT-SERVICE-CENTRE

| Parameter name | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Invoke Id | M | M(=) | M(=) | M(=) |
| MSIsdn-Alert | M | M(=) |  |  |
| IMSI | C | C(=) |  |  |
| Correlation ID | C | C(=) |  |  |
| Service Centre Address | M | M(=) |  |  |
| Maximum UE Availability Time | C | C(=) |  |  |
| SMS-GMSC Alert Event | C | C(=) |  |  |
| SMS-GMSC Diameter Address | C | C(=) |  |  |
| New SGSN Number | C | C(=) |  |  |
| New SGSN Diameter Address | C | C(=) |  |  |
| New MME Number | C | C(=) |  |  |
| New MME Diameter Address | C | C(=) |  |  |
| New MSC Number | C | C(=) |  |  |
| Reset-ID LIST | C | C(=) |  |  |
| User error |  |  | C | C(=) |
| Provider error |  |  |  | O |

### 12.5.3	Parameter use

Invoke id

See definition in clause 7.6.1.

MSIsdn-Alert

See definition in clause 7.6.2.

When the service is used between the HLR and the SMS-IWMSC, the provided MSISDN shall be the one which is stored in the Message Waiting Data file. If no MSISDN is available, the dummy MSISDN value (see clause 3 of 3GPP TS 23.003 [17]) shall be sent and an IMSI or Correlation ID (SIP-URI-B) shall be present.

When the service is used between the HLR and the SMS-IWMSC after HLR restart due to a loss of Message Waiting Data, the dummy MSISDN value (see clause 3 of 3GPP TS 23.003 [17]) shall be sent and a Reset-ID LIST shall be present.

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, the dummy MSISDN value (see clause 3 of 3GPP TS 23.003 [17]) shall be sent and an IMSI shall be present.

IMSI

When the service is used between the HLR and the SMS-IWMSC, the provided IMSI shall be the identifier which is stored in the Message Waiting Data file if no MSISDN is available in the context of T4 device triggering (see 3GPP TS 23.682 [148]).

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, this information element shall contain the IMSI in the request sent from the MME (via an IWF), SGSN or MSC, or the User Identifier Alert previously sent in the MT Forward Short Message response, when the request is sent from the SMS Router to the SMS-GMSC.

Correlation ID

When the service is used between the HLR and the SMS-IWMSC, the provided SIP-URI-B within the Correlation ID parameter shall be the identifier which is stored in the Message Waiting Data file if no MSISDN is available in a retry context of SMS for IMS UE to IMS UE without MSISDN (see 3GPP TS 23.204 [134]). HLR-ID and SIP-URI-A shall be absent.

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, this information element shall not be included.

Service Centre Address

See definition in clause 7.6.2.

When the service is used between the HLR and the SMS-IWMSC, this information element shall contain the E.164 number of the Service Center.

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, this information element shall contain the E.164 number of the SMS-GMSC (or SMS Router) previously received in the SMS-GMSC Address IE in the MT Forward Short Message Request.

Maximum UE Availability Time

See definition in clause 7.6.8.

When the service is used between the HLR and the SMS-IWMSC, this information element shall be included, if available.

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, this information element shall not be included.

SMS-GMSC Alert Event

See definition in clause 7.6.8.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, this information element shall either indicate that the MS is now available for MT SMS or that the MS has moved under the coverage of another MME, SGSN or MSC.

New SGSN Number

See definition in clause 7.6.8.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MME (via an IWF) or SGSN and the SMS-GMSC, this information element may be included if the SMS-GMSC Alert Event indicates that the MS has moved under the coverage of another SGSN. When present, it shall contain the E.164 number of the new SGSN serving the MS.

New MME Number

See definition in clause 7.6.8.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MME (via an IWF) or SGSN and the SMS-GMSC, this information element may be included if the SMS-GMSC Alert Event indicates that the MS has moved under the coverage of another MME. When present, it shall contain the E.164 number of the new MME serving the MS.

New SGSN Diameter Address

See definition in clause 7.6.8.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MME (via an IWF) or SGSN and the SMS-GMSC, this information element shall be included if available and if the SMS-GMSC Alert Event indicates that the MS has moved under the coverage of another SGSN. When present, it shall contain the Diameter Identity of the new SGSN serving the MS.

New MME Diameter Address

See definition in clause 7.6.8.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MME (via an IWF) or SGSN and the SMS-GMSC, this information element shall be included if available and if the SMS-GMSC Alert Event indicates that the MS has moved under the coverage of another MME. When present, it shall contain the Diameter Identity of the new MME serving the MS.

SMS-GMSC Diameter Address

See definition in clause 7.6.2.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MME (via an IWF), SGSN or MSC and the SMS-GMSC, this information element shall contain, if available, the Diameter Identity of the SMS-GMSC (or SMS Router) previously received in the SMS-GMSC Diameter Address IE in the MT Forward Short Message Request.

New MSC Number

See definition in clause 7.6.8.33.

When the service is used between the HLR and the SMS-IWMSC, this information element shall not be included.

When the service is used between an MSC and the SMS-GMSC, this information element may be included if the SMS-GMSC Alert Event indicates that the MS has moved under the coverage of another MSC. When present, it shall contain the E.164 number of the new MSC serving the MS.

Reset-ID LIST

The Reset-ID LIST is a list of Reset-IDs. It shall be present if the service is used by the HLR after HLR restart, and shall be used to identify waiting short messages for subscribers potentialy affected by the restart (loss of MWD).

User error

The following errors defined in clause 7.6.1 may be used, depending on the nature of the fault:

-	System Failure;

-	Unexpected Data Value;

-	Data missing.

Provider error

For definition of provider errors see clause 7.6.1.

* * * Next Change * * * *

## 12.6	MAP-INFORM-SERVICE-CENTRE service

### 12.6.1	Definition

This service is used between the HLR and the gateway MSC (transiting an SMS Router, if present) to inform the Service Centre which MSISDN number is stored in the Message Waiting Data file. If the stored MSISDN number is not the same as the one received from the gateway MSC in the MAP-SEND-ROUTING-INFO-FOR-SM service primitive the stored MSISDN number is included in the message.

The service may also be used by the HLR to inform the Service Centre about the Reset-Id associated to the resource in the HLR (e.g. Message Waiting Data List).

Additionally the status of MCEF, MNRF and MNRG flags and the inclusion of the particular Service Centre address in the Message Waiting Data list is informed to the gateway MSC when appropriate.

If the HLR has stored a single MNRR, the value is included in the Absent Subscriber Diagnostic SM parameter.

If the HLR has stored a second MNRR, the value of the MNRR for the MSC is included in the Absent Subscriber Diagnostic SM parameter and the value of the MNRR for the SGSN is included in the Additional Absent Subscriber Diagnostic SM parameter.

The MAP-INFORM-SERVICE-CENTRE service is a non-confirmed service using the primitives from table 12.6/1.

### 12.6.2	Service primitives

Table 12.6/1: MAP-INFORM-SERVICE-CENTRE

| Parameter name | Request | Indication |
| --- | --- | --- |
| Invoke Id | M | M(=) |
| MSIsdn-Alert | C | C(=) |
| MWD Status | C | C(=) |
| Absent Subscriber Diagnostic SM | C | C(=) |
| Additional Absent Subscriber Diagnostic SM | C | C(=) |
| Reset-Id | U | C(=) |

### 12.6.3	Parameter use

Invoke id

See definition in clause 7.6.1.

MSIsdn-Alert

See definition in clause 7.6.2. This parameter refers to the MSISDN stored in a Message Waiting Data file in the HLR.

MWD Status

See definition in clause 7.6.8. This parameter indicates the status of the MCEF, MNRF and MNRG flags and the status of the particular SC address presence in the Message Waiting Data list.

Absent Subscriber Diagnostic SM

See definition in clause 7.6.8.

Additional Absent Subscriber Diagnostic SM

See definition in clause 7.6.8.

Reset-Id

The Reset-Id identifies a resource in the HLR (e.g.Message Waiting Data list) that may get lost due to restart.

* * * Next Change * * * *

### 17.7.1	Mobile Service data types

.$MAP-MS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version18 (18)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

*************text not shown for clarity************

AddressString,

ISDN-AddressString,

ISDN-SubaddressString,

FTN-AddressString,

AccessNetworkSignalInfo,

IMSI,

IMEI,

TMSI,

HLR-List,

LMSI,

Identity,

GlobalCellId,

CellGlobalIdOrServiceAreaIdOrLAI,

Ext-BasicServiceCode,

NAEA-PreferredCI,

EMLPP-Info,

MC-SS-Info,

SubscriberIdentity,

AgeOfLocationInformation,

LCSClientExternalID,

LCSClientInternalID,

Ext-SS-Status,

LCSServiceTypeID,

ASCI-CallReference,

TBCD-STRING,

LAIFixedLength,

PLMN-Id,

EMLPP-Priority,

GSN-Address,

DiameterIdentity,

Time,

E-UTRAN-CGI,

NR-CGI,

TA-Id,

RAIdentity,

NetworkNodeDiameterAddress,

Reset-Id

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

*************text not shown for clarity************

*************text not shown for clarity************

* * * Next Change * * * *

### 17.7.6	Short message data types

.$MAP-SM-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SM-DataTypes (16) version18 (18)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

*************text not shown for clarity************

IMPORTS

AddressString,

ISDN-AddressString,

SignalInfo,

IMSI,

LMSI,

ASCI-CallReference,

Time,

NetworkNodeDiameterAddress,

HLR-Id

Reset-Id

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

*************text not shown for clarity************

InformServiceCentreArg ::= SEQUENCE {

storedMSISDN	ISDN-AddressString	OPTIONAL,

mw-Status	MW-Status	OPTIONAL,

extensionContainer	ExtensionContainer	OPTIONAL,

... ,

absentSubscriberDiagnosticSM	AbsentSubscriberDiagnosticSM	OPTIONAL,

additionalAbsentSubscriberDiagnosticSM	[0]	AbsentSubscriberDiagnosticSM	OPTIONAL,

reset-Id		[1] Reset-Id	OPTIONAL }

-- additionalAbsentSubscriberDiagnosticSM may be present only if

-- absentSubscriberDiagnosticSM is present.

-- if included, additionalAbsentSubscriberDiagnosticSM is for GPRS and

-- absentSubscriberDiagnosticSM is for non-GPRS

*************text not shown for clarity************

* * * Next Change * * * *

### 17.7.7	Error data types

.$MAP-ER-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-ER-DataTypes (17) version18 (18)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

*************text not shown for clarity************

IMPORTS

SS-Status

FROM MAP-SS-DataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-SS-DataTypes (14) version18 (18)}

SignalInfo,

BasicServiceCode,

NetworkResource,

AdditionalNetworkResource,

IMSI,

Time,

Reset-Id

FROM MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

*************text not shown for clarity************

AbsentSubscriberSM-Param ::= SEQUENCE {

absentSubscriberDiagnosticSM	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- AbsentSubscriberDiagnosticSM can be either for non-GPRS

-- or for GPRS

extensionContainer	ExtensionContainer	OPTIONAL,

...,

additionalAbsentSubscriberDiagnosticSM  	[0] 	AbsentSubscriberDiagnosticSM	OPTIONAL,

-- if received, additionalAbsentSubscriberDiagnosticSM

-- is for GPRS and absentSubscriberDiagnosticSM is

-- for non-GPRS

imsi		[1] IMSI	OPTIONAL,

-- when sent from HLR to IP-SM-GW, IMSI shall be present if UNRI is not set

-- to indicate that the absent condition is met for CS and PS but not for IMS.

requestedRetransmissionTime	[2] Time	OPTIONAL,

userIdentifierAlert	[3] IMSI	OPTIONAL,

reset-Id		[4] Reset-Id	OPTIONAL }

*************text not shown for clarity************

* * * Next Change * * * *

### 17.7.8	Common data types

.$MAP-CommonDataTypes {

itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS

-- general data types and values

AddressString,

ISDN-AddressString,

maxISDN-AddressLength,

FTN-AddressString,

ISDN-SubaddressString,

ExternalSignalInfo,

Ext-ExternalSignalInfo,

AccessNetworkSignalInfo,

SignalInfo,

maxSignalInfoLength,

AlertingPattern,

TBCD-STRING,

DiameterIdentity,

Time,

HLR-Id,

-- data types for numbering and identification

IMSI,

TMSI,

Identity,

SubscriberId,

IMEI,

HLR-List,

LMSI,

GlobalCellId,

NetworkResource,

AdditionalNetworkResource,

NAEA-PreferredCI,

NAEA-CIC,

ASCI-CallReference,

SubscriberIdentity,

PLMN-Id,

E-UTRAN-CGI,

NR-CGI,

TA-Id,

RAIdentity,

NetworkNodeDiameterAddress,

Reset-Id

*************text not shown for clarity************

-- data types for numbering and identification

Reset-Id ::= OCTET STRING (SIZE (1..4))

-- Reset-Ids shall be unique within the HPLMN.

*************text not shown for clarity************

* * * Next Change * * * *

### 19.3.2	HLR fault recovery procedures

#### 19.3.2.1	General

For the HLR, periodic back-up of data to non-volatile memory is mandatory.

Data that have been changed after the last back-up and before the restart of the HLR cannot be recovered by reload from the non-volatile memory. Therefore, a restoration procedure is triggered for each IMSI record that has been affected by the HLR fault at the first authenticated radio contact with the MS concerned.

Similarly, Message Waiting Data that have been changed after the last back-up and before the restart of the HLR cannot be recovered by reload from non-volatile memory. Therefore, a restoration procedure may be triggered for each SMS Service Centre from a pre-configured list.

As an implementation option, a notification can be forwarded to the MS to alert the subscriber to check the parameters for supplementary services that allow subscriber controlled input (MAP_FORWARD_CHECK_SS_INDICATION service). If the VLR receives this notification from the HLR it shall forward the notification to the MS. If the Gs-interface is implemented the VLR shall not forward this notification.

A restoration procedure may also be triggered for IMSI records that shares subscription data with other IMSI records when the shared subscription data is modified, added or deleted. This option presumes the support of Reset-IDs.

The message flow for HLR restoration for a non-GPRS subscriber is shown in figure 19.3.2/1.

The message flow for HLR restoration for a GPRS subscriber is shown in figure 19.3.2/2.

The message flow for HLR restoration for SMS is shown in figure 23.4/3a.

1)	MAP_RESET_req/ind

2)	MAP_PROCESS_ACCESS_REQUEST_req/ind

3)	MAP_UPDATE_LOCATION_req/ind

4)	MAP_ACTIVATE_TRACE_MODE_req/ind (Note 1, Note 2)

5)	MAP_ACTIVATE_TRACE_MODE_rsp/cnf (Note 1, Note 2)

6)	MAP_INSERT_SUBSCRIBER_DATA_req/ind

7)	MAP_INSERT_SUBSCRIBER_DATA_rsp/cnf

8)	MAP_UPDATE_LOCATION_rsp/cnf

9)	MAP_FORWARD_CHECK_SS_INDICATION_req/ind (Note 1)

10)	MAP_FORWARD_CHECK_SS_INDICATION_req/ind (Note 1)

NOTE 1:	Services printed in italics are optional.

NOTE 2:	If subscriber tracing is active in the HLR.

Steps 2 to 10 may be skipped if the procedure is used to update shared subscription data.

Figure 19.3.2/1: Message flow for HLR restoration (non-GPRS)

1)	MAP_RESET_req/ind

2)	MAP_UPDATE_GPRS_LOCATION_req/ind

3)	MAP_ACTIVATE_TRACE_MODE_req/ind (Note 1, Note 2)

4)	MAP_ACTIVATE_TRACE_MODE_rsp/cnf (Note 1, Note 2)

5)	MAP_INSERT_SUBSCRIBER_DATA_req/ind

6)	MAP_INSERT_SUBSCRIBER_DATA_rsp/cnf

7)	MAP_UPDATE_GPRS_LOCATION_rsp/cnf

NOTE 1:	Services printed in italics are optional.

NOTE 2:	If subscriber tracing is active in the HLR.

Steps 2 to 7 may be skipped if the procedure is used to update shared subscription data.

Figure 19.3.2/2: Message flow for HLR restoration (GPRS)

#### 19.3.2.2	Procedure in the HLR

The MAP process in the HLR to notify the relevant serving nodes that the HLR has restarted is shown in figure 19.3.2/3.

The SGSN address list includes one instance of the address of each SGSN in which (according to the HLR data retrieved from the non-volatile memory) there is at least one subscriber registered who is affected by the HLR restart.

The VLR address list includes one instance of the address of each VLR in which (according to the HLR data retrieved from the non-volatile memory) there is at least one subscriber registered who is affected by the HLR restart.

The IWMSC address list may be preconfigured in the HLR.

The MAP process in the HLR to notify a VLR that the HLR has restarted is shown in figure 19.3.2/4. The MAP process invokes a macro not defined in this clause; the definition of this macro can be found as follows:

Receive_Open_Cnf			see subclause 25.1.2.

The MAP process in the HLR to notify an SGSN that the HLR has restarted is shown in figure 19.3.2/5. The MAP process invokes a macro not defined in this clause; the definition of this macro can be found as follows:

Receive_Open_Cnf			see subclause 25.1.2.

#### 19.3.2.3	Procedure in the VLR

The MAP process in the VLR to handle a notification that an HLR has restarted is shown in figure 19.3.2/6. The MAP process invokes a macro not defined in this clause; the definition of this macro can be found as follows:

Receive_Open_Ind			see subclause 25.1.1.

When Reset-IDs are not supported or not present in the MAP_RESET indication, the VLR uses the HLR number or the HLR identity list included in the MAP_RESET indication to identify the IMSI records which are affected by the HLR restart. 
When Reset-IDs are supported and present in the MAP_RESET indication, the VLR uses the Reset-IDs included in the MAP_RESET indication to identify the affected IMSI records.

The task "Update Data" includes any required action to let the update come into effect.
If the update of shared subscription data requires only local updates in the VLR (i.e., the update of the profile does not imply to initiate any signalling interaction towards other network nodes), the updates should be performed immediately (e.g. deleting an Operator Determined Barring).
If the update of shared subscription data implies initiating a signalling interaction towards other nodes, the signalling towards other nodes should be deferred to the next authenticated radio contact with that UE.

NOTE:	The rational for the recommendation to defer signalling towards other nodes until the next authenticated radio contact is to consider impacts to the network only when the updates are required, and to spread the signalling towards other nodes over some time, based on user’s activity.

To avoid high processing/signalling load resulting from shared subscription data update, processing/signalling actions resulting from data updates in the VLR may take a maximum operator configuration-depending time.

#### 19.3.2.4	Procedure in the SGSN

The MAP process in the SGSN to handle a notification that an HLR has restarted is shown in figure 19.3.2/7. The MAP process invokes a macro not defined in this clause; the definition of this macro can be found as follows:

Receive_Open_Ind			see subclause 25.1.1.

When Reset-IDs are not supported or not present in the MAP_RESET indication, the SGSN uses the HLR number or the HLR identity list included in the MAP_RESET indication to identify the IMSI records which are affected by the HLR restart. 
When Reset-IDs are supported and present in the MAP_RESET indication, the SGSN uses the Reset-IDs included in the MAP_RESET indication to identify the affected IMSI records.
The task "Update Data" includes any required action to let the update come into effect.

If the update of shared subscription data requires only local updates in the SGSN (i.e., the update of the profile does not imply to initiate any signalling interaction towards other network nodes), the updates should be performed immediately (e.g. deleting an Operator Determined Barring).
If the update of shared subscription data implies initiating a signalling interaction towards other nodes, the signalling towards other nodes should be deferred to the next authenticated radio contact with that UE.

NOTE:	The rational for the recommendation to defer signalling towards other nodes until the next authenticated radio contact is to consider impacts to the network only when the updates are required, and to spread the signalling towards other nodes over some time, based on user’s activity.

To avoid high processing/signalling load resulting from shared subscription data update, processing/signalling actions resulting from data updates in the SGSN may take a maximum operator configuration-depending time.

Figure 19.3.2/3: Process Restart_HLR

Figure 19.3.2/4: Process Send_Reset_To_VLR_HLR

Figure 19.3.2/5: Process Send_Reset_To_SGSN_HLR

Figure 19.3.2/6: Process Receive_Reset_VLR

Figure 19.3.2/7: Process Receive_Reset_SGSN

* * * Next Change * * * *

## 23.4	The Short Message Alert procedure

The Short Message Alert procedure is used to alert the Service Centre when the mobile subscriber is active after a short message transfer has failed because the mobile subscriber is not reachable, or when the MS has indicated that it has memory capacity to accept a short message.

The Short Message Alert procedure may also be used by the HLR to indicate to the Service Centre that the HLR has lost its Message Waiting Data e.g. due to a restart.

The message flow for the Short Message Alert procedure for the case when the mobile subscriber was not reachable is shown in figure 23.4/1.

1)	CM Service Request (**), Page response or Location Updating (3GPP TS 24.008 [35]).

2)	MAP_PROCESS_ACCESS_REQUEST / MAP_UPDATE_LOCATION_AREA (**).

3)	MAP_READY_FOR_SM (Mobile Present) / MAP_UPDATE_LOCATION /
Supplementary Service Control Request (*).

4)	MAP_READY_FOR_SM_ACK (*).

5)	MAP_ALERT_SERVICE_CENTRE (notes 1 and 2).

6)	Alert Service Centre (3GPP TS 23.040).

7)	MAP_ALERT_SERVICE_CENTRE_ACK.

NOTE 1:	To all Service Centres in the Message Waiting List.

NOTE 2:	The HLR initiates the MAP_ALERT_SERVICE_CENTRE service only if the MS Memory Capacity Exceeded flag is clear.

(*)	For GPRS, messages 3) and 4) are sent/received by the SGSN.

(**)	These messages are not used by the SGSN.

Figure 23.4/1: Short message alert procedure (Mobile is present)

The message flow for the Short Message Alert procedure for the case where the MS indicates that it has memory capacity to accept one or more short messages is shown in figure 23.4/2.

1)	SM memory capacity available ( 3GPP TS 24.011 [37]).

2)	MAP_READY_FOR_SM (Memory Available) (*).

3)	MAP_READY_FOR_SM (Memory Available) (**).

4)	MAP_READY_FOR_SM_ACK (**).

5)	MAP_READY_FOR_SM_ACK (*).

6)	SM memory capacity available (Acknowledge) (3GPP TS 24.011 [37]).

7)	MAP_ALERT_SERVICE_CENTRE (note).

8)	Alert Service Centre (3GPP TS 23.040).

9)	MAP_ALERT_SERVICE_CENTRE_ACK.

NOTE:	To all Service Centres in the Message Waiting List.

(*)	Messages 2) and 5) are not used by the SGSN.

(**)	For GPRS, messages 3) and 4) are sent/received by the SGSN.

Figure 23.4/2: Short message alert procedure (MS memory capacity available)

In addition the following MAP services are used in the MS memory available case:

MAP_PROCESS_ACCESS_REQUEST	(see subclause 8.3); (*)

MAP_AUTHENTICATE	(see subclause 8.5); (*)

MAP_SET_CIPHERING_MODE	(see subclause 8.6); (*)

MAP_PROVIDE_IMSI	(see subclause 8.9); (*)

MAP_CHECK_IMEI	(see subclause 8.7);

MAP_FORWARD_NEW_TMSI	(see subclause 8.9); (*)

MAP_TRACE_SUBSCRIBER_ACTIVITY	(see subclause 9.1). (*)

(*)	These services are not used by the SGSN.

The Short Message Alert procedure when the MS indicates successful transfer after polling is shown in figure 23.4/3.

1)	MAP_REPORT_SM_DELIVERY_STATUS (Successful Transfer).

2)	MAP_REPORT_SM_DELIVERY_STATUS_ACK.

3)	MAP_ALERT_SERVICE_CENTRE (note).

4)	Alert Service Centre (3GPP TS 23.040).

5)	MAP_ALERT_SERVICE_CENTRE_ACK.

NOTE:	To all Service Centres in the Message Waiting List.

Figure 23.4/3: Short message alert procedure (Successful transfer after polling)

The Short Message Alert procedure when the HLR restarts is shown in figure 23.4/3a.

1)	MAP_ALERT_SERVICE_CENTRE (note).

2)	Alert Service Centre (3GPP TS 23.040).

3)	MAP_ALERT_SERVICE_CENTRE_ACK.

NOTE:	To all Service Centres in a preconfigured list.

Figure 23.4/3a: Short message alert procedure (HLR restart)

* * * Next Change * * * *

### 23.4.4	Procedure in the HLR

The process starts when the HLR receives a dialogue opening request using the application context mwdMngtContext. The MAP process invokes macros not defined in this clause; the definitions of these macros can be found as follows:

Receive_Open_Ind		see subclause 25.1.1;

Check_Indication		see subclause 25.2.1;

Alert_Service_Centre_HLR	see subclause 25.10.3.

Sheet 1: If the dialogue opening request is from an SGSN, version 2 and version 1 of the application context are not applicable.

The short message alert process in the HLR is shown in figure 23.4/7.

For HLR restart procedure see figure 19.3.2/3.

* * * End Of Change * * * *