3GPP TSG-CT4 Meeting #90	C4-191227

# Xi’an, China, 08th April 2019 - 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.334 | CR | 0126 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Iq interface enhancement to support DBI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Nomor Research GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E2E_DELAY-CT |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA4 introduced the feature Delay Budget Information (DBI) in TS 26.114. For the optional capability to support DBI the IMS-ALG and IMS-AGW need to support DBI signalling for voice and video media streams using RTCP feedback messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update subclause 5.16 SDP "a-line" information element to include "rtcp-fb" attribute with the "3gpp-delay-budget" feedback parameter. - Introduce new Signalling Object "DBI" applicable for the H.245 Local Descriptor and Remote Descriptor in subclause 5.17.1. - Update of the Configure AGW Connection Point Request and the AGW Connection Point Request Acknowledge to support DBI in subclause 5.17.2.3. - Update of the Reserve and Configure AGW Connection Point Request and the Reserve and Configure AGW Connection Point Request Acknowledge to support DBI in subclause 5.17.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delay Budget Information signalling not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.16, 5.17.1, 5.17.2.3, 5.17.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS, TS ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations defined in 3GPP TR 21.905 [1] apply, with the following additions. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ABNF	Augmented Backus-Naur Form

ATCF	Access Transfer Control Function

ATGW	Access Transfer Gateway

B-ALG	Bearer Level Application-Level Gateway

BFCP	Binary Floor Control Protocol

CCM	Codec Control Messages

CVO	Coordination of Video Orientation

DBI	Delay Budget Information

DSCP	Differentiated Service Code Point

e2ae	End-to-Access-Edge (security model)

ECN	Explicit Congestion Notification

eIMS-AGW	IMS Access Gateway enhanced for WebRTC

eP-CSCF	P-CSCF enhanced for WebRTC

FECC	Far End Camera Control

FIR	Full Intra Request

GTT	Global Text Telephony

ICE	Interactive Connectivity Establishment

IMS-AGW	IMS Access Gateway

IMS-ALG	IMS Application Level Gateway

IP	Internet Protocol

LD	Local Descriptor (H.248 protocol element)

MG	Media Gateway

MGC	Media Gateway Controller

MPS	Multimedia Priority Service

MSRP	Message Session Relay Protocol

MTSI	Multimedia Telephony Service for IMS

NA	Not Applicable

NAPT	Network Address and Port Translation

NAPT-PT	NAPT and Protocol Translation

NAT	Network Address Translation

RD	Remote Descriptor (H.248 protocol element)

ROI	Region of Interest

RTCP	RTP Control Protocol

SCTP	Stream Control Transport Protocol

SDP	Session Description Protocol

SDPCapNeg	SDP Capability Negotiation

SRVCC	Single Radio Voice Call Continuity

STUN	Session Traversal Utilities for NAT

TCP	Transmission Control Protocol

TLS	Transport Layer Security (protocol)

TMMBN	Temporary Maximum Media Stream Bit Rate Notification

TMMBR	Temporary Maximum Media Stream Bit Rate Request

ToS	Type-of-Service

TISPAN	Telecommunications and Internet converged Services and Protocols for Advanced Networking

WebRTC	Web Real Time Communication

WIC	WebRTC IMS Client

WWSF	WebRTC Web Server Function

* * * Next Change * * * *

## 5.16	Optional support of SDP and Annex C information elements

Specifies what SDP attributes and Annex C information elements may be supported.

Table 5.16.1: Optional Annex C and SDP information elements

| Information Element | Annex C Support | SDP Support |
| --- | --- | --- |
| a-line | "SDP_A " | 1) Application "RTCP transport address control": a) Default mode "without RTP/RTCP transport multiplexing": The attribute "a=rtcp" line may either contain (a=rtcp: <port>) or (a=rtcp: <port> <network type> <address type> <connection address>) when the "a=" line is used for RTCP transport port and optionally network address transmission (see IETF RFC 3605 [21]). . The MGC shall supply the "a=rtcp" line in the RD when non-default RTCP network address or transport port values are used by the peer media entity. "RTCP transport address control" should be supported by MG (NOTE 2). b) Optional extension mode "with RTP/RTCP transport multiplexing": The attribute "a=rtcp-mux" (see IETF RFC 5761 [59]) is used for indicating RTP/RTCP transport multiplexing. Tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5] define the appropriate RTCP port allocation rules. 2) Media related parameters in general: The "a=" line provides the complementary information for the "m=" line with regards to a specified media type/format (e.g. an optional SDP „a=ptime" line for a particular media format). For a dynamic RTP payload type, for each media information on the codec type shall be provided in a separate SDP "a=rtpmap"line and possibly additional SDP "a=fmtp"-line(s). 3) Application " Media interworking (transcoding)": See "a=" line specification in (2). Media interworking is limited to audio transcoding only (NOTE 1). 4) IMS media plane security related parameters: 4.1) SRTP-specific security parameters: The attribute "a=crypto" (see IETF RFC 4568 [29]) shall be provided for an m-line in the local and remote descriptor of an access network termination if the IMS-ALG wants that the corresponding media is encrypted, decrypted and/or integrity protected by the IMS-AGW (IMS end-to-access-edge media plane security). For each m-line, only a single "a=crypto" attribute shall be provisioned (i.e. only information related to a single crypto suite is provisioned to the IMS-AGW). The "a=crypto" attribute may contain several master keys. An IMS-AGW supporting end-to-access-edge media plane security shall support parameters within the "a=crypto" attribute in accordance with the profile in Annex of 3GPP TS 33.328 [34]. 4.2) (D)TLS-specific security parameters: The attribute(s) "a=fingerprint" (see IETF RFC 8122 [55]) shall be provided in accordance with ITU-T Recommendation H.248.90 [48] for an "m="-line in the local and remote descriptor of an access network termination if the IMS-ALG wants that the corresponding media is encrypted, decrypted and/or integrity protected by the IMS-AGW (IMS end-to-access-edge media plane security). 5) Coordination of Video Orientation The attribute "a=extmap" (see IETF RFC 5285 [41]) may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information, see also 3GPP TS 26.114 [26]. 6) Generic Image Attribute The attribute "a=imageattr" (see IETF RFC 6236 [42]) may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the generic image attributes, see also 3GPP TS 26.114 [26]. The local descriptor indicates the image sizes which the IMS-AGW supports in the receiving direction for the selected payload type and corresponds to the "recv" keyword (see IETF RFC 6236 [42]) in the "a=imageattr" that the IMS-ALG will send within the SDP body on the Mw/Mx interface. The remote descriptor indicates the image sizes which the IMS-AGW supports in the sending direction for the selected payload type and corresponds to the "send" keyword (see IETF RFC 6236 [42]) in the "a=imageattr" that the IMS-ALG will send within the SDP body on the Mw/Mx interface. 7) ICE support The attributes "a=candidate", "a=ice-pwd", and "a=ice-ufrag" (see IETF RFC 5245 [44]) may be provided for an SDP m-line in the local and remote descriptor if the IMS-AGW supports ICE, see also 3GPP TS 24.229 [45]. In the local descriptor, the IMS-ALG shall provide "a=ice-pwd", and "a=ice-ufrag" with wildcard sign "$" to request the allocation of a password and user name fragment, and the "a=candidate" of type "host" with the transport, port and priority parameters with wildcard sign "$" to request the allocation of a host candidate. The IMS-AGW shall then reply with completed "a=ice-pwd", and "a=ice-ufrag" and "a=candidate" attributes in the local descriptor, and shall include "a=ice-lite" if it only supports ICE lite. In the remote descriptor, the IMS-ALG may provide the "a=candidate", "a=ice-pwd", and "a=ice-ufrag". 8) state-agnostic and state-aware TCP handling: The attribute "a=setup" (see IETF RFC 4145 [20]) shall be provided for TCP-based media, in accordance with ITU-T Recommendation H.248.84 [46], when triggering an end-to-end TCP simultaneous open (leading to a TCP merge mode in the IMS-AGW) or other TCP modes of operation. 9) Application-aware interworking for MSRP traffic: The attribute "a=path" (see IETF RFC 4975 [11]) shall be provided, when enabling a bearer level application gateway (B-ALG) function for MSRP traffic, according to ITU-T Recommendation H.248.78 [56]. 10) Handling of RTCP APP messages when transcoding between EVS and non EVS codecs: The attribute "a=3gpp_mtsi_app_adapt" (see 3GPP TS 26.114 [26]) containing the allowed RTCP APP message types shall be provided when the IMS-AGW is allowed to send RTCP APP messages. 11) Pre-defined Video Region-of-Interest (ROI): The attribute "a=rtcp-fb" with the "Predefined ROI" type expressed by the parameter "3gpp-roi-predefined" may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the Predefined ROI mode, see also 3GPP TS 26.114 [26]. In addition, the attribute "a=extmap" (see IETF RFC 5285 [41]) may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the extended RTP header for carriage of pre-defined video Region of Interest (ROI) information in the sent video, see also 3GPP TS 26.114 [26]. 12) Arbitrary Video Region of Interest (ROI): The attribute a=rtcp-fb" with the "Arbitrary ROI" type expressed by the parameter "3gpp-roi-arbitrary" may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the Arbitrary ROI mode, see also 3GPP TS 26.114 [26]. In addition, the attribute "a=extmap" (see IETF RFC 5285 [41]) may be provided for an m-line in the local and remote descriptor if the IMS-AGW supports the extended RTP header for carriage of arbitrary video Region of Interest (ROI) information in the sent video, see also 3GPP TS 26.114 [26]. . 13) WebRTC data channel: The attributes "a=sctp-port" and "a=max-message-size" shall be provided in the remote descriptor (see IETF draft-ietf-mmusic-sctp-sdp [68]). In the local descriptor, the IMS-ALG shall provide "a=sctp-port" with omission sign "-" to indicate that the IMS-ALG shall use the same port as for UDP, and "a=max-message-size" with wildcard sign "$". The IMS-AGW shall then reply with completed "a=sctp-port" and "a=max-message-size" attributes in the local descriptor, The attribute "a=dcmap" shall be provided in the local and remote descriptor, with the parameter "subprotocol" either set to "-" (Application-agnostic data channel configuration) or with real value (Application-aware data channel configuration). 14) Application aware interworking of traffic within a WebRTC data channel: The attribute "a=dcsa" may be provided in the local descriptor and/or remote descriptor (see IETF draft-ietf-mmusic-data-channel-sdpneg [69], NOTE 3). T.140 (see ITU-T Recommendation T.140 75) is used for Global Text Telephony (GTT). Application aware interworking between the transport according to IETF draft-schwarz-mmusic-t140-usage-data-channel 77 within WebRTC data channels and the transport according to IETF RFC 4103 76 in the IMS core network should be applied for T.140 according to ITU-T Recommendation H.248.94 [67] Appendix 2. For application aware T.140 interworking, the "a= rtpmap" attribute shall be provided with "t140" payload type for the termination towards the IMS core network and "t140" value of the "subprotocol" parameter of the SDP "a=dcmap" attribute shall be provided for the termination towards the served WIC. 15) SDP Capability Negotiation: The attributes of "a=acap", "a=tcap", "a=pcfg" and "a=acfg" (see IETF RFC 5939 [72]) may be provided in the local descriptor and/or remote descriptor. 16) Rate adaptation for media endpoints: If the IMS-AGW performs media transcoding and if the rate adaptation for media endpoints using the enhanced bandwidth negotiation is supported by the IMS-AGW, attribute(s) "a=bw-info" with direction "send" or "sendrecv" may be provided for an m-line and the selected IP payload type and applicable IP version in the remote descriptor. The following bandwidth properties, as defined in 3GPP TS 26.114 [26], clause 19, may be included in "a=bw-info" line: <payload type> <dir> <MaxSupBw>, <MaxDesBw>, <MinDesBw>, <MinSupBw> and <IpVer>. 17) "RTP-level pause and resume" signalling: The "rtcp-fb" SDP attribute with the "ccm" feedback parameter and the "pause" ccm parameter as defined in IETF RFC 7728 [79] may be provided for an m-line in the local and remote descriptor to indicate that the IMS-AGW shall forward RTCP feedback "CCM PAUSE-RESUME" messages transparently. 18) "RTCP Codec Control Commands and Indications" signalling: The "rtcp-fb" SDP attribute with the "ccm" feedback parameter and the "fir" and/or "tmmbr" ccm parameters as defined in IETF RFC 5104 [78] may be provided for an m-line in the local and remote descriptor to indicate that the IMS-AGW shall be prepared to receive and is allowed to send, respectively, the RTCP CCM feedback messages FIR, and/or TMMBR and TMMBN. 19) DBI signalling: The "rtcp-fb" SDP attribute with the "3gpp-delay-budget" feedback parameter as defined in 3GPP TS 26.114 [26] may be provided for an m-line in the local and remote descriptor to indicate that the IMS-AGW shall forward RTCP feedback "DBI" messages transparently. |
| NOTE 1: Media Interworking is optional. NOTE 2: Table 1 in ITU-T Recommendation H.248.57 [5] provides the correspondent RTCP port allocation rules. NOTE 3: See IETF draft-ietf-mmusic-msrp-usage-data-channel [70] for WebRTC data application 'MSRP'. |  |  |

Editor's Note:	The support for video transcoding is required for vSRVCC but should be changed from Rel-11, separate CRs would be required for this change.

* * * Next Change * * * *

### 5.17.1	Formats and Codes

Table 5.17.1.1 shows the parameters which are required for the procedures defined in the following clauses.

The coding rules applied in ITU-T Recommendation H.248.1 [10] for the applicable coding technique shall be followed for the UMTS capability set.

The binary encoding rules which are applicable to the defined Abstract Syntaxes are the Basic Encoding Rules for Abstract Syntax Notation One, defined in ITU-T Recommendation X.690 [22].  Specifically in accordance with ITU-T Recommendation X.690 [22] section 7.3, alternative encodings based on the definite and indefinite form of length are permitted by the basic encoding rules as a sender's option. Receivers shall support both alternatives.

Unsupported values of parameters or properties may be reported by the IMS-AGW and shall be supported by the IMS-ALG as such by using H.248.1 error code #449 "Unsupported or Unknown Parameter or Property Value ". The unsupported or unknown value is included in the error text in the error descriptor.

Table 5.17.1.1: Information Elements Used in Procedures

| Signalling Object | H.248 Descriptor | Coding |  |
| --- | --- | --- | --- |
| Additional Bandwidth Properties | Remote Descriptor | The "a=bw-info" SDP attribute defined in 3GPP TS 26.114 [26], see table 5.16.1. |  |
| Allowed RTCP APP message types | Remote Descriptor | The "a=3gpp_mtsi_app_adapt" SDP attribute defined in 3GPP TS 26.114 [26]. |  |
| Alternate MGC Id | ServiceChange | The MGCIdToTry parameter in ITU-T Recommendation H.248.1 [10]. |  |
| Arbitrary ROI | Local Descriptor or Remote Descriptor | The "rtcp-fb" SDP attribute defined in IETF RFC 4585 [21] to indicate the "Arbitrary ROI" RTCP feedback message expressed by the "3gpp-roi-arbitrary" parameter, as described in 3GPP TS 26.114 [26]. |  |
| Available Realms | Termination State | According to Available Realms property in ITU-T Recommendation H.248.41 [8]. |  |
| Application-aware MSRP interworking request | LocalControl |  | This is the ptbalg property from ITU-T Recommendation H.248.78 [56] concerning the configuration of a B-ALG service (for MSRP traffic). |
| BNC Release | Events, ObservedEvents | As for the Events/ObservedEvents Descriptor in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "Cause" |  |
| CCM BASE | Local Descriptor or Remote Descriptor | "rtcp-fb" SDP attribute (defined in IETF RFC 4585 [25]) with the "ccm" feedback parameter and the "fir" and/or "tmmbr" ccm parameters as defined in IETF RFC 5104 [78]. |  |
| CCM pause-resume | Local Descriptor or Remote Descriptor | "rtcp-fb" SDP attribute (defined in IETF RFC 4585 [25]) with the "ccm" feedback parameter (defined in IETF RFC 5104 [78]) and the "pause" ccm parameter as defined in IETF RFC 7728 [79]. |  |
| Cause | ObservedEvents | As for the ObservedEvent Parameter in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "General cause" |  |
| Changed Realms | Observed Events | According to Observed Events Parameters for Available Realms Changed event in ITU-T Recommendation H.248.41 [8]. |  |
| Codec List | Local Descriptor or Remote Descriptor | <fmt list> in a single SDP m-line. For a static RTP payload type, the codec type should be implied by the RTP payload type, if not then each codec type shall be provided in a separate SDP "a=rtpmap"-line and possibly additional SDP "a=fmtp"-line(s). For a dynamic RTP payload type, for each codec information on the codec type shall be provided in a separate SDP "a=rtpmap"-line and possibly additional SDP "a=fmtp"-line(s). |  |
| Connectivity Mode | LocalControl | ITU-T Recommendation H.248.1 [10] Mode property. Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 Annex A [10] "streamMode" Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 Annex B [10] "streamMode". |  |
| Context ID | NA | Binary Encoding: As per ITU-T Recommendation H.248.1 [10] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [10] Annex B. |  |
| Cryptographic SDES Attribute | Local Descriptor or Remote Descriptor | "crypto" attribute in SDP a-line as defined in IETF RFC 4568 [29], see 5.16 |  |
| DBI | Local Descriptor or Remote Descriptor | "rtcp-fb" SDP attribute defined in IETF RFC 4585 [25] with the "3gpp-delay-budget" feedback parameter defined in 3GPP TS 26.114 [26]. |  |
| Delay Variation Tolerance | LocalControl | This is the tman/dvt property from ITU-T Recommendation H.248.53 [7]. |  |
| Diffserv Code Point | LocalControl | Defined according to the Differentiated Services Code Point property in ITU-T Recommendation H.248.52 [12]. |  |
| Diffserv Tagging Behaviour | LocalControl | Defined according to the Tagging Behaviour property in ITU-T Recommendation H.248.52 [12]. |  |
| Discard Incoming TCP Connection Establishment Requests Indicator | LocalControl | Defined according to the Incoming bearer connection establishment blocking property (tcpbcc/bceb) in ITU-T Recommendation H.248.89 [47]. |  |
| ECN Enabled | Local Descriptor or Remote Descriptor | Defined according to the "ECN Enabled" property in ITU-T Recommendation H.248.82 [40]. |  |
| ECN Failure | Events, Observed Events | Defined according to the "ECN Failure" Event in ITU-T Recommendation H.248.82 [40]. |  |
| ECN Failure Type | ObservedEvents Descriptor | As for the ObservedEventsDescriptor Parameter "Failure Type" in ITU-T Recommendation H.248.82 [40]. |  |
| ECN Initiation Method | Local Descriptor or Remote Descriptor | Defined according to "Initiation Method" property in ITU-T Recommendation H.248.82 [40]. |  |
| Emergency Call Indication | NA | ITU-T Recommendation H.248.1 [10] 6.1.1 Emergency Call Indicator Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex A "Emergency" context attribute Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex B " EmergencyToken" context attribute |  |
| Establish (D)TLS session | Signals | Defined according to the Establish BNC signal (tlsbsc/EstBNC) in ITU-T Recommendation H.248.90 [48]. |  |
| Extended Header For CVO | Local Descriptor or Remote Descriptor | "extmap" attribute in SDP a-line as defined in IETF RFC 5285 [41], see 5.16 |  |
| Extended RTP Header for Sent ROI | Local Descriptor or Remote Descriptor | "extmap" attribute in SDP a-line to pass on the ROI extended RTP header as defined by IETF RFC 5285 [41] for carriage of predefined and/or arbitrary ROI information, see 5.16 |  |
| Forward Incoming TCP Connection Establishment Requests Indicator | LocalControl | Defined according to the Interlinkage topology property (seplink/linktopo) in ITU-T Recommendation H.248.93 [50]. |  |
| Generic Image Attribute | Local Descriptor or Remote Descriptor | "imageattr" attribute in SDP a-line as defined in IETF RFC 6236 [46], see table 5.16.1. |  |
| ICE host candidate request | Local Descriptor | The "a=candidate" SDP attribute defined in IETF RFC 5245 [44] of type "host" with the transport, port and priority parameters with wildcard sign "$" to request the allocation of a host candidate |  |
| ICE host candidate | Local Descriptor | The "a=candidate" SDP attribute defined in IETF RFC 5245 [44] |  |
| ICE lite indication | Local Descriptor | The "a=ice-lite" SDP attribute defined in IETF RFC 5245 [44]. |  |
| ICE password request | Local Descriptor | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [44] with wildcard sign "$". |  |
| ICE password | Local Descriptor | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [44]. |  |
| ICE received candidate | Remote Descriptor | The "a=candidate" SDP attribute defined in IETF RFC 5245 [44] |  |
| ICE received password | Remote Descriptor | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [44]. |  |
| ICE received Ufrag | Remote Descriptor | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [44]. |  |
| ICE Ufrag request | Local Descriptor | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [44] with wildcard sign "$". |  |
| ICE Ufrag | Local Descriptor | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [44]. |  |
| ICE Connectivity Check Result | Events, Observed Events | Defined according to Connectivity Check Result event in ITU-T Recommendation H.248.50 [43]. |  |
| ICE Send Connectivity Check | Signals | Defined as the ostuncc/scc signal in ITU-T Recommendation H.248.50 [43]. |  |
| ICE New Peer Reflexive Candidate | Events, Observed Events | Defined according to New Peer Reflexive Candidate event in ITU-T Recommendation H.248.50 [43], only applicable for full ICE. |  |
| ICE Send Additional Connectivity Check | Signals | Defined as the ostuncc/sacc signal in ITU-T Recommendation H.248.50 [43], only applicable for full ICE. |  |
| Consent freshness test request | Signals | Defined according to stnconfres/contest signal in ITU-T Recommendation H.248.50 [43]. |  |
| STUN consent freshness test failure | Events, Observed Events | Defined according to stnconfres/confail event in ITU-T Recommendation H.248.50 [43]. |  |
| Inactivity Timer | Events, Observed Events | Defined according to Inactivity Timeout event in ITU-T Recommendation H.248.14 [11]. |  |
| IP Address | Local Descriptor or Remote Descriptor | <connection address> in SDP "c-line" |  |
| IP Realm | LocalControl | According to IP Realm Identifier property in ITU-T Recommendation H.248.41 [8]. |  |
| IP Version | Local Descriptor or Remote Descriptor | <address type> in SDP "c-line", see 5.15 |  |
| Latching | Signals | This is the ipnapt/latch signal in ITU-T Recommendation H.248.37 [4]. |  |
| Local certificate fingerprint | Local Descriptor | "fingerprint" attribute in SDP "a="-line as defined in IETF RFC 8122 [55] see table 5.16.1. |  |
| Local certificate fingerprint Request | Local Descriptor | "fingerprint" attribute in SDP "a="-line as defined in IETF RFC 8122 [55] with wildcard choose "$". |  |
| Local Dcmap | Local Descriptor | The SDP attribute "a=dcmap" (see IETF draft-ietf-mmusic-data-channel-sdpneg [69]). |  |
| Local Dcsa | Local Descriptor | The SDP attribute "a=dcsa" (see IETF draft-ietf-mmusic-data-channel-sdpneg [69]). |  |
| Local SCTP maximum message size Request | Local Descriptor | The SDP attribute "a= max-message-size" (see IETF draft-ietf-mmusic-sctp-sdp [68]) with wilcard sign "$". |  |
| Local SCTP maximum message size | Local Descriptor | The SDP attribute "a= max-message-size" (see IETF draft-ietf-mmusic-sctp-sdp [68]) |  |
| Local SCTP Port Request | Local Descriptor | The SDP attribute "a= sctp-port" (see IETF draft-ietf-mmusic-sctp-sdp [68]) with omission sign "-" to indicate that the same port as for UDPshall be used. |  |
| Local SCTP Port | Local Descriptor | The SDP attribute "a= sctp-port" (see IETF draft-ietf-mmusic-sctp-sdp [68]) |  |
| Maximum Burst Size | LocalControl | This is the tman/mbs property from ITU-T Recommendation H.248.53 [7] |  |
| Media Inactivity Detection | Events, Observed Events | Defined according to ipstop event in ITU-T Recommendation H.248.40 [24]. |  |
| Media Inactivity Detection Time | Events | As for the Event Parameter in ITU-T Recommendation H.248.40 [24] "Detection Time" |  |
| Media Inactivity Detection Direction | Events | As for the Event Parameter in ITU-T Recommendation H.248.40 [24] "Direction" |  |
| Media Type | Local Descriptor or Remote Descriptor | <media> in SDP m-line "audio" or "video" or "-" |  |
| MSRP Path | Remote Descriptor |  | The "a=path" SDP attribute defined in IETF RFC 4975 [18]. |
| Notify (D)TLS session establishment Failure Event | ObservedEvents | As for the ObservedEvent Parameter in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "General cause" |  |
| Notify TCP Connection Establishment Failure Event | ObservedEvents | As for the ObservedEvent Parameter in subclause E.1.2.1/ ITU-T Recommendation H.248.1 [10] "General cause" |  |
| Overload Notification | Events, ObservedEvents | This is the chp/mgcon event from ITU-T Recommendation H.248.10 [14] or the ocp/mg_overload event from ITU-T Recommendation H.248.11 [13]. |  |
| Peak Data Rate | LocalControl | This is the tman/pdr property from ITU-T Recommendation H.248.53 [7]. |  |
| Policing Required | LocalControl | This is the tman/pol property from ITU-T Recommendation H.248.53 [7]. |  |
| Port | Local Descriptor or Remote Descriptor | <port> in SDP m-line. |  |
| Predefined ROI | Local Descriptor or Remote Descriptor | The "rtcp-fb" SDP attribute defined in IETF RFC 4585 [25] to indicate the "Predefined ROI" RTCP feedback message expressed by the "3gpp-roi-predefined" parameter, as described in 3GPP TS 26.114 [26]. |  |
| Priority Information | NA | Priority Indicator (subclause 6.1.1 of ITU-T Recommendation H.248.1 [10]) Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex A "priority" context attribute Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex B "priority" context attribute |  |
| Realm Availability Change | Events, Observed Events | According to Available Realms Changed event in ITU-T Recommendation H.248.41 [8]. |  |
| Received SCTP Stream Reset Request | Events, ObservedEvents | Defined according to the Detect outgoing SCTP Stream reset event (sctpreset/detreset) in ITU-T Recommendation H.248.97 [66]. |  |
| Received SCTP Stream Reset Response | Events, ObservedEvents | Defined according to the Outgoing SCTP Stream reset result event (sctpreset/result) in ITU-T Recommendation H.248.97 [66]. |  |
| Reduction | ObservedEvent Descriptor | As for the ObserverdEventDescriptor in subclause 4.2.1/ ITU-T Recommendation H.248.10 [14] "MGCongestion". |  |
| Release (D)TLS session | Signals | Defined according to the Release BNC signal (tlsbsc/RelBNC) in ITU-T Recommendation H.248.90 [48]. |  |
| Remote certificate fingerprint | Remote Descriptor | "fingerprint" attribute in SDP "a="-line as defined in IETF RFC 8122 [55], see table 5.16.1. |  |
| Remote Dcmap | Remote Descriptor | The SDP attribute "a=dcmap" (see IETF draft-ietf-mmusic-data-channel-sdpneg [69]). |  |
| Remote Dcsa | Remote Descriptor | The SDP attribute "a=dcsa" (see IETF draft-ietf-mmusic-data-channel-sdpneg [69]). |  |
| Remote SCTP maximum message size | RemoteDescriptor | The SDP attribute "a= max-message-size" (see IETF draft-ietf-mmusic-sctp-sdp [68]) |  |
| Remote SCTP Port | Remote Descriptor | The SDP attribute "a= sctp-port" (see IETF draft-ietf-mmusic-sctp-sdp [68]) |  |
| Remote Source Address Filtering | LocalControl | Defined according to Remote Source Address Filtering property in ITU-T Recommendation H.248.43 [6]. |  |
| Remote Source Address Mask | LocalControl | Defined according to Remote Source Address Mask property in ITU-T Recommendation H.248.43 [6]. |  |
| Remote Source Port Filtering | LocalControl | Defined according to Remote Source Port Filtering property in ITU-T Recommendation H.248.43 [6]. |  |
| Remote Source Port | LocalControl | Defined according to Remote Source Port property in ITU-T Recommendation H.248.43 [6]. |  |
| Remote Source Port Range | LocalControl | Defined according to Remote Source Port Range property in ITU-T Recommendation H.248.43 [6]. |  |
| Reserve_Value | LocalControl | ITU-T Recommendation H.248.1 [10] Reserve property. Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex A "reserveValue " Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [10] Annex B "reservedValueMode". |  |
| ROOT Properties | Termination State | The properties in subclause E.2.1/ ITU-T Recommendation H.248.1 [10] |  |
| RTCP allocation ((NOTE 1) | Local Control | Defined according to RTCP Allocation Specific Behaviour property in ITU-T Recommendation H.248.57 [5]. |  |
| RTP/RTCP transport multiplexing | Local Descriptor or Remote Descriptor | The SDP attribute "a=rtcp-mux" according to IETF RFC 5761 [59]. (NOTE 2) |  |
| explicit RTCP transport address | Remote Descriptor | The SDP attribute "a=rtcp:" according to IETF RFC 3605 [21]. |  |
| RtcpbwRR | Local Descriptor or Remote Descriptor | <bandwidth> in SDP "b:RR"-line. see 5.15 |  |
| RtcpbwRS | Local Descriptor or Remote Descriptor | <bandwidth> in SDP "b:RS"-line. see 5.15 |  |
| Rtpbw | Local Descriptor or Remote Descriptor | <bandwidth> in SDP "b:AS"-line. see 5.15 |  |
| RTPpayload | Local Descriptor or Remote Descriptor | <fmt list> in SDP m-line. This may be set to CHOOSE ($) in a LD sent from the IMS-ALG toward the IMS-AGW. |  |
| SCTP Group Semantics | Termination State | Group Semantics (mgroup/groupse) defined in ITU-T Recommendation H.248.96 [65] with semantics "SCTP" defined in ITU-T Recommendation H.248.97 [66] |  |
| SCTP stream deaggregation | LocalControl | Stream Deaggregation (mgroup/strdeagg) related semantics "SCTP" defined in ITU-T Recommendation H.248.96 [65] |  |
| SCTP stream ID | LocalControl | SCTP StreamID (sctpbcc/sctpid) defined in ITU-T Recommendation H.248.97 [66] |  |
| SDPCapNeg configuration | Local Descriptor or Remote Descriptor | The SDP attributes for SDP capability negotiation as defined in IETF RFC 5939 [72]. |  |
| SDPCapNeg Supported Capabilities | Termination State | Defined according to SDPCapNeg Extensions property in ITU-T Recommendation H.248.80 [73]. |  |
| Send SCTP Association Establishment Requests Indicator | Signals | Defined according to the Establish BNC signal (sctpbcc/EstBNC) in ITU-T Recommendation H.248.97 [66]. |  |
| Send SCTP Stream Reset Requests Indicator | Signals | Defined according to the Initiate Outgoing SCTP Stream Reset signal (sctpreset/initreset) in ITU-T Recommendation H.248.97 [66]. |  |
| Send SCTP Stream Reset Response Indicator | Signals | Defined according to the Outgoing SCTP Stream Reset Response signal (sctpreset/resetresp) in ITU-T Recommendation H.248.97 [66]. |  |
| Send TCP Connection Establishment Requests Indicator | Signals | Defined according to the Establish BNC signal (tcpbcc/EstBNC) in ITU-T Recommendation H.248.89 [47]. |  |
| Stream Number | Stream | Encoding as per ITU-T Recommendation H.248.1 [10] Annex B "Stream"/"ST". For a single stream, this may be omitted by the IMS-ALG. |  |
| STUN server request | LocalControl | Encoding as per ITU-T Recommendation H.248.50 [43] "MG Act-as STUN Server" (mgastuns) package "Act-as STUN Server" (astuns, 0x0001) property. |  |
| Sustainable Data Rate | LocalControl | This is the tman/sdr property from ITU-T Recommendation H.248.53 [7]. |  |
| TCP State-aware Handling Indicator and Setup Direction | Local Descriptor or Remote Descriptor | The "a=setup" SDP attribute as per subclause 13.5.1 of ITU-T Recommendation H.248.84 [46]. |  |
| Termination heartbeat | Events ObservedEvents | As per Termination Heartbeat defined in ITU-T Recommendation H.248.36 [9] Clause 5.2.1. |  |
| Termination ID | NA | Binary Encoding: As per ITU-T Recommendation H.248.1 [10] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [10] Annex B. |  |
| Transaction ID | NA | Binary Encoding: As per ITU-T Recommendation H.248.1 [10] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [10] Annex B. |  |
| Transport | Local Descriptor or Remote Descriptor | <transport> in SDP m-line, see 5.15 |  |
| NOTE 1: Signalling element "RTCP allocation" corresponds to the stage 2 information element "RTCP handling". NOTE 2: RTP/RTCP transport multiplexing can be agreed with the served UE using the SDP attribute(s) "a=rtcp-mux" and/or "a=rtcp-mux-only" (defined in IETF draft-ietf-mmusic-mux-exclusive [74]), see 3GPP TS 23.334 [23]. However, the SDP attribute "rtcp-mux-only" is not used on Iq interface. |  |  |  |

* * * Next Change * * * *

#### 5.17.2.3	Configure AGW Connection Point

This procedure is used to configure the AGW connection point during session establishment or to reconfigure it during session establishment or after the session is established

The IMS-ALG sends a MODIFY request command as in Table 5.17.2.3.1.

Table 5.17.2.3.1: Configure AGW Connection Point Request

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| If local resources are modified: Local Descriptor { Port IP Address IP Version } If remote resources are modified: Remote Descriptor { Port IP Address IP Version } | Transaction ID = x Context ID = C1 Termination ID = T1 If MPS priority is modified: Priority Indicator = x (NOTE 4) If Stream Number specified: Stream Number If Resources for multiple Codecs required: Reserve_Value If diffserv required:- Diffserv Code Point If tagging behaviour Diffserv Tagging Behaviour If Remote Source Address Filtering required:- Remote Source Address Filtering If Remote Source Address range required: Remote Source Address Mask If Remote Source Port Filtering required:- Remote Source Port Filtering If individual port: Remote Source Port If range of ports Remote Source Port Range NotificationRequested (Event ID = x, "termination heartbeat") If IP Realm specified:- IP Realm (NOTE 1) If Latching Required:- Latching If Sustainable Data Rate Policing Required:- Policing Required Sustainable Data Rate Maximum Burst Size If Peak Data Rate Policing Required: Policing Required Peak Data Rate If Delay Variation Required Delay Variation Tolerance If Media Inactivity Detection Required: NotificationRequested (Event ID = x, "Media Inactivity Detection( Media Inactivity Detection Time, Media Inactivity Detection Direction)") (NOTE 2) If RTCP handling required: RTCP allocation If ECN transparent support required: ECN Enable = "True" Initiation Method = "inactive" If ECN Endpoint support required ECN Enable = "True" Initiation Method = "ECN Initiation Method" (NOTE 3) If notification of ECN Failure Report: NotificationRequested (Event ID = x,"ECN Failure") If full ICE is applied: Send Connectivity Check ("Control") If notification of ICE Connectivity Check Result Report: NotificationRequested (Event ID= xx, "Connectivity Check Result") If notification of New Peer Reflexive Candidate: NotificationRequested (Event ID = xy,"New Peer Reflexive Candidate") Send Additional Connectivity Check ("Control") If Discard Incoming TCP connection establishment request required: Discard Incoming TCP Connection Establishment Requests Indicator If Forward Incoming TCP connection establishment request required: Forward Incoming TCP Connection Establishment Requests Indicator If TCP connection establishment required: Send TCP Connection Establishment Request Indicator If indication on TCP connection establishment failure requested: NotificationRequested (Event ID = x, "TCP connection establishment failure") If (D)TLS session establishment required: Establish (D)TLS session If indication on (D)TLS session establishment failure requested: NotificationRequested (Event ID = x, "(D)TLS session establishment failure") If (D)TLS session release required: Release (D)TLS session If media is "message": If B-ALG for MSRP required: Application-aware MSRP interworking request If SCTP association for WebRTC data channels: SCTP Group Semantics SCTP stream deaggregation SCTP stream ID NotificationRequested (Event ID = x, "Received SCTP Stream Reset Response") If reset of SCTP Stream for WebRTC data channels: Send SCTP Stream Reset Requests Indicator If Notification of reset result desired: NotificationRequested (Event ID = x, "Received SCTP Stream Reset Response") | If local resources are modified: Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE 9) If IMS media plane security required: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO required: Extended Header For CVO (NOTE 5) If imageattr negotiation: Generic Image Attribute (NOTE 6) If Predefined ROI required: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI required: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If SCTP association for WebRTC data channels: Local Dcmap If application aware interworking Local Dcsa If SDPCapNeg is signalled to the gateway: SDPCapNeg configuration } If remote resources are modified: Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If rate adaptation for media endpoints: Additional Bandwidth Properties (NOTE 10) If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE 9) If RTCP handling required: explicit RTCP transport address (NOTE 8) If IMS media plane security required: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP APP messages allowed Allowed RTCP APP message Types If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "message" or "application" or "-": If IMS media plane security required: Remote certificate fingerprint If media is "video": If CVO required: Extended Header For CVO (NOTE 5) If imageattr negotiation: Generic Image Attribute (NOTE 6) If Predefined ROI required: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI required: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI If media is "message": If B-ALG for MSRP required: MSRP Path If ICE is applied: ICE received candidate ICE received password ICE received Ufrag (NOTE 7) If STUN consent freshness test required: STUN consent freshness request NotificationRequested(Event ID= x, "STUN consent freshness test failure") If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If SCTP association for WebRTC data channels: Remote SCTP Port Remote SCTP maximum message size Remote Dcmap If application aware interworking Remote Dcsa If SDPCapNeg is signalled to the gateway: SDPCapNeg configuration } |
| NOTE 1: This can only be set to the same realm as at the reservation stage. If a different realm is specified, the IMS-AGW shall return error 501 "Not Implemented". NOTE 2: The event parameters "Media Inactivity Detection Time" and "Media Inactivity Detection Direction" are optional. NOTE 3: This shall be set to a value other than "inactive". See Table 5.14.3.15.1. NOTE 4: The support of the modification of the Priority Indicator value is optional for the IMS-AGW and depends on implementation solution for Priority call/session authorisation (see 3GPP TS 23.334 [23]). NOTE 5: If the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information it shall pass any received extended RTP header with CVO bits on to outgoing RTP streams. If the IMS-AGW is transcoding between video payloads and it supports the extended RTP header with Coordination of Video Orientation information it shall convey received RTP header bytes on the outgoing RTP stream after transcoding associated packets as specified in 3GPP TS 26.114 [26], subclause 7.4.5. NOTE 6: The support of the generic image attributes is optional for the IMS-AGW. The list of image sizes per payload type supported by the IMS-AGW is preconfigured in the IMS-ALG. If none of the image sizes received within an SDP body on Mx/Mw interface is supported by the IMS-AGW then the IMS-ALG will not send the generic image attribute parameter to the IMS-AGW. NOTE 7: The support of ICE received candidate, ICE received password, ICE received Ufrag are optional for ICE lite, as specified in 3GPP TS 23.334 [23]. NOTE 8: The basic RTCP port allocation rules are defined by table 1 in ITU-T Recommendation H.248.57 [5], which summarizes all rules, with and without the "explicit RTCP transport address" element. NOTE 9: This element is optional. The RTCP port allocation rules are specified in tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5]. NOTE 10: The support of rate adaptation for media endpoints using the additional bandwidth properties is optional for the IMS-AGW. If media transcoding is required the IMS-ALG may provide for the selected payload type and the used IP version the additional bandwidth properties. |  |  |

The IMS-AGW responds as in Table 5.17.2.3.2.

Table 5.17.2.3.2: Configure AGW Connection Point Request Acknowledge

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| If local resources were provided in request: Local Descriptor { Port IP Address IP Version } If remote resources are provided in request: Remote Descriptor { Port IP Address IP Version } NOTE | Transaction ID = x Context ID = C1 Termination ID = T1 If Stream Number Specified: Stream Number | If local resources were provided in request: Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in request: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute If Predefined ROI provided in the request: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI provided in the request: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI } If remote resources are provided in request: Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If rate adaptation for media endpoints: Additional Bandwidth Properties If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute If Predefined ROI provided in the request: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI provided in the request: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI } NOTE |
| NOTE: Sending of the Remote Descriptor is optional. |  |  |

* * * Next Change * * * *

#### 5.17.2.4	Reserve and Configure AGW Connection Point

The IMS-ALG sends an ADD request command as in Table 5.17.2.4.1.

Table 5.17.2.4.1: Reserve and Configure AGW Connection Point Request

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| Local Descriptor { Port = $ IP Address = $ IP Version = IPv4 or IPv6 } Remote Descriptor { Port IP Address IP Version } | Transaction ID = x If Context Requested: Context ID = $ If Emergency Call: Emergency Call Indication If MPS call/session: Priority Indicator = x If Context Provided: Context ID = c1 Termination ID = $ If Stream Number Specified: Stream Number If Resources for multiple Codecs shall be reserved: Reserve_Value If IP Interface Type: IP interface = "IP interface type" If indication on Bearer Released requested: NotificationRequested (Event ID = x, "BNC Release") If diffserv required:- Diffserv Code Point If tagging behaviour Diffserv Tagging Behaviour If Remote Source Address Filtering required:- Remote Source Address Filtering If Remote Source Address range required: Remote Source Address Mask If Remote Source Port Filtering required:- Remote Source Port Filtering If individual port: Remote Source Port If range of ports Remote Source Port Range NotificationRequested (Event ID = x, "termination heartbeat") If IP Realm specified:- IP Realm If Latching Required:- Latching If Sustainable Data Rate Policing Required:- Policing Required Sustainable Data Rate Maximum Burst Size If Peak Data Rate Policing Required: Policing Required Peak Data Rate If Delay Variation Required Delay Variation Tolerance If Media Inactivity Detection Required: NotificationRequested (Event ID = x, "Media Inactivity Detection( Media Inactivity Detection Time, Media Inactivity Detection Direction)") (NOTE 1) If RTCP handling required: RTCP allocation If ECN transparent support required: ECN Enable = "True" Initiation Method = "inactive" If ECN Endpoint support required ECN Enable = "True" Initiation Method = "ECN Initiation Method" (NOTE 2) If notification of ECN Failure Report: NotificationRequested (Event ID = x,"ECN Failure") If ICE is applied: STUN server request If full ICE is applied Send Connectivity Check ("Control") If notification of ICE Connectivity Check Result Report: NotificationRequested (Event ID = xx, "Connectivity Check Result") If notification of New Peer Reflexive Candidate: NotificationRequested (Event ID = xy,"New Peer Reflexive Candidate") If Discard Incoming TCP connection establishment request required: Discard Incoming TCP Connection Establishment Requests Indicator If Forward Incoming TCP connection establishment request required: Forward Incoming TCP Connection Establishment Requests Indicator If indication on TCP connection establishment failure requested: NotificationRequested (Event ID = x, "TCP connection establishment failure") If (D)TLS session establishment required: Establish (D)TLS session If indication on (D)TLS session establishment failure requested: NotificationRequested (Event ID = x, "(D)TLS session establishment failure") If media is "message": If B-ALG for MSRP required: Application-aware MSRP interworking request If SCTP association for WebRTC data channels: SCTP Group Semantics SCTP stream deaggregation SCTP stream ID NotificationRequested (Event ID = x, "Received SCTP Stream Reset Request") | Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE 7) If IMS media plane security required: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO required: Extended Header For CVO (NOTE 3) If imageattr negotiation: Generic Image Attribute (NOTE 4) If Predefined ROI required: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI required: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI If ICE is applied: ICE host candidate request ICE password request ICE Ufrag request If media is "message" or "application" or "-": If IMS media plane security required: Local certificate fingerprint Request If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If SCTP association for WebRTC data channels: Local SCTP Port Request Local SCTP maximum message size Request Local Dcmap If application aware interworking Local Dcsa If SDPCapNeg is signalled to the gateway: SDPCapNeg configuration } Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If rate adaptation for media endpoints: Additional Bandwidth Properties (NOTE 8) If RTCP bandwidth RtcpbwRS RtcpbwRR If RTCP handling required: RTP/RTCP transport multiplexing (NOTE 7) If RTCP handling required: explicit RTCP transport address (NOTE 6) If IMS media plane security required: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP APP messages allowed Allowed RTCP APP message types If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO required: Extended Header For CVO (NOTE 3) If imageattr negotiation: Generic Image Attribute (NOTE 4) If Predefined ROI required: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI required: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI If media is "message": If B-ALG for MSRP required: MSRP Path If ICE is applied: ICE received candidate ICE received password ICE received Ufrag (NOTE 5) If STUN consent freshness test required: STUN consent freshness request NotificationRequested(Event ID= x, "STUN consent freshness test failure") If media is "message" or "application" or "-": If IMS media plane security required: Remote certificate fingerprint If TCP state-aware handling required: TCP State-aware Handling Indicator and Setup Direction If SCTP association for WebRTC data channels: Remote SCTP Port Remote SCTP maximum message size Remote Dcmap If application aware interworking Remote Dcsa If SDPCapNeg is signalled to the gateway: SDPCapNeg configuration } |
| NOTE 1: The event parameters "Media Inactivity Detection Time" and "Media Inactivity Detection Direction" are optional. NOTE 2: This shall be set to a value other than "inactive". See Table 5.14.3.15.1. NOTE 3: If the IMS-AGW supports the extended RTP header with Coordination of Video Orientation information it shall pass any received extended RTP header with CVO bits on to outgoing RTP streams. If the IMS-AGW is transcoding between video payloads and it supports the extended RTP header with Coordination of Video Orientation information it shall convey received RTP header bytes on the outgoing RTP stream after transcoding associated packets as specified in 3GPP TS 26.114 [26], subclause 7.4.5. NOTE 4: The support of the generic image attributes is optional for the IMS-AGW. The list of image sizes per payload type supported by the IMS-AGW is preconfigured in the IMS-ALG. If none of the image sizes received within an SDP body on Mx/Mw interface is supported by the IMS-AGW then the IMS-ALG will not send the generic image attribute parameter to the IMS-AGW. NOTE 5: The support of ICE received candidate, ICE received password, ICE received Ufrag are optional for ICE lite, as specified in 3GPP TS 23.334 [23]. NOTE 6: The basic RTCP port allocation rules are defined by table 1 in ITU-T Recommendation H.248.57 [5], which summarizes all rules, with and without the "explicit RTCP transport address" element. NOTE 7: This element is optional. The RTCP port allocation rules are specified in tables 4/1 to 4/5 in ITU-T Recommendation H.248.57 [5]. NOTE 8: The support of rate adaptation for media endpoints using the additional bandwidth properties is optional for the IMS-AGW. If media transcoding is required the IMS-ALG may provide for the selected payload type and the used IP version the additional bandwidth properties. |  |  |

The IMS-AGW  responds as in Table 5.17.2.4.2.

Table 5.17.2.4.2: Reserve and Configure AGW Connection Point Request Acknowledge

| Address Information | Control information | Bearer information |
| --- | --- | --- |
| Local Descriptor { Port IP Address IP Version } Remote Descriptor { Port IP Address IP Version } NOTE | Transaction ID = x Context ID = C1 Termination ID = T1 Stream Number | Local Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute If Predefined ROI provided in the request: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI provided in the request: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI If ICE is applied: ICE host candidate ICE password ICE Ufrag If ICE lite implementation ICE lite indication If media is "message" or "application" or "-": If Local certificate fingerprint was requested: Local certificate fingerprint If SCTP association for WebRTC data channels: Local SCTP Port Local SCTP maximum message size If SDPCapNeg is signalled to the gateway: SDPCapNeg configuration Remote Descriptor { If media is "audio" or "video": Codec List RTP Payloads Rtpbw If rate adaptation for media endpoints: Additional Bandwidth Properties If RTCP bandwidth RtcpbwRS RtcpbwRR If IMS media plane security was provided in the request: Cryptographic SDES Attribute If RTP-level pause and resume: CCM pause-resume If RTCP Codec Control Commands and Indications: CCM BASE If Delay Budget Information: DBI If media is "video": If CVO extension header provided in the request: Extended Header For CVO If image attribute negotiation: Generic Image Attribute If Predefined ROI provided in the request: RTCP feedback for Predefined ROI Extended Header for Sent ROI If Arbitrary ROI provided in the request: RTCP feedback for Arbitrary ROI Extended Header for Sent ROI If SDPCapNeg is signalled to the gateway: SDPCapNeg configuration } NOTE |
| NOTE: Sending of the Remote Descriptor is optional. |  |  |

* * * End of Change * * * *