3GPP TSG-CT WG4 Meeting #90	C4-191274

Xi'An, China; 8th April -  12th April 2019 	was C4-191154

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0024 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Positioning Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-03-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As specified in 24.501, during Registration procedures UE shall always indicate the support of "LPP in N1 mode" to the network: ##### "5.5.1.2.2 Initial registration initiation … If the UE supports the LTE positioning protocol (LPP) in N1 mode as specified in 3GPP TS 36.355 [26], the UE shall set the LPP bit to "LPP in N1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. … ##### 5.5.1.3.2 Mobility and periodic registration update initiation … If the UE supports the LTE positioning protocol (LPP) in N1 mode as specified in 3GPP TS 36.355 [26], the UE shall set the LPP bit to "LPP in N1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. …" This secures AMF always knowing the UE LPP in N1 mode support. As specified in 29.171, MME can pass the UE Positioning capability to SMLC, indicating UE support LPP or not. If not, SMLC will select positioning methods which doesn't require LPP procedure to get UE assistant data. This can avoid unnecessary network traffic as well as avoid negative impacts network KPI due to LPP procedure failure. As 5G counterpart, AMF should pass the UE LPP support to LMF to achieve the same functionality. Rev1: Update dependent SA2 CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Describe how LMF handling the UE LCS capability in 5.2.2.2.2 2. Define New data type UeLcsCapability 3. Add attribute in InputData in 6.1.6.2.2 4. Update accordingly in 6.1.6.1 and Nlmf_Location OpenAPI specification A.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage 2 requirement, resulting unnecessary traffics and negative impacts to network KPI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 6.1.6.1, 6.1.6.2.2. 6.1.6.2.x (New), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR 23.502 CR 1143 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI file for API Nlmf_Location. CAT F 23.502 CR1143 issued in SA2. |  |  |  |  |  |  |  |  |

* * * Information * * * *

UE Positioning Capability definition in 3GPP TS 29.171:

### 7.4.8	UE Positioning Capability

This parameter provides information about the LCS capabilities of the target UE.

Table 7.4.8-1: UE Positioning Capability

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| LPP Support | M |  | BOOLEAN | Defines if the UE supports LPP or not. TRUE means supported. | – |  |

* * * First Change * * * *

##### 5.2.2.2.2	Retrieve UE Location

This procedure allows a consumer NF to request the location information (geodetic location and, optionally, civic location).

Figure 5.2.2.2.2-1: DetermineLocation Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the "determine-location" custom operation. The input parameters for the request (external client type, LCS correlation identifier, serving cell identifier, location QoS, supported GAD shapes….) shall be included in the HTTP POST request body.

If UE LCS Capability is received in the request indicating LPP is not supported by the UE, the LMF shall not send LPP messages to the UE in subsequent positioning procedures.

2a.	On success, "200 OK" shall be returned. The response body shall contain the parameters related to the determined position of the UE (geodetic position, civic location, positioning methods…).

2b.	On failure, one of the HTTP status code listed in Table 6.1.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.4.2.2-2.

* * * Next Change * * * *

#### 6.1.6.1	General

This subclause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nlmf service based interface protocol.

Table 6.1.6.1-1: Nlmf specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| InputData | 6.1.6.2.2 |  |
| LocationData | 6.1.6.2.3 |  |
| GeographicalCoordinates | 6.1.6.2.4 |  |
| GeographicArea | 6.1.6.2.5 |  |
| Point | 6.1.6.2.6 |  |
| PointUncertaintyCircle | 6.1.6.2.7 |  |
| PointUncertaintyEllipse | 6.1.6.2.8 |  |
| Polygon | 6.1.6.2.9 |  |
| PointAltitude | 6.1.6.2.10 |  |
| PointAltitudeUncertainty | 6.1.6.2.11 |  |
| EllipsoidArc | 6.1.6.2.12 |  |
| LocationQoS | 6.1.6.2.13 |  |
| CivicAddress | 6.1.6.2.14 |  |
| PositioningMethodAndUsage | 6.1.6.2.15 |  |
| GnssPositioningMethodAndUsage | 6.1.6.2.16 |  |
| VelocityEstimate | 6.1.6.2.17 |  |
| HorizontalVelocity | 6.1.6.2.18 |  |
| HorizontalWithVerticalVelocity | 6.1.6.2.19 |  |
| HorizontalVelocityWithUncertainty | 6.1.6.2.20 |  |
| HorizontalWithVerticalVelocityAndUncertainty | 6.1.6.2.21 |  |
| UncertaintyEllipse | 6.1.6.2.22 |  |
| UeLcsCapability | 6.1.6.2.x | Indicates the LCS capability supported by the UE. |
| Altitude | 6.1.6.3.2 |  |
| Angle | 6.1.6.3.2 |  |
| Uncertainty | 6.1.6.3.2 |  |
| Orientation | 6.1.6.3.2 |  |
| Confidence | 6.1.6.3.2 |  |
| Accuracy | 6.1.6.3.2 |  |
| InnerRadius | 6.1.6.3.2 |  |
| CorrelationID | 6.1.6.3.2 |  |
| AgeOfLocationEstimate | 6.1.6.3.2 |  |
| HorizontalSpeed | 6.1.6.3.2 |  |
| VerticalSpeed | 6.1.6.3.2 |  |
| SpeedUncertainty | 6.1.6.3.2 |  |
| BarometricPressure | 6.1.6.3.2 |  |
| ExternalClientType | 6.1.6.3.3 |  |
| SupportedGADShapes | 6.1.6.3.4 |  |
| ResponseTime | 6.1.6.3.5 |  |
| PositioningMethod | 6.1.6.3.6 |  |
| GnssMethod | 6.1.6.3.7 |  |
| GnssId | 6.1.6.3.8 |  |
| Usage | 6.1.6.3.9 |  |
| LcsPriority | 6.1.6.3.10 |  |
| VelocityRequested | 6.1.6.3.11 |  |
| AccuracyFulfilmentIndicator | 6.1.6.3.12 |  |
| VerticalDirection | 6.1.6.3.13 |  |

Table 6.1.6.1-2 specifies data types re-used by the Nlmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf service based interface.

Table 6.1.6.1-2: Nlmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Supi | 3GPP TS 29.571 [8] |  |
| Pei | 3GPP TS 29.571 [8] |  |
| Gpsi | 3GPP TS 29.571 [8] |  |
| Ecgi | 3GPP TS 29.571 [8] |  |
| Ncgi | 3GPP TS 29.571 [8] |  |
| NfInstanceId | 3GPP TS 29.571 [8] |  |

* * * Next Change * * * *

##### 6.1.6.2.2	Type: InputData

Table 6.1.6.2.2-1: Definition of type InputData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| externalClientType | ExternalClientType | O | 0..1 |  |
| correlationID | CorrelationID | O | 0..1 |  |
| amfId | NfInstanceId | O | 0..1 | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |
| locationQoS | LocationQoS | O | 0..1 |  |
| supportedGADShapes | array(SupportedGADShapes) | O | 1..N |  |
| supi | Supi | O | 0..1 |  |
| pei | Pei | O | 0..1 |  |
| gpsi | Gpsi | O | 0..1 |  |
| ecgi | Ecgi | O | 0..1 | When present, this IE shall indicate the identifier of the E-UTRAN cell serving the UE. (NOTE 2) |
| ncgi | Ncgi | O | 0..1 | When present, this IE shall indicate the identifier of the NR cell serving the UE. (NOTE 2) |
| priority | LcsPriority | O | 0..1 |  |
| velocityRequested | VelocityRequested | O | 0..1 |  |
| ueLcsCap | UeLcsCapability | O | 0..1 | When present, this IE shall indicate the LCS capability supported by the UE. |
| NOTE 1: At least one of the attributes defined in this table shall be present in the InputData structure. NOTE 2: Attribute "ecgi" and "ncgi" shall not be present at the same time. |  |  |  |  |

* * * Next Change * * * *

### 6.1.6.2.x	Type: UeLcsCapability

Table 6.1.6.2.x-1: Definition of type UeLcsCapability

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| lppSupport | boolean | O | 0..1 | Indicates whether the UE supports LPP or not. - true (default): LPP supported by the UE - false: LPP not supported by the UE |
| NOTE: At least one of the attributes defined in this table shall be present in the UeLcsCapability structure. |  |  |  |  |  |

* * * Next Change * * * *

## A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.0.1'

title: 'LMF Location'

description: 'LMF Location Service'

…

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nlmf-loc: Access to the Nlmf_Location API

schemas:

#

# COMPLEX TYPES

#

InputData:

type: object

not:

required: [ ecgi, ncgi ]

properties:

externalClientType:

$ref: '#/components/schemas/ExternalClientType'

correlationID:

$ref: '#/components/schemas/CorrelationID'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

priority:

$ref: '#/components/schemas/LcsPriority'

velocityRequested:

$ref: '#/components/schemas/VelocityRequested'

ueLcsCap:

$ref: '#/components/schemas/UeLcsCapability'

…

HorizontalWithVerticalVelocityAndUncertainty:

type: object

required:

- hSpeed

- bearing

- vSpeed

- vDirection

- hUncertainty

- vUncertainty

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

vSpeed:

$ref: '#/components/schemas/VerticalSpeed'

vDirection:

$ref: '#/components/schemas/VerticalDirection'

hUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

vUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

UeLcsCapability:

type: object

properties:

lppSupport:

type: boolean

default: true

#

# SIMPLE TYPES

#

…

* * * End of Changes * * *