3GPP TSG-CT WG4 Meeting #90	C4-191514

# Xi'an, P.R.China; 08th – 12th April 2019

Source:	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Cisco Systems, SoftBank Corp

Title:	Pseudo-CR on Evaluation of SRv6 solutions – Additional Considerations

Spec:	3GPP TR 29.892 v1.0.0

Agenda item:	6.1.1

Document for:	Decision

1. Reason for Change

This pCR provides text for the evaluation of the SRv6 solutions regarding the additional consideration points.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 29.892 v1.0.0.

* * * First Change * * * *

## 7.2	Additional Considerations

### 7.2.3	SRv6

Table 7.2.3-1 describes the SRv6 Traditional Mode solution on the additional considerations defined in subclause 7.2.1.

Table 7.2.3-2 evaluates the SRv6 Enhanced Mode on the additional evaluation criteria defined in subclause 7.2.1.

Table 7.2.3-1: Additional Considerations for SRv6 in Traditional Mode

| Additional Considerations | Description |
| --- | --- |
| A1. Proven technology / Time of Availability of used standards | IETF work on SRv6 related improvements is still in progress. SRv6 as a replacement of GTP-U would require new 3GPP standardization work. |
| A2. Enabling separation between 3GPP User Plane and Transport | 3GPP User Plane and IP transport are collapsed/integrated, with the IP destination address encoding the IP address of the peer UPF, the TEID of the specific user plane tunnel and other 3GPP related information (e.g. QFI, RQI). With SRv6 as a GTP-U replacement, 3GPP User Plane and Transport are tied in one technology. |
| A3. Transport network requirements | SRv6 essentially relies on IPv6. IPv4 networks might be considered but at the expense of extra signalling overhead and operational costs. The length of the IPv6 prefix is constrained to keep enough bits to the Function and Arguments of the SID (see Figure 6.2.2.3.2.1). |
| A4. Co-existence with existing User Plane solution | Co-existence with 3GPP user plane entities supporting only GTP-U is required. Use of SRv6 in Traditional Mode needs to be negotiated via control plane signalling during the setup of the user plane tunnel (in a backward compatible manner to take into account implementations that do not support SRv6 and corresponding negotiation signalling). |
| A5. Interworking with RAN | The solution requires interworking GTP-U over N3 and SRv6 over N9, including interworking the PDU Session User Plane Protocol (in GTP-U extension header over N3, and in the Destination IP address in SRv6 in Traditional Mode), causing impacts in UPF. |
| A6. Interworking with EPS | GTP-U is supported on user plane interfaces in the EPS. The solution requires the UPF/PGW-U to support different user plane protocols and to switch from one to the other as the PDU session moves between EPS and 5GS. |
| A7. Impacts to GSMA GRX/IPX | GSMA would need to study the support of SRv6 in GRX/IPX as a replacement of GTP-U (including security aspects, see A8). |
| A8. Security | Due to security considerations mentioned in section 8.2 of IETF RFC 8402 [5], packets sent over SRv6 from one PLMN to another across N9 in home routed roaming cases will be filtered/dropped. Security requirements and proposed 3GPP extensions to existing IETF security requirements for use of SRv6 across SR Domain (e.g. inter-PLMN) need to be reviewed and assessed by SA3 (see subclause 6.2.2.7). GSMA has specified user plane security solution for GTP-U in EPC (based on GTP-U firewall). GSMA and 3GPP SA3 are further studying GTP-U security in 5G System. GSMA and 3GPP SA3 would need to be involved to study SRv6 user plane security in 5G System. |
| A9. Minimize number of protocols in network | The solution would add one new 3GPP user plane protocol solution, for use over the N9 interface. Other interfaces would continue to support GTP-U, e.g. N3, N4-u, S5/S8. |
| A10. Reusability of existing 3GPP implementations | SMFs and UPFs need to be modified to implement SRv6 as a replacement of GTP-U and reproduce existing GTP-U functionalities (e.g. End Markers, PDU Session User Plane Protocol). |
| A11. Protocol Extensibility | Editor's Note: protocol extensibility of the solution is FFS. |
| A12. Protocol Overhead | No UDP (8 octets) and GTP-U (12 octets) headers. If SRv6 is supported over IPv4: extra IPv6 header (40 octets). |
| A13. Resource-efficiency | Per 3GPP system architecture requirements, 3GPP user plane functionalities are controlled in UPF by the SMF over the N4 interface; the UPF shall keep states for the established PFCP associations and PFCP sessions, like for GTP-U. |
| A14. Routing capabilities | Routing capabilities supported by IPv6 or the underlying transport technologies, like GTP-U. The solution supports error detection and error reporting capabilities. Fast Rerouting is also supported. |

Table 7.2.3-2: Additional Considerations for SRv6 in Enhanced Mode

| Additional Considerations | Description |
| --- | --- |
| A1. Proven technology / Time of Availability of used standards | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A2. Enabling separation between 3GPP User Plane and Transport | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). In addition, SRH headers also encode IP addresses of intermediate IP routers on the path, requiring the UPF (mobile network) to be configured with information about the SIDs and thus topology of the transport network. This essentially assumes mobile and transport networks from a same operator. |
| A3. Transport network requirements | To use the full set of functionalities, the solution relies on IPv6 and SRv6 capable networks. |
| A4. Co-existence with existing User Plane solution | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1), but with negotiating the use of SRv6 in Enhanced Mode. |
| A5. Interworking with RAN | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A6. Interworking with EPS | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A7. Impacts to GSMA GRX/IPX | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A8. Security | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A9. Minimize number of protocols in network | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A10. Reusability of existing 3GPP implementations | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A11. Protocol Extensibility | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A12. Protocol Overhead | No UDP (8 octets) and GTP-U (8 to 12 octets) headers. Generic Routing Extension header: 4 octets SRH header: 4 octets + 16 octets per SID + optional objects (if any) If SRv6 is supported over IPv4: extra IPv6 header (40 octets). |
| A13. Resource-efficiency | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). |
| A14. Routing capabilities | Same as for SRv6 in Traditional Mode (see Table 7.2.2.1.4-1). In addition, SRv6 in Enhance Mode enables to route packets through intermediate IP routers on the path between two UPFs. |

Editor's Note: the considerations in the above table are based on the current content of the TR.

* * * End of Changes * * * *