3GPP TSG-CT WG4 Meeting #93	C4-193292

# Wroclaw, Poland; 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.003 | CR | 0543 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE radio capability ID format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 agreed at SA2#133 to introduce support for RACS in both EPS and 5GS in Rel-16. RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface. Consequently, the format of the UE radio capability ID needs to be defined in TS 23.003. SA2 concluded at the end of their study on RACS that the manufacturer-assigned UE radio capability ID is accompanied with the UE manufacturer information (e.g. TAC field), see the following text in TR 23.743 subclause 8: - Manufacturer-specific: The UE Capability ID may be assigned by the UE manufacturer in which case it is accompanied with the UE manufacturer information (e.g. TAC field in the PEI). This is also reflected in TS 23.501 by the following text in subclause 5.4.4.1a: When a PLMN decides to switch to operate based on manufacturer-assigned UE Radio Capability ID for a particular type of UE (e.g. based on TAC and SV): It is thus proposed that the manufacturer-assigned UE radio capability ID includes the TAC and SVN. Since the TAC and SVN in the IMEI are currently defined as strings of decimal digits (see TS 23.003 subclause 6.2), it is proposed to re-use the same format and to define the manufacturer-assigned UE radio capability ID as a string of decimal digits with variable length, with the first digit containing the type of UE radio capability ID (manufacturer-assigned), the next 8 digits containing the TAC, the next 2 digits containing the SVN, and the next n digits containing the radio configuration identifier. In order to enable operators who wish to do so to also include the TAC and SVN in network-assigned UE radio capability IDs (so as to enable e.g. deletion of a list of network-assigned UE radio capability IDs by signalling only the TAC+SV from the UCMF to the AMF/MME), it is further proposed that the network-assigned UE radio capability ID can also optionally include the TAC and SVN, and is thus also defined as a string a decimal digits with variable length, with the first digit containing the type of UE radio capability ID (network-assigned), optionally the first 8 digits containing the TAC, optionally the next 2 digits containing the SVNm and the next n digits containing the radio configuration identifier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The formats of the manufacturer-assigned UE radio capability ID and network-assigned UE radio capability ID were specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The format of the UE radio capability ID will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.2, 2X (New), 2X.1 (New), 2X.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 1.2	Abbreviations

For the purposes of the present document, the abbreviations defined in 3GPP TS 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-GUTI	5G Globally Unique Temporary Identifier

5GS	5G System

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

AMF	Access and Mobility Management Function

EPS	Evolved Packet System

ER	EAP Re-authentication

ERP	EAP Re-authentication Protocol

GUAMI	Globally Unique AMF Identifier

GUTI	Globally Unique Temporary UE Identity

ICS	IMS Centralized Services

MTC	Machine Type Communication

NCGI	NR Cell Global Identity

NCI	NR Cell Identity

OCS	Online Charging System

PEI	Permanent Equipment Identifier

RACS	Radio Capability Signalling Optimisation

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TWAP	Trusted WLAN AAA Proxy

UUID	Universally Unique Identifier

V2X	Vehicle-to-Everything

WebRTC	Web Real-Time Communication

WWSF	WebRTC Web Server Function

*** Next change ***

# 2X	Numbering, addressing and identification for RACS

## 2X.1	Introduction

This clause describes the format of the parameters used for Radio Capability Signalling Optimisation (RACS). For further information on the use of the parameters see 3GPP TS 23.401 [72] and 3GPP TS 23.501 [119].

## 2X.2	UE radio capability ID

The UE radio capability ID is composed as shown in figure 2X.2-1.

Figure 2X.2-1: Structure of UE radio capability ID

The UE radio capability ID is composed of the following elements (each element shall consist of decimal digits only):

1)	Type Field (TF): identifies the type of UE radio capability ID. The following values are defined:

-	0: manufacturer-assigned UE radio capability ID;

-	1: network-assigned UE radio capability ID with TAC and SVN fields;

-	2: network-assigned UE radio capability ID without TAC and SVN fields; and

-	3 to 9: spare values for future use.

2)	Type Allocation Code (TAC). Its length is 8 digits. This field is present only if the Type Field is set to 0 or 1;

3)	Software Version Number (SVN): identifies the software version number of the mobile equipment. Its length is 2 digits. This field is present only if the Type Field is set to 0 or 1;

4)	Radio Configuration Identifier (RCI): identifies the UE radio configuration. It has a variable length.

*** End of changes ***