3GPP TSG-CT WG4 Meeting #93	C4-193556

# Wroclaw, Poland; 26th – 30th August 2019

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	Pseudo-CR on Authentication vector generation in UDM

Spec:	3GPP TS 23.632 v0.3.0

Agenda item:	6.1.8

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

TR 23.732 concludes in favour of solution 2 that includes the option of vector generation in the UDM (see 23.732 subclause 6.2.3.3.2).

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.632 v0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.002: "Network Architecture".

[4]	3GPP TS 23.380: "IMS Restoration Procedures".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[xx]	3GPP TS 33.401: “3GPP System Architecture Evolution (SAE); Security architecture”.

[xx]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[xx]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[xx]	3GPP TS 33.220: "3G security; Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

* * * Next Change * * * *

### 4.2.1	General

A subscriber's authentication subscription data, including the subscriber’s long term key(s) and sequence number, shall be stored in a single repository so that a single sequence number can be maintained for the subscriber.

The subscriber’s long term key(s) shall not be transferred over Nxx reference point between HSS and UDM. Therefore,  subscriber's authentication vectors for subscribers with both 5G and EPS subscriptions shall be calculated in one single place in the system which is either the UDM/ARPF or the HSS. Subscription data stored in the 5GS-UDR and EPS-UDR or alternatively local configuration in the UDM and HSS indicates which option is to be used for a given subscriber.

The following clauses specify the system procedures for these different alternatives.

* * * Next Change * * * *

### 4.2.2	Vector Generation in HSS

This clause specifies the procedures for authentication vector request when the subscriber’s authentication subscription data is stored at the EPS-UDR. In this case, the UDM requests the generation of the Authentication Vector for 5GS to the HSS.

NOTE: 	The HSS acts as ARPF rather than as AuC and it is required to generate authentication vectors as defined in 3GPP TS 33.501 [6].

Editor’s Note: The possibility to collocate the ARPF in HSS (i.e. not within UDM as described in TS 33.501 [6]) needs to be confirmed by SA3.

When the UDM receives an authentication information Request from the AUSF it shall check (by means of an 5GS-UDR query or local configuration in the UDM) whether the subscribed authentication method is 5G_AKA or EAP_AKA_PRIME and if so whether 5G authentication vector generation for the identified subscriber shall be done in the HSS. If so, the UDM shall make use of the Nhss_UEAuthentication_Get service operation to retrieve a 5G authentication vector from the HSS.

Figure 4.2.2-1 shows the scenario where the authentication vector request for a 5G subscriber who also has an EPS subscription is received by the UDM.

Figure 4.2.2-1: Authentication for 5G subscriber with authentication vector generation in HSS

1.	The UDM receives an Authentication Vector request, containing the identity of the user (SUPI or SUCI). If SUCI is received, the UDM performs SUCI to SUPI de-concealment. For details of the Nudm_UEAuthentication Service see 3GPP TS 23.502 [5] and 3GPP TS 33.501 [6].

Editor’s Note: 	According to TS 33.501 [6], the ARPF holds the home network private key that is used by the SIDF to deconceal the SUCI and reconstruct the SUPI.  It needs to be confirmed if UDM would still be able to deconceal the SUPI in the case that the ARPF is colocated with the HSS rather than with the UDM.

2.	If the 5GS-UDR is used, the UDM queries the 5GS-UDR using the SUPI to retrieve Authentication Subscription Information. In this scenario the Authentication Subscription Information contains a subscribed authentication method of 5G_AKA or EAP_AKA_PRIME and an indicator indicating that authentication vector generation shall be performed in the HSS. Optionally, the indication that the authentication vector generation shall be performed in the HSS could be locally configured at the UDM/ARPF.

3.	The UDM uses the Nhss_UEAuthentication_Get service operation to retrieve an authentication vector from the HSS. The request contains the IMSI the authentication method and serving network name.

4.	The HSS reads authentication subscription data from the EPS-UDR. This step is omitted if all relevant authentication subscription data are stored locally in the HSS.

5.	The HSS (AuC/ARPF) calculates the requested authentication vector taking into account the serving network name and authentication method received in step 3 and the authentication subscription information retrieved from the EPS-UDR.

6.	The calculated authentication vector is sent to the UDM.

7.	The HSS updates the EPS-UDR with the new sequence number. This step is omitted if the sequence number is stored locally in the HSS.

8.	The UDM forwards the authentication vector to the AUSF.

* * * Next Change * * * *

### 4.2.3	Vector Generation in UDM/ARPF

This clause specifies the procedures for authentication vector request when the subscriber’s authentication subscription data is stored at the 5GS-UDR. In this case, the HSS requests the generation of the Authentication Vector for EPS and/or IMS to the UDM.

NOTE: 	The UDM acts as AuC rather than as ARPF and it is required to generate authentication vectors as defined in 3GPP TS 33.401 [xx], 3GPP TS 33.402 [xx], 3GPP TS 33.203 [xx] and 3GPP TS 33.220 [xx].

When the HSS receives an authentication vector request from a serving node (e.g. MME, SGSN, VLR, S-CSCF, BSF) it shall check (by means of an EPC-UDR query) whether authentication vector generation for the identified subscriber shall be done in the UDM. If so, the HSS shall make use of the Nudm_UEAuthentication Get service operation to retrieve authentication vectors from the UDM.

Figure 4.2.3-1 shows the scenario where an authentication vector request for a subscriber is received by the HSS and subscription data stored in the EPS-UDR indicate that for the subscriber authentication vector generation is to be performed in the UDM.

Figure 4.2.3-1: Authentication for a subscriber with authentication vector generation in UDM

1.	The HSS receives an Authentication Vector request, containing the identity of the user (IMSI, or Public User Identity and/or Private User Identity).

2.	The HSS queries the EPS-UDR using the identity of the user to retrieve Authentication Subscription Information. In this scenario the Authentication Subscription Information contains an indicator indicating that authentication vector generation shall be performed in the UDM.

3.	The HSS uses the Nudm_UEAuthentication Get service operation to retrieve an authentication vector from the UDM. The request contains the identity of the user, the type of the requested vector (E-UTRAN/UTRAN or GERAN/ IMS-AKA and when available the visited PLMN-ID.

4.	The UDM reads authentication subscription data from the 5GS-UDR.

5.	The UDM (ARPF) calculates the requested authentication vectors taking into account the information received in step 3 and the authentication subscription information retrieved from the 5GS-UDR.

6.	The calculated authentication vectors are sent to the HSS.

7.	The UDM updates the 5GS-UDR with the new sequence number.

8.	The HSS forwards the authentication vectors to the serving node.

* * * Next Change * * * *

### 5.1.2	Nhss_UEAuthentication service

#### 5.1.2.1	Nhss_UEAuthentication_Get service operation

Service operation name: Nhss_UEAuthentication_Get

Description: Requester NF gets the authentication vector from HSS. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

Inputs, Required: IMSI, authentication method, serving network name.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Authentication vector.

Outputs, Optional: None.

* * * Next Change * * * *

### 5.2.1	General

The following table illustrates the UDM Services.

Table 5.2.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| UECM | P-CSCF-RestorationTrigger | Request/Response | HSS |
|  | Get | Request/Response | HSS |
| MT | ProvideDomainSelectionInfo | Request/Response | HSS |
| EventExposure | Subscribe | Subscribe/Notify | HSS |
|  | Notify |  |  |
| UEAU | Get | Request/Response | HSS |

* * * Next Change * * * *

### 5.2.X	Nudm_UEAuthentication service

#### 5.2.X.1	Nudm_UEAuthentication_Get service operation

Service operation name: Nudm_UEAuthentication_Get

Description: Requester NF gets the authentication vector from UDM. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

Inputs, Required: SUPI, authentication method (e.g. 5G-AKA, EAP-AKA’, EPS-AKA, IMS-AKA), serving network name.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Authentication vector.

Outputs, Optional: None.

* * * End Of Change * * * *