3GPP TSG-CT WG4 Meeting #93	C4-193616

# Wroclaw, Poland; 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0039 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Binding between NF Service Consumer and NF Service Producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-07-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 6.3.1.0 of TS 23.501 introduces requirements and principles for binding, selection and reselection. Binding can be used to indicate suitable target NF producer instance(s) for NF service instance selection, reselection and routing of subsequent requests associated with a specific context. This allows the NF producer to indicate that the NF consumer, for a particular context, should be bound to an NF service instance, NF instance, NF service set or NF set depending on local policies and other criteria (e.g. at what point it is in the middle of a certain procedure, considering performance aspects etc). The NF service producer may provide a binding indication to the NF service consumer as part of the Direct or Indirect Communication procedures, to be used in subsequent related service requests. The level of binding indication provided by the NF service producer to the NF consumer indicates if the producer is either bound to NF service instance, NF instance, NF Service Set or NF set as specified in Table 6.3.1.0-1. The binding indication may include NF Service Set ID, NF Set ID, NF instance ID, or NF service instance ID, for use by the NF consumer or SCP for NF Service Producer (re-)selection. If the resource is created in the NF Service Producer, the NF Service Producer provides resource information which includes the endpoint address of the NF service producer. Clause 4.17.12 of TS 23.502 specifies the corresponding call flows for Binding between NF service consumer and NF service producer: - Binding created as part of service response: Producer may return a binding information to the consumer in a service response; consumer stores the received binding and uses it for subsequent related requests. Producer may update the binding indication and resource information in subsequent responses. - Binding created as part of service request: Consumer may include binding information if it can also be a NF service producer for later communication from the contacted producer. - Binding for subscription requests: Binding can be created as part of the implicit subscription request: in this case, Producer returns a binding indication in the response to the service request that creates the implicit subscription, to be used for subsequent operations on the subscription. The stage 3 requirements to support binding need to be specified. A binding indication can be sent in a service request or response, with or without a payload body (e.g. 204 No Content, DELETE request). It may be used by the NF service consumer or SCP for the selection or reselection of the NF Service Producer. Besides, a request or response may include different types of binding information, e.g. a NF Service Producer may send a response with a producer binding information and an implicit subscription binding information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new 3GPP custom header "3gpp-Sbi-Binding" is defined, that allows to signal binding information in an HTTP request or response, including: - the binding type, i.e. whether the binding information refers to the NF Service producer, NF Service consumer or to an implicit subscription; - the binding level, i.e. a binding to either a NF Service Instance, a NF Service Set, a NF Instance or a NF set; - one or more Binding ID values (NF Instance ID, NF Set ID, NF Service Instance ID and/or NF Service Set ID). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.1, 5.2.3.2.x (new), 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.003 CR 0536 TS 23.003 CR 0537 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | With binding indication defined as a new 3GPP custom header, no further change should be needed in the different API TS to support binding, assuming they already reference the mandatory headers to support specified in TS 29.500. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: An Editor's note is added regarding how to encode multiple binding indications of different types in a same message. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.2.3.2	Mandatory to support custom headers

#### 5.2.3.2.1	General

The 3GPP NF Services shall support the HTTP custom headers specified in Table 5.2.3.2-1 below. A description of each custom header and the normative requirements on when to include them are also provided in Table 5.2.3.2-1.

Table 5.2.3.2-1: Mandatory HTTP custom headers

| Name | Reference | Description |
| --- | --- | --- |
| 3gpp-Sbi-Message-Priority | Subclause 5.2.3.2.1 | This header is used to specify the HTTP/2 message priority for 3GPP service based interfaces. This header shall be included in HTTP/2 messages when a priority for the message needs to be conveyed (e.g HTTP/2 messages related to Multimedia Priority Sessions). |
| 3gpp-Sbi-Callback | Subclause 5.2.3.2.3 | This header is used to indicate if a HTTP/2 message is a callback (e.g notification). This header shall be included in HTTP POST messages for callbacks towards NF service consumer(s) in another PLMN via the SEPP (See 3GPP TS 29.573 [27]). |
| 3gpp-Sbi-Binding | Subclause 5.2.3.2.x | This header is used to indicate a binding between a NF service consumer and a NF service producer (see clause 6.x). |

* * * Next Change * * * *

#### 5.2.3.2.x	3gpp-Sbi-Binding

The header contains a binding indication (see clause 6.x.2).

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Binding = "3gpp-Sbi-Binding" ":" OWS "bt=" btvalue ";" "bl=" blvalue 1*( OWS ";" parameter)

btvalue			= "consumer" / "producer" / "subscription"

blvalue			= "nfinstance" / "nfset" / "nfserviceinstance" / "nfserviceset"

parameter			 =  parametername "=" token

parametername	= "nfinst" / "nfset" / "nfservinst" / "nfserviceset"

The following parameters are defined:

- bt (binding type): indicates whether the binding information refers to the NF Service producer, NF Service consumer or to an implicit subscription respectively.

- bl (binding level): indicates a binding to either a NF Instance, a NF set, a NF Service Instance or a NF Service Set.

- nfinst (NF instance): indicates a NF Instance ID, as defined in 3GPP TS 29.510 [8].

- nfset (NF set): indicates a NF Set ID, as defined in 3GPP TS 23.003 [15].

- nfservinst (NF service instance): indicates a NF Service Instance ID

- nfserviceset (NF service set): indicates a NF Service Set ID, as defined in 3GPP TS 23.003 [15].

EXAMPLE 1:	Binding to SMF set "345_12_SMF_set1": 

3gpp-Sbi-Binding: bt=producer; bl=nfset; nfset=345_12_SMF_set1

EXAMPLE 2:	Binding to an SMF instance within SMF set "345_12_SMF_set1": 

3gpp-Sbi-Binding: bt=producer; bl=nfinstance; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset=345_12_SMF_set1

EXAMPLE 3:	Binding to a SMF Service Set within an SMF instance within SMF set "345_12_SMF_set1"

3gpp-Sbi-Binding: bt=producer; bl=nfserviceset; nfservset=345_12_54804518-4191-46b3-955c-ac631f953ed8_nsmf-pdusession_set-3; nfinst=54804518-4191-46b3-955c-ac631f953ed8; nfset=345_12_SMF_set1

EXAMPLE 4:	Binding to AMF set 1 within AMF region 48 (hexadecimal)
3gpp-Sbi-Binding: bt=producer; bl=nfset; nfset=345_12_AMF_region48-set001

Editor's note:	how to encode multiple binding indications of different types in a same message is FFS, e.g. the ABNF of the binding header could allow concatenating different binding indications in a same binding header.

* * * Next Change * * * *

## 6.x	Binding between NF Service Consumer and NF Service Producer

### 6.x.1	General

A NF Service Producer may provide a binding indication to a NF Service Consumer as part of the Direct or Indirection Communication procedures, to be used in subsequent related service requests. This allows the NF Service Producer to indicate that the NF Service Consumer, for a particular context, should be bound to an NF service instance, NF instance, NF service set or NF set. See clause 6.3.1.0 of 3GPP TS 23.501 [3] and clause 4.17.12 of 3GPP TS 23.502 [4].

A binding may be established or updated as part of a:

1)	service response creating or modifying a resource, to be used for subsequent requests targeting the resource (see clause 4.17.12.2 of 3GPP TS 23.502 [4]), for any API that defines resources;

2)	service request, if the NF Service Consumer can also act as an NF Service Producer for later communication from the contacted NF Service Producer, to be used for subsequent service requests initiated by the contacted NF Service Producer (see clause 4.17.12.3 of 3GPP TS 23.502 [4]);

3)	service response creating an implicit subscription, to be used for subsequent operations on the subscription (see clause 4.17.12.4 of 3GPP TS 23.502 [4]).

### 6.x.2	Binding Indication

A binding indication shall be provided in a service request or response by including a 3gpp-Sbi-Binding header (see clause 5.2.3.2.x.) in the HTTP request or response with:

-	the binding type (bt) parameter set to "producer", "consumer" or "subscription" to indicate whether the binding information refers to the NF Service producer, NF Service consumer or to an implicit subscription respectively;

-	the binding level (bl) parameter indicating a binding to either a NF Service Instance, a NF Service Set, a NF Instance or a NF set;

-	at least one of the NF Service Instance (nfservinst), NF Service Set (nfservset), NF instance (nfinst) and NF Set (nfset) parameters, set to a NF Service Instance ID, NF Service Set ID, NF Instance ID and NF Set ID respectively, as described in Table 6.3.1.0-1 of 3GPP TS 23.502 [4].

NOTE 1:	A binding indication can be part of a HTTP request or response with or without payload body, e.g. in a 204 No Content or in a DELETE request.

NOTE 2:	The binding type value "consumer" is only used in a service request and for binding information referring to the consumer, i.e. in case 2) of clause 6.x.1. When later on the contacted NF Service Producer initiates service request for services offered by the NF instance that was acting as a NF Service Consumer, the binding type is set to "producer".

A service request or response may include multiple binding indications with different binding types, i.e.:

-	a NF Service Consumer may send a request to a NF Service Producer including a producer binding information received earlier and a consumer binding information;

-	a NF Service Producer may send a response with a producer binding information and an implicit subscription binding information.

* * * End of Changes * * * *