3GPP TSG-CT WG4 Meeting #94	C4-194023

# Portoroz, Slovenia; 07th – 11th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.003 | CR | 0544 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Format of NF (Service) Set ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-07-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The definition and structure of the NF Set ID and NF Service Set ID have been agreed and specified in TS 23.003, but the exact format was left for FFS. It was questioned in particular whether an FQDN format would be more suitable. The permitted character set for the Set ID part was also left for further study. NF Set ID and NF Service Set ID are identifiers that in general do not need to be resolvable via DNS. In specific cases, an FQDN may be used, e.g. for an SMF Set where the SMFs of an SMF Set may be discovered by the UPF using DNS procedures. Whether other NF (Service) Set ID are provisioned in DNS is up to operator’s deployment but is not necessarily required. Also the case of alphabetic characters needs to be significant in these identifiers (e.g. for use in URI query parameters), whereas it is not for FQDN. Thus it is proposed to format these IDs as string, with a format that allows to easily derive an FQDN when necessary, and that allows to be easily extended e.g. to add a NID for Standalone Non-Public Networks. AMF Set ID has also been defined as a string in Rel-15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NF Set ID and NF Service Set ID are formatted as strings: set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC> set<Set ID>.sn<Service Name>.nfi<NF Instance ID>.5gc.mnc<MNC>.mcc<MCC> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 28.3.2.9, 28.12, 28.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 28.3.2.9	SMF Set FQDN

An SMF Set within an operator's network is identified by its NF Set ID as defined in clause 28.12, with NFType set to "SMF".

A subdomain name shall be derived from the MNC and MCC by adding the label "SMFset" to the beginning of the Home Network Realm/Domain (see clause 28.2).

The SMF Set FQDN shall be constructed as follows:

set<Set Id>.SMFset.5gc.mnc<MNC>.mcc<MCC>.3gppnetwork.org

where

-	<MNC> = 3 digits

-	<MCC> = 3 digits

If there are only 2 significant digits in the MNC, one "0" digit shall be inserted at the left side to fill the 3 digits coding of MNC in the AMF Set FQDN.

-	<Set Id> is the string representing the Set ID part within the NF Set ID defined in clause 28.12.

EXAMPLE:	"set12. SMFset.5gc.mnc012.mcc345.3gppnetwork.org" (for the SMF set from MCC 345, MNC 12 and SetID "12")

NOTE:	The labels preceding the ".3gppnetwork.org" domain correspond to the NF Set ID definition in clause 28.12.

* * * Next Change * * * *

## 28.12	NF Set Identifier (NF Set ID)

A NF Set Identifier is a globally unique identifier of a set of equivalent and interchangeable CP NFs from a given network that provide distribution, redundancy and scalability (see clause 5.21.3 of 3GPP TS 23.501 [119]).

An NF Set Identifier shall be constructed from the MCC, MNC, NF type and a Set ID.

A NF Set Identifier shall be formatted as the following string:

set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>

where:

-	the <MCC> and <MNC> shall identify the PLMN of the NF Set and shall be encoded as follows:

-	<MCC> = 3 digits

-	<MNC> = 3 digits

If there are only 2 significant digits in the MNC, one "0" digit shall be inserted at the left side to fill the 3 digits coding of MNC.

-	the <NFType> shall identify the NF type of the NFs within the NF set, as defined by 3GPP TS 29.510 [130];

-	the Set ID shall be a NF type specific Set ID within the PLMN, chosen by the operator, that shall consist of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit;

-	the case of alphabetic characters is significant. 

For an AMF set, the Set ID shall be set to "<AMF Set ID>.region<AMF Region ID>", with the AMF Region ID and AMF Set ID encoded as defined in 3GPP TS 29.571 [y].

EXAMPLE 1:	setxyz.SMFset.5gc.mnc012.mcc345

EXAMPLE 2:	set12.PCFset.5gc.mnc012.mcc345

EXAMPLE 3:	set1.region48.AMFset.5gc.mnc012.mcc345 (for AMF Region 48 (hexadecimal) and AMF Set 1)

NOTE:	If needed, an FQDN can be derived from a given NF Set ID by appending the ".3gppnetwork.org" domain to the NF Set ID, see e.g. SMF Set FQDN in clause 28.3.2.9.

* * * Next Change * * * *

## 28.13	NF Service Set Identifier (NF Service Set ID)

A NF Service Set Identifier is a globally unique identifier of a set of equivalent and interchangeable CP NF service instances within a NF instance from a given network that provide distribution, redundancy and scalability (see clause 5.21.3 of 3GPP TS 23.501 [119]).

An NF Service Set Identifier shall be constructed from the MCC, MNC, NF instance Identifier, service name and a Set ID.

A NF Service Set Identifier shall be formatted as the following string:

set<Set ID>.sn<Service Name>.nfi<NF Instance ID>.5gc.mnc<MNC>.mcc<MCC>

where:

-	the <MCC> and <MNC> shall identify the PLMN of the NF Service Set and shall be encoded as follows:

-	<MCC> = 3 digits

-	<MNC> = 3 digits

If there are only 2 significant digits in the MNC, one "0" digit shall be inserted at the left side to fill the 3 digits coding of MNC.

-	the NFInstanceID shall identify the NF instance of the NF Service set, as defined by 3GPP TS 23.501 [119] and 3GPP TS 29.510 [130];

-	the ServiceName shall identify the NF service of the NF Service set, as defined by 3GPP TS 29.510 [130];

-	the Set ID shall be a service specific Set ID within the NF instance, chosen by the operator that shall consist of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit;

-	the case of alphabetic characters is significant.

EXAMPLE 1:	setxyz.snnsmf-pdusession.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345

EXAMPLE 2:	set2.snnpcf-smpolicycontrol.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345

NF service instances from different NF instances are equivalent NF service instances if they share the same MCC, MNC, ServiceName and Set ID.

* * * End of Changes * * * *