3GPP TSG-CT WG4 Meeting #94	C4-194146

# Portoroz, Slovenia; 07th – 11th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | CR0311 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to pre-defined rule handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | CUPS-CT |  |  |  |  |  | Date: |  |  | 2019-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 29.244 actually specifies pre-defined rule handling, but for some resaon clause 5.19 refers to pre-defined PDRs (see DISC in C4-194145 for more details). Number of statements should be updated by perlacing "pre-defined PDRs" with "pre-defined rules". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pre-defiend rule definition was added and statements in clause 5.19 and 8.2.25 were corrected to refer to "pre-defined rules". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in pre-defined rule handling that may lead to interopetion issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.19, 8.2.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Match Field: a field of the Packet Detection Information of a Packet Detection Rule against which a packet is attempted to be matched.

Matching: comparing the set of header fields of a packet to the match fields of the Packet Detection Information of a Packet Detection Rule.

CP function: A node with a Control Plane function (see 3GPP TS 23.214[2]) supporting one or more PFCP entities. A Control Plane function, i.e. a Control Plane Node, is identified by the Node ID, that is set to either an FQDN or an IP address.

UP function: A node with a User Plane function (see 3GPP TS 23.214[2]) supporting one or more PFCP entities. A User Plane function, i.e. a User Plane Node, is identified by the Node ID, that is set to either a FQDN or an IP address.

PFCP Entity: An endpoint in a CP (or UP) function supporting PFCP, that is identified by the IP address. The IP address of a PFCP entity may or may not be the IP address included in the Node ID.

Pre-defined rules: A rule, e.g. FAR, QER, etc., which is pre-configured in UP function. In order to activate, or deactivate the pre-defined rule, CP function needs to send the name of the required rule to the UP function.

* * * 2nd Change * * * *

## 5.19	Activation and Deactivation of Pre-defined Rules

To reduce the signalling overhead for establishing a PFCP session (for a PDU session or a PDN connection) and improve the signalling efficiency, the CP and UP functions may support the Activation and Deactivation of a Pre-defined rules (ADPDR) feature as described below.

NOTE:	CP function does no need to signal ADPDR support to UPF during the PFCP Association Setup procedure. If UPF indicates ADPDR support, the CP function may activate and deactivate pre-defined rules at any time by signalling the pre-defined rule name with Activate Predefined Rules IE.

When both the CP function and the UP function support the ADPDR feature, the CP function may activate one or more pre-defined rules for a PFCP session during a PFCP Session Establishment or a PFCP Session Modification procedure. A pre-defined rule shall be configured in the UP function before it can be activated in a PFCP session.

A pre-defined rule may contain all the necessary packets detection information to identify a service data flow or application traffic which may be common to many PFCP sessions, and it may be configured in the UPF associated with a pre-defined FAR, one or more pre-defined QER(s), and/or one or more pre-defined URR(s).

Any PFCP session specific information, e.g. traffic endpoint information, may not be part of a pre-defined rule and is provisioned before or during the activation of the pre-defined rule.

To activate one or more pre-defined rules, the CP function shall provide one or more Activate Predefined Rules IE(s) in a Create PDR IE in a PFCP Session Establishment Request, or in a Create PDR IE or an Update PDR IE in a PFCP Session Modification Request message, that references a pre-defined rule configured in the UP function, with the following information in the Create PDR IE or Update PDR IE:

-	the traffic endpoint that the traffic shall match (e.g. Local F-TEID, UE IP Address or Traffic Endpoint ID);

-	the QFI that the traffic shall match, e.g. for a UL PDR where UL QoS flow binding verification is required (see clause 5.4.2);

-	the precedence to be applied by the UP function among all PDRs of the PFCP session;

-	a FAR containing instructions related to the processing of the packets matching the pre-defined PDR(s); when present, the UP function shall enforce it instead of the one defined in the pre-defined PDR(s) if any;

-	one or more URRs to be used in addition to any URR(s) specified in the pre-defined PDR(s) (e.g. for session level Usage Monitoring);

-	 one or more QERs to be applied in addition to any QER(s) specified in the pre-defined PDR (e.g. for APN-AMBR enforcement).

When a pre-defined rule is activated for a given PDR, an incoming packet matches the PDR if it matches the traffic endpoint, and the QFI if provisioned and one of the activated pre-defined PDR(s).

The CP function may update the use of pre-defined rules that are already activated in a PFCP session by including one or more Activate Predefined Rules IE(s) or Deactivate Predefined Rules IE(s) in a PFCP Session Modification Request and/or by updating the parameters provisioned in the PDR.

NOTE:	The CP function can not change a pre-defined PDR via PFCP session related procedure.

The CP function may deactivate a pre-defined rule that is already activated in a PFCP session by including the Deactivate Predefined Rules IE in a PFCP Session Modification Request requesting to deactivate the pre-defined rules.

In addition, this feature allows to define a group of pre-defined rules which can be activated, updated, and deactivated together. This allows the CP function to further optimize the signaling towards the UP function.

To activate, update, or deactivate a group of pre-defined rules, the CP shall follow the same procedure as for activating, updating, and deactivating a pre-defined rules, but it shall use an Activate Predefined Rules IE that refers to a group of pre-defined rules.

* * * 3rd Change * * * *

### 8.2.25	UP Function Features

The UP Function Features IE indicates the features supported by the UP function. It is coded as depicted in Figure 8.2.25-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 43 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 6 | Supported-Features |  |  |  |  |  |  |  |  |
|  | 7 to 8 | Additional Supported-Features 1 |  |  |  |  |  |  |  |  |
|  | 9 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.25-1: UP Function Features

The UP Function Features IE takes the form of a bitmask where each bit set indicates that the corresponding feature is supported. Spare bits shall be ignored by the receiver. The same bitmask is defined for all PFCP interfaces.

The following table specifies the features defined on PFCP interfaces and the interfaces on which they apply.

Table 8.2.25-1: UP Function Features

| Feature Octet / Bit | Feature | Interface | Description |
| --- | --- | --- | --- |
| 5/1 | BUCP | Sxa, N4 | Downlink Data Buffering in CP function is supported by the UP function. |
| 5/2 | DDND | Sxa, N4 | The buffering parameter 'Downlink Data Notification Delay' is supported by the UP function. |
| 5/3 | DLBD | Sxa, N4 | The buffering parameter 'DL Buffering Duration' is supported by the UP function. |
| 5/4 | TRST | Sxb, Sxc, N4 | Traffic Steering is supported by the UP function. |
| 5/5 | FTUP | Sxa, Sxb, N4 | F-TEID allocation / release in the UP function is supported by the UP function. |
| 5/6 | PFDM | Sxb, Sxc, N4 | The PFD Management procedure is supported by the UP function. |
| 5/7 | HEEU | Sxb, Sxc, N4 | Header Enrichment of Uplink traffic is supported by the UP function. |
| 5/8 | TREU | Sxb, Sxc, N4 | Traffic Redirection Enforcement in the UP function is supported by the UP function. |
| 6/1 | EMPU | Sxa, Sxb, N4 | Sending of End Marker packets supported by the UP function. |
| 6/2 | PDIU | Sxa, Sxb, Sxc, N4 | Support of PDI optimised signalling in UP function (see clause 5.2.1A.2). |
| 6/3 | UDBC | Sxb, Sxc, N4 | Support of UL/DL Buffering Control |
| 6/4 | QUOAC | Sxb, Sxc, N4 | The UP function supports being provisioned with the Quota Action to apply when reaching quotas. |
| 6/5 | TRACE | Sxa, Sxb, Sxc, N4 | The UP function supports Trace (see clause 5.15). |
| 6/6 | FRRT | Sxb, N4 | The UP function supports Framed Routing (see IETF RFC 2865 [37] and IETF RFC 3162 [38]). |
| 6/7 | PFDE | Sxb, N4 | The UP function supports a PFD Contents including a property with multiple values. |
| 6/8 | EPFAR | Sxa, Sxb, Sxc, N4 | The UP function supports the Enhanced PFCP Association Release feature (see clause 5.18). |
| 7/1 | DPDRA | Sxb, Sxc, N4 | The UP function supports Deferred PDR Activation or Deactivation. |
| 7/2 | ADPDP | Sxa, Sxb, Sxc, N4 | The UP function supports the Activation and Deactivation of Pre-defined rules (see clause 5.19). |
| 7/3 | UEIP | N4 | The UPF supports allocating UE IP addresses or prefixes (see clause 5.21). |
| 7/4 | SSET | N4 | UPF support of PFCP sessions successively controlled by different SMFs of a same SMF Set (see clause 5.22). |
| 7/5 | MNOP | Sxa, Sxb, Sxc, N4 | UPF supports measurement of number of packets which is instructed with the flag 'Measurement of Number of Packets' in a URR. See also 5.2.2.2.1. |
| 7/6 | MTE | N4 | UPF supports multiple instances of Traffic Endpoint IDs in a PDI. |
| 7/7 | BUNDL | Sxa, Sxb, Sxc, N4 | PFCP messages bunding (see clause 6.5) is supported by the UP function. |
| 7/8 | GCOM | N4 | UPF support of 5G VN Group Communication. (See clause 5.23) |
| 8/1 | MPAS | N4 | UPF support for multiple PFCP associations to the SMFs in an SMF set (see clause 5.22.3). |
| 8/2 | RTTL | N4 | The UP function supports redundant transmission at transport layer. |
| 8/3 | VTIME | Sxb,N4 | UPF support of quota validity time feature. |
| Feature Octet / Bit: The octet and bit number within the Supported-Features IE, e.g. "5 / 1". Feature: A short name that can be used to refer to the octet / bit and to the feature. Interface: A list of applicable interfaces to the feature. Description: A clear textual description of the feature. |  |  |  |

* * * End of Changes * * * *