3GPP TSG-CT WG4 Meeting #94	C4-194415

Portoroz, Slovenia; 07th – 11th October 2019 	was C4-194176

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.336 | CR | 0157 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Battery Indication for Communication pattern enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNAPIs |  |  |  |  |  | Date: |  |  | 2019-9-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 23.682 the Communication Pattern parameters provisioning procedure is enhanced to support Battery indication. The Battery indication identifies power consumption criticality for the UE. It indicates if the UE is battery powered with not rechargeable/not replaceable battery, battery powered with rechargeable/replaceable battery, or not battery powered. 23.682 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Battery indicator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Battery indicator is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1, 8.4.26, 8.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR will introduce backward compatible new feature. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Correct typos on the cover page. - Correct typos and the format problems in 8.4.x. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.4.1	General

The following table specifies the Diameter AVPs defined for the S6t interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32, bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x00000001 should be used.

Table 8.4.1-1: S6t specific Diameter AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| AESE-Communication-Pattern | 3113 | 8.4.25 | Grouped | M,V |  |  |  | No |
| Communication-Pattern-Set | 3114 | 8.4.26 | Grouped | M,V |  |  |  | No |
| Periodic-Communication-Indicator | 3115 | 8.4.27 | Unsigned32 | M,V |  |  |  | No |
| Communication-Duration-Time | 3116 | 8.4.28 | Unsigned32 | M,V |  |  |  | No |
| Periodic-time | 3117 | 8.4.29 | Unsigned32 | M,V |  |  |  | No |
| Scheduled-Communication-Time | 3118 | 8.4.30 | Grouped | M,V |  |  |  | No |
| Stationary-Indication | 3119 | 8.4.31 | Unsigned32 | M,V |  |  |  | No |
| AESE-Communication-Pattern-Config-Status | 3120 | 8.4.32 | Grouped | M,V |  |  |  | No |
| AESE-Error-Report | 3121 | 8.4.33 | Grouped | M,V |  |  |  | No |
| Monitoring-Event-Configuration | 3122 | 8.4.2 | Grouped | M,V |  |  |  | No |
| Monitoring-Event-Report | 3123 | 8.4.3 | Grouped | M,V |  |  |  | No |
| SCEF-Reference-ID | 3124 | 8.4.4 | Unsigned32 | M,V |  |  |  | No |
| SCEF-ID | 3125 | 8.4.5 | DiameterIdentity | M,V |  |  |  | No |
| SCEF-Reference-ID-for-Deletion | 3126 | 8.4.6 | Unsigned32 | M,V |  |  |  | No |
| Monitoring-Type | 3127 | 8.4.7 | Unsigned32 | M,V |  |  |  | No |
| Maximum-Number-of-Reports | 3128 | 8.4.8 | Unsigned32 | M,V |  |  |  | No |
| UE-Reachability-Configuration | 3129 | 8.4.9 | Grouped | M,V |  |  |  | No |
| Monitoring-Duration | 3130 | 8.4.10 | Time | M,V |  |  |  | No |
| Maximum-Detection-Time | 3131 | 8.4.11 | Unsigned32 | M,V |  |  |  | No |
| Reachability-Type | 3132 | 8.4.12 | Unsigned32 | M,V |  |  |  | No |
| Maximum Latency | 3133 | 8.4.13 | Unsigned32 | M,V |  |  |  | No |
| Maximum Response Time | 3134 | 8.4.14 | Unsigned32 | M,V |  |  |  | No |
| Location-Information-Configuration | 3135 | 8.4.15 | Grouped | M,V |  |  |  | No |
| MONTE-Location-Type | 3136 | 8.4.16 | Unsigned32 | M,V |  |  |  | No |
| Accuracy | 3137 | 8.4.17 | Unsigned32 | M,V |  |  |  | No |
| Association-Type | 3138 | 8.4.18 | Unsigned32 | M,V |  |  |  | No |
| Roaming-Information | 3139 | 8.4.19 | Unsigned32 | M,V |  |  |  | No |
| Reachability-Information | 3140 | 8.4.20 | Unsigned32 | M,V |  |  |  | No |
| IMEI-Change | 3141 | 8.4.22 | Unsigned32 | M,V |  |  |  | No |
| Monitoring-Event-Config-Status | 3142 | 8.4.24 | Grouped | M,V |  |  |  | No |
| Supported-Services | 3143 | 8.4.40 | Grouped | M,V |  |  |  | No |
| Supported-Monitoring-Events | 3144 | 8.4.41 | Unsigned64 | M,V |  |  |  | No |
| CIR-Flags | 3145 | 8.4.39 | Unsigned32 | M,V |  |  |  | No |
| Service-Result | 3146 | 8.4.37 | Grouped | M,V |  |  |  | No |
| Service-Result-Code | 3147 | 8.4.38 | Unsigned32 | M,V |  |  |  | No |
| Reference-ID-Validity-Time | 3148 | 8.4.42 | Time | M,V |  |  |  | No |
| Event-Handling | 3149 | 8.4.43 | Unsigned32 | M,V |  |  |  | No |
| NIDD-Authorization-Request | 3150 | 8.4.44 | Grouped | M,V |  |  |  | No |
| NIDD-Authorization-Response | 3151 | 8.4.45 | Grouped | M,V |  |  |  | No |
| Service-Report | 3152 | 8.4.47 | Grouped | M,V |  |  |  | No |
| Node-Type | 3153 | 8.4.48 | Unsigned32 | M,V |  |  |  | No |
| S6t-HSS-Cause | 3154 | 8.4.50 | Unsigned32 | M,V |  |  |  | No |
| Enhanced-Coverage-Restriction | 3155 | 8.4.51 | Grouped | V |  |  | M | No |
| Enhanced-Coverage-Restriction-Data | 3156 | 8.4.52 | Grouped | V |  |  | M | No |
| Restricted-PLMN-List | 3157 | 8.4.53 | Grouped | V |  |  | M | No |
| Allowed-PLMN-List | 3158 | 8.4.54 | Grouped | V |  |  | M | No |
| Requested-Validity-Time | 3159 | 8.4.55 | Time | V |  |  | M | No |
| Granted-Validity-Time | 3160 | 8.4.56 | Time | V |  |  | M | No |
| NIDD-Authorization-Update | 3161 | 8.4.57 | Grouped | V |  |  | M | No |
| Loss-Of-Connectivity-Reason | 3162 | 8.4.58 | Unsigned32 | V |  |  | M | No |
| Group-Reporting-Guard-Timer | 3163 | 8.4.59 | Unsigned32 | V |  |  | M | No |
| CIA-Flags | 3164 | 8.4.60 | Unsigned32 | V |  |  | M | No |
| Group-Monitoring-Event-Report | 3165 | 8.4.61 | Grouped | V |  |  | M | No |
| Group-Monitoring-Event-Report-Item | 3166 | 8.4.62 | Grouped | V |  |  | M | No |
| RIR-Flags | 3167 | 8.4.63 | Unsigned32 | V |  |  | M | No |
| Type-Of-External-Identifier | 3168 | 8.4.64 | Unsigned32 | V |  |  | M | No |
| APN-Validity-Time | 3169 | 8.4.65 | Grouped | V |  |  | M | No |
| Suggested-Network-Configuration | 3170 | 8.4.66 | Grouped | V |  |  | M | No |
| Monitoring-Event-Report-Status | 3171 | 8.4.67 | Grouped | V |  |  | M | No |
| PLMN-ID-Requested | 3172 | 8.4.68 | Enumerated | V |  |  | M | No |
| AdditionalIdentifiers | 3173 | 8.4.69 | Grouped | V |  |  | M | No |
| NIR-Flags | 3174 | 8.4.70 | Unsigned32 | V |  |  | M | No |
| Reporting-Time-Stamp | 3175 | 8.4.71 | Time | V |  |  | M | No |
| NIA-Flags | 3176 | 8.4.72 | Unsigned32 | V |  |  | M | No |
| Group-User-Identifier | 3177 | 8.4.73 | Grouped | V |  |  | M | No |
| MTC-Provider-Info | 3178 | 8.4.74 | Grouped | V |  |  | M | No |
| MTC-Provider-ID | 3179 | 8.4.75 | UTF8String | V |  |  | M | No |
| PDN-Connectivity-Status-Configuration | 3180 | 8.4.76 | Grouped | V |  |  | M | No |
| PDN-Connectivity-Status-Report | 3181 | 8.4.77 | Grouped | V |  |  | M | No |
| PDN-Connectivity-Status-Type | 3182 | 8.4.78 | Unsigned32 | V |  |  | M | No |
| Traffic-Profile | 3183 | 8.4.79 | Unsigned32 | V |  |  | M | No |
| Battery-Indicator | x1 | 8.4.x | Unsigned32 | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M" indicates whether support of the AVP is required. The AVP header bit denoted as "V" indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [23]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table specifies the Diameter AVPs re-used by the S6t interface protocol from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within S6t.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter base protocol specified in IETF RFC 6733 [23], do not need to be supported. The AVPs from Diameter base protocol specified in IETF RFC 6733 [23] are not included in table 8.4.1-2, but they may be re-used for the S6t protocol.

Table 8.4.1-2: S6t re-used Diameter AVPs

| Attribute Name | Reference |  | Comments | M-bit |
| --- | --- | --- | --- | --- |
| User-Identifier | 6.4.2 |  | see 8.4.36 |  |
| External-Identifier | 6.4.11 |  |  |  |
| MSISDN | 3GPP TS 29.329 [10] |  |  |  |
| User-Name | IETF RFC 6733 [23] |  | This AVP shall contain the IMSI of the UE |  |
| Supported-Features | 3GPP TS 29.229 [7] |  | see 8.4.23 |  |
| Feature-List-ID | 3GPP TS 29.229 [7] |  |  |  |
| Feature-List | 3GPP TS 29.229 [7] |  |  |  |
| OC-Supported-Features | IETF RFC 7683 [15] |  | See 6.4.16 | Must not set |
| OC-OLR | IETF RFC 7683 [15] |  | See 6.4.17 | Must not set |
| Visited PLMN Id | 3GPP TS 29.272 [14] |  |  |  |
| Charged-Party | 3GPP TS 32.299 [16] |  |  |  |
| EPS-Location-Information | 3GPP TS 29.272 [14] |  | see 8.4.21 |  |
| MME-Location-Information | 3GPP TS 29.272 [14] |  | see 8.4.34 |  |
| SGSN-Location-Information | 3GPP TS 29.272 [14] |  | see 8.4.35 |  |
| E-UTRAN-Cell-Global-Identity | 3GPP TS 29.272 [14] |  |  |  |
| Tracking-Area-Identity | 3GPP TS 29.272 [14] |  |  |  |
| Current-Location-Retrieved | 3GPP TS 29.272 [14] |  |  |  |
| Age-Of-Location-Information | 3GPP TS 29.272 [14] |  |  |  |
| User-CSG-Information | 3GPP TS 29.272 [14] |  |  |  |
| Cell-Global-Identity | 3GPP TS 29.272 [14] |  |  |  |
| Service-Area-Identity | 3GPP TS 29.272 [14] |  |  |  |
| Routing-Area-Identity | 3GPP TS 29.272 [14] |  |  |  |
| eNodeB-ID | 3GPP TS 29.217 [17] |  |  |  |
| Day-Of-Week-Mask | IETF RFC 5777 [18] |  |  |  |
| Time-Of-Day-Start | IETF RFC 5777 [18] |  |  |  |
| Time-Of-Day-End | IETF RFC 5777 [18] |  |  |  |
| DRMP | IETF RFC 7944 [20] |  | see 8.4.46 | Must not set |
| Service-Selection | IETF RFC 5778 [21] |  | See 8.4.49 |  |
| Load | IETF RFC 8583 [22] |  | See 6.4.20 | Must not set |
| DL-Buffering-Suggested-Packet-Count |  | 3GPP TS 29.272 [14] |  |  |
| Extended-eNodeB-ID | 3GPP TS 29.217 [17] |  |  | Must not set |
| Maximum-UE-Availability-Time | 3GPP TS 29.338 [12] |  |  |  |
| Idle-Status-Indication | 3GPP TS 29.128 [24] |  |  | Must not set |
| Active-Time | 3GPP TS 29.128 [24] |  |  | Must not set |
| DL-Buffering-Suggested-Packet-Count | 3GPP TS 29.272 [14] |  |  | Must not set |
| Subscribed-Periodic-RAU-TAU-Timer | 3GPP TS 29.272 [14] |  |  | Must not set |
| IMSI-Group-Id | 3GPP TS 29.272 [14] |  |  | Must not set |
| Number-of-UEs | 3GPP TS 29.154 [25] |  |  | Must not set |
| Terminal-Information | 3GPP TS 29.272 [14] |  |  | Must not set |
| PDN-Type | 3GPP TS 29.272 [14] |  |  | Must not set |
| Non-IP-PDN-Type-Indicator | 3GPP TS 29.272 [14] |  |  | Must not set |
| Non-IP-Data-Delivery-Mechanism | 3GPP TS 29.272 [14] |  |  | Must not set |
| Served-Party-IP-Address | 3GPP TS 32.299 [16] |  |  | Must not set |

* * * Next Change * * * *

8.4.26	Communication-Pattern-Set

The Communication-Pattern-Set AVP is of type Grouped, and it shall contain a set of Communication-Pattern.

AVP format

Communication-Pattern-Set ::= <AVP header: 3114 10415>

[ Periodic-Communication-Indicator ]

[ Communication-Duration-Time ]

[ Periodic-Time ]

*[ Scheduled-Communication-Time ]

[ Stationary-Indication ]

[ Reference-ID-Validity-Time ]

[ Traffic-Profile ]

[ Battery-Indicator ]

*[ AVP ]

Communication-duration-time and Periodic-Time shall be only provided when the Periodic-Communication-Indicator is set to PERIODICALLY.

If the Reference-ID-Validity-Time AVP is absent, it indicates that there is no expiration time defined for the Communication-Pattern-Set.

* * * Next Change * * * *

8.4.x	Battery-Indicator

The Battery-Indicator AVP is of type Unsigned32 and it shall contain a bit mask. The meaning of the bits shall be as defined in table 8.4.x-1:

# Table 8.4.x-1: Battery-Indicator

| Bit | Name | Description |
| --- | --- | --- |
| 0 | NO_BATTERY | When this bit is set it indicates that UE has not battery powered. |
| 1 | BATTERY_REPLACEABLE_INDICATION | When this bit is set it indicates that UE is powered with replaceable battery, when this bit is not set it indicates that UE is powered without replaceable battery. |
| 2 | BATTERY_RECHARGEABLE_INDICATION | When this bit is set it indicates that UE is powered with rechargeable battery, when this bit is not set it indicates that UE is powered without rechargeable battery. |
| NOTE1: Bits not defined in this table shall be cleared by the sender and discarded by the receiver of the command. NOTE2: If bit 0 is set, bit 1 and bit 2 shall be cleared. |  |  |

* * * End of Changes * * * *