3GPP TSG-CT WG4 Meeting #95	C4-195120

# Reno, US; 11th – 15th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.008 | CR | 0574 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding support for subscription-based access restriction in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN2#107, RAN2 sent LS R2-1911533 asking SA2 and CT1 to complete the normative work on subscription-based access restriction, policy and charging functionalities for NR accessing through unlicensed bands (NR-U). At SA3#135, SA2 agreed which CR 1847 to TS 23.501 (S2-1910729) which introduces supports for RAT restriction in 5GS by adding the following text: The UDM shall provide to the AMF the following information about the subscriber's NR or E-UTRA access restriction set by the operator determined e.g. by subscription scenario and roaming scenario: - For NR: - NR not allowed as primary access. - NR not allowed as secondary access. - NR in unlicensed bands not allowed as primary access. - NR in unlicensed bands not allowed as secondary access. - For E-UTRA: - E-UTRA not allowed as primary access. - E-UTRA not allowed as secondary access. - NB-IoT not allowed as primary access. In order to enforce all primary access restrictions, the related access has to be deployed in different Tracking Area Codes and the subscriber shall not be allowed to access the network in TAs using the particular access. With all secondary access restrictions, the subscriber shall not be allowed to use this access as secondary access. This needs to be reflected in TS 23.008. It is not note that even though SA2 did not include “E-UTRA in unlicensed bands” in the above text, the RAN2 running CR does include support for “LTE in unlicensed bands” as PCell or PScell, hence RAT restriction for E-UTRA in unlicensed bands also needs to be covered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause under clause 2.25 is created for Access Restriction Data in 5GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements on RAT restriction in 5GS will remain missing from TS 23.008 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.25.xx (New), 5.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 1847 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

### 2.25.xx	Access Restriction Data

The use of this data is described in 3GPP TS 23.501 [112] clause 5.3.4.1.1.

The Access Restriction Data is permanent subscriber data and is conditionally stored per PLMN in the UDM and the AMF.

The parameter takes either of the following values:

-	NR not allowed as primary RAT, the subscriber shall not be allowed to access the network in TAs using NR radio access. Valid for Idle and Connected mode;

-	NR not allowed as secondary RAT, the subscriber shall not be allowed to access the network using NR radio access as secondary RAT. Valid for Idle and Connected mode;

-	NR in unlicensed bands not allowed as primary RAT, the subscriber shall not be allowed to access the network in TAs using NR radio access in unlicensed bands. Valid for Idle and Connected mode;

-	NR in unlicensed bands not allowed as secondary RAT, the subscriber shall not be allowed to access the network using NR radio access in unlicensed bands as secondary RAT. Valid for Idle and Connected mode;

-	E-UTRA not allowed as primary RAT, the subscriber shall not be allowed to access the network in TAs using E-UTRA radio access. Valid for Idle and Connected mode;

-	E-UTRA not allowed as secondary RAT, the subscriber shall not be allowed to access the network using E-UTRA radio access as secondary RAT. Valid for Idle and Connected mode;

-	E-UTRA in unlicensed bands not allowed as primary RAT, the subscriber shall not be allowed to access the network in TAs using E-UTRA radio access in unlicensed bands. Valid for Idle and Connected mode;

-	E-UTRA in unlicensed bands not allowed as secondary RAT, the subscriber shall not be allowed to access the network using E-UTRA radio access in unlicensed bands as secondary RAT. Valid for Idle and Connected mode;

-	NB-IoT not allowed, the subscriber shall not be allowed to access the network in TAs using NB-IoT radio access. Valid for Idle and Connected mode.

Only the access restriction for WB-E-UTRA and NB-IoT may include a different value per PLMN.

The use of this parameter for TA update procedures is described in 3GPP TS 23.501 [112].

*** Next change ***

## 5.2B	PS Network Access Mode Data (5GS)

Table 5.2B-1: Overview of data used for PS Network Access Mode (5GS)

| PARAMETER | Clause | UDM | AMF | SMF | SMSF | AUSF | NEF |  | TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subscribed UE-AMBR | 2.13.30 | C | C | - | - | - | - | - | P |
| TAI | 2.13.45 | - | C | C | - | - | - | - | T |
| Restricted RAT Types | 2.13.126 | C | C | - | - | - | - | - | P |
| IMSI Group Identifier List | 2.13.153 | C | C |  | - | - | - | - | P |
| UE Usage Type | 2.13.154 | C | C |  | - | - | - | - | P |
| SUPI | 2.25.1 | M | C | C | C | - | C | - | P |
| GPSI | 2.25.2 | M | C | C | - | - | C | - | P |
| Internal Group ID-list | 2.25.3 | C | C | C | - | - | - | - | P |
| Subscribed S-NSSAI | 2.25.4 | C | C | C | - | - | - | - | P |
| Default S-NSSAI | 2.25.5 | C | C | - | - | - | - | - | P |
| 5G QOS Parameter | 2.25.6 | C | - | C | - | - | - | - | P |
| Forbidden Area | 2.25.7 | C | C | - | - | - | - | - | P |
| Service Area Restriction | 2.25.8 | C | C | - | - | - | - | - | P |
| RFSP Index | 2.25.9 | C | C | - | - | - | - | - | P |
| Priority Service | 2.25.10 | C | C | - | - | - | - | - | P |
| DNN Info | 2.25.11 | C | - | C | - | - | - | - | P |
| DNN Configuration | 2.25.12 | C | - | C | - | - | - | - | P |
| Steering of Roaming Information | 2.25.13 | C | C | - |  | C |  |  | T |
| SMS supported Data | 2.25.14 | C | C | - | C | - | - | - | T |
| SMSF Registrations | 2.25.15 | C | C | - | M | - | - | - | P |
| SMS subscribed | 2.25.16 | M | - | - | C | - | - | - | P |
| SMS barring | 2.25.17 | M | - | - | C | - | - | - | P |
| SMS Roaming | 2.25.18 | C | C | - | C | - | - | - | P |
| Core Network Type Restriction | 2.25.19 | C | C | - | - | - | - | - | P |
| LADN Information | 2.25.20 | C | C | - | - | - | - | - | P |
| Subscribed periodic Registration timer | 2.25.21 | C | C | - | - | - | - | - | P |
| PEI | 2.25.22 | C | C | C | - | - | - | - | T |
| 5G GUTI | 2.25.23 | - | C | - | - | - | - | - | T |
| PLMN ID | 2.25.24 | C | C | C | C | - | - | - | P |
| PDU session | 2.25.25 | C | C | C | - | - | - | - | T |
| Trace Data | 2.25.26 | C | C | C | - | - | - | - | P |
| Mobile Initiated Connection Only (MICO) mode | 2.25.27 | C | C | - | - | - | - | - | P |
| UE purged in AMF flag | 2.25.28 | M | - | - | - | - | - | - | T |
| GUAMI | 2.25.29 | C | C | C | - | - | - | - | T |
| RAND, AUTN, XRES, CK', IK' | 2.25.30 | - | - | - | - | - | - | - | T |
| RAND, AUTN, XRES* | 2.25.31 | - | - | - | - | - | - | - | T |
| RAND, AUTN, HXRES* | 2.25.32 | - | - | - | - | - | - | - | T |
| Kausf | 2.25.33 | - | - | - | - | C | - | - | T |
| Kseaf | 2.25.34 | - | M | - | - | - | - | - | T |
| AuthenticationMethod | 2.25.35 | M | - | - | - | - | - | - | P |
| PermanentKey | 2.25.36 | C | - | - | - | - | - | - | P |
| SequenceNumber | 2.25.37 | C | - | - | - | - | - | - | T |
| AuthenticationManagementField | 2.25.38 | C | - | - | - | - | - | - | P |
| VectorAlgorithm | 2.25.39 | C | - | - | - | - | - | - | P |
| Milenage | 2.25.40 | C | - | - | - | - | - | - | P |
| Tuak | 2.25.41 | C | - | - | - | - | - | - | P |
| Opc | 2.25.42 | C | - | - | - | - | - | - | P |
| Topc | 2.25.43 | C | - | - | - | - | - | - | P |
| Authentication Status | 2.25.44 | C | - | - | - | - | - | - | T |
| SoR-XMAC-IUE | 2.25.45 | C | - | - | - | - | - | - | T |
| Shared Data Ids | 2.25.46 | C | C | C | C | - | - | - | P |
| Access Restriction Data | 2.25.xx | C | C | - | - | - | - | - | P |

NOTE:	Where the node physically stores data and/or from where it retrieves data for usage (e.g. interactions with the UDR) are not reflected in this table.

*** End of changes ***