3GPP TSG-CT WG4 Meeting #96e	C4-200582

# E-Meeting, 17th – 28th February 2020

Source:	Ericsson

Title:	Completion of authentication schemes

Spec:	3GPP TS 29.562 v0.3.0

Agenda item:	6.1.14

Document for:	Agreement

1. Introduction

-

2. Reason for Change

Complete authentication schemes.

3. Conclusions

-

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.562 v0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 29.335: "User Data Repository Access Protocol over the Ud interface; Stage 3".

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 7807: "Problem Details for HTTP APIs".

[12]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[13]	3GPP TS 23.003: "Numbering, addressing and identification".

[14]	3GPP TS 33.203: "Access security for IP-based services".

[15]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[zz] 	IETF RFC 4740: "Diameter Session Initiation Protocol (SIP) Application".

* * * Next Change * * * *

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the structured data types defined for the Nhss_imsUEAU service API.

Table 6.3.6.1-1: Nhss_imsUEAU specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SipAuthenticationInfoRequest | 6.3.6.2.2 | Contains SIP Authentication Schema, Authentication Items and Resynchronization Information |
| SipAuthenticationInfoResult | 6.3.6.2.3 | Contains authentication information (e.g. authentication vectors, line id) |
| ResynchronizationInfo | 6.3.6.2.4 | Contains RAND and AUTS |
| 3GAkaAv | 6.3.6.2.5 | Contains RAND, XRES, AUTN, CK, and IK |

Table 6.3.6.1-2 specifies data types re-used by the Nhss_imsUEAU service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_imsUEAU.

Table 6.3.6.1-2: Nhss_imsUEAU re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [16] | Common data type used in response bodies |
| Autn | 3GPP TS 29.503 [15] |  |
| Auts | 3GPP TS 29.503 [15] |  |
| Rand | 3GPP TS 29.503 [15] |  |
| Xres | 3GPP TS 29.503 [15] |  |

* * * Next Change * * * *

##### 6.3.6.2.2	Type: SipAuthenticationInfoRequest

Table 6.3.6.2.3-1: Definition of type AuthenticationInfoRequest

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sipAuthenticationScheme | SipAuthenticationScheme | M | 1 | SIP Authentication Scheme requested by service consumer (S-CSCF) |
| sipNumberAuthItems | SipNumberAuthItems | O | 1 | Number of authentication items (AVs). Absence indicates that only one AV is requested. This attribute shall be absent for authentication schemes other than IMS AKA. |
| resynchronizationInfo | ResynchronizationInfo | O | 1 | Contains RAND and AUTS; see 3GPP TS 33.203 [14]. This attribute shall be included only if a resynchronization procedure needs to be triggered. This attribute shall be absent for authentication schemes other than IMS AKA. |

* * * Next Change * * * *

##### 6.3.6.2.3	Type: SipAuthenticationInfoResult

Table 6.3.6.2.3-1: Definition of type AuthenticationInfoResult

| Attribute name |  |  | Data type |  | P | Cardinality | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| sipAuthenticationScheme |  |  |  | SipAuthenticationScheme | M | 1 | SIP Authentication Scheme used by service producer (HSS) |
| 3gAkaAvs |  |  | array(3GAkaAv) |  | O | 1..N | If IMS AKA is used, it shall be present and will contains Authentication Vectors. |
| digestAuth | DigestAuthentication | O | 0..1 | If SIP Digest is used, it shall be present and shall contain the digest realm, algorithm, QoP and HA1. |
| lineIdentifierList |  | array(LineIdentifier) |  |  | O | 0..N | If NBA is used, it shall be present and shall contain a list of fixed broadband access line identifiers associated to the user. |
| ipAddress | IpAddress | O | 0..1 | If GIBA is used, it shall be present and shall contain an IP address (IPv4, or IPv6, or IPv6 prefix) |

* * * Next Change * * * *

##### 6.3.6.2.x1	Type: DigestAuthentication

Table 6.3.6.2.3-1: Definition of type DigestAuthentication

| Attribute name |  |  | Data type |  | P | Cardinality | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| digestRealm |  |  |  | string | M | 1 | Digest realm. See IETF RFC 4740 [zz]. |
| digestAlgorithm |  |  | SipDigestAlgorithm |  | M | 1 | Digest Algorithm. See IETF RFC 4740 [zz]. |
| digestQop | SipDigestQop | M | 1 | Quality of Protection. See IETF RFC 4740 [zz]. |
| ha1 |  | string |  |  | M | 1 | Hash H(A1). See IETF RFC 4740 [zz]. |

* * * Next Change * * * *

##### 6.3.6.3.2	Simple data types

The simple data types defined in table 6.3.6.3.2-1 shall be supported.

Table 6.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| SipNumberAuthItems | integer | Number of authentication items Minimum: 1 |
| Impi | string | IMS Private User Identity |
| Ck | string | pattern: "^[A-Fa-f0-9]{32}$" |
| Ik | string | pattern: "^[A-Fa-f0-9]{32}$" |
| LineIdentifier | string | Line Identifier for the wireline access |

* * * Next Change * * * *

##### 6.3.6.3.x1	Enumeration: SipAuthenticationScheme

The enumeration SipAuthenticationScheme represents an IMS authentication scheme. It shall comply with the provisions defined in table 6.3.6.3.x1-1.

Table 6.3.6.3.x1-1: Enumeration SipAuthenticationScheme

| Enumeration value | Description |
| --- | --- |
| "DIGEST-AKAV1-MD5" | IMS-AKA authentication scheme |
| "DIGEST-HTTP" | SIP Digest authentication scheme |
| "NBA" | NASS Bundled authentication scheme |
| "GIBA" | GPRS-IMS-Bundled authentication scheme |
| "UNKNOWN" | Authentication scheme to be used is unknown by the service consumer |

* * * Next Change * * * *

##### 6.3.6.3.x2	Enumeration: SipDigestAlgorithm

The enumeration SipDigestAlgorithm represents the algorithm for SIP Digest scheme. It shall comply with the provisions defined in table 6.3.6.3.x2-1.

Table 6.3.6.3.x2-1: Enumeration SipDigestAlgorithm

| Enumeration value | Description |
| --- | --- |
| "MD5" |  |
| "MD5_SESS" |  |

* * * Next Change * * * *

##### 6.3.6.3.x3	Enumeration: SipDigestQop

The enumeration SipDigestQop represents the quality of protection for SIP Digest scheme. It shall comply with the provisions defined in table 6.3.6.3.x3-1.

Table 6.3.6.3.x3-1: Enumeration SipDigestQop

| Enumeration value | Description |
| --- | --- |
| "AUTH" | Indicates authentication |
| "AUTH_INT" | Indicates authentication with integrity protection |

* * * Next Change * * * *

##### 6.3.6.3.x4	Type: IpAddress

The type IpAddress represents the IP address for GIBA scheme. It shall comply with the provisions defined in table 6.3.6.3.x4-1.

Table 6.3.6.3.x4-1: Enumeration IpAddress

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 |  |
| ipv6Addr | Ipv6Addr | C | 0..1 |  |
| ipv6Prefix | Ipv6Prefix | C | 0..1 |  |
| NOTE: Either ipv4Addr, or ipv6Addr, or ipv6Prefix shall be present. |  |  |  |  |

* * * Next Change * * * *

## A.4	Nhss_imsUEAU API

openapi: 3.0.0

info:

version: '1.0.0'

title: 'Nhss_imsUEAU'

description: |

Nhss Subscriber Data Management Service for IMS.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 HSS Services for Interworking with IMS, version 16.0.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-ueau/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nhss-ims-ueau

- {}

paths:

/{impi}/security-information/generate-sip-auth-data:

post:

summary: Generate authentication data for the UE based on the Auth-Scheme provided

operationId: GenerateSipAuthData

tags:

- Generate Auth Data

parameters:

- name: impi

in: path

description: IMS Private Identity for the UE (IMPI)

required: true

schema:

$ref: '#/components/schemas/Impi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SipAuthenticationInfoRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SipAuthenticationInfoResult'

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

description: Not implemented

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

description: Gateway timeout

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

# COMPLEX TYPES:

SipAuthenticationInfoRequest:

type: object

required:

- sipAuthenticationScheme

properties:

sipAuthenticationScheme:

$ref: '#/components/schemas/SipAuthenticationScheme'

sipNumberAuthItems:

$ref: '#/components/schemas/SipNumberAuthItems'

resynchronizationInfo:

$ref: '#/components/schemas/ResynchronizationInfo'

ResynchronizationInfo:

type: object

required:

- rand

- auts

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

auts:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Auts'

SipAuthenticationInfoResult:

type: object

required:

- impi

properties:

impi:

$ref: '#/components/schemas/Impi'

3gAkaAvs:

type: array

items:

$ref: '#/components/schemas/3GAkaAv'

minItems: 1

digestAuth:

$ref: '#/components/schemas/DigestAuthentication'

lineIdentifierList:

type: array

items:

$ref: '#/components/schemas/LineIdentifier'

minItems: 1

ipAddress:

$ref: '#/components/schemas/IpAddress'

3GAkaAv:

type: object

required:

- rand

- xres

- autn

- ck

- ik

properties:

rand:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

xres:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Xres'

autn:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Autn'

ck:

$ref: '#/components/schemas/Ck'

ik:

$ref: '#/components/schemas/Ik'

DigestAuthentication:

type: object

required:

- digestRealm

- digestAlgorithm

- digestQop

- ha1

properties:

digestRealm:

type: string

digestAlgorithm:

$ref: '#/components/schemas/SipDigestAlgorithm'

digestQop:

$ref: '#/components/schemas/SipDigestQop'

ha1:

type: string

IpAddress:

type: object

oneOf:

- required:

- ipv4Addr

- required:

- ipv6Addr

- required:

- ipv6Prefix

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

# SIMPLE TYPES:

SipNumberAuthItems:

type: integer

minimum: 1

Impi:

type: string

Ck:

type: string

pattern: '^[A-Fa-f0-9]{32}$'

Ik:

type: string

pattern: '^[A-Fa-f0-9]{32}$'

LineIdentifier:

type: string

# ENUMS:

SipAuthenticationScheme:

anyOf:

- type: string

enum:

- DIGEST-AKAV1-MD5

- DIGEST-HTTP

- NBA

- GIBA

- UNKNOWN

- type: string

SipDigestAlgorithm:

anyOf:

- type: string

enum:

- MD5

- MD5_SESS

- type: string

SipDigestQop:

anyOf:

- type: string

enum:

- AUTH

- AUTH_INT

- type: string

* * * End of Changes * * * *