3GPP TSG-CT4 Meeting #96-e	C4-200479

# Online, , 17th Feb 2020 - 28th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.008 | CR | 0575 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce support for 5G SRVCC in PS Network Access Mode Data (5GS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Limited, Hewlett Packard Enterprise, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SRVCC |  |  |  |  |  | Date: |  |  | 2020-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PS Network Access Mode Data (5GS) do not mention STN-SR and UE SRVCC Capability applicable to 5G SRVCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce STN-SR and UE SRVCC Capability in PS Network Access Mode Data (5GS). NIT: Made explanation given in clause 4 re: M, C, T and P also applicable to table 5.2B-1 Rev2: - Much like the pattern followed for describing STN-SR in 5GS, GPRS or EPS, introduce a new subclause to capture UE SRVCC Capability in 5GS (to differentiate from the existing UE SRVCC Capability in EPS) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G-SRVCC doesn’t work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.21.1, 2.21.1.1 (NEW), 2.21.1.2 (NEW), 2.21.1.3 (NEW), 2.21.2, 2.21.2.1 (NEW), 2.21.2.2 (NEW), 2.21.2.3 (NEW), 5, 5.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

### 2.21.1	STN-SR

#### 2.21.1.1	General

The STN-SR identifies the Session Transfer Number for SRVCC or 5G-SRVCC (see 3GPP TS 23.003 [5]).

The STN-SR is temporary data and is conditionally stored in the UDM, HSS, AMF, MME and SGSN (see 3GPP TS 23.216 [80]).

#### 2.21.1.2	STN-SR in EPS or GPRS

The existence or absence of a STN-SR data in the HSS indicates that the user is SRVCC subscribed or not SRVCC subscribed. This information on SRVCC subscription is independent of the STN-SR value. The creation or deletion of the STN-SR data in the HSS is ensured by provisioning.

NOTE:	Backward compatibility cases (e.g. Pre-Release 10 AS) can still require provisioning of the STN-SR.

#### 2.21.1.3	STN-SR in 5GS

The existence or absence of a STN-SR data in the UDM indicates that the user is 5G-SRVCC subscribed or not 5G-SRVCC subscribed. This information on SRVCC subscription is independent of the STN-SR value. The creation or deletion of the STN-SR data in the UDM is ensured by provisioning.

*** Next change ***

### 2.21.2	UE SRVCC Capability

#### 2.21.2.1	General

This information element indicates the SRVCC capability of the UE and is described in 3GPP TS 23.237 [87].

#### 2.21.2.2	UE SRVCC Capability in EPS

The UE SRVCC Capability is temporary data conditionally stored in the HSS, MME and SGSN.

#### 2.21.2.3	UE SRVCC Capability in 5GS

The UE 5G SRVCC Capability is temporary data conditionally stored in the UDM and AMF.

Editor's note: it is FFS whether the 5GS needs to store UE SRVCC Capability as in clause 2.21.2.2 in order to handle a request as in TS 29.562 to retrieve the SRVCC data for a UE that is not registered in the EPS.

*** Next change ***

# 5	Accessing subscriber or PSI data

It shall be possible to retrieve or store subscriber data concerning a specific MS from the HSS by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI);

-	Mobile Station ISDN Number (MSISDN).

It shall be possible to retrieve CSG subscriber data concerning a specific roaming MS from the CSS by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI).

It may be possible to retrieve subscriber data concerning a specific roaming MS from the CSS by use of each of the following references:

-	Mobile Station ISDN Number (MSISDN).

It shall be possible to retrieve or store subscriber IP Multimedia service data concerning a specific IMS subscription from the HSS by use of each of the following references:

-	Private User Identity;

-	Public User Identity.

It shall be possible to retrieve or store PSI IP Multimedia service data from the HSS by use of each of the following references:

-	Public Service Identity.

It shall be possible to retrieve or store subscriber data concerning a specific MS from the VLR by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI);

-	Temporary Mobile Subscriber Identity (TMSI).

It shall be possible to retrieve or store subscriber data concerning a specific MS from the SGSN by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI);

-	Packet Temporary Mobile Subscriber identity (P-TMSI);

-	International Mobile Equipment Identity (IMEI) for emergency attached UEs without IMSI (e.g; UICCless UE) or with an unauthenticated IMSI.

It shall be possible to retrieve or store subscriber data concerning a specific MS from the GGSN by use of the following reference:

-	International Mobile Subscriber Identity (IMSI);

-	International Mobile Equipment Identity (IMEI) for emergency attached UEs without IMSI (e.g; UICCless UE) or with an unauthenticated IMSI.

It shall be possible to retrieve or store subscriber data concerning a specific MS from the MME by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI);

-	Globally Unique Temporary Identity (GUTI);

-	International Mobile Equipment Identity (IMEI) for emergency attached UEs without IMSI (e.g; UICCless UE) or with an unauthenticated IMSI.

It shall be possible to retrieve or store subscriber data concerning a specific MS from the S-GW and P-GW by use of the following reference:

-	International Mobile Subscriber Identity (IMSI);

-	International Mobile Equipment Identity (IMEI) for emergency attached UEs without IMSI (e.g; UICCless UE) or with an unauthenticated IMSI.

It shall be possible to retrieve or store subscriber data concerning a specific MS from the 3GPP AAA Server by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI);

-	Mobile Subscriber ISDN Number (MSISDN).

-	International Mobile Equipment Identity (IMEI) for emergency attached UEs without IMSI (e.g; UICCless UE) or with an unauthenticated IMSI.

It shall be possible to retrieve or store subscriber data concerning a specific MS from the 3GPP AAA Proxy by use of the following reference:

-	Mobile Subscriber ISDN Number (MSISDN).

It shall be possible to retrieve or store subscriber data concerning a specific MS from the WAG by use of the following reference:

-	Mobile Subscriber ISDN Number (MSISDN).

It shall be possible to retrieve or store subscriber data concerning a specific MS from the PDG by use of each of the following references:

-	International Mobile Subscriber Identity (IMSI);

-	Mobile Subscriber ISDN Number (MSISDN).

-	International Mobile Equipment Identity (IMEI) for emergency attached UEs without IMSI (e.g; UICCless UE) or with an unauthenticated IMSI.

NOTE:	See clause 4 for explanation of M, C, T and P in table 5.1, table 5.2, table 5.2A-1, table 5.2A-2, table 5.2A-3, table 5.2B-1 and table 5.3.

*** Next change ***

## 5.2B	PS Network Access Mode Data (5GS)

Table 5.2B-1: Overview of data used for PS Network Access Mode (5GS)

| PARAMETER | Clause | UDM | AMF | SMF | SMSF | AUSF | NEF |  | TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subscribed UE-AMBR | 2.13.30 | C | C | - | - | - | - | - | P |
| TAI | 2.13.45 | - | C | C | - | - | - | - | T |
| Restricted RAT Types | 2.13.126 | C | C | - | - | - | - | - | P |
| IMSI Group Identifier List | 2.13.153 | C | C |  | - | - | - | - | P |
| UE Usage Type | 2.13.154 | C | C |  | - | - | - | - | P |
| SUPI | 2.25.1 | M | C | C | C | - | C | - | P |
| GPSI | 2.25.2 | M | C | C | - | - | C | - | P |
| Internal Group ID-list | 2.25.3 | C | C | C | - | - | - | - | P |
| Subscribed S-NSSAI | 2.25.4 | C | C | C | - | - | - | - | P |
| Default S-NSSAI | 2.25.5 | C | C | - | - | - | - | - | P |
| 5G QOS Parameter | 2.25.6 | C | - | C | - | - | - | - | P |
| Forbidden Area | 2.25.7 | C | C | - | - | - | - | - | P |
| Service Area Restriction | 2.25.8 | C | C | - | - | - | - | - | P |
| RFSP Index | 2.25.9 | C | C | - | - | - | - | - | P |
| Priority Service | 2.25.10 | C | C | - | - | - | - | - | P |
| DNN Info | 2.25.11 | C | - | C | - | - | - | - | P |
| DNN Configuration | 2.25.12 | C | - | C | - | - | - | - | P |
| Steering of Roaming Information | 2.25.13 | C | C | - |  | C |  |  | T |
| SMS supported Data | 2.25.14 | C | C | - | C | - | - | - | T |
| SMSF Registrations | 2.25.15 | C | C | - | M | - | - | - | P |
| SMS subscribed | 2.25.16 | M | - | - | C | - | - | - | P |
| SMS barring | 2.25.17 | M | - | - | C | - | - | - | P |
| SMS Roaming | 2.25.18 | C | C | - | C | - | - | - | P |
| Core Network Type Restriction | 2.25.19 | C | C | - | - | - | - | - | P |
| LADN Information | 2.25.20 | C | C | - | - | - | - | - | P |
| Subscribed periodic Registration timer | 2.25.21 | C | C | - | - | - | - | - | P |
| PEI | 2.25.22 | C | C | C | - | - | - | - | T |
| 5G GUTI | 2.25.23 | - | C | - | - | - | - | - | T |
| PLMN ID | 2.25.24 | C | C | C | C | - | - | - | P |
| PDU session | 2.25.25 | C | C | C | - | - | - | - | T |
| Trace Data | 2.25.26 | C | C | C | - | - | - | - | P |
| Mobile Initiated Connection Only (MICO) mode | 2.25.27 | C | C | - | - | - | - | - | P |
| UE purged in AMF flag | 2.25.28 | M | - | - | - | - | - | - | T |
| GUAMI | 2.25.29 | C | C | C | - | - | - | - | T |
| RAND, AUTN, XRES, CK', IK' | 2.25.30 | - | - | - | - | - | - | - | T |
| RAND, AUTN, XRES* | 2.25.31 | - | - | - | - | - | - | - | T |
| RAND, AUTN, HXRES* | 2.25.32 | - | - | - | - | - | - | - | T |
| Kausf | 2.25.33 | - | - | - | - | C | - | - | T |
| Kseaf | 2.25.34 | - | M | - | - | - | - | - | T |
| AuthenticationMethod | 2.25.35 | M | - | - | - | - | - | - | P |
| PermanentKey | 2.25.36 | C | - | - | - | - | - | - | P |
| SequenceNumber | 2.25.37 | C | - | - | - | - | - | - | T |
| AuthenticationManagementField | 2.25.38 | C | - | - | - | - | - | - | P |
| VectorAlgorithm | 2.25.39 | C | - | - | - | - | - | - | P |
| Milenage | 2.25.40 | C | - | - | - | - | - | - | P |
| Tuak | 2.25.41 | C | - | - | - | - | - | - | P |
| Opc | 2.25.42 | C | - | - | - | - | - | - | P |
| Topc | 2.25.43 | C | - | - | - | - | - | - | P |
| Authentication Status | 2.25.44 | C | - | - | - | - | - | - | T |
| SoR-XMAC-IUE | 2.25.45 | C | - | - | - | - | - | - | T |
| Shared Data Ids | 2.25.46 | C | C | C | C | - | - | - | P |
| Access Restriction Data | 2.25.47 | C | C | - | - | - | - | - | P |
| STN-SR | 2.21.1 | C | C | - | - | - | - | - | T |
| UE 5G SRVCC Capability | 2.21.2 | C | C | - | - | - | - | - | T |

NOTE:	Where the node physically stores data and/or from where it retrieves data for usage (e.g. interactions with the UDR) are not reflected in this table.

*** No more changes ***