3GPP TSG-CT WG4 Meeting #96e	C4-200795

# E-Meeting, 17th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.272 | CR | 0813 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IAB-Operation-Permission to subscriber data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC-CT |  |  |  |  |  | Date: |  |  | 2020-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In CR 3555 to TS 23.401 (S2-1910282) and CR 3570 to TS 23.401 (S2-1911918), SA2 agreed to add an “IAB-Operation Allowed” indication to the UE subscription data in EPS. This indication from the HSS needs to be stored in the UE’s MM context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new AVP for IAB-Operation-Permission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB node authorization will not be supported in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.3.1, 7.3.2, 7.3.xxx (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AVP	Attribute Value Pair

C	Conditional

CSS	CSG Subscriber Server

DCN	Dedicated Core Network

DRMP	Diameter Routing Message Priority

DSCP	Differentiated Services Code Point

EIR	Equipment Identity Register

ESM	EPS Session Management

HSS	Home Subscriber Server

IAB	Integrated Access and Backhaul

IE	Information Element

LAA	Licensed Assisted Access

LWA	LTE/WLAN Aggregation

LWIP	LTE/WLAN Radio Level Integration with IPsec Tunnel

M	Mandatory

MME	Mobility Management Entity

NR	New Radio

O	Optional

ODB	Operator Determined Barring

SCEF	Service Capability Exposure Function

URRP-MME	User Reachability Request Parameter for MME

URPP-SGSN	User Reachability Request Parameter for SGSN

*** Next change ***

### 7.3.1	General

The following table specifies the Diameter AVPs defined for the S6a/S6d interface protocol, the S7a/S7d interface protocol and the S13/S13' interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32, e.g., ULR-Flags, DSR-Flags, PUA-Flags, etc., bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x0001 should be used.

Table 7.3.1/1: S6a/S6d, S7a/S7d and S13/S13' specific DiameterAVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| Subscription-Data | 1400 | 7.3.2 | Grouped | M, V |  |  |  | No |
| Terminal-Information | 1401 | 7.3.3 | Grouped | M, V |  |  |  | No |
| IMEI | 1402 | 7.3.4 | UTF8String | M, V |  |  |  | No |
| Software-Version | 1403 | 7.3.5 | UTF8String | M, V |  |  |  | No |
| QoS-Subscribed | 1404 | 7.3.77 | OctetString | M, V |  |  |  | No |
| ULR-Flags | 1405 | 7.3.7 | Unsigned32 | M, V |  |  |  | No |
| ULA-Flags | 1406 | 7.3.8 | Unsigned32 | M, V |  |  |  | No |
| Visited-PLMN-Id | 1407 | 7.3.9 | OctetString | M, V |  |  |  | No |
| Requested-EUTRAN-Authentication-Info | 1408 | 7.3.11 | Grouped | M, V |  |  |  | No |
| Requested-UTRAN- GERAN-Authentication-Info | 1409 | 7.3.12 | Grouped | M, V |  |  |  | No |
| Number-Of-Requested-Vectors | 1410 | 7.3.14 | Unsigned32 | M, V |  |  |  | No |
| Re-Synchronization-Info | 1411 | 7.3.15 | OctetString | M, V |  |  |  | No |
| Immediate-Response-Preferred | 1412 | 7.3.16 | Unsigned32 | M, V |  |  |  | No |
| Authentication-Info | 1413 | 7.3.17 | Grouped | M, V |  |  |  | No |
| E-UTRAN-Vector | 1414 | 7.3.18 | Grouped | M, V |  |  |  | No |
| UTRAN-Vector | 1415 | 7.3.19 | Grouped | M, V |  |  |  | No |
| GERAN-Vector | 1416 | 7.3.20 | Grouped | M, V |  |  |  | No |
| Network-Access-Mode | 1417 | 7.3.21 | Enumerated | M, V |  |  |  | No |
| HPLMN-ODB | 1418 | 7.3.22 | Unsigned32 | M, V |  |  |  | No |
| Item-Number | 1419 | 7.3.23 | Unsigned32 | M, V |  |  |  | No |
| Cancellation-Type | 1420 | 7.3.24 | Enumerated | M, V |  |  |  | No |
| DSR-Flags | 1421 | 7.3.25 | Unsigned32 | M, V |  |  |  | No |
| DSA-Flags | 1422 | 7.3.26 | Unsigned32 | M, V |  |  |  | No |
| Context-Identifier | 1423 | 7.3.27 | Unsigned32 | M, V |  |  |  | No |
| Subscriber-Status | 1424 | 7.3.29 | Enumerated | M, V |  |  |  | No |
| Operator-Determined-Barring | 1425 | 7.3.30 | Unsigned32 | M, V |  |  |  | No |
| Access-Restriction-Data | 1426 | 7.3.31 | Unsigned32 | M, V |  |  |  | No |
| APN-OI-Replacement | 1427 | 7.3.32 | UTF8String | M, V |  |  |  | No |
| All-APN-Configurations-Included-Indicator | 1428 | 7.3.33 | Enumerated | M, V |  |  |  | No |
| APN-Configuration-Profile | 1429 | 7.3.34 | Grouped | M, V |  |  |  | No |
| APN-Configuration | 1430 | 7.3.35 | Grouped | M, V |  |  |  | No |
| EPS-Subscribed-QoS-Profile | 1431 | 7.3.37 | Grouped | M, V |  |  |  | No |
| VPLMN-Dynamic-Address-Allowed | 1432 | 7.3.38 | Enumerated | M, V |  |  |  | No |
| STN-SR | 1433 | 7.3.39 | OctetString | M, V |  |  |  | No |
| Alert-Reason | 1434 | 7.3.83 | Enumerate | M, V |  |  |  | No |
| AMBR | 1435 | 7.3.41 | Grouped | M, V |  |  |  | No |
| CSG-Subscription-Data | 1436 | 7.3.78 | Grouped | M. V |  |  |  | No |
| CSG-Id | 1437 | 7.3.79 | Unsigned32 | M, V |  |  |  | No |
| PDN-GW-Allocation-Type | 1438 | 7.3.44 | Enumerated | M, V |  |  |  | No |
| Expiration-Date | 1439 | 7.3.80 | Time | M, V |  |  |  | No |
| RAT-Frequency-Selection-Priority-ID | 1440 | 7.3.46 | Unsigned32 | M, V |  |  |  | No |
| IDA-Flags | 1441 | 7.3.47 | Unsigned32 | M, V |  |  |  | No |
| PUA-Flags | 1442 | 7.3.48 | Unsigned32 | M, V |  |  |  | No |
| NOR-Flags | 1443 | 7.3.49 | Unsigned32 | M, V |  |  |  | No |
| User-Id | 1444 | 7.3.50 | UTF8String | V |  |  | M | No |
| Equipment-Status | 1445 | 7.3.51 | Enumerated | M, V |  |  |  | No |
| Regional-Subscription-Zone-Code | 1446 | 7.3.52 | OctetString | M, V |  |  |  | No |
| RAND | 1447 | 7.3.53 | OctetString | M, V |  |  |  | No |
| XRES | 1448 | 7.3.54 | OctetString | M, V |  |  |  | No |
| AUTN | 1449 | 7.3.55 | OctetString | M, V |  |  |  | No |
| KASME | 1450 | 7.3.56 | OctetString | M, V |  |  |  | No |
| Trace-Collection-Entity | 1452 | 7.3.98 | Address | M, V |  |  |  | No |
| Kc | 1453 | 7.3.59 | OctetString | M, V |  |  |  | No |
| SRES | 1454 | 7.3.60 | OctetString | M, V |  |  |  | No |
| PDN-Type | 1456 | 7.3.62 | Enumerated | M, V |  |  |  | No |
| Roaming-Restricted-Due-To-Unsupported-Feature | 1457 | 7.3.81 | Enumerated | M, V |  |  |  | No |
| Trace-Data | 1458 | 7.3.63 | Grouped | M, V |  |  |  | No |
| Trace-Reference | 1459 | 7.3.64 | OctetString | M, V |  |  |  | No |
| Trace-Depth | 1462 | 7.3.67 | Enumerated | M, V |  |  |  | No |
| Trace-NE-Type-List | 1463 | 7.3.68 | OctetString | M, V |  |  |  | No |
| Trace-Interface-List | 1464 | 7.3.69 | OctetString | M, V |  |  |  | No |
| Trace-Event-List | 1465 | 7.3.70 | OctetString | M, V |  |  |  | No |
| OMC-Id | 1466 | 7.3.71 | OctetString | M, V |  |  |  | No |
| GPRS-Subscription-Data | 1467 | 7.3.72 | Grouped | M, V |  |  |  | No |
| Complete-Data-List-Included-Indicator | 1468 | 7.3.73 | Enumerated | M, V |  |  |  | No |
| PDP-Context | 1469 | 7.3.74 | Grouped | M, V |  |  |  | No |
| PDP-Type | 1470 | 7.3.75 | OctetString | M, V |  |  |  | No |
| 3GPP2-MEID | 1471 | 7.3.6 | OctetString | M, V |  |  |  | No |
| Specific-APN-Info | 1472 | 7.3.82 | Grouped | M, V |  |  |  | No |
| LCS-Info | 1473 | 7.3.84 | Grouped | M, V |  |  |  | No |
| GMLC-Number | 1474 | 7.3.85 | OctetString | M, V |  |  |  | No |
| LCS-PrivacyException | 1475 | 7.3.86 | Grouped | M, V |  |  |  | No |
| SS-Code | 1476 | 7.3.87 | OctetString | M, V |  |  |  | No |
| SS-Status | 1477 | 7.3.88 | OctetString | M, V |  |  |  | No |
| Notification-To-UE-User | 1478 | 7.3.89 | Enumerated | M, V |  |  |  | No |
| External-Client | 1479 | 7.3.90 | Grouped | M, V |  |  |  | No |
| Client-Identity | 1480 | 7.3.91 | OctetString | M, V |  |  |  | No |
| GMLC-Restriction | 1481 | 7.3.92 | Enumerated | M, V |  |  |  | No |
| PLMN-Client | 1482 | 7.3.93 | Enumerated | M, V |  |  |  | No |
| Service-Type | 1483 | 7.3.94 | Grouped | M, V |  |  |  | No |
| ServiceTypeIdentity | 1484 | 7.3.95 | Unsigned32 | M, V |  |  |  | No |
| MO-LR | 1485 | 7.3.96 | Grouped | M, V |  |  |  | No |
| Teleservice-List | 1486 | 7.3.99 | Grouped | M, V |  |  |  | No |
| TS-Code | 1487 | 7.3.100 | OctetString | M, V |  |  |  | No |
| Call-Barring-Info | 1488 | 7.3.101 | Grouped | M, V |  |  |  | No |
| SGSN-Number | 1489 | 7.3.102 | OctetString | M, V |  |  |  | No |
| IDR-Flags | 1490 | 7.3.103 | Unsigned32 | M, V |  |  |  | No |
| ICS-Indicator | 1491 | 7.3.104 | Enumerated | V |  |  | M | No |
| IMS-Voice-Over-PS-Sessions-Supported | 1492 | 7.3.106 | Enumerated | V |  |  | M | No |
| Homogeneous-Support-of-IMS-Voice-Over-PS-Sessions | 1493 | 7.3.107 | Enumerated | V |  |  | M | No |
| Last-UE-Activity-Time | 1494 | 7.3.108 | Time | V |  |  | M | No |
| EPS-User-State | 1495 | 7.3.110 | Grouped | V |  |  | M | No |
| EPS-Location-Information | 1496 | 7.3.111 | Grouped | V |  |  | M | No |
| MME-User-State | 1497 | 7.3.112 | Grouped | V |  |  | M | No |
| SGSN-User-State | 1498 | 7.3.113 | Grouped | V |  |  | M | No |
| User-State | 1499 | 7.3.114 | Enumerated | V |  |  | M | No |
| MME-Location Information | 1600 | 7.3.115 | Grouped | V |  |  | M | No |
| SGSN-Location-Information | 1601 | 7.3.116 | Grouped | V |  |  | M | No |
| E-UTRAN-Cell-Global-Identity | 1602 | 7.3.117 | OctetString | V |  |  | M | No |
| Tracking-Area-Identity | 1603 | 7.3.118 | OctetString | V |  |  | M | No |
| Cell-Global-Identity | 1604 | 7.3.119 | OctetString | V |  |  | M | No |
| Routing-Area-Identity | 1605 | 7.3.120 | OctetString | V |  |  | M | No |
| Location-Area-Identity | 1606 | 7.3.121 | OctetString | V |  |  | M | No |
| Service-Area-Identity | 1607 | 7.3.122 | OctetString | V |  |  | M | No |
| Geographical-Information | 1608 | 7.3.123 | OctetString | V |  |  | M | No |
| Geodetic-Information | 1609 | 7.3.124 | OctetString | V |  |  | M | No |
| Current-Location-Retrieved | 1610 | 7.3.125 | Enumerated | V |  |  | M | No |
| Age-Of-Location-Information | 1611 | 7.3.126 | Unsigned32 | V |  |  | M | No |
| Active-APN | 1612 | 7.3.127 | Grouped | V |  |  | M | No |
| Error-Diagnostic | 1614 | 7.3.128 | Enumerated | V |  |  | M | No |
| Ext-PDP-Address | 1621 | 7.3.129 | Address | V |  |  | M | No |
| UE-SRVCC-Capability | 1615 | 7.3.130 | Enumerated | V |  |  | M | No |
| MPS-Priority | 1616 | 7.3.131 | Unsigned32 | V |  |  | M | No |
| VPLMN-LIPA-Allowed | 1617 | 7.3.132 | Enumerated | V |  |  | M | No |
| LIPA-Permission | 1618 | 7.3.133 | Enumerated | V |  |  | M | No |
| Subscribed-Periodic-RAU-TAU-Timer | 1619 | 7.3.134 | Unsigned32 | V |  |  | M | No |
| Ext-PDP-Type | 1620 | 7.3.75A | OctetString | V |  |  | M | No |
| SIPTO-Permission | 1613 | 7.3.135 | Enumerated | V |  |  | M | No |
| MDT-Configuration | 1622 | 7.3.136 | Grouped | V |  |  | M | No |
| Job-Type | 1623 | 7.3.137 | Enumerated | V |  |  | M | No |
| Area-Scope | 1624 | 7.3.138 | Grouped | V |  |  | M | No |
| List-Of-Measurements | 1625 | 7.3.139 | Unsigned32 | V |  |  | M | No |
| Reporting-Trigger | 1626 | 7.3.140 | Unsigned32 | V |  |  | M | No |
| Report-Interval | 1627 | 7.3.141 | Enumerated | V |  |  | M | No |
| Report-Amount | 1628 | 7.3.142 | Enumerated | V |  |  | M | No |
| Event-Threshold-RSRP | 1629 | 7.3.143 | Unsigned32 | V |  |  | M | No |
| Event-Threshold-RSRQ | 1630 | 7.3.144 | Unsigned32 | v |  |  | M | No |
| Logging-Interval | 1631 | 7.3.145 | Enumerated | V |  |  | M | No |
| Logging-Duration | 1632 | 7.3.146 | Enumerated | V |  |  | M | No |
| Relay-Node-Indicator | 1633 | 7.3.147 | Enumerated | V |  |  | M | No |
| MDT-User-Consent | 1634 | 7.3.148 | Enumerated | V |  |  | M | No |
| PUR-Flags | 1635 | 7.3.149 | Unsigned32 | V |  |  | M | No |
| Subscribed-VSRVCC | 1636 | 7.3.150 | Enumerated | V |  |  | M | No |
| Equivalent-PLMN-List | 1637 | 7.3.151 | Grouped | V |  |  | M | No |
| CLR-Flags | 1638 | 7.3.152 | Unsigned32 | V |  |  | M | No |
| UVR-Flags | 1639 | 7.3.153 | Unsigned32 | M, V |  |  |  | No |
| UVA-Flags | 1640 | 7.3.154 | Unsigned32 | M, V |  |  |  | No |
| VPLMN-CSG-Subscription-Data | 1641 | 7.3.155 | Grouped | M, V |  |  |  | No |
| Time-Zone | 1642 | 7.3.163 | UTF8String | V |  |  | M | No |
| A-MSISDN | 1643 | 7.3.157 | OctetString | V |  |  | M | No |
| MME-Number-for-MT-SMS | 1645 | 7.3.159 | OctetString | V |  |  | M | No |
| SMS-Register-Request | 1648 | 7.3.162 | Enumerated | V |  |  | M | No |
| Local-Time-Zone | 1649 | 7.3.156 | Grouped | V |  |  | M | No |
| Daylight-Saving-Time | 1650 | 7.3.164 | Enumerated | V |  |  | M | No |
| Subscription-Data-Flags | 1654 | 7.3.165 | Unsigned32 | V |  |  | M | No |
| Measurement-Period-LTE | 1655 | 7.3.166 | Enumerated | V |  |  | M | No |
| Measurement-Period-UMTS | 1656 | 7.3.167 | Enumerated | V |  |  | M | No |
| Collection-Period-RRM-LTE | 1657 | 7.3.168 | Enumerated | V |  |  | M | No |
| Collection-Period-RRM-UMTS | 1658 | 7.3.169 | Enumerated | V |  |  | M | No |
| Positioning-Method | 1659 | 7.3.170 | OctetString | V |  |  | M | No |
| Measurement-Quantity | 1660 | 7.3.171 | OctetString | V |  |  | M | No |
| Event-Threshold-Event-1F | 1661 | 7.3.172 | Integer32 | V |  |  | M | No |
| Event-Threshold-Event-1I | 1662 | 7.3.173 | Integer32 | V |  |  | M | No |
| Restoration-Priority | 1663 | 7.3.174 | Unsigned32 | V |  |  | M | No |
| SGs-MME-Identity | 1664 | 7.3.175 | UTF8String | V |  |  | M | No |
| SIPTO-Local-Network-Permission | 1665 | 7.3.176 | Unsigned32 | V |  |  | M | No |
| Coupled-Node-Diameter-ID | 1666 | 7.3.177 | DiameterIdentity | V |  |  | M | No |
| WLAN-offloadability | 1667 | 7.3.181 | Grouped | V |  |  | M | No |
| WLAN-offloadability-EUTRAN | 1668 | 7.3.182 | Unsigned32 | V |  |  | M | No |
| WLAN-offloadability-UTRAN | 1669 | 7.3.183 | Unsigned32 | V |  |  | M | No |
| Reset-ID | 1670 | 7.3.184 | OctetString | V |  |  | M | No |
| MDT-Allowed-PLMN-Id | 1671 | 7.3.185 | OctetString | V |  |  | M | No |
| Adjacent-PLMNs | 1672 | 7.3.186 | Grouped | V |  |  | M | No |
| Adjacent-Access-Restriction-Data | 1673 | 7.3.187 | Grouped | V |  |  | M | No |
| DL-Buffering-Suggested-Packet-Count | 1674 | 7.3.188 | Integer32 | V |  |  | M | No |
| IMSI-Group-Id | 1675 | 7.3.189 | Grouped | V |  |  | M | No |
| Group-Service-Id | 1676 | 7.3.190 | Unsigned32 | V |  |  | M | No |
| Group-PLMN-Id | 1677 | 7.3.191 | OctetString | V |  |  | M | No |
| Local-Group-Id | 1678 | 7.3.192 | OctetString | V |  |  | M | No |
| AIR-Flags | 1679 | 7.3.201 | Unsigned32 | V |  |  | M | No |
| UE-Usage-Type | 1680 | 7.3.202 | Unsigned32 | V |  |  | M | No |
| Non-IP-PDN-Type-Indicator | 1681 | 7.3.204 | Enumerated | V |  |  | M | No |
| Non-IP-Data-Delivery-Mechanism | 1682 | 7.3.205 | Unsigned32 | V |  |  | M | No |
| Additional-Context-ID | 1683 | 7.3.206 | Unsigned32 | V |  |  | M | No |
| SCEF-Realm | 1684 | 7.3.207 | DiameterIdentity | V |  |  | M | No |
| Subscription-Data-Deletion | 1685 | 7.3.208 | Grouped | V |  |  | M | No |
| Preferred-Data-Mode | 1686 | 7.3.209 | Grouped | V |  |  | M | No |
| Emergency-Info | 1687 | 7.3.210 | Grouped | V |  |  | M | No |
| V2X-Subscription-Data | 1688 | 7.3.212 | Grouped | V |  |  | M | No |
| V2X-Permission | 1689 | 7.3.213 | Unsigned32 | V |  |  | M | No |
| PDN-Connection-Continuity | 1690 | 7.3.214 | Unsigned32 | V |  |  | M | No |
| eDRX-Cycle-Length | 1691 | 7.3.215 | Grouped | V |  |  | M | No |
| eDRX-Cycle-Length-Value | 1692 | 7.3.216 | OctetString | V |  |  | M | No |
| UE-PC5-AMBR | 1693 | 7.3.217 | Unsigned32 | V |  |  | M | No |
| MBSFN-Area | 1694 | 7.3.219 | Grouped | V |  |  | M | No |
| MBSFN-Area-ID | 1695 | 7.3.220 | Unsigned32 | V |  |  | M | No |
| Carrier-Frequency | 1696 | 7.3.221 | Unsigned32 | V |  |  | M | No |
| RDS-Indicator | 1697 | 7.3.222 | Enumerated | V |  |  | M | No |
| Service-Gap-Time | 1698 | 7.3.223 | Unsigned32 | V |  |  | M | No |
| Aerial-UE-Subscription-Information | 1699 | 7.3.224 | Unsigned32 | V |  |  | M | No |
| Broadcast-Location-Assistance-Data-Types | 1700 | 7.3.225 | Unsigned64 | V |  |  | M | No |
| Paging-Time-Window | 1701 | 7.3.226 | Grouped | V |  |  | M | No |
| Operation-Mode | 1702 | 7.3.227 | Unsigned32 | V |  |  | M | No |
| Paging-Time-Window-Length | 1703 | 7.3.228 | OctetString | V |  |  | M | No |
| Core-Network-Restrictions | 1704 | 7.3.230 | Unsigned32 | V |  |  | M | No |
| eDRX-Related-RAT | 1705 | 7.3.229 | Grouped | V |  |  | M | No |
| Interworking-5GS-Indicator | 1706 | 7.3.231 | Enumerated | V |  |  | M | No |
| Ethernet-PDN-Type-Indicator | 1707 | 7.3.232 | Enumerated | V |  |  | M | No |
| Subscribed-ARPI | 1708 | 7.3.233 | Unsigned32 | V |  |  | M | No |
| IAB-Operation-Permission | xxxx | 7.3.xxx | Enumerated | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M", indicates whether support of the AVP is required. The AVP header bit denoted as "V", indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [61]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table specifies the Diameter AVPs re-used by the S6a/S6d interface protocol from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within S6a and S6d.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter base protocol specified in IETF RFC 6733 [61], do not need to be supported. The AVPs from the Diameter base protocol specified in IETF RFC 6733 [61] are not included in table 7.3.1/2, but they may be re-used for the S6a/S6d protocol, the S7a/S7protocol and the S13/S13' protocol.

Table 7.3.1/2: S6a/S6d, S7a/S7d and S13/S13' re-used Diameter AVPs

| Attribute Name | Reference | Comments | M-bit |
| --- | --- | --- | --- |
| Service-Selection | IETF RFC 5778 [20] | See clause 7.3.36 |  |
| 3GPP-Charging-Characteristics | 3GPP TS 29.061 [21] | See 3GPP TS 32.251 [33] Annex A and 3GPP TS 32.298 [22] clause 5.1.2.2.7 This attribute holds the EPS PDN Connection Charging Characteristics data for an EPS APN Configuration, or the PDP context Charging Characteristics for GPRS PDP context, or the Subscribed Charging Characteristics data for the subscriber level 3GPP Charging Characteristics; refer to 3GPP TS 23.008 [30]. |  |
| Supported-Features | 3GPP TS 29.229 [9] |  |  |
| Feature-List-ID | 3GPP TS 29.229 [9] |  |  |
| Feature-List | 3GPP TS 29.229 [9] | See clause 7.3.10 |  |
| Served-Party-IP-Address | 3GPP TS 32.299 [8] | holds the PDN IP Address of the user |  |
| QoS-Class-Identifier | 3GPP TS 29.212 [10] |  |  |
| Allocation-Retention-Priority | 3GPP TS 29.212 [10] | See clause 7.3.40 |  |
| Priority-Level | 3GPP TS 29.212 [10] |  |  |
| Pre-emption-Capability | 3GPP TS 29.212 [10] |  |  |
| Pre-emption-Vulnerability | 3GPP TS 29.212 [10] |  |  |
| Max-Requested-Bandwidth-DL | 3GPP TS 29.214 [11] |  |  |
| Max-Requested-Bandwidth-UL | 3GPP TS 29.214 [11] |  |  |
| Extended-Max-Requested-BW-DL | 3GPP TS 29.214 [11] |  |  |
| Extended-Max-Requested-BW-UL | 3GPP TS 29.214 [11] |  |  |
| RAT-Type | 3GPP TS 29.212 [10] | See clause 7.3.13 | Must set |
| MSISDN | 3GPP TS 29.329 [25] |  |  |
| MIP6-Agent-Info | IETF RFC 5447 [26] |  |  |
| MIP-Home-Agent-Address | IETF RFC 4004 [27] |  |  |
| MIP-Home-Agent-Host | IETF RFC 4004 [27] |  |  |
| PDP-Address | 3GPP TS 32.299 [8] |  |  |
| Confidentiality-Key | 3GPP TS 29.229 [9] | See clause 7.3.57 |  |
| Integrity-Key | 3GPP TS 29.229 [9] | See clause 7.3.58 |  |
| Visited-Network-Identifier | 3GPP TS 29.229 [9] | See clause 7.3.105 | Must not set |
| GMLC-Address | 3GPP TS 29.173 [37] | See clause 7.3.109 | Must not set |
| User-CSG-Information | 3GPP TS 32.299 [8] |  | Must not set |
| ProSe-Subscription-Data | 3GPP TS 29.344 [49] | See clause 7.3.180 | Must not set |
| OC-Supported-Features | IETF RFC 7683 [50] | See clause 7.3.178 | Must not set |
| OC-OLR | IETF RFC 7683 [50] | See clause 7.3.179 | Must not set |
| SCEF-Reference-ID | 3GPP TS 29.336 [54] |  | Must not set |
| SCEF-ID | 3GPP TS 29.336 [54] |  | Must not set |
| AESE-Communication-Pattern | 3GPP TS 29.336 [54] | see clause 7.3.193 | Must not set |
| Communication-Pattern-set | 3GPP TS 29.336 [54] | see clause 7.3.194 | Must not set |
| Monitoring-Event-Configuration | 3GPP TS 29.336 [54] | See clause 7.3.195 | Must not set |
| Monitoring-Event-Report | 3GPP TS 29.336 [54] | See clause 7.3.196 | Must not set |
| UE-Reachability-Configuration | 3GPP TS 29.336 [54] | See clause 7.3.197 | Must not set |
| eNodeB-ID | 3GPP TS 29.217 [56] | See clause 7.3.198 | Must not set |
| SCEF-Reference-ID-for-Deletion | 3GPP TS 29.336 [54] |  | Must not set |
| Monitoring-Type | 3GPP TS 29.336 [54] |  | Must not set |
| Maximum-Number-of-Reports | 3GPP TS 29.336 [54] |  | Must not set |
| Monitoring-Duration | 3GPP TS 29.336 [54] |  | Must not set |
| Charged-Party | 3GPP TS 29.336 [54] |  | Must not set |
| UE-Reachability-Configuration | 3GPP TS 29.336 [54] |  | Must not set |
| Location-Information-Configuration | 3GPP TS 29.336 [54] |  | Must not set |
| Reachability-Type | 3GPP TS 29.336 [54] |  | Must not set |
| Maximum-Response-Time | 3GPP TS 29.336 [54] |  | Must not set |
| Reachability-Information | 3GPP TS 29.336 [54] |  | Must not set |
| Monitoring-Event-Config-Status | 3GPP TS 29.336 [54] |  | Must not set |
| Supported-Services | 3GPP TS 29.336 [54] | See clause 7.3.199 | Must not set |
| Supported-Monitoring-Events | 3GPP TS 29.336 [54] | See clause 7.3.200 | Must not set |
| DRMP | IETF RFC 7944 [57] | See clause 7.3.203 | Must not set |
| Reference-ID-Validity-Time | 3GPP TS 29.336 [54] |  | Must not set |
| Maximum-UE-Availability-Time | 3GPP TS 29.338 [48] | See clause 5.3.3.22 | Must not set |
| Emergency-Services | 3GPP TS 29.273 [59] |  |  |
| Load | IETF RFC 8583 [60] | See clause 7.3.211 | Must not set |
| Extended-eNodeB-ID | 3GPP TS 29.217 [56] | See clause 7.3.218 | Must not set |
| External-Identifier | 3GPP TS 29.336 [54] |  | Must not set |
| Loss-Of-Connectivity-Reason | 3GPP TS 29.336 [54] |  | Must not set |
| Active-Time | 3GPP TS 29.128 [63] |  | Must not set |
| Idle-Status-Indication | 3GPP TS 29.128 [63] |  | Must not set |
| MTC-Provider-Info | 3GPP TS 29.336 [54] |  | Must not set |
| Traffic-Profile | 3GPP TS 29.336 [54] |  | Must not set |
| PDN-Connectivity-Status-Configuration | 3GPP TS 29.336 [54] | See clause 7.3.195 | Must not set |
| PDN-Connectivity-Status-Report | 3GPP TS 29.336 [54] | See clause 7.3.196 | Must not set |
| Battery-Indicator | 3GPP TS 29.336 [54] |  | Battery-Indicator |
| NOTE 1: The M-bit settings for re-used AVPs override those of the defining specifications that are referenced. Values include: "Must set", "Must not set". If the M-bit setting is blank, then the defining specification applies. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |

### 7.3.2	Subscription-Data

The Subscription-Data AVP is of type Grouped. It shall contain the information related to the user profile relevant for EPS and GERAN/UTRAN.

AVP format:

Subscription-Data ::= <AVP header: 1400 10415>

[ Subscriber-Status ]

[ MSISDN ]

[ A-MSISDN ]

[ STN-SR ]

[ ICS-Indicator ]

[ Network-Access-Mode ]

[ Operator-Determined-Barring ]

[ HPLMN-ODB ]

*10[ Regional-Subscription-Zone-Code ]

[ Access-Restriction-Data ]

[ APN-OI-Replacement ]

[ LCS-Info ]

[ Teleservice-List ]

*[ Call-Barring-Info ]

[ 3GPP-Charging-Characteristics ]

[ AMBR ]

[ APN-Configuration-Profile ]

[ RAT-Frequency-Selection-Priority-ID ]

[ Trace-Data]

[ GPRS-Subscription-Data ]

*[ CSG-Subscription-Data ]

[ Roaming-Restricted-Due-To-Unsupported-Feature ]

[ Subscribed-Periodic-RAU-TAU-Timer ]

[ MPS-Priority ]

[ VPLMN-LIPA-Allowed ]

[ Relay-Node-Indicator ]

[ MDT-User-Consent ]

[ Subscribed-VSRVCC ]

[ ProSe-Subscription-Data ]

[ Subscription-Data-Flags ]

*[ Adjacent-Access-Restriction-Data ]

[ DL-Buffering-Suggested-Packet-Count ]

*[ IMSI-Group-Id ]

[ UE-Usage-Type ]

*[ AESE-Communication-Pattern ]

*[ Monitoring-Event-Configuration ]

[ Emergency-Info ]

[ V2X-Subscription-Data ]

*[ eDRX-Cycle-Length ]

[ External-Identifier ]

[ Active-Time ]

[ Service-Gap-Time ]

[ Broadcast-Location-Assistance-Data-Types ]

[ Aerial-UE-Subscription-Information ]

[ Core-Network-Restrictions ]

*[ Paging-Time-Window ]

[ Subscribed-ARPI ]

[ IAB-Operation-Permission ]

*[ AVP ]

The AMBR included in this grouped AVP shall include the AMBR associated to the user's subscription (UE-AMBR); Max-Requested-Bandwidth-UL and Max-Requested-Bandwidth-DL within this AVP shall not both be set to "0".

The APN-OI-Replacement included in this grouped AVP shall include the UE level APN-OI-Replacement associated to the user's subscription.

When multiple External Identifiers are defined for the same subscription, the External-Identifier in this grouped AVP shall contain a default External Identifier determined by the HSS.

*** Next change ***

### 7.3.xxx	IAB-Operation-Permission

The IAB-Operation-Permission AVP is of type Enumerated. It shall indicate to the MME or SGSN whether the UE is allowed for IAB operation. See 3GPP TS 23.401 [2].

The following values are defined:

IAB_OPERATION_ALLOWED (0)

IAB_OPERATION_NOTALLOWED (1)

*** End of changes ***