3GPP TSG-CT WG4 Meeting #97e	C4-202344

# E-Meeting, 15th – 24th April 2020

# Title:	LS on Presence of PEI in Nudm_UECM_Registration Requests

# Release:	Rel-15 and Rel-16

Source:	CT4

To:	SA2

Cc:	CT1

Contact Person:

Name:	Ulrich Wiehe

Tel. Number:

E-mail Address:	ulrich.wiehe@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-202049

1. Overall Description:

CT4 have detected a misalignment between TS 23.502 and TS 29.503 with regard to the presence of PEI in Nudm_UECM_Registration requests sent from AMF to UDM:

TS 23.502 indicates in clause 5.2.3.2.1 that PEI is a required input.

However, in other clauses, it gives the impression that PEI is optional; for example, clause 4.2.2.2.1 says:

If the AMF needs the PEI in the initial registration, it should retrieve the PEI as it establishes the NAS security context with a Security Mode Command.

And also, clause 4.2.2.2.2 says:

The PEI may be retrieved in initial registration from the UE as described in clause 4.2.2.2.1.

Regarding stage-3, TS 29.503 has defined PEI as an optional attribute in Amf3GppAccessRegistration and AmfNon3GppAccessRegistration data types (see clauses 6.2.6.2.2 and 6.2.6.2.3).

CT4 have discussed the attached CR to TS 29.503 which tries to align with TS 23.502 by making PEI mandatory in Amf3GppAccessRegistration and AmfNon3GppAccessRegistration data types. During discussions concerns were raised, suggesting to relax the requirement from TS 23.502 and to allow absence of PEI from Nudm_UECM_Registrations.

CT4 kindly ask SA2 to either confirm that PEI is a required input for Nudm_UECM_Registration (and, if so, to consider updating the clauses mentioned above that might suggest otherwise), or to remove this requirement in TS 23.502 clause 5.2.3.2.1.

2. Actions:

To SA2 group.

ACTION: CT4 kindly ask SA2 to clarify whether PEI is required or optional input to Nudm_UECM_Registration

3. Date of Next CT4 Meetings:

3GPP TSG CT4#98e	2nd – 11th June 2020	E-Meeting