Joint-Working-Group (Parlay, ETSI TISPAN Project OSA, 3GPP CT5)	C5-070566

Meeting CT5#40, 6 - 8 August 2007, Trondheim, NORWAY

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.199-02 | CR | 0014 |  rev | - |  Current version: | 7.4.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add service scenario to Third Party Call web service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OSA8 |  |  |  |  |  |  | Date:  |  | 05/09/2007 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Add service scenario to show the relationship among ECC web services such as Third Party Call, Call Notification, Audio Call, and Multimedia Conference web servics. Correct API in figure 4.1. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add service scenario supported by combining TPC and AC web services in order to explain the relationship between them. Change ‘makeACall’ into ‘MakeCallSession’ in figure 4.1. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | It might cause lack of understanding the ECC web services. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4, New 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 4	Detailed service description

Currently, in order to perform a third party call in telecommunication networks we have to write applications using specific protocols to access Call Control functions provided by network elements (specifically operations to initiate a call from applications). This approach requires a high degree of network expertise. We can also use the OSA gateway approach, invoking standard interfaces to gain access to call control capabilities, but these interfaces are usually perceived to be quite complex by application IT developers. Developers must have advanced telecommunication skills to use Call Control OSA interfaces.

In this subclause we describe a Parlay X Web Service, Third Party Call, for creating and managing a call initiated by an application (third party call). The overall scope of this Web Service is to provide functions to application developers to create a call in a simple way. Using the Third Party Call Web Service, application developers can invoke call handling functions without detailed telecommunication knowledge.

The underlying model of the service is based on the following entities:

- Call Session: a call (uniquely identified) to which participants can be added/removed.

- Call Participant: each of the call parties (uniquely identified) involved in the call session.

- Media: the call can utilize multiple media types to support the participants' communication. In particular both audio and video streams are available, including the specific stream direction (i.e. incoming, outgoing, bidirectional).

NOTE 1:	Call participants in a Call Session are anticipated to be uniquely identifiable using their URI address.

An application setting up a call session must initially invoke the makeCallSession. The result of such invocation is the creation of a "context" that represents a call session with usually two participants, or at a minimum one participant connected, a unique identifier is assigned to the just-created call session. Subsequently the application may wish to add, remove, park or transfer call participants. In order to do so the operations addCallParticipant, transferCallParticipant, deleteCallParticipant can be used. Furthermore the call session or call participant status including the media details can be read. In order to do so the operations getCallParticipantInformation, and getCallSessionInformation can be used. It is also possible to retrieve the media details on its own using the getMediaForParticipant or getMediaForCall operations of the Audio Call web service.

The application can also force the call session and all its participants to be terminated with the operation endCallSession.

NOTE 2:	A call session allows the application to avail of other web service features that can add value to the created call session. For example the Audio Call web service can provide multimedia message delivery to call participants in the call session (playXXXMessage operation) and furthermore control of the media types for the call participants thus enabling conversational multimedia communication including voice, video, chat, and data. Media can be added/removed for each participant.

Figure 4.1 shows a scenario using the Third Party Call Web Service to handle third party call functions. The application invokes a Web Service to retrieve stock quotes and a Parlay X Interface to initiate a third party call between a broker and his client.

In the scenario, whenever a particular stock quote reaches a threshold value (1) and (2), the client application invokes a third party call between one or more brokers and their corresponding customers to decide actions to be taken. After invocation (3) by the application, the Third Party Call Web Service invokes a Parlay API operation (4) using the Parlay/OSA SCS-CC (Call control) interface. This SCS handles the invocation and sends a message (5) to an MSC to set-up a call between user A and user B.

In an alternative scenario, the Parlay API interaction involving steps (4) and (5) could be replaced with a direct  interaction between the Third Party Call Web Service and the Mobile network

Figure 4.1: Third party call scenario

Figure 4.1: Third party call scenario

Next Modified Section

## 6.2	Third party call with announcement between broker and customer

This sequence shows the third party call with announcement between broker and customer in addition to basic flow of figure 4.1. It could be realized through invoking TPC and AC web services with same session identifier. If the broker gets the announcement of information related to this call, he can get more information before talking to his customer.

End of modifications