3GPP TSG-CT WG6 #110-e 	C6-220170

- E-meeting; 22nd – 25th February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0943 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding eDRX parameters in the USIM for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ARCH_NR_REDCAP |  |  |  |  |  |  | Date: |  | 2022-02-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the work item ’ ARCH_NR_REDCAP’, CT1 stated the need for the update of the related eDRX parameter for NR devices in the TS 24.008 (C1-22041), CT6 stated that there are potential update of related eDRX parameters and UE configuration parameters(C6-210395).At the same time the subscription data defined in TS 23.502 contained ‘Extended idle mode DRX cycle length’. Therefore, the relevant parameters should also be supplemented in the USIM to support eDRX in NR RAT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add eDRX parameters in the USIM to provide the "Extended idle mode DRX cycle length" in order to support NR RAT type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will not meet the relevant requirements in the work item and the negotiation process based on eDRX parameters cannot be well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8,4.4.11,4.7,5.3, Annex A, Annex E, H.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

- *****************Start of change*******************

- 4.2.8    EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n°1 to n°8 |  |  | M |  | 1 byte |
| 2 | Services n°9 to n°16 |  |  | O |  | 1 byte |
| 3 | Services n°17 to n°24 |  |  | O |  | 1 byte |
| 4 | Services n°25 to n°32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n°(8X-7) to n°(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | Service n°3: | Extension 2 |
|  | Service n°4: | Service Dialling Numbers (SDN) |
|  | Service n°5: | Extension3 |
|  | Service n°6: | Barred Dialling Numbers (BDN) |
|  | Service n°7: | Extension4 |
|  | Service n°8: | Outgoing Call Information (OCI and OCT) |
|  | Service n°9: | Incoming Call Information (ICI and ICT) |
|  | Service n°10: | Short Message Storage (SMS) |
|  | Service n°11: | Short Message Status Reports (SMSR) |
|  | Service n°12: | Short Message Service Parameters (SMSP) |
|  | Service n°13: | Advice of Charge (AoC) |
|  | Service n°14: | Capability Configuration Parameters 2 (CCP2) |
|  | Service n°15: | Cell Broadcast Message Identifier |
|  | Service n°16: | Cell Broadcast Message Identifier Ranges |
|  | Service n°17: | Group Identifier Level 1 |
|  | Service n°18: | Group Identifier Level 2 |
|  | Service n°19: | Service Provider Name |
|  | Service n°20: | User controlled PLMN selector with Access Technology |
|  | Service n°21: | MSISDN |
|  | Service n°22: | Image (IMG) |
|  | Service n°23: | Support of Localised Service Areas (SoLSA) |
|  | Service n°24: | Enhanced Multi-Level Precedence and Pre-emption Service |
|  | Service n°25: | Automatic Answer for eMLPP |
|  | Service n°26: | RFU |
|  | Service n°27: | GSM Access |
|  | Service n°28: | Data download via SMS-PP |
|  | Service n°29: | Data download via SMS-CB |
|  | Service n°30: | Call Control by USIM |
|  | Service n°31: | MO-SMS Control by USIM |
|  | Service n°32: | RUN AT COMMAND command |
|  | Service n°33: | shall be set to '1' |
|  | Service n°34: | Enabled Services Table |
|  | Service n°35: | APN Control List (ACL) |
|  | Service n°36: | Depersonalisation Control Keys |
|  | Service n°37: | Co-operative Network List |
|  | Service n°38: | GSM security context |
|  | Service n°39: | CPBCCH Information |
|  | Service n°40: | Investigation Scan |
|  | Service n°41: | MexE |
|  | …….. | ……… |
|  | Service n°111 | ePDG configuration Information for Emergency Service configured |
|  | Service n°112 | eCall Data over IMS |
|  | Service n°113 | URI support for SMS-PP DOWNLOAD as defined in 3GPP TS 31.111 [12] |
|  | Service n°114 | From Preferred |
|  | Service n°115 | IMS configuration data |
|  | Service n°116 | TV configuration |
|  | Service n°117 | 3GPP PS Data Off |
|  | Service n°118 | 3GPP PS Data Off Service List |
|  | Service n°119 | V2X |
|  | Service n°120 | XCAP Configuration Data |
|  | Service n°121 | EARFCN list for MTC/NB-IOT UEs |
|  | Service n°122 | 5GS Mobility Management Information |
|  | Service n°123 | 5G Security Parameters |
|  | Service n°124 | Subscription identifier privacy support |
|  | Service n°125 | SUCI calculation by the USIM |
|  | Service n°126 | UAC Access Identities support |
|  | Service n°127 | Control plane-based steering of UE in VPLMN |
|  | Service n°128 | Call control on PDU Session by USIM |
|  | Service n°129 | 5GS Operator PLMN List |
|  | Service n°130 | Support for SUPI of type NSI or GLI or GCI |
|  | Service n°131 | 3GPP PS Data Off separate Home and Roaming lists |
|  | Service n°132 | Support for URSP by USIM |
|  | Service n°133 | 5G Security Parameters extended |
|  | Service n°134 | MuD and MiD configuration data |
|  | Service n°135 | Support for Trusted non-3GPP access networks by USIM |
|  | Service n°136 | Support for multiple records of NAS security context storage for multiple registration |
|  | Service n°137 | Pre-configured CAG information list |
|  | Service n°138 | SOR-CMCI storage in USIM |
|  | Service n°xxx | Pre-configured eDRX parameters |

***** next change *****

### 4.4.1	Contents of files at the DF SoLSA level

This only applies if the Support of Localised Service Areas is supported, as indicated by Service Number 23 in the USIM Service Table and specified in TS 23.073 [23] .

The Efs contain information about the users subscribed local service areas.

### 4.4.11.1	Introduction

This clause describes the files that are specific for 5GS.

DF5GS shall be present at the ADFUSIM level if any of the following services are "available" in EFUST (USIM Service Table):

| Service n°122 | 5GS Mobility Management Information |
| --- | --- |
| Service n°123 | 5G Security Parameters |
| Service n°124 | Subscription identifier privacy support |
| Service n°125 | SUCI calculation by the USIM |
| Service n°126 | UAC Access Identities support |
| Service n°127 | Control plane-based steering of UE in VPLMN |
| Service n°128 | Call control on PDU Session by USIM |
| Service n°129 | 5GS Operator PLMN List |
| Service n°130 | Support for SUPI of type NSI or GLI or GCI |
| Service n°132 | Support for URSP by USIM |
| Service n°133 | 5G Security Parameters extended |
| Service n°134 | MuD and MiD configuration data |
| Service n°135 | Support for Trusted non-3GPP access networks by USIM |
| Service n°136 | Support for multiple records of NAS security context storage for multiple registration |
| Service n°137 | Pre-configured CAG information list |
| Service n°138 | SOR-CMCI storage in USIM |
| Service n°xxx | Pre-configured eDRX parameters |

.

***** next change *****

### 4.4.11.x	EF5GSEDRX (5GS eDRX Parameters)

If service n°xxx is "available" in EFUST, this file shall be present.

This EF contains the following 5GS eDRX parameters:

-	ratType: Radio Access Technology Type;

-	edrxValue: Extended idle mode DRX cycle length.

| Identifier: '4F0X' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '0X' |  |  |  |  |  |  |
| File size: 2 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | 5GS ratType |  |  | M |  | 1 byte |
| 2 | The extended idle mode DRX cycle length value |  |  | M |  | 1 byte |

-	5GS ratType.

Contents:

The NG-RAN rat type which eDRX values are applicable to.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1:the rat type is NG-RAN 0:RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1:the rat type is Satellite NG-RAN 0:RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |

-	The extended idle mode DRX cycle length value.

Contents:

The extended idle mode DRX cycle length value.

Coding:

This data object contains the extended idle mode DRX cycle length value, the coding of which is specified in 3GPP TS 24.008 table 10.5.5.32.

***** next change *****

## 4.7	Files of USIM

This clause contains two figures depicting the file structure of the UICC and the ADFUSIM. ADFUSIM shall be selected using the AID and information in EFDIR.

|  |  |  |  |  |  | DF5GS '5FC0' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | EF5GS3GPPLOCI |  |  |  |  | EF5GSN3GPPLOCI |  |  |  |  |  |  | EF5GS3GPPNSC |  |  |  |  | EF5GSN3GPPNSC |  |  |  |  | EF5GAUTHKEYS |  |  |
|  |  |  |  |  |  |  |  |  |  |  | '4F01' |  |  |  |  | '4F02' |  |  |  |  |  |  | '4F03' |  |  |  |  | '4F04' |  |  |  |  | '4F05' |  |  |
|  |  |  |  |  |  |  |  |  |  |  | EFUAC_AIC |  |  |  |  | EFSUCI_Calc_Info |  |  |  |  |  |  | EFOPL5G |  |  |  |  | EFSUPI_NAI |  |  |  |  | EFRouting_Indicator |  |  |
|  |  |  |  |  |  |  |  |  |  |  | '4F06' |  |  |  |  | '4F07' |  |  |  |  |  |  | '4F08' |  |  |  |  | '4F09' |  |  |  |  | '4F0A' |  |  |
|  |  |  |  |  |  |  |  |  |  |  | EFURSP |  |  |  |  | EFTN3GPPSNN |  |  |  |  |  |  | EFCAG |  |  |  |  | EFSSOR-CMCI |  |  |  |  | EF5GSEDRX |  |  |
|  |  |  |  |  |  |  |  |  |  |  | '4F0B' |  |  |  |  | '4F0C' |  |  |  |  |  |  | '4F0D' |  |  |  |  | '4F0E' |  |  |  |  | ‘4F0X' |  |  |

***** next change *****

## 5.3	Subscription related procedures

### 5.3.xx	5GS eDRX procedure for ME

Requirement:	Service n°xxx "available".

Request:	The ME performs the reading procedure with EF5GSEDRX.

***** next change *****

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |
| --- | --- | --- |
| '2F00' | Application directory | Caution |
| '2F05' | Preferred languages | Yes |
| '2F06' | Access rule reference | Caution |
| '2F08' | UICC Maximum Power Consumption | No |
| '2FE2' | ICC identification | No |
| '4F01' | ProSe Monitoring Parameters | Yes |
| '4F01' | ACDC List | Yes |
| '4F01' | MCS Service Table | Yes |
| '4F01' | V2X Service Table | Yes |
| '4F01' | 5GS 3GPP location information | Caution (Note 1) |
| '4F02' | V2X configuration data | Yes |
| '4F02' | ProSe Announcing Parameters | Yes |
| '4F02' | MCS configuration data | Yes |
| '4F02' | 5GS non-3GPP location information | Caution (Note 1) |
| '4F03' | HPLMN ProSe Function | Yes |
| '4F03' | 5GS 3GPP Access NAS Security Context | Caution |
| '4F03' | V2X data policy over PC5 | Yes |
| '4F04' | ProSe Direct Communication Radio Parameters | Yes |
| '4F04' | 5GS non-3GPP Access NAS Security Context | Caution |
| '4F04' | V2X data policy over Uu | Yes |
| '4F05' | ProSe Direct Discovery Monitoring Radio Parameters | Yes |
| '4F05' | 5G authentication keys | No |
| '4F06' | ProSe Direct Discovery Announcing Radio Parameters | Yes |
| '4F06' | UAC Access Identities Configuration | Caution |
| '4F07' | ProSe Policy Parameters | Yes |
| '4F07' | Subscriber Concealed Identifier Calculation Information | Yes |
| '4F09' | SUPI as Network Access Identifier | Caution |
| '4F09' | ProSe Group Counter | No |
| '4F0A' | Routing Indicator | Yes (Note 5) |
| '4F0B' | UE Route Selection Policies | Yes |
| '4F0C' | Trusted non-3GPP Serving network name list | Yes |
| '4F0D' | Pre-configured CAG information list | Caution |
| '4F0E' | Steering of roaming connected mode control information | Yes |
| ‘4F0X' | 5GS eDRX Parameters | Yes |
| '4F10' | ProSe Service Table | Caution |
| '4F11' | ProSe UsageInformationReportingConfiguration | Caution (Note 4) |
| '4F12' | ProSe Group Member Discovery Parameters | Yes |
| '4F13' | ProSe Relay Parameters | Yes |
| '4F14' | ProSe Relay Discovery Parameters | Yes |
| '4F20' | Image data | Yes |
| '4F20' | GSM Ciphering key Kc | No |
| '4FXX' | Image Instance data Files | Yes |
| '4FXX' | ACDC OS Configuration | Yes |
| '4F21' | ICE graphics | Yes |
| '4FXX' | Unique identifier | Yes |
| '4F22' | Phone book synchronisation counter | Yes |
| '4F23' | Change counter | Yes |
| '4F24' | Previous unique identifier | Yes |
| '4F30' | Phone book reference file | Yes |
| '4FXX' | Capability configuration parameters 1 | Yes |
| '4F30' | SoLSA Access Indicator | Caution |
| '4F31' | SoLSA LSA List | Caution |
| '4FXX' | LSA Descriptor files | Caution |
| '4F52' | GPRS Ciphring key KcGPRS | No |
| '4F63' | CPBCCH Information | No |
| …… | …… | … |
| '6FE4' | EPS NAS Security Context | Caution |
| '6FE5' | Public Service Identity of the SM-SC | Yes |
| '6FE6' | USAT Facility Control | Caution |
| '6FE7' | UICC IARI | Caution (Note 3) |
| '6FE8' | Non Access Stratum Configuration | Yes |
| '6FE9' | UICC certificate | Yes |
| '6FEA' | Relay Node ID | Yes |
| '6FEB' | Max value of Secure Channel counter | Caution |
| '6FEC' | Public Warning System | Yes |
| '6FED' | FDN URI | Yes (Note 2) |
| '6FEE' | BDN URI | Yes |
| '6FEF' | SDN URI | Yes (Note 2) |
| '6FF0' | IMEI(SV) Allowed List | Yes |
| '6FF1' | IMEI(SV) Pairing Status | Caution |
| '6FF2' | IMEI(SV) Pairing Devices | Caution |
| '6FF3' | Home ePDG Identifier | Yes |
| '6FF4' | ePDG Selection Information | Yes |
| '6FF5' | Emergency ePDG Identifier | Yes |
| '6FF6' | ePDG Selection Information for Emergency Services | Yes |
| '6FF7' | From Preferred | Yes |
| '6FF8' | IMSConfigData | Caution |
| '6FF9' | 3GPPPSDATAOFF | Caution |
| '6FFA' | 3GPPPSDATAOFFservicelist | Caution |
| '6FFB' | TV Configuration | Yes |
| '6FFC' | XCAP Configuration Data | Yes |
| '6FFD' | EARFCN List for MTC/NB-IOT UEs | Yes |
| 6FFE | MuD and MiD configuration data | Yes |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, EFEPSLOCI, EF5GS3GPPLOCI and EF5GSN3GPPLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. NOTE5: If EFRouting_Indicator is changed, the UICC shall issue File Change Notification REFRESH, as defined in 3GPP TS 31.111 [12]. |  |  |

Editor's Note:	The REFRESH for SUPI_NAI update needs to be further specified.

***** next change *****

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description | Value |
| --- | --- | --- |
| '2F00' | Application directory | Card issuer/operator dependent |
| '2F05' | Preferred languages | 'FF…FF' |
| '2F06' | Access rule reference | Card issuer/operator dependent |
| '2F08' | UICC Maximum Power Consumption | Card issuer/operator dependent |
| '2FE2' | ICC identification | operator dependent |
| '4F01' | ProSe Monitoring Parameters | Operator dependent |
| '4F01' | ACDC List | Operator dependent |
| '4F01' | MCS Service Table | '0000' |
| '4F01' | V2X Service Table | Operator dependent |
| '4F01' | 5GS 3GPP location information | 'FFFFFFFFFFFFFFFFFFFFFFFFFF xxxxxx 000000 01' (see NOTE 2) |
| '4F02' | V2X configuration data | Operator dependent |
| '4F02' | ProSe Announcing Parameters | Operator dependent |
| '4F02' | MCS configuration data | Operator dependent |
| '4F02' | 5GS non-3GPP location information | 'FFFFFFFFFFFFFFFFFFFFFFFFFF xxxxxx 000000 01' (see NOTE 2) |
| '4F03' | HPLMN ProSe Function | Operator dependent |
| '4F03' | 5GS 3GPP Access NAS Security Context | 'FF…FF' |
| '4F03' | V2X data policy over PC5 | Operator dependent |
| '4F04' | ProSe Direct Communication Radio Parameters | Operator dependent |
| '4F04' | 5GS non-3GPP Access NAS Security Context | 'FF…FF' |
| '4F04' | V2X data policy over Uu | Operator dependent |
| '4F05' | ProSe Direct Discovery Monitoring Radio Parameters | Operator dependent |
| '4F05' | 5G authentication keys | 'FF…FF' |
| '4F06' | ProSe Direct Discovery Announcing Radio Parameters | Operator dependent |
| '4F06' | UAC Access Identities Configuration | Operator dependent |
| '4F07' | ProSe Policy Parameters | Operator dependent |
| '4F07' | Subscriber Concealed Identifier Calculation Information | Operator dependent |
| '4F08' | 5GS Operator PLMN List | Operator dependent |
| '4F08' | ProSe PLMN Parameters | Operator dependent |
| '4F09' | SUPI as Network Access Identifier | Operator dependent |
| '4F09' | ProSe Group Counter | 'FF…FF' |
| '4F0A' | Routing Indicator | 'F0FFFFFF' |
| '4F0B' | UE Route Selection Policies | Operator dependent |
| '4F0C' | Trusted non-3GPP Serving network name list | Operator dependent |
| '4F0D' | Pre-configured CAG information list | Operator dependent |
| '4F0E' | Steering of roaming connected mode control information | Operator dependent |
| ‘4F0X' | 5GS eDRX Parameters | Operator dependent |
| '4F10' | ProSe Service Table | Operator dependent |
| '4F11' | ProSe UsageInformationReportingConfiguration | Operator dependent |
| '4F12' | ProSe Group Member Discovery Parameters | Operator dependent |
| '4F13' | ProSe Relay Parameters | Operator dependent |
| '4F14' | ProSe Relay Discovery Parameters | Operator dependent |
| '4F20' | Image data | '00FF...FF' |
| '4F20' | GSM Ciphering key Kc | 'FF...FF07' |
| '4FXX' | Image instance data files | 'FF…FF' |
| '4FXX' | ACDC OS Configuration | Operator dependent |
| '4FXX' | TV User Service Description | Operator dependent |
| '4F21' | ICE graphics | 'FF…FF' |
| '4FXX' | Unique identifier | '0000' |
| .... | ….. | …. |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxxxxx stands for any valid MCC and MNC, coded according to TS 24.008 [9].

***** next change *****

# H.9	List of SFI Values at the DF 5GS Level

| File Identification | SFI | Description |
| --- | --- | --- |
| '4F01' | '01' | 5GS 3GPP location information (Note) |
| '4F02' | '02' | 5GS non-3GPP location information (Note) |
| '4F03' | '03' | 5GS 3GPP Access NAS Security Context |
| '4F04' | '04' | 5GS non-3GPP Access NAS Security Context |
| '4F05' | '05' | 5G authentication keys |
| '4F06' | '06' | UAC Access Identities Configuration |
| '4F07' | '07' | Subscriber Concealed Identifier Calculation Information |
| '4F08' | '08' | 5GS Operator PLMN List |
| '4F09' | '09' | SUPI as Network Access Identifier |
| '4F0A' | '0A' | Routing Indicator |
| '4F0B' | '0B' | UE Route Selection Policies |
| '4F0C' | '0C' | Trusted non-3GPP Serving network name list |
| '4F0D' | '0D' | Pre-configured CAG information list |
| '4F0E' | '0E' | Steering of roaming connected mode control information |
| ‘4F0X' | '0X' | 5GS eDRX Parameters |
| NOTE: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EF5GS3GPPLOCI and EF5GSN3GPPLOCI accordingly. |  |  |

All other SFI values are reserved for future use.

*****End of change *****