3GPP TSG-CT WG6 Meeting #114	C6-230059

# Athens, Greece, 28th Feb – 03rd March 2023	    revision of C6-230058, C6-230044, C6-230039

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0978 | rev | 3 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EF to store parameters for SENSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IDEMIA, Deutsche Telekom, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SENSE |  |  |  |  |  | Date: |  |  | 2023-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Within the work item SENSE a possibility to store parameters in the USIM has been defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of an EF to store parameters for the "Operator controlled signal threshold per access technology" as defined in 3GPP TS 23.122 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8, 4.2.94, 4.2.114, 4.2.11x (new), 4.7, 5.xx (new), Annex A, Annex D, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C6-230058 was C6-230044 was C6-230039 |  |  |  |  |  |  |  |  |

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n8 |  |  | M |  | 1 byte |
| 2 | Services n9 to n16 |  |  | O |  | 1 byte |
| 3 | Services n17 to n24 |  |  | O |  | 1 byte |
| 4 | Services n25 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n(8X-7) to n(8X) |  |  | O |  | 1 byte |

| -Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Contents: | Service n°1: |  | Local Phone Book |  |
|  | Service n°2: |  | Fixed Dialling Numbers (FDN) |  |
|  | Service n°3: |  | Extension 2 |  |
|  | Service n°4: |  | Service Dialling Numbers (SDN) |  |
|  | … |  |  |  |
|  | … |  |  |  |
|  |  | Service n°144 | Multiplier Coefficient for Higher Priority PLMN search via NG-RAN satellite access |
|  |  | Service n°145 | KAUSF derivation configuration |
|  |  | Service n°xxx | UE configured for using the "Operator controlled signal threshold per access technology" as defined in 3GPP TS 23.122 [31] |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

Service n°46 can only be declared "available" if service n°45 is declared "available".

Service n°95, n°99 and n°115 shall not be declared "available" if an ISIM application is present on the UICC.

Service n°125 shall only be taken into account if Service n°124 is declared "available". If Service n°124 and Service n°125 are declared "available", the "SUCI calculation is to be performed by the USIM". If Service n°124 is declared "available" and Service n°125 is not declared "available", the "SUCI calculation is to be performed by the ME".

Coding:

1 bit is used to code each service:
bit = 1: service available;
bit = 0: service not available.

-	Service available means that the USIM has the capability to support the service and that the service is available for the user of the USIM unless the service is identified as "disabled" in EFEST.
Service not available means that the service shall not be used by the USIM user, even if the USIM has the capability to support the service.

First byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | B4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°4 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°5 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°6 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°7 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°8 |

Second byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | B4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°9 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°10 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°11 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°12 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°13 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°14 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°16 |

etc.

### ********************************* NEXT CHANGE *******************************

### 4.2.94	EFNASCONFIG (Non Access Stratum Configuration)

If service n°96 is "available", this file shall be present. This EF contains some of the NAS configuration parameters defined in TS 24.368 [65]. For each of these NAS configuration parameters, a parameter provided in EFNASCONFIG shall take precedence over the corresponding parameter stored in the ME's non-volatile memory.

| Identifier: '6FE8' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: Z bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM ACTIVATE ADM DEACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to Z | NAS configuration parameter TLV objects |  |  | M |  | Z bytes |

NAS configuration parameter information

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| NAS signalling priority Tag | '80' | O | 1 |
| Length | L1 | C | Note 1 |
| NAS signalling priority value | -- | C | L1 |
| NMO I Behaviour Tag | '81' | O | 1 |
| Length | L2 | C | Note 1 |
| NMO I Behaviour value | -- | C | L2 |
| Attach with IMSI Tag | '82' | O | 1 |
| Length | L3 | C | Note 1 |
| Attach with IMSI value | -- | C | L3 |
| Minimum Periodic Search Timer Tag | '83' | O | 1 |
| Length | L4 | C | Note 1 |
| Minimum Periodic Search Timer value | -- | C | L4 |
| Extended access barring Tag | '84' | O | 1 |
| Length | L5 | C | Note 1 |
| Extended access barring value | -- | C | L5 |
| Timer T3245 Behaviour Tag | '85' | O | 1 |
| Length | L6 | C | Note 1 |
| Timer T3245 Behaviour value | -- | C | L6 |
| Override NAS signalling low priority Tag | '86' | O | 1 |
| Length | L7 | C | Note 1 |
| Override NAS signalling low priority value | -- | C | L7 |
| Override Extended access barring Tag | '87' | O | 1 |
| Length | L8 | C | Note 1 |
| Override Extended access barring value | -- | C | L8 |
| Fast First Higher Priority PLMN Search Tag | '88' | O | 1 |
| Length | L9 | C | Note 1 |
| Fast First Higher Priority PLMN value | -- | C | L9 |
| E-UTRA Disabling Allowed for EMM cause #15 Tag | '89' | O | 1 |
| Length | L10 | C | Note 1 |
| E-UTRA Disabling Allowed for EMM cause #15 Value | - | C | L10 |
| SM_RetryWaitTime Tag | '8A' | O | 1 |
| Length | L11 | C | Note 1 |
| SM_RetryWaitTime value | -- | C | L11 |
| SM_RetryAtRATChange Tag | '8B' | O | 1 |
| Length | L12 | C | Note 1 |
| SM_RetryAtRATChange value | -- | C | L12 |
| Default_DCN_ID Tag | '8C' | O | 1 |
| Length | L13 | C | Note 1 |
| Default_DCN_ID value | -- | C | L13 |
| Exception Data Reporting Allowed Tag | '8D' | O | 1 |
| Length | L14 | C | Note 1 |
| Exception Data Reporting Allowed value | -- | C | L14 |
| RLOSPreferredPLMNList Tag | '8E' | O | 1 |
| Length | L15 | C | 4n (Note 1) |
| RLOSPreferredPLMNList value | -- | C | L15 |
| RLOSAllowedMCCList Tag | '8F' | O | 1 |
| Length | L16 | C | 2n (Note 1) |
| RLOSAllowedMCCList value | -- | C | L16 |
| No E-UTRA Disabling In 5GS Tag | '90' | O | 1 |
| Length | L17 | C | Note 1 |
| No E-UTRA Disabling In 5GS value | -- | C | L17 |
| UE_configured_for_using_ the "Operator controlled signal threshold per access technology" tag | '91' | O | 1 |
| Length | L18 | C | Note 1 |
| UE_configured_for_using_ the "Operator controlled signal threshold per access technology" value | -- | C | L18 |
| Note 1 : The length is coded according to ISO/IEC 8825-1 [35] Note 2: C; if the Tag is present, this is mandatory. |  |  |  |

-  NAS signalling priority

Contents:

As described in TS 24.368 [65], used to determine the NAS signalling priority included in NAS messages.

Coding:

As defined for the NAS_SignallingPriority leaf in TS 24.368 [65].

-  NMO I Behaviour

Content:

As described in TS 24.368 [65], indicates whether the "NMO I, Network Mode of Operation I" indication is applied by the UE.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the NMO_I_Behaviour leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

- Attach with IMSI

Content:

As described in TS 24.368 [65], indicates whether attach with IMSI is performed when moving to a non-equivalent PLMN.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the AttachWithIMSI leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

- Minimum Periodic Search Timer

Content:

As described in TS 24.368 [65], gives a minimum value in minutes for the timer T controlling the periodic search for higher prioritized PLMNs. The content applies a minimum value that may override the value in file EFHPPLMN (Higher Priority PLMN search period), as specified in TS 23.122 [31] clause 4.4.3.3.1.

Coding:

As defined for the MinimumPeriodicSearchTimer leaf in TS 24.368 [65].

- Extended access barring

Content:

As described in TS 24.368 [65], indicates whether the UE applies extended access barring.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for ExtendedAccessBarring leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

- Timer T3245 Behaviour

Content:

As described in TS 24.368 [65], indicates whether the timer T3245 and the related functionality is used by the UE.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for Timer_T3245_Behaviour leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-  Override NAS signalling low priority

Contents:

As described in TS 24.368 [65], used to determine whether the NAS signalling priority included in NAS messages can be overriden.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the Override_NAS_SignallingLowPriority leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

The Override NAS signalling low priority tag and the Override Extended access barring tag shall be set to the same value, e.g., if the UE is configured to override the NAS signalling low access priority indicator, then it has also to be configured to override Extended access barring (see 3GPP TS 23.401 [69] clause 4.3.17.4).

-  Override Extended access barring

Contents:

As described in TS 24.368 [65], used to determine whether the Extended access barring included in NAS messages can be overriden.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the Override_ ExtendedAccessBarring leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

The Override Extended access barring tag and the Override NAS signalling low priority tag shall be set to the same value, e.g., if the UE is configured to override Extended access barring, then it has also to be configured to override the NAS signalling low access priority indicator (see 3GPP TS 23.401 [69] clause 4.3.17.4).

-	Fast First Higher Priority PLMN Search

Contents:

Determine whether the UE can perform Fast First Higher Priority PLMN Search upon selecting a VPLMN as specified in 3GPP TS 23.122 [31].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the FastFirstHigherPriorityPLMNSearch leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	E-UTRA Disabling Allowed for EMM cause #15

Contents:

Determine whether the UE is allowed to disable the E-UTRA capability when it receives the Extended EMM IE with value cause "E-UTRAN not allowed" as specified in 3GPP TS 24.301 [51].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the EUTRADisablingAllowedforEMMcause15 leaf in 3GPP TS 24.368 [65] |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	SM_RetryWaitTime

Contents:

As described in TS 24.368 [65], provides a configured UE retry wait time value applicable when in HPLMN or EHPLMN (see 3GPP TS 23.122 [31]) for controlling the UE session management retry behaviour when prior session management request was rejected by the network with cause value #8, #27, #32, #33 as specified in 3GPP TS 24.008 [9] and 3GPP TS 24.301 [51], or when prior session management request was rejected by the network with cause value #8, #27, #32, #33, #70 as specified in 3GPP TS 24.501 [104].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | bits b1 to b8 to be interpreted as defined for the SM_RetryWaitTime leaf in 3GPP TS 24.368 [65] |

-	SM_RetryAtRATChange

Contents:

As described in TS 24.368 [65], indicates the UE's retry behaviour when in HPLMN or EHPLMN (see 3GPP TS 23.122 [31]) after inter-system change between S1 mode and A/Gb or Iu mode or N1 mode as specified in 3GPP TS 24.008 [9] and 3GPP TS 24.301 [51] and 3GPP TS 24.501 [104].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the SM_RetryAtRATChange leaf in 3GPP TS 24.368 [65] |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	Default_DCN_ID

Contents:

As described in 3GPP TS 24.368 [65], indicates the default DCN-ID which is provided by NAS to the lower layers at establishment of the NAS signalling connection as specified in 3GPP TS 24.301 [51].

Coding:

As the DCN-ID value inside the DCN-ID IEI defined in TS 24.008 [9] clause 10.5.5.35.

-	Exception Data Reporting Allowed

Contents:

As described in 3GPP TS 24.368 [65], for the UE in NB-S1 mode indicates whether the UE is allowed to use the RRC establishment cause mo-ExceptionData, as specified in 3GPP TS 24.301 [51].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the ExceptionDataReportingAllowed leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	RLOSPreferredPLMNList

Contents:

As described is 3GPP TS 24.368 [65], it contains in descending priority order, a list of preferred RLOS PLMNs configured to the UE for selection of a PLMN offering access to RLOS as specified in 3GPP TS 23.122 [31].

Coding:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 to 3 | 1st PLMN (highest priority) | M | 3 bytes |
| 4 | 1st PLMN Config | M | 1 byte |
| 5 to 7 | 2nd PLMN | O | 3 bytes |
| 8 | 2nd PLMN Config | O | 1 byte |
| 9 to 11 | 3rd PLMN | O | 3 bytes |
| 12 | 3rd PLMN Config | O | 1 byte |
| : | : | : | : |
| (4n-3) to (4n-1) | nth PLMN | O | 3 bytes |
| 4n | nth PLMN Config | O | 1 byte |

Coding of PLMN Config:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1=0: This PLMN has higher priority than the next PLMN in the list b1=1: This PLMN has same priority than the next PLMN in the list |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	RLOSAllowedMCCList

Contents:

As described is 3GPP TS 24.368 [65], it contains a list of allowed MCC configured to the UE to allow the UE to check whether the MCC of the PLMN ID of the serving network that advertises RLOS service is present in the list and whether the MCC part of the IMSI configured in the USIM is present in the list of MCCs before initiating the RLOS connection as specified in 3GPP TS 23.122 [31].

Coding:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 to 2 | 1st MCC | M | 2 bytes |
| 3 to 4 | 2nd MCC | O | 2 bytes |
| 5 to 6 | 3rd MCC | O | 2 bytes |
| : | : | : | : |
| (2n-1) to (2n) | nth MCC | O | 2 bytes |

Coding of MCC: according to TS 24.008 [9].

-	A BCD value of 'D' in any of the MCC digits shall be used to indicate a "wild" value for that corresponding MCC digit. A value of 'DDD' represents "any MCC" value.

-	No E-UTRA Disabling In 5GS

Contents:

As described in TS 24.368 [65], used to indicate whether E-UTRA Disabling In 5GS is disabled or enabled.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for NoEUTRADisablingIn5GS leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	UE_configured_for_using_ the "Operator controlled signal threshold per access technology"

Contents:

As described in TS 24.368 [65], used to indicate whether the UE is configured to use the "Operator controlled signal threshold per access technology" or not.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for UE_configured_for_using_ the "Operator controlled signal threshold per access technology" leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

If any of these NAS configuration parameters is neither included in EFNASCONFIG nor stored in the ME's non-volatile memory, the default value as defined for the corresponding leaf in TS 24.368 [65] shall apply.

Unused bytes shall be set to 'FF'.

### ********************************* NEXT CHANGE *******************************

### 4.2.114	EFeAKA (enhanced AKA support)

The file EFeAKA stores information about the support of the enhanced AKA. The content of the EF may be updated via OTA. The content of this EF is internal to the USIM.

It contains information if the enhanced SQN calculation as defined in 3GPP TS 33.102 [13] Annex J is supported by the USIM. This information can be used by the USIM internally to identify which SQN calculation to use.

.

| Identifier: '6F01' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X bytes, (X1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ADM UPDATE ADM ACTIVATE ADM DEACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 - X | eAKA support |  |  | M |  | X bytes |

-	eAKA support.

Contents:

-	indicates the support of the enhanced SQN calculation

Coding:

-	the support is indicated by bit 1 in Byte 1 as defined below

Byte 1:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1=0: enhanced SQN calculation not supported b1=1: enhanced SQN calculation supported |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |

All other bytes are RFU.

### 4.2.11x	EFSENSE ("Operator controlled signal threshold per access technology")

If service n°xxx is "available", this file shall be present.

The file EFSENSE stores parameters to support the feature "Operator controlled signal threshold per access technology"

The ME shall use the information in this EF only if the UE_configured_for_using_ the "Operator controlled signal threshold per access technology"  value in EFNASCONFIG is set to '1'.

| Identifier: '6F02' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X bytes, (X1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM ACTIVATE ADM DEACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 - X | the "Operator controlled signal threshold per access technology" parameter TLV object |  |  | M |  | X bytes |

SENSE parameter TLV data object:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Operator controlled signal threshold per access technology tag | '80' | M | 1 |
| Length | 4n | M | Note |
| Access Technology 1 | -- | C | 2 |
| Operator controlled signal threshold per access technology 1 | -- | C | 2 |
| … |  |  |  |
| Access Technology n | -- | C | 2 |
| Operator controlled signal threshold per access technology n | -- | C | 2 |
| Note: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

-	Access Technology:

Contents:

- The Access Technology of the PLMN for which an operator controlled signal threshold per access technology is defined.

Coding:

-	See EFPLMNwACT for coding.

-	Operator controlled signal threshold per access technology:

Contents:

- Operator controlled signal threshold per access technology value for an access technology

Coding:

- the Operator controlled signal threshold per access technology value for an access technology is coded on 2 bytes using BCD coding. The maximum number of digits is 4. Unused digits are padded with 'F'. The Operator controlled signal threshold per access technology value is to be interpreted as Dezibel Milliwatt (dBm).

NOTE 1:	Digits are not swapped, i.e. for instance the value "-105dBm" is coded as '10 5F',						 the value "-80dBm" is coded as '80 FF'.

NOTE 2:	The considered range of the "Operator controlled signal threshold per access technology" is between the cell selection criterion and the high quality signal defined for an access technology in the appropriate AS specifications.

Unused bytes shall be set to 'FF'.

***************************** NEXT CHANGE *********************************

## 4.7	Files of USIM

This clause contains two figures depicting the file structure of the UICC and the ADFUSIM. ADFUSIM shall be selected using the AID and information in EFDIR.

|  |  |  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |

…..

…..

|  | ADFUSIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | EFLI |  |  | EFARR |  |  |  | EFIMSI |  |  | EFKeys |  |  | EFKeysPS |  |
|  |  |  |  |  |  |  | '6F05' |  |  | '6F06' |  |  |  | '6F07' |  |  | '6F08' |  |  | '6F09' |  |
|  |  |  |  |  |  |  | EFDCK |  |  | EFHPPLMN |  |  |  | EFCNL |  |  | EFACMmax |  |  | EFUST |  |
|  |  |  |  |  |  |  | '6F2C' |  |  | '6F31' |  |  |  | '6F32' |  |  | '6F37' |  |  | '6F38' |  |
|  |  |  |  |  |  |  | EFACM |  |  | EFFDN |  |  |  | EFSMS |  |  | EFGID1 |  |  | EFGID2 |  |
|  |  |  |  |  |  |  | '6F39' |  |  | '6F3B' |  |  |  | '6F3C' |  |  | '6F3E' |  |  | '6F3F' |  |
|  |  |  |  |  |  |  | EFMSISDN |  |  | EFPUCT |  |  |  | EFSMSP |  |  | EFSMSS |  |  | EFCBMI |  |
|  |  |  |  |  |  |  | '6F40' |  |  | '6F41' |  |  |  | '6F42' |  |  | '6F43' |  |  | '6F45' |  |
|  |  |  |  |  |  |  | EFSPN |  |  | EFSMSR |  |  |  | EFCBMID |  |  | EFSDN |  |  | EFEXT2 |  |
|  |  |  |  |  |  |  | '6F46' |  |  | '6F47' |  |  |  | '6F48' |  |  | '6F49' |  |  | '6F4B' |  |
|  |  |  |  |  |  |  | EFEXT3 |  |  | EFBDN |  |  |  | EFEXT5 |  |  | EFCCP2 |  |  | EFCBMIR |  |
|  |  |  |  |  |  |  | '6F4C' |  |  | '6F4D' |  |  |  | '6F4E' |  |  | '6F4F' |  |  | '6F50' |  |
|  |  |  |  |  |  |  | EFEXT4 |  |  | EFEST |  |  |  | EFACL |  |  | EFCMI |  |  | EFSTART-HFN |  |
|  |  |  |  |  |  |  | '6F55' |  |  | '6F56' |  |  |  | '6F57' |  |  | '6F58' |  |  | '6F5B' |  |
|  |  |  |  |  |  |  | EFTHRESHOLD |  |  | EFPLMNwAcT |  |  |  | EFOPLMNwAcT |  |  | EFHPLMNwAcT |  |  | EFPSLOCI |  |
|  |  |  |  |  |  |  | '6F5C' |  |  | '6F60' |  |  |  | '6F61' |  |  | '6F62' |  |  | '6F73' |  |
|  |  |  |  |  |  |  | EFACC |  |  | EFFPLMN |  |  |  | EFLOCI |  |  | EFICI |  |  | EFOCI |  |
|  |  |  |  |  |  |  | '6F78' |  |  | '6F7B' |  |  |  | '6F7E' |  |  | '6F80' |  |  | '6F81' |  |
|  |  |  |  |  |  |  | EFICT |  |  | EFOCT |  |  |  | EFAD |  |  | EFVGCS |  |  | EFVGCSS |  |
|  |  |  |  |  |  |  | '6F82' |  |  | '6F83' |  |  |  | '6FAD' |  |  | '6FB1' |  |  | '6FB2' |  |
|  |  |  |  |  |  |  | EFVBS |  |  | EFVBSS |  |  |  | EFeMLPP |  |  | EFAaeM |  |  | EFECC |  |
|  |  |  |  |  |  |  | '6FB3' |  |  | '6FB4' |  |  |  | '6FB5' |  |  | '6FB6' |  |  | '6FB7' |  |
|  |  |  |  |  |  |  | EFHiddenkey |  |  | EFNETPAR |  |  |  | EFPNN |  |  | EFOPL |  |  | EFMBDN |  |
|  |  |  |  |  |  |  | '6FC3' |  |  | '6FC4' |  |  |  | '6FC5' |  |  | '6FC6' |  |  | '6FC7' |  |
|  |  |  |  |  |  |  | EFEXT6 |  |  | EFMBI |  |  |  | EFMWIS |  |  | EFCFIS |  |  | EFEXT7 |  |
|  |  |  |  |  |  |  | '6FC8' |  |  | '6FC9' |  |  |  | '6FCA' |  |  | '6FCB' |  |  | '6FCC' |  |
|  |  |  |  |  |  |  | EFSPDI |  |  | EFMMSN |  |  |  | EFEXT8 |  |  | EFMMSICP |  |  | EFMMSUP |  |
|  |  |  |  |  |  |  | '6FCD' |  |  | '6FCE' |  |  |  | '6FCF' |  |  | '6FD0' |  |  | '6FD1' |  |
|  |  |  |  |  |  |  | EFMMSUCP |  |  | EFNIA |  |  |  | EFVGCSCA |  |  | EFVBSCA |  |  | EFGBAP |  |
|  |  |  |  |  |  |  | '6FD2' |  |  | '6FD3' |  |  |  | '6FD4' |  |  | '6FD5' |  |  | '6FD6' |  |
|  |  |  |  |  |  |  | EFMSK |  |  | EFMUK |  |  |  | EFEHPLMN |  |  | EFGBANL |  |  | EFEHPLMNPI |  |
|  |  |  |  |  |  |  | '6FD7' |  |  | '6FD8' |  |  |  | '6FD9' |  |  | '6FDA' |  |  | '6FDB' |  |
|  |  |  |  |  |  |  | EFLRPLMNSI |  |  | EFNAFKCA |  |  |  | EFSPNI |  |  | EFPNNI |  |  | EFNCP-IP |  |
|  |  |  |  |  |  |  | '6FDC' |  |  | '6FDD' |  |  |  | '6FDE' |  |  | '6FDF' |  |  | '6FE2' |  |
|  |  |  |  |  |  |  | EFEPSLOCI |  |  | EFEPSNSC |  |  |  | EFUFC |  |  | EFUICCIARI |  |  | EFNASCONFIG |  |
|  |  |  |  |  |  |  | '6FE3' |  |  | '6FE4' |  |  |  | '6FE6' |  |  | '6FE7' |  |  | '6FE8' |  |
|  |  |  |  |  |  |  | EFPWS |  |  | EFFDNURI |  |  |  | EFBDNURI |  |  | EFSDNURI |  |  | EFIAL |  |
|  |  |  |  |  |  |  | '6FEC' |  |  | '6FED' |  |  |  | '6FEE' |  |  | '6FEF' |  |  | '6FF0' |  |
|  |  |  |  |  |  |  | EFIPS |  |  | EFIPD |  |  |  | EFePDGId |  |  | EFePDGSelection |  |  | EFePDGIdEm |  |
|  |  |  |  |  |  |  | '6FF1' |  |  | '6FF2' |  |  |  | '6FF3' |  |  | '6FF4' |  |  | '6FF5' |  |
|  |  |  |  |  |  |  | EFePDGSelectionEm |  |  | EFFromPreferred |  |  |  | EFIMSConfigData |  |  | EF3GPPPSDATAOFF |  |  | EF3GPPPSDATAOFFservicelist |  |
|  |  |  |  |  |  |  | '6FF6' |  |  | '6FF7' |  |  |  | '6FF8' |  |  | '6FF9' |  |  | '6FFA' |  |
|  |  |  |  |  |  |  | EFTVCONFIG |  |  | EFXCAPConfigData |  |  |  | EFEARFCNList |  |  | EFMuDMiDConfigData |  |  | EFeAKA |  |
|  |  |  |  |  |  |  | '6FFB' |  |  | '6FFC' |  |  |  | '6FFD' |  |  | '6FFE' |  |  | '6F01' |  |
|  |  |  |  |  |  |  | EFSENSE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '6F02' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

….

******************************* NEXT CHANGE ********************************************

## 5.xx	SENSE related procedures

### 5.xx.1	Non Access Stratum Configuration

An ME supporting SENSE shall read the UE_configured_for_using_ the "Operator controlled signal threshold per access technology"  parameter in EFNASCONFIG (Non Access Stratum Configuration). If the valie is set to "1", the ME shall read EFSENSE, if available.

### 5.xx.2	SENSE parameter related procedure

Requirement:	Service n°xxx is "available".

Request:	The ME performs the reading procedure with EFSENSE.

The ME shall follow the procedures as specified in 3GPP TS 23.122 [31] for  the "Operator controlled signal threshold per access technology".

******************************* NEXT CHANGE ********************************************

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |  |
| --- | --- | --- | --- |
| '2F00' | Application directory | Caution |  |
| '2F05' | Preferred languages | Yes |  |
| '4F86' | Operator HNB name | Yes |  |
| … |  |  |  |
| … |  |  |  |
| '6F01' | enhanced AKA support | Caution |  |
| '6F02' | SENSE | Yes |  |
| '6F05' | Language indication | Yes |  |
| '6F06' | Access rule reference (under ADFUSIM and DFTELECOM) | Caution |  |
| … |  |  |  |
| … |  |  |  |
| '6FFC' | XCAP Configuration Data | Yes |
| '6FFD' | EARFCN List for MTC/NB-IOT UEs | Yes |
| '6FFE' | MuD and MiD configuration data | Yes |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, EFEPSLOCI, EF5GS3GPPLOCI and EF5GSN3GPPLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. NOTE5: If EFRouting_Indicator is changed, the UICC shall issue File Change Notification REFRESH, as defined in 3GPP TS 31.111 [12]. |  |  |  |

Editor's Note:	The REFRESH for SUPI_NAI update needs to be further specified.

************************************ NEXT CHANGE **************************************

Annex D (informative):
Tags defined in 31.102

| Tag | Name of Data Element |  | Usage |
| --- | --- | --- | --- |
| '43' | Full name for network IEI |  | PLMN Network Name (EFPNN) |
| '45' | Short name for network IEI |  | PLMN Network Name (EFPNN) |
| '53' | MBMS Data Object |  | AUTHENTICATE command parameter, in MBMS security context |
| '53' | MBMS operation response Data Object The following tags are encapsulated within '53' 'DB' successful MBMS operation tag |  | Response to AUTHENTICATE command, in MBMS security context |
| '73' | Key Derivation Data Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag '81' Counter limit tag '82' Request MAC tag '83' NAF_ID tag '84' Terminal_ID tag '85' Terminal_appli_ID_tag '86' UICC_appli_ID tag '87' RANDx tag 'A0' Key Identifier tag |  | AUTHENTICATE command parameter, in Local Key Establishment security context |
|  | …. |  |  |
| '80' | Protection Scheme Identifier List data object tag | Protection Scheme Identifier List (EFSUCI_Calc_Info) |  |
| '80' | Network Specific Identifier TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |  |
| '80' | SOR-CMCI data object tag | EFSOR-CMCI |  |
| '80' | Operator controlled signal threshold per access technology tag | EFSENSE |  |
| '81' | Global Line Identifier Tag TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |  |
| '82' | Global Cable Identifier TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |  |
|  | …. |  |  |
| '90' | No E-UTRA Disabling In 5GS Tag |  | Non Access Stratum Configuration (EFNASCONFIG) |
| '91' | UE_configured_for_using_SENSE tag |  | Non Access Stratum Configuration (EFNASCONFIG) |
| 'A0' | MUK ID tag The following tags are encapsulated within 'A0' '80' MUk IDr tag '82' MUk IDi tag |  | MBMS User Key (EFMUK) |
|  | …. |  |  |
|  | …. |  |  |

************************************ NEXT CHANGE **************************************

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification |  | Description |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| '2F00' |  | Application directory |  | Card issuer/operator dependent |  |
| '2F05' |  | Preferred languages |  | 'FF…FF' |  |
| '2F06' |  | Access rule reference |  | Card issuer/operator dependent |  |
| … |  |  |  |  |  |
| … |  |  |  |  |  |
| '4F86' |  | Operator HNB name |  | Operator dependent |  |
| '6F01' |  | enhanced AKA support |  | Operator dependent |  |
| '6F02' |  | SENSE |  | 'FF…FF' |  |
| '6F05' |  | Language indication |  | 'FF…FF' |  |
| '6F06' |  | Access rule reference (under ADFUSIM and DFTELECOM) |  | Card issuer/operator dependent |  |
| … |  |  |  |  |  |
| … |  |  |  |  |  |
| '6FFE' |  | MuD and MiD Configuration Data |  | Operator dependent |  |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxxxxx stands for any valid MCC and MNC, coded according to TS 24.008 [9].

************************************* END OF CHANGES ****************************************