3GPP TSG-CT WG6 Meeting #114	C6-230073

# Athens, Greece, 28th Feb – 03rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0690 | rev | 1 | Current version: | 16.10.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |
| + |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test sequence for 27.22.4.7- REFRESH (steering of roaming in NG-RAN context) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R16 |  |  |  |  |  | Date: |  |  | 2023-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NG-RAN case not covered by REFRESH (steering of roaming) test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR updates : - Table B.1 test sequence reference 3.xx added, - Clause 27.22.4.7.3.2 conformance requirements to add missed requirements, - Clause 27.22.4.7.3.4 test sequence 3.xx added, - reception of first REFRESH (steering of roaming) - update EF FPLMN - ME does not register to another NG-SS - reception of second REFRESH (steering of roaming) - update EF FPLMN - ME registers to second NG-SS - reception of third REFRESH (steering of roaming) - No update of EF FPLMN - ME registers to first NG-SS - Clause 27.22.4.7.3.5 test requirement to include 3.xx test sequence coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case covering the REFRESH (Steering of roaming) in NG-RAN context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.4.7.3.2, 27.22.4.7.3.4, 27.22.4.7.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C6-230056 |  |  |  |  |  |  |  |  |

***** first change *****

## 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

Table B.1: Applicability of tests

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Rel-16 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PROFILE DOWNLOAD 27.22.1 | R99 | 1 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/1 | No |  |  |
| 2 | Contents of the TERMINAL PROFILE command 27.22.2 | R99 |  | M | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/1 | No |  |  |
| 3 | Servicing of Proactive UICC Commands 27.22.3 | R99 |  | M | M | M | M | M | M | M | M | M | M | M | M | M | M |  | No |  |  |
| 4 | DISPLAY TEXT 27.22.4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Unpacked | R99 | 1.1 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Screen busy | R99 | 1.2 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | high priority | R99 | 1.3 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Packed | R99 | 1.4 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | clear after delay | R99 | 1.5 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | long text up to 160 bytes | R99 | 1.6 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Backwards move in USIM session | R99 | 1.7 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 | No |  |  |
|  | Session terminated by user | R99 | 1.8 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 | No |  |  |

…

| 10 | REFRESH 27.22.4.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | USIM initialization, enabling FDN mode | R99 | 1.1 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | file change notification of FDN file | R99 | 1.2 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | USIM initialization and file change notification of ADN | R99 | 1.3 | C168 AND C177 AND C178 | C168 AND C177 AND C178 | C168 AND C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/24 AND E.1/110 AND E.1/111 | No |  |  |
|  | USIM initialization and full file change notification, enabling FDN mode | R99 | 1.4 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UICC reset | R99 | 1.5 |  |  |  | M | M | M | M | M | M | M | M | M | M | M | E.1/24 | No |  |  |

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Rel-16 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | USIM Initialization after SMS-PP data download | R99 | 1.6 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | USIM Application Reset | R99 | 1.7 |  |  |  | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UICC Reset for IMSI Changing procedure | R99 | 2.1 |  |  |  |  |  | M | M | M | M | M | M | M | M | M | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | USIM Application Reset for IMSI Changing procedure | R99 | 2.2 |  |  |  | M | M | M | M | M | M | M | M | M | M | M | E.1/24 | Yes |  |  |
|  | 3G Session Reset for IMSI Changing procedure | R99 | 2.3 |  |  |  |  |  | M | M | M | M | M | M | M | M | M | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | reject 3G Session Reset for IMSI Changing procedure during CScall | R99 | 2.4 |  |  |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E 1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Rel-16 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | reject UICC Reset for IMSI Changing procedure during CS call | R99 | 2.5 |  |  |  |  |  | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UICC Reset for IMSI Changing procedure during active PDP context | R99 | 2.6 |  |  |  |  |  | C215 | C215 | C215 | C215 | C215 | C215 | C215 | C215 | C215 | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | 3G Session Reset for IMSI Change procedure during active PDP context | R99 | 2.7 |  |  |  |  |  | C215 | C215 | C215 | C215 | C215 | C215 | C215 | C215 | C215 | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | Steering of roaming, UTRAN | Rel-7 | 3.1 |  |  |  |  | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/24 AND E.1/236 | UMTS System Simulator only |  |  |
|  | Steering of roaming, InterRAT | Rel-7 | 3.2 |  |  |  |  | C167 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | E.1/24 AND E.1/236 | UMTS System Simulator and System Simulator |  |  |
|  | Steering of roaming, E-UTRAN | Rel-8 | 3.3 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | C222 | E.1/24 AND E.1/135 AND E.1/236 | E-USS or NB-SS (See NOTE) |  |  |
|  | Steering of roaming, NG-RAN | Rel-16 | 3.xx |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/24 AND E.1/236 | NG-SS only |  |  |
|  | Refresh with AID, E-UTRAN or UTRAN | Rel-8 | 4.1 |  |  |  |  | C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | E.1/24 | E-USS only or UMTS System Simulator |  |  |

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Rel-16 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UICC Reset for IMSI Changing procedure, E-UTRAN | Rel-8 | 5.1 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | C222 | E.1/24 | E-USS or NB-SS (See NOTE) |  |  |
|  | 3G Session Reset for IMSI Changing procedure, E-UTRAN | Rel-8 | 5.2 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | C222 | E.1/24 | E-USS or NB-SS (See NOTE) |  |  |
|  | REFRESH, UICC Reset for IMSI Changing procedure, NG-RAN | Rel-16 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | REFRESH, 3G Session Reset for IMSI Changing procedure, NG-RAN | Rel-16 | 6.2 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | REFRESH, UICC Reset for SUPI_NAI Changing procedure, NG-RAN | Rel-16 | 7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |
|  | REFRESH, 3G Session Reset for SUPI_NAI Changing procedure, NG-RAN | Rel-16 | 7.2 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |

***** next change *****

##### 27.22.4.7.3	REFRESH (Steering of roaming)

27.22.4.7.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.3.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.1, clause 6.4.7, clause 6.6.13, clause 7.5.4,  clause 8.2, clause 8.6, clause 8.7 and clause 8.90.

Consequently the Rel-7 or later ME shall support the steering of roaming procedure as defined in:

-	TS 23.122 [29] clause 4.4.6.

27.22.4.7.3.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier. This shall require the ME to perform:

-	the steering of roaming procedure,

-	a successfull return of the result of the execution of the command in the TERMINAL RESPONSE command send to the UICC.

27.22.4.7.3.4	Method of test

27.22.4.7.3.4.1	Initial conditions

For sequences 3.1 and 3.2 the ME is connected to the USIM Simulator and connected to the USS/SS.

For sequence 3.3 the ME supporting E-UTRAN/NB-IoT is connected to the USIM Simulator and connected to the E-USS/NB-SS.

For sequence 3.xx the ME supporting NG-RAN is connected to the USIM Simulator and connected to the NG-SS.

For sequences 3.1 and 3.2:

The elementary files are coded as Toolkit default with the following exceptions:

EFFPLMN

Logically:	PLMN1:	254 002 (MCC MNC)

PLMN2:	254 003

PLMN3:	254 004

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 00 | 52 | 34 | 00 | 52 | 44 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |  |  |
|  | 32 | 54 | 00 | 32 | 64 | 00 |  |  |  |  |  |  |

EFOPLMNwACT

Logically:	1st PLMN:	254 001 (MCC MNC)

1st ACT:	UTRAN

2nd PLMN:	254 001

2nd ACT:	GSM

3rd PLMN:	274 002

3rd ACT:	UTRAN

4th PLMN:	274 003

4th ACT:	UTRAN

5th PLMN:	274 004

5th ACT:	UTRAN

6th PLMN:	274 005

6th ACT:	UTRAN

7th PLMN:	274 006

7th ACT:	UTRAN

8th PLMN:	274 007

8th ACT:	UTRAN

| Coding: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 14 | 00 | 80 | 00 | 52 | 14 | 00 | 00 | 80 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 72 | 24 | 00 | 80 | 00 | 72 | 34 | 00 | 80 | 00 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 72 | 44 | 00 | 80 | 00 | 72 | 54 | 00 | 80 | 00 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 72 | 64 | 00 | 80 | 00 | 72 | 74 | 00 | 80 | 00 |

For sequence 3.3:

The default E-UTRAN UICC, the default E-USS/NB-SS parameters and the following parameters are used:

EFFPLMN

Logically:	PLMN1:	254 002 (MCC MNC)

PLMN2:	254 003

PLMN3:	254 004

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 00 | 52 | 34 | 00 | 52 | 44 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |  |  |
|  | 32 | 54 | 00 | 32 | 64 | 00 |  |  |  |  |  |  |

EFOPLMNwACT

Logically:	1st PLMN:	254 001 (MCC MNC)

1st ACT:	E-UTRAN, UTRAN

2nd PLMN:	254 001

2nd ACT:	GSM

3rd PLMN:	274 002

3rd ACT:	E-UTRAN

4th PLMN:	274 003

4th ACT:	E-UTRAN

5th PLMN:	274 004

5th ACT:	E-UTRAN

6th PLMN:	274 005

6th ACT:	E-UTRAN

7th PLMN:	274 006

7th ACT:	E-UTRAN

8th PLMN:	274 007

8th ACT:	UTRAN

| Coding: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 14 | 00 | C0 | 00 | 52 | 14 | 00 | 00 | 80 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 72 | 24 | 00 | 40 | 00 | 72 | 34 | 00 | 40 | 00 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 72 | 44 | 00 | 40 | 00 | 72 | 54 | 00 | 40 | 00 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 72 | 64 | 00 | 40 | 00 | 72 | 74 | 00 | 80 | 00 |

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For sequence 3.xx:

The default NG-RAN UICC, the default NG-SS parameters and the following parameters are used:

EFFPLMN

Logically:	PLMN1:	254 002 (MCC MNC)

PLMN2:	254 003

PLMN3:	254 004

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 00 | 52 | 34 | 00 | 52 | 44 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |  |  |
|  | 32 | 54 | 00 | 32 | 64 | 00 |  |  |  |  |  |  |

EFOPLMNwACT

Logically:	1st PLMN:	254 001 (MCC MNC)

1st ACT:	NG-RAN

2nd PLMN:	254 001

2nd ACT:	GSM

3rd PLMN:	274 002

3rd ACT:	E-UTRAN

4th PLMN:	274 003

4th ACT:	E-UTRAN

5th PLMN:	274 004

5th ACT:	E-UTRAN

6th PLMN:	274 005

6th ACT:	E-UTRAN

7th PLMN:	274 006

7th ACT:	E-UTRAN

8th PLMN:	274 007

8th ACT:	UTRAN

| Coding: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 14 | 00 | 08 | 00 | 52 | 14 | 00 | 00 | 80 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 72 | 24 | 00 | 40 | 00 | 72 | 34 | 00 | 40 | 00 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 72 | 44 | 00 | 40 | 00 | 72 | 54 | 00 | 40 | 00 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 72 | 64 | 00 | 40 | 00 | 72 | 74 | 00 | 80 | 00 |

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

***** Next change *****

27.22.4.7.3.4.2	Procedure

Expected Sequence 3.1 (REFRESH, Steering of roaming, UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USS | The first UMTS USS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - LAI (MCC/MNC/LAC): 254/001/0001. - Access control: unrestricted. The second UMTS USS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - LAI (MCC/MNC/LAC): 254/002/0001. - Access control: unrestricted. |  |
| 2 | ME USS | The ME registers to the first USS. |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.1.1 | [Setting up LOCATION STATUS Event] |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1 |  |
| 6a | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1 |  |
| 6b | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.1.2 | This step applies only if A.1/171 |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 3.1.1 | Note: Step 11 can occur at any time during execution of steps 10a to 10d |
| 10a | UICC | Void |  |
| 10b | ME UICC | Update of EF FPLMN | [Deletion of the entries with PLMN 254/003 and PLMN 254/004] |
| 10c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entries with PLMN 254/003 and PLMN 254/004] |
| 10d | ME USS | From steps 9 -13: The ME does not register to another USS than the currently selected and shall not send new LOCATION STATUS event to the UICC. |  |
| 11 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.1 | [normal ending] Note: For a pre-release 11 ME, the UICC simulator does not need to evaluate the response |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 |  | Wait approx. 180 seconds | [The ME does not register to another USS than the currently selected.] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.2 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: REFRESH 3.1.2 | Note: Step 18 can occur at any time during execution of steps 17a to 17c |
| 17a | UICC | Void |  |
| 17b | ME UICC | Update of EF FPLMN | [Deletion of the entry with PLMN 254/002] |
| 17c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entry with PLMN 254/002] |
| 18 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.2 | [normal ending] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | ME USS | The ME registers to the second USS. | Note: The ME might have registered to the second USS also before steps 18/19. |
| 21 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.1.1 | PLMN MCC/MNC: 254/002, Normal service Note: The ME send the Envelope after registration to the second USS, thus might have sent the Envelope also before steps 18/19. |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: REFRESH 3.1.3 | Note: Step 26 can occur at any time during execution of steps 25a to 25c |
| 25a | UICC | Void |  |
| 25b | UICC | EF FPLMN | [PLMN entries 254/003 and PLMN 254/001 not existent in EF FPLMN] |
| 25c | ME | ME's internal memory | [Not explicitly verified: PLMN entries 254/003 and PLMN 254/001 not existent in FPLMN list] |
| 26 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.2 | [normal ending] |
| 27 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 28 | ME USS | The ME registers to the first USS. | Note: The ME might have registered to the first USS also before steps 26/27. |
| 29 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.1.2 | PLMN MCC/MNC: 254/001 Note: The ME send the Envelope after registration to the first USS, thus might have sent the Envelope also before steps 26/27. |
| 30 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.2.1 |  |
| 31 | ME  UICC | FETCH |  |
| 32 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1 | [Event LOCATION STATUS download removed] |
| 33 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1 | The content of the Terminal Response is not part of the evaluation of the test case |
| 34 | USER  ME | SWITCH OFF ME |  |

PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in clause 27.22.7.4.1.4.2.

TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1

Same as TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 in clause 27.22.7.4.1.4.2.

PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.3.2 in clause 27.22.4.16.1.4.2.

TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1

Same as TERMINAL RESPONSE: SET UP EVENT LIST 1.3.2 in clause 27.22.4.16.1.4.2.

…

Expected Sequence 3.xx (REFRESH, Steering of roaming, NG-RAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | The first NG-SS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - TAI (MCC/MNC/TAC): 254/001/000001. - Access control: unrestricted. The second NG-SS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - TAI (MCC/MNC/TAC): 254/002/000001. - Access control: unrestricted. |  |
| 2 | ME NG-SS | The ME registers to the first NG-SS. |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.1.1 | [Setting up LOCATION STATUS Event] |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1 | IF A.1/171 THEN ME sends a ENVELOPE: EVENT DOWNLOAD - Location Status 3.xx.3 |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.xx.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 3.xx.1 | Note: Step 11 can occur at any time during execution of steps 10a to 10d |
| 10a | UICC | Void |  |
| 10b | ME UICC | Update of EF FPLMN | [Deletion of the entries with PLMN 254/003 and PLMN 254/004] |
| 10c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entries with PLMN 254/003 and PLMN 254/004] |
| 10d | ME NG-SS | From steps 9-13: The ME does not register to another NG-SS than the currently selected and shall not send new LOCATION STATUS event to the UICC. |  |
| 11 | ME UICC | TERMINAL RESPONSE: REFRESH 3.xx.1 | [normal ending] Note: For a pre-release 11 ME, the UICC simulator does not need to evaluate the response |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 |  | Wait approx. 180 seconds | [The ME does not register to another NG-SS than the currently selected.] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.xx.2 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: REFRESH 3.xx.2 | Note: Step 18 can occur at any time during execution of steps 17a to 17c |
| 17a | UICC | Void |  |
| 17b | ME UICC | Update of EF FPLMN | [Deletion of the entry with PLMN 254/002] |
| 17c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entry with PLMN 254/002] |
| 18 | ME UICC | TERMINAL RESPONSE: REFRESH 3.xx.2 | [normal ending] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | ME NG-SS | The ME registers to the second NG-SS | Note: The ME might have registered to the second NG-SS also before steps 18/19. |
| 21 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.xx.2 | PLMN MCC/MNC: 254/002 Note: The ME send the Envelope after registration to the second NG-SS, thus might have sent the Envelope also before steps 18/19. |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.xx.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: REFRESH 3.xx.3 | Note: Step 26 can occur at any time during execution of steps 25a to 25c |
| 25a | UICC | Void |  |
| 25b | UICC | EF FPLMN | [PLMN entries 254/003 and PLMN 254/001 not existent in EF FPLMN] |
| 25c | ME | ME's internal memory | [Not explicitly verified: PLMN entries 254/003 and PLMN 254/001 not existent in FPLMN list] |
| 26 | ME UICC | TERMINAL RESPONSE: REFRESH 3.xx.3 | [normal ending] |
| 27 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 28 | ME NG-SS | The ME registers to the first NG-SS. | Note: The ME might have registered to the first NG-SS also before steps 26/27. |
| 29 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.xx.3 | PLMN MCC/MNC: 254/001 Note: The ME send the Envelope after registration to the first NG-SS, thus might have sent the Envelope also before steps 26/27. |
| 30 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.2.1 |  |
| 31 | ME UICC | FETCH |  |
| 32 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1 | [Event LOCATION STATUS download removed] |
| 33 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1 | The content of the Terminal Response is not part of the evaluation of the test case |
| 34 | USER ME | SWITCH OFF ME |  |

PROACTIVE COMMAND: REFRESH 3xx.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	E-UTRAN, UTRAN

2ndPLMN:	254/004

2ndACT:	GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | C0 | 00 | 52 | 44 | 00 | 00 | 80 |  |

TERMINAL RESPONSE: REFRESH 3.xx.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 3.xx.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/002

1stACT:	NG-RAN/UTRAN/GERAN

2ndPLMN:	254/001

2ndACT:	E-UTRAN/UTRAN/GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 24 | 00 | 88 | 80 | 52 | 14 | 00 | C0 | 80 |  |

TERMINAL RESPONSE: REFRESH 3.xx.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 3.xx.2

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/002)

TAC	000001

NR Cell id:	0000000001 (36bits)

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 0B | 52 | 24 | 00 | 00 | 00 | 01 | 00 | 00 | 00 | 00 |
|  | 1F |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: REFRESH 3.xx.3

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	E-UTRAN/UTRAN/GERAN

2ndPLMN:	254/001

2ndACT:	NG-RAN/UTRAN/GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | C0 | 80 | 52 | 14 | 00 | 88 | 80 |  |

EVENT DOWNLOAD - LOCATION STATUS 3.xx.3

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/001)

TAC	000001

NR Cell id:	0000000001 (36bits)

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 0B | 52 | 14 | 00 | 00 | 00 | 01 | 00 | 00 | 00 | 00 |
|  | 1F |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.7.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.xx.

##### 27.22.4.7.5	REFRESH (IMSI changing procedure, E-UTRAN)

27.22.4.7.5.1	Definition and applicability

See clause 3.2.2.

***** End of change *****