3GPP TSG-CT6 Meeting #117	C6-230724

# Chicago, US, 14nd Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0997 | rev | 4 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add EF of IMS Data Channel configuration to the USIM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As ‘Reply LS on UICC configuration for IMS Data Channel’(C6-230626) says: The operator needs to able to control whether IMS Data Channel is allowed for the UE or not. Secondly, if IMS Data Channel is allowed for the UE, SA2 has specified following two options when an IMS Data Channel is to be established: 1.UE establishes an IMS Data Channel simultaneously while establishing an IMS audio/video session or, 2.UE upgrades an ongoing IMS audio/video session to an IMS Data Channel session It is suggested to add configuration for IMS data channel setup option on UICC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add configuration for IMS data channel setup option according to C6-230626. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement of IMS Data Channel Establishment is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.8, 4.2.xx(new), 4.7, 5.xx(new), Annex A, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C6-230699, C6-230668, C6-230652 |  |  |  |  |  |  |  |  |

****************************start of changes ********************************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.111: "USIM and IC Card Requirements".

[2]	3GPP TS 22.011: "Service accessibility".

...

- 3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[xxx]	3GPP TS 23.228: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

****************************next of changes ********************************

4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n8 |  |  | M |  | 1 byte |
| 2 | Services n9 to n16 |  |  | O |  | 1 byte |
| 3 | Services n17 to n24 |  |  | O |  | 1 byte |
| 4 | Services n25 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n(8X-7) to n(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | Service n°3: | Extension 2 |
|  | ... | ... |
|  | Service n°147 | 5MBS UE pre-configuration |
|  | Service n°148 | UE configured for using "Operator controlled signal threshold per access technology" |
|  | Service n°1xx | IMS Data Channel indication |

****************************next of changes ********************************

4.2.xx	EFIMSDCI (IMS Data Channel indication)

If service n°1xx is "available", this file shall be present.

This EF contains an indication to the ME for IMS Data Channel.

| Identifier: '6Fxx' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 1 byte |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | IMS DC Establishment Indication |  |  | M |  | 1 byte |

-	IMS DC Establishment Indication:

Contents:

-    IMS Data Channel Establishment Indication

Coding:

-	'00' -IMS Data Channel is not allowed

-	'01' -IMS data channel is to be setup after an IMS session is established

-	'02' -IMS data channel is to be setup simultaneously while establishing an IMS session

-	All other values are RFU

****************************next of changes ********************************

4.7	Files of USIM

......

|  | ADFUSIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | EFLI |  |  | EFARR |  |  |  | EFIMSI |  |  | EFKeys |  |  | EFKeysPS |  |
|  |  |  |  |  |  |  | '6F05' |  |  | '6F06' |  |  |  | '6F07' |  |  | '6F08' |  |  | '6F09' |  |
|  |  |  |  |  |  |  | EFDCK |  |  | EFHPPLMN |  |  |  | EFCNL |  |  | EFACMmax |  |  | EFUST |  |
|  |  |  |  |  |  |  | '6F2C' |  |  | '6F31' |  |  |  | '6F32' |  |  | '6F37' |  |  | '6F38' |  |
|  |  |  |  |  |  |  | EFACM |  |  | EFFDN |  |  |  | EFSMS |  |  | EFGID1 |  |  | EFGID2 |  |
|  |  |  |  |  |  |  | '6F39' |  |  | '6F3B' |  |  |  | '6F3C' |  |  | '6F3E' |  |  | '6F3F' |  |
|  |  |  |  |  |  |  | EFMSISDN |  |  | EFPUCT |  |  |  | EFSMSP |  |  | EFSMSS |  |  | EFCBMI |  |
|  |  |  |  |  |  |  | '6F40' |  |  | '6F41' |  |  |  | '6F42' |  |  | '6F43' |  |  | '6F45' |  |
|  |  |  |  |  |  |  | EFSPN |  |  | EFSMSR |  |  |  | EFCBMID |  |  | EFSDN |  |  | EFEXT2 |  |
|  |  |  |  |  |  |  | '6F46' |  |  | '6F47' |  |  |  | '6F48' |  |  | '6F49' |  |  | '6F4B' |  |
|  |  |  |  |  |  |  | EFEXT3 |  |  | EFBDN |  |  |  | EFEXT5 |  |  | EFCCP2 |  |  | EFCBMIR |  |
|  |  |  |  |  |  |  | '6F4C' |  |  | '6F4D' |  |  |  | '6F4E' |  |  | '6F4F' |  |  | '6F50' |  |
|  |  |  |  |  |  |  | EFEXT4 |  |  | EFEST |  |  |  | EFACL |  |  | EFCMI |  |  | EFSTART-HFN |  |
|  |  |  |  |  |  |  | '6F55' |  |  | '6F56' |  |  |  | '6F57' |  |  | '6F58' |  |  | '6F5B' |  |
|  |  |  |  |  |  |  | EFTHRESHOLD |  |  | EFPLMNwAcT |  |  |  | EFOPLMNwAcT |  |  | EFHPLMNwAcT |  |  | EFPSLOCI |  |
|  |  |  |  |  |  |  | '6F5C' |  |  | '6F60' |  |  |  | '6F61' |  |  | '6F62' |  |  | '6F73' |  |
|  |  |  |  |  |  |  | EFACC |  |  | EFFPLMN |  |  |  | EFLOCI |  |  | EFICI |  |  | EFOCI |  |
|  |  |  |  |  |  |  | '6F78' |  |  | '6F7B' |  |  |  | '6F7E' |  |  | '6F80' |  |  | '6F81' |  |
|  |  |  |  |  |  |  | EFICT |  |  | EFOCT |  |  |  | EFAD |  |  | EFVGCS |  |  | EFVGCSS |  |
|  |  |  |  |  |  |  | '6F82' |  |  | '6F83' |  |  |  | '6FAD' |  |  | '6FB1' |  |  | '6FB2' |  |
|  |  |  |  |  |  |  | EFVBS |  |  | EFVBSS |  |  |  | EFeMLPP |  |  | EFAaeM |  |  | EFECC |  |
|  |  |  |  |  |  |  | '6FB3' |  |  | '6FB4' |  |  |  | '6FB5' |  |  | '6FB6' |  |  | '6FB7' |  |
|  |  |  |  |  |  |  | EFHiddenkey |  |  | EFNETPAR |  |  |  | EFPNN |  |  | EFOPL |  |  | EFMBDN |  |
|  |  |  |  |  |  |  | '6FC3' |  |  | '6FC4' |  |  |  | '6FC5' |  |  | '6FC6' |  |  | '6FC7' |  |
|  |  |  |  |  |  |  | EFEXT6 |  |  | EFMBI |  |  |  | EFMWIS |  |  | EFCFIS |  |  | EFEXT7 |  |
|  |  |  |  |  |  |  | '6FC8' |  |  | '6FC9' |  |  |  | '6FCA' |  |  | '6FCB' |  |  | '6FCC' |  |
|  |  |  |  |  |  |  | EFSPDI |  |  | EFMMSN |  |  |  | EFEXT8 |  |  | EFMMSICP |  |  | EFMMSUP |  |
|  |  |  |  |  |  |  | '6FCD' |  |  | '6FCE' |  |  |  | '6FCF' |  |  | '6FD0' |  |  | '6FD1' |  |
|  |  |  |  |  |  |  | EFMMSUCP |  |  | EFNIA |  |  |  | EFVGCSCA |  |  | EFVBSCA |  |  | EFGBAP |  |
|  |  |  |  |  |  |  | '6FD2' |  |  | '6FD3' |  |  |  | '6FD4' |  |  | '6FD5' |  |  | '6FD6' |  |
|  |  |  |  |  |  |  | EFMSK |  |  | EFMUK |  |  |  | EFEHPLMN |  |  | EFGBANL |  |  | EFEHPLMNPI |  |
|  |  |  |  |  |  |  | '6FD7' |  |  | '6FD8' |  |  |  | '6FD9' |  |  | '6FDA' |  |  | '6FDB' |  |
|  |  |  |  |  |  |  | EFLRPLMNSI |  |  | EFNAFKCA |  |  |  | EFSPNI |  |  | EFPNNI |  |  | EFNCP-IP |  |
|  |  |  |  |  |  |  | '6FDC' |  |  | '6FDD' |  |  |  | '6FDE' |  |  | '6FDF' |  |  | '6FE2' |  |
|  |  |  |  |  |  |  | EFEPSLOCI |  |  | EFEPSNSC |  |  |  | EFUFC |  |  | EFUICCIARI |  |  | EFNASCONFIG |  |
|  |  |  |  |  |  |  | '6FE3' |  |  | '6FE4' |  |  |  | '6FE6' |  |  | '6FE7' |  |  | '6FE8' |  |
|  |  |  |  |  |  |  | EFPWS |  |  | EFFDNURI |  |  |  | EFBDNURI |  |  | EFSDNURI |  |  | EFIAL |  |
|  |  |  |  |  |  |  | '6FEC' |  |  | '6FED' |  |  |  | '6FEE' |  |  | '6FEF' |  |  | '6FF0' |  |
|  |  |  |  |  |  |  | EFIPS |  |  | EFIPD |  |  |  | EFePDGId |  |  | EFePDGSelection |  |  | EFePDGIdEm |  |
|  |  |  |  |  |  |  | '6FF1' |  |  | '6FF2' |  |  |  | '6FF3' |  |  | '6FF4' |  |  | '6FF5' |  |
|  |  |  |  |  |  |  | EFePDGSelectionEm |  |  | EFFromPreferred |  |  |  | EFIMSConfigData |  |  | EF3GPPPSDATAOFF |  |  | EF3GPPPSDATAOFFservicelist |  |
|  |  |  |  |  |  |  | '6FF6' |  |  | '6FF7' |  |  |  | '6FF8' |  |  | '6FF9' |  |  | '6FFA' |  |
|  |  |  |  |  |  |  | EFTVCONFIG |  |  | EFXCAPConfigData |  |  |  | EFEARFCNList |  |  | EFMuDMiDConfigData |  |  | EFeAKA |  |
|  |  |  |  |  |  |  | '6FFB' |  |  | '6FFC' |  |  |  | '6FFD' |  |  | '6FFE' |  |  | '6F01' |  |
|  |  |  |  |  |  |  | EFOCST '6F02' |  |  | EFIMSDCI '6Fxx' |  |  |  |  |  |  |  |  |  |  |  |

****************************next of changes ********************************

5.xx		   IMS DC Establishment procedures

5.xx.1 IMS DC Establishment indication procedure

Requirement:	Service n°1xx is "available".

Request:	 The ME performs the reading procedure with EFIMSDCI.

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |  |
| --- | --- | --- | --- |
| '2F00' | Application directory | Caution |  |
| ...... | ...... |  |
| '6Fxx' | IMS DC Establishment Indication | Yes |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, EFEPSLOCI, EF5GS3GPPLOCI and EF5GSN3GPPLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. NOTE5: If EFRouting_Indicator is changed, the UICC shall issue File Change Notification REFRESH, as defined in 3GPP TS 31.111 [12]. |  |  |  |

Editor's Note:	The REFRESH for SUPI_NAI update needs to be further specified.

****************************next of changes ********************************

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification |  | Description |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| '2F00' |  | Application directory |  | Card issuer/operator dependent |  |
| '2F05' |  | Preferred languages |  | 'FF…FF' |  |
| ...... |  | ...... |  |  |  |
| '6Fxx' |  | IMS DC Establishment Indication |  | Operator dependent |  |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxxxxx stands for any valid MCC and MNC, coded according to TS 24.008 [9].

****************************end of changes ********************************