3GPP TSG-CT6 Meeting #118	C6-240085

# Athens, Greece, 27th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0537 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce voice centric support for 31.121 5G UICC tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently 31.121 test cases with 5G Registration can’t be executed with voice centric UEs since when IMS voice is not available over 3GPP access, if the UE's usage setting is "voice centric", UE disables the N1 mode capability for 3GPP access (ref 24.501 clause 4.3.2). This issue is not limited to 31.121 UAC TCs and impacts all the 5G UICC TCs expecting successful 5G Registration. Reasons for the change done under common defaults applicable for 5G UICC TCs: - The change in this CR applicable for all the 5G UICC TCs expecting successful 5G Registration. - Avoided making changes to any individual TCs, instead the change is under the common defaults for 5G TCs. - The IMS Registration procedure introduced in this CR can also be used in future IMS/voice TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduced Default NG-SS and UE configuration for different Usage setting in clause 4y and added a reference to it in clause 3.5 (Generic procedures for E-UTRAN/UTRAN/GERAN/IMS/NB-IoT/5G-NR). - Defined the PIXIT pc_ue_usage_voice_centric for the NG-SS to support IMS/voice capabilities if the UE is voice centric. Else, it is not required. - Added Definition of 5G-NR ISIM-UICC that is required for voice centric testing, and it can also be used for future 5G IMS specific tests. - Added 5G-NR generic procedure for 5G Registration on 3GPP access with IMS service and it can also be used for future 5G IMS specific tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 31.121 test cases with 5G Registration can’t be executed with voice centric UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.5, 3.7, 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

…..

[42]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

…..

[xx]	3GPP TS 33.203: “Technical Specification Group Services and System Aspects; 3G security; Access security for IP-based services”

[yy]	3GPP TS 34.229-5: “Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Part 5: Protocol conformance specification using 5G System (5GS)”

**** Next Change ****

## 3.5	Generic procedures for E-UTRAN/UTRAN/GERAN/IMS/NB-IoT/5G-NR

If a test case contains the statement "This test applies to Terminals accessing E-UTRAN", the procedures defined in TS 36.508 [29] shall be the basis for all performed procedures during the test. The procedures in clause 4.5 describe the default behaviour of a conformant UE regarding the specified protocols to be used for E-UTRAN and the required procedures from the NAS.

If a test case contains the statement "This test applies to Terminals accessing UTRAN", the procedures defined in TS 34.108 [21], clause 7.2 shall be the basis for all performed procedures during the test. The procedures in clause 7 describe the default behaviour of a conformant UE regarding the specified protocols to be used for UTRAN and the required procedures from the NAS.

If a test case contains the statement "This test applies to Terminals accessing GERAN", the procedures defined in TS 51.010-1 [22], clause 10 shall be the basis for all performed procedures during the test. The procedures in clause 10 describe the default behaviour of a conformant UE regarding the specified protocols to be used for GERAN and the required procedures from the NAS.

3GPP TS 34.229-1[33], Annex C describes the generic test procedures for IMS.

If a test case contains the statement "This test applies to Terminals accessing NB", the procedures defined in TS 36.508 [29] shall be the basis for all performed procedures during the test. The procedures in TS 36.508 [29] clause 8.1.5 describe the default behaviour of a conformant UE regarding the specified protocols to be used for NB-IoT and the required procedures from the NAS.

If a test case contains the statement "This test applies to Terminals accessing 5G-NR", the procedures defined in TS 38.508-1 [40] shall be the basis for all performed procedures during the test. The procedures in clause 4.5 describe the default behaviour of a conformant UE regarding the specified protocols to be used for 5G-NR and the required procedures from the NAS. Depending on the Usage Setting of the Terminal NG-SS and UE shall use the default configuration as defined in clause 4x. If the usage setting of the terminal is voice centric pc_ue_usage_voice_centric shall be set to TRUE.

5G-NR generic procedure for 5G Registration on 3GPP access with IMS service is defined in Annex Z.1.

**** Next Change ****

# 3.7	Table of optional features

Support of several features is optional or release dependent for the terminal equipment. However, if an ME states conformance with a specific 3GPP release, it is mandatory for the ME to support all mandatory functions of that release, as stated in table A.1 with the exception of the functions:

Table A.1: Options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Support of CS | O |  | O_CS |
| 2 | Support of a feature requiring PIN2 entry (such as e.g. AoC or FDN) | O |  | O_PIN2_ENTRY_FEAT |
| 3 | Support of UTRAN access | C001 |  | O_UTRAN |
| 4 | Support of GERAN access | C002 |  | O_GERAN |
| 5 | Support of Fixed Dialling Numbers | O |  | O_FDN |
| 6 | Support of Advice of Charge Charging | O |  | O_AoCC |
| 7 | Support of Higher priority PLMN selector with Access Technology service (Implementation is optional in Rel-6 and onwards) | C003 |  | O_HPLMNwACT |
| 8 | Support of local phonebook | O NOTE 1 |  | O_Local_PB |
| 9 | Support of global phonebook | C004 |  | O_Global_PB |
| 10 | Support of storing received Class 2 Short Messages in the USIM | O |  | O_Store_Received_SMS |
| 11 | Support of MMS | O |  | O_MMS |
| 12 | Support of usage of MMS related data stored on the USIM | C005 |  | O_MMS_USIM_DATA |
| 13 | Supported of unselected user MMS connectivity parameters | O |  | O_NO_USER_MMS_CONF_SELEC |
| 14 | Support of MMS notification storage on the USIM | O |  | O_MMS_NOTIF_STORAGE |
| 15 | Support of ACL | O NOTE 2 |  | O_ACL |
| 16 | Support of SDN | O |  | O_SDN |
| 17 | Support of numerical entry of PLMN codes in EF PLMNwACT | O |  | O_EFPLMNwACT_numerical entry |
| 18 | Terminal does support speech call | O |  | O_Speech_Calls |
| 19 | Terminal support PIN MMI strings | O |  | O_PIN_MMI_Strings |
| 20 | Terminal does support eFDD | O |  | pc_eFDD |
| 21 | Terminal does support eTDD | O |  | pc_eTDD |
| 22 | Terminal does support CSG list handling (for E-UTRA) | O |  | pc_Allowed_CSG_list |
| 23 | Terminal supports SM-over-IP-receiver | O |  | pc_SM-over-IP receiver |
| 24 | Terminal supports reading SMS' stored in EF SMS on the USIM if USIM and ISIM are present | O |  | pc_USIM_EF_SMS_reading_support_if_USIM_ISIM both present |
| 25 | Terminal supports reading SMS' stored in EF SMS on the ISIM if USIM and ISIM are present | O |  | pc_ISIM_EF_SMS_reading_support_if_USIM_ISIM both present |
| 26 | Terminal can store more than 1000 text messages | O |  | O_LARGE_SMS_STORAGE |
| 27 | Support for multiple PDN connections | O |  | pc_Multiple_PDN |
| 28 | Terminal does support CSG (for UTRA) | O |  | pc_CSG |
| 29 | Support of manual CSG selection | O |  | pc_manual_CSG_selection |
| 30 | Support of PS | O |  | O_PS |
| 31 | Terminal does support display | O |  | O_Display |
| 32 | Terminal does support keypad | O |  | O_Keypad |
| 33 | Terminal supports E-UTRA Disabling Allowed for EMM cause #15 | O |  | O_EUTRA_Disabling_EMM_cause#15 |
| 34 | Terminal supports Override NAS signalling low priority | O |  | O_Override_NAS_signalling_low_priority |
| 35 | Terminal supports T3245 timer | O |  | O_T3245 |
| 36 | Terminal supports Override Extended access barring | O |  | O_Override_EAB |
| 37 | Terminal does support NB-IoT | O |  | pc_NB |
| 38 | MS maintains a list of PLMN-specific attempt counters | O |  | O_PLMN_specific_attempt_counters |
| 39 | Terminal does support deactivation of the UICC in PSM. | O |  | O_PSM_DEAC_UICC |
| 40 | Terminal does support deactivation of the UICC during extended DRX | O |  | O_eDRX_DEAC_UICC |
| 41 | Terminal does support the UICC suspension mechanism in PSM. | O |  | O_PSM_SUSPEND_UICC |
| 42 | Terminal does support the UICC suspension mechanism during extended DRX | O |  | O_eDRX_SUSPEND_UICC |
| 43 | UE supports 5G Core Network | O |  | pc_5GC |
| 44 | Support of NR access | O |  | pc_NR |
| 45 | Support of URSP by USIM | O |  | O_URSP_by_USIM |
| 46 | Terminal supports SUPI as Network Access Identifier (NSI, GLI or GCI) | O |  | O_SUPI_NAI |
| 47 | Supports RRC_INACTIVE | O |  | pc_inactiveState |
| aa | Support of UE usage voice centric | O |  | pc_ue_usage_voice_centric |
| C001 If terminal is 3G terminal then M else N/A C002 If terminal is 2G terminal then M else O C003 If Higher priority PLMN selector with Access Technology service is implemented according to Rel-6 or later then O else M C004 If (A.1/18 is supported) AND (A.1/31 is supported) AND (A.1/32 is supported) AND (terminal is implemented according to Rel-6 or later) then M, else O C005 If ((A.1/11 is NOT supported) OR (terminal is implemented according to R99)) then N/A else if terminal is implemented according to Rel-4 then O else M C006 void NOTE 1: The support of this feature was made optional by CR#0214. See conditions in TS 31.102 [4] NOTE 2: The support of this feature was made optional by CR#0200. |  |  |  |  |

**** Next Change ****

# 4	Default Values

All tests defined in the subsequent clauses apply to Terminals using card types specified in ETSI TS 102 221 [5], unless otherwise stated.

The following sequence of tests confirms:

a)	the correct interpretation of data read from the USIM (Universal Subscriber Identification Module) by the Terminal;

b)	the correct writing of data to the USIM by the Terminal;

c)	the initiation of appropriate procedures by the Terminal;

d)	High level protocols.

All tests apply to the USIM application on the UICC.

…..

…..

## 4.y	Definition of 5G-NR ISIM-UICC

### 4.y.1	Applications on the 5G-NR ISIM-UICC

The 5G-NR ISIM-UICC shall contain a USIM as defined in clause 4.9 and an ISIM as defined in clause 4.5.3.

### 4.y.2	Default ISIM values on 5G-NR ISIM-UICC

The 5G-NR ISIM-UICC shall contain an ISIM for IMS access with the values as defined in clause 4.5.3.

## 4x	Default NG-SS and UE configuration for different Usage setting

<Editor’s note: ‘x’ in 4x can be ‘a’, ‘b’, or any other letter applicable>.

For ‘voice centric’ usage, NG-SS shall support IMS voice over PS session over 3GPP access and the IMS SIP Registration procedure, and clauses 4x.1 and 4x.2 are applicable. IMS specific configuration as defined in clause 4x.3 shall be used in the UE.

For ‘data centric’ usage, NG-SS may not support IMS voice over PS session over 3GPP access as in clause 4x.1.2 and only the clause 4x.1.1 may be applicable. UE may attempt to initiate a PDU Session establishment for IMS and it is up to NG-SS implementation to accept it and perform IMS registration procedure as in clause 4x.2 or reject the PDU Session establishment. IMS specific configuration as defined in clause 4x.3 may be used in the UE.

Depending on the optional feature pc_ue_usage_voice_centric as defined in clause 3.7 test system shall switch the test case behaviour to ‘voice centric’ or ‘data centric’.

### 4x.1	Default NG-SS configuration for 3GPP access with IMS service

### 4x.1.1	Default NG-SS configuration for IMS Registration support

NG-SS is configured with the IMSI within the USIM application, the P-CSCF address, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the ISIM UICC.

- NG-SS is listening to SIP default port 5060 for both UDP and TCP protocols.

- NG-SS is able to perform IMS AKA authentication for the IMPI, according to 3GPP TS 33.203 [xx] clause 6.1.

- SIB 1 in NG-SS Cell shall not include uac-BarringInfo with UAC_BarringInfo for AccessCategory 9, IMS Signalling.

### 4x.1.2	Default NG-SS configuration for PS voice support

Following parameters apply to all the 5G test cases in voice centric usage.

- 5GS network feature support element includes ‘IMS voice over PS session over 3GPP access indicator’ (IMS-VoPS-3GPP) in the REGISTRATION ACCEPT.

### 4x.2	Default NG-SS procedure for 5G Registration on 3GPP access with IMS service

The NG-SS and UE shall perform 5G Registration procedure and IMS SIP Registration procedure as per the generic procedure defined in this clause.

5G-NR generic procedure for 5G Registration on 3GPP access with IMS service defined in Annex Z.1 shall be the default procedure for 5G Registration on 3GPP access with IMS service.

Steps under the Procedure in each 5G test case may describe only the steps necessary for the Test Purpose. However, in voice centric usage, NG-SS and UE shall support additional steps defined in Annex Z.1.

### 4x.3	Default UE configuration for IMS service

UE shall be configured with the DNN type as ‘ims’ and DNN as IMS.

IMS private user identity, Home Network Domain Name, IMS public user identity, and P-CSCF Address required for IMS SIP Registration shall be configured in the 5G-NR ISIM-UICC as defined in clause 4.y.

Annex Z

### Z.1 	5G-NR generic procedure for 5G Registration on 3GPP access with IMS service

| Step | Direction | Action | Information |
| --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up with SIB1 as defined in the test case initial conditions and the additional requirements as defined in clause 4.x. | pc_ue_usage_voice_centric is TRUE |
| 2 | UE | The UE is powered on |  |
| 3 | UE <> TT | UE camps on NG-SS Cell and establishes RRC connection |  |
| 4 | UE > TT | UE sends REGISTRATION REQUEST |  |
| 5 | TT > UE | NG-SS sends AUTHENTICATION REQUEST message. |  |
| 6 | UE | UE passes the RAND and AUTN values to the USIM |  |
| 7 | UE > TT | UE sends AUTHENTICATION RESPONSE message with RES value calculated according to TS 24.501 [42] | The UE shall return RES equal to the XRES calculated at the NG-SS |
| 8 | TT > UE | NG-SS sends SECURITY MODE COMMAND message |  |
| 9 | UE > TT | UE send SECURITY MODE COMPLETE |  |
| 10 | TT > UE | NG-SS sends REGISTRATION ACCEPT message |  |
| 11 | UE > TT | UE sends REGISTRATION COMPLETE |  |
| 12 | UE > TT | The UE sends an ULInformationTransfer message with a PDU SESSION ESTABLISHMENT REQUEST for the DNN type ‘ims’. | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST (DNN=IMS) |
| 13 | TT > UE | The NG-SS sends an RRCReconfiguration message with an PDU SESSION ESTABLISHMENT ACCEPT | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT |
| 14 | UE > TT | The UE sends an RRCReconfigurationComplete message |  |
| 15 | UE <> TT | IMS Registration procedure according TS 34.229-5 [yy] subclause A.2 (steps 1-8). |  |

**** End of Changes ****