3GPP TSG-CT WG6 Meeting #118	C6-240103

# Athens, Greece; 27th Feb – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 1023 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Uu MBS configuration and AS MBS configuration policies for V2X services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MBS4V2X |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 have added support of Uu MBS configuration and AS MBS configuration policies for V2X services. Those new configuration for policies over Uu interface are in TS 24.588. Hence, TS 31.102 needs to consider all this information so there is also support in the USIM for the Rel-18 version of 3GPP specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing V2X data policy over Uu data file (i.e., EFV2XP_Uu) is updated in order to include MBS configuration and AS MBS configuration policies for V2X services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misliagnment with the policies over Uu interface defined by TS 24.588. MBS configuration and AS MBS configuration policies for V2X services over Uu interface cannot be provided by the USIM. This results in that the use of MBS policies over Uu are not possible by use of the USIM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.588 CR0038 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | V1: the proposal has been reworded so that it matches the previous sentence in the specification to keep consistency. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C1-240090 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.6.5.5	EFV2XP_Uu (V2X data policy over Uu)

If service n°3 is "available" in EFVST, this file shall be present. This EF contains V2X in 5GS UE policies over Uu. The format of the V2X in 5GS UE policies over Uu are specified in 3GPP TS 24.588 [113].

| Identifier: '4F04' |  | Structure: Transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: Optional |  |  |  |  |  |  |
| File size: X bytes bytes, (X ≥ 3) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | V2X data policy over Uu TLV objects |  |  | M |  | X bytes |

The V2X in 5GS UE policies over Uu data object parameters tags:

| Description | Tag Value |
| --- | --- |
| V2X data policy over Uu Tag | 'A0' |
| V2X service identifier to PDU session parameters mapping rules Tag | '80' |
| PLMN infos Tag | '81' |

The V2X in 5GS UE policies over Uu contents:

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| V2X data policy over Uu Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer | -- | M | X1 |
| Indicator bits | -- | M | 1 |
| V2X service identifier to PDU session parameters mapping rules Tag | '80' | M | 1 |
| Length | X2 | M | Note 2 |
| V2X service identifier to PDU session parameters mapping rules information | -- | M | X2 |
| PLMN infos Tag | '81' | O | 1 |
| Length | X3 | O | Note 2 |
| PLMN infos information | -- | O | X3 |

-	Validity timer

Contents:

-	The validity timer contains the timer for controlling the validity of 5GS UE policies for V2X communication over PC5.

Contents:

-	The validity timer is encoded as shown in figure 5.4.1.1 and table 5.4.1.1 of 3GPP TS 24.588 [113].

-	Indicator bits

Contents:

-	The indicator bits contains bit indications about 5GS UE policies for V2X communication over PC5.

Contents:

-	The indicator bits are encoded as shown in figure 5.4.1.1 and table 5.4.1.1 of 3GPP TS 24.588 [113].

-	V2X service identifier to PDU session parameters mapping rules Tag '80'

Contents:

-	The V2X service identifier to PDU session parameters mapping rules contains configuration parameters for V2X service identifier to PDU session parameters mapping rules.

Contents:

-	The V2X service identifier to PDU session parameters mapping rules is encoded as shown in figures 5.4.1.17 to 5.4.1.18 and tables 5.4.1.17 to 5.4.1.18 of 3GPP TS 24.588 [113].

-	PLMN infos Tag '81'

Contents:

-	The PLMN infos a list of PLMNs in which the UE is configured to use V2X communication over Uu.

Contents:

-	The not served by E-UTRA and not served by NR is encoded as shown in figures 5.4.1.2 to 5.4.1.16 and tables 5.4.1.2 to 5.4.1.16 of 3GPP TS 24.588 [113]. The V2X MBS configuration and the V2X AS MBS configuration are encoded as shown in figures 5.4.1.8a to 5.4.1.8e and tables 5.4.1.8a to 5.4.1.8e of 3GPP TS 24.588 [113].

V2X in 5GS UE policies over Uu data policy object tag:

| V2X in 5GS UE policies over Uu data policy | Tag Values | Condition |
| --- | --- | --- |
| V2X data policy over Uu | 'A0' | Shall be present if service n°3 is "available" in the V2X service table |

* * * End of Changes * * * *