3GPP TSG-CT WG6 Meeting #118	C6-240127

# Athens, Greece; 27th Feb – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.115 | CR | 0036 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 31.115 alignment to ETSI Release 17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Old release of ETSI TS 102 224 (8.0.0) is referenced, and clause 'Proof of receipt mechanisms' updated. Old release of ETSI TS 102 225 (12.1.0) is referenced. Now these references have to take into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference to ETSI TS 102 224 8.0.0 replaced with ETSI TS 102 224 15.0.0. Clause 6.4 updated to refer ETSI TS 102 224 15.0. Reference to ETSI TS 102 225 12.1.0 replaced with ETSI TS 102 225 17.0.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Very old ETSI specifications version referenced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI TS 102 224 V15.0.0: "Smart Cards; Security mechanisms for UICC based Applications – Functional requirements".

[3]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[4]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[5]	ETSI TS 101 220 "Smart Cards; ETSI numbering system for telecommunication application providers".

[6]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[7]	3GPP TS 24.012: "Short Message Service Cell Broadcast (SMSCB) support on the mobile radio interface".

[8]	3GPP TS 23.038: "Alphabets and language-specific information".

[9]	ETSI TS 102 225 V17.0.0: "Smart Cards; Secured packet structure for UICC based applications".

[10]	3GPP TS 24.090: "Unstructured Supplementary Service Data (USSD) – Stage 3".

* * * Next Change * * * *

## 6.4	Structure of the Response Packet contained in concatenated USSD Messages

If the Response Packet, which is structured as described in clause 6.3, is longer than 159 bytes (including the Response Header) then it shall be handled as follows.

-	The entire Response Packet including the Response Header shall be separated into its component concatenated parts.

-	The Response Packet is handled as a Concatenated USSD Message as described in annex X of the present document.

-	The Response Packet Header will only be present in the first segment of a concatenated message.

If the data is ciphered, then it is ciphered as described above, before being broken down into individual concatenated elements.

RPI, RPL and RHL shall be included in the calculation of the RC/CC/DS.

An example illustrating a Response Packet split over a sequence of three messages is shown below.

Figure 5: Example of Response split using concatenated USSD messages

If it is indicated in the SPI2 of a Command Packet to send back a PoR  and if the Response Packet is too large to be contained in a single USSD String, then:

-	One single Response Packet shall be sent back to the SE using the Return Result Component contained in the subsequent Facility message. This Response Packet:

-	Shall not contain any additional response data.

-	Shall contain the Response Status Code set to '0C' ('Actual response data to be sent using a ProcessUnstructuredSS-Request invoke component (i.e. using SEND USSD proactive command) ').

-	The security applied to this Response Packet shall be the one indicated in the SPI2 of the Command Packet and following ETSI TS 102.224 [2] clause 6.1.5.

-	This shall be followed by a complete Response Packet, contained in a concatenated USSD Message as defined above.

* * * End of Changes * * * *