3GPP TSG-CT WG6 Meeting #119	C6-240241

# Hyderabad, India; 28th – 31st May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 1035 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of configuration parameter for 5GMM cause #15 extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 has discussed an issue called “Restricting satellite access for a PLMN” almost for a year. There are scenarios where the restriction on the satellite service is not localized in a PLMN, but rather the whole PLMN does not allow the UE to access the network via a satellite cell while it is allowed to access via non-satellite or terrestrial cells, based on the subscription, service level agreement or the operator’s policy. Under the circumstances, if the UE has sent any NAS request message in order to acquire a normal service, the network should be able to reject the request with a proper cause code. Current 5GMM cause values (e.g. #7, #11 or #78) cannot handle those scenario properly, since those cause values will block the UE from the whole services, not only the services via satellite cells. Also other 5GMM cause values such as #12 or #15 can be used as it is for specific scenario where the deployment of satellite access only reside in a limited number of tracking areas, which does not fully resolve the issue. After a long discussion on the issue, CT1 is now discussing on extending the existing cause code #15 with additional IE called “Extended 5GMM cause”, which is similar to “the Extended EMM cause IE” defined in TS 24.301. Also the new IE will be used with the 5GMM cause code #15 in order to disable 3GPP satellite NG-RAN access technology. Similar to what has done for EMM cause #15, the network and the UE need to be sure that the “disabling satellite access” functionality should be enabled or disabled by the network operator beforehand. For EMM cause #15, TS 31.102 has defined a NAS configuration parameter called “E-UTRA Disabling Allowed for EMM cause #15” as part of EFNASCONFIG. For the 5GMM cause #15 for disabling satellite access, similar parameter is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New NAS configuration parameter called “Satellite Disabling Allowed for 5GMM cause #15” is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and the network cannot disable/enable the functionality via USIM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.94, Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.501 CR 6179 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Tdoc number for the related CR in C1#149 is C1-243251 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.2.94	EFNASCONFIG (Non Access Stratum Configuration)

If service n°96 is "available", this file shall be present. This EF contains some of the NAS configuration parameters defined in TS 24.368 [65]. For each of these NAS configuration parameters, a parameter provided in EFNASCONFIG shall take precedence over the corresponding parameter stored in the ME's non-volatile memory.

| Identifier: '6FE8' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: Z bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM ACTIVATE ADM DEACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to Z | NAS configuration parameter TLV objects |  |  | M |  | Z bytes |

NAS configuration parameter information

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| NAS signalling priority Tag | '80' | O | 1 |
| Length | L1 | C | Note 1 |
| NAS signalling priority value | -- | C | L1 |
| NMO I Behaviour Tag | '81' | O | 1 |
| Length | L2 | C | Note 1 |
| NMO I Behaviour value | -- | C | L2 |
| Attach with IMSI Tag | '82' | O | 1 |
| Length | L3 | C | Note 1 |
| Attach with IMSI value | -- | C | L3 |
| Minimum Periodic Search Timer Tag | '83' | O | 1 |
| Length | L4 | C | Note 1 |
| Minimum Periodic Search Timer value | -- | C | L4 |
| Extended access barring Tag | '84' | O | 1 |
| Length | L5 | C | Note 1 |
| Extended access barring value | -- | C | L5 |
| Timer T3245 Behaviour Tag | '85' | O | 1 |
| Length | L6 | C | Note 1 |
| Timer T3245 Behaviour value | -- | C | L6 |
| Override NAS signalling low priority Tag | '86' | O | 1 |
| Length | L7 | C | Note 1 |
| Override NAS signalling low priority value | -- | C | L7 |
| Override Extended access barring Tag | '87' | O | 1 |
| Length | L8 | C | Note 1 |
| Override Extended access barring value | -- | C | L8 |
| Fast First Higher Priority PLMN Search Tag | '88' | O | 1 |
| Length | L9 | C | Note 1 |
| Fast First Higher Priority PLMN value | -- | C | L9 |
| E-UTRA Disabling Allowed for EMM cause #15 Tag | '89' | O | 1 |
| Length | L10 | C | Note 1 |
| E-UTRA Disabling Allowed for EMM cause #15 Value | - | C | L10 |
| SM_RetryWaitTime Tag | '8A' | O | 1 |
| Length | L11 | C | Note 1 |
| SM_RetryWaitTime value | -- | C | L11 |
| SM_RetryAtRATChange Tag | '8B' | O | 1 |
| Length | L12 | C | Note 1 |
| SM_RetryAtRATChange value | -- | C | L12 |
| Default_DCN_ID Tag | '8C' | O | 1 |
| Length | L13 | C | Note 1 |
| Default_DCN_ID value | -- | C | L13 |
| Exception Data Reporting Allowed Tag | '8D' | O | 1 |
| Length | L14 | C | Note 1 |
| Exception Data Reporting Allowed value | -- | C | L14 |
| RLOSPreferredPLMNList Tag | '8E' | O | 1 |
| Length | L15 | C | 4n (Note 1) |
| RLOSPreferredPLMNList value | -- | C | L15 |
| RLOSAllowedMCCList Tag | '8F' | O | 1 |
| Length | L16 | C | 2n (Note 1) |
| RLOSAllowedMCCList value | -- | C | L16 |
| No E-UTRA Disabling In 5GS Tag | '90' | O | 1 |
| Length | L17 | C | Note 1 |
| No E-UTRA Disabling In 5GS value | -- | C | L17 |
| Additional NAS configuration parameters Tag | '91' | O | 1 |
| Length | L18 | C | Note 1 |
| Additional NAS configuration parameters value | -- | C | L18 |
| Satellite Disabling Allowed for 5GMM cause #15 Tag | '92' | O | 1 |
| Length | L19 | C | Note 1 |
| Satellite Disabling Allowed for 5GMM cause #15 Value | -- | C | L19 |
| Note 1 : The length is coded according to ISO/IEC 8825-1 [35] Note 2: C; if the Tag is present, this is mandatory. |  |  |  |

-  NAS signalling priority

Contents:

As described in TS 24.368 [65], used to determine the NAS signalling priority included in NAS messages.

Coding:

As defined for the NAS_SignallingPriority leaf in TS 24.368 [65].

-  NMO I Behaviour

Content:

As described in TS 24.368 [65], indicates whether the "NMO I, Network Mode of Operation I" indication is applied by the UE.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the NMO_I_Behaviour leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

- Attach with IMSI

Content:

As described in TS 24.368 [65], indicates whether attach with IMSI is performed when moving to a non-equivalent PLMN.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the AttachWithIMSI leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

- Minimum Periodic Search Timer

Content:

As described in TS 24.368 [65], gives a minimum value in minutes for the timer T controlling the periodic search for higher prioritized PLMNs. The content applies a minimum value that may override the value in file EFHPPLMN (Higher Priority PLMN search period), as specified in TS 23.122 [31] clause 4.4.3.3.1.

Coding:

As defined for the MinimumPeriodicSearchTimer leaf in TS 24.368 [65].

- Extended access barring

Content:

As described in TS 24.368 [65], indicates whether the UE applies extended access barring.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for ExtendedAccessBarring leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

- Timer T3245 Behaviour

Content:

As described in TS 24.368 [65], indicates whether the timer T3245 and the related functionality is used by the UE.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for Timer_T3245_Behaviour leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-  Override NAS signalling low priority

Contents:

As described in TS 24.368 [65], used to determine whether the NAS signalling priority included in NAS messages can be overriden.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the Override_NAS_SignallingLowPriority leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

The Override NAS signalling low priority tag and the Override Extended access barring tag shall be set to the same value, e.g., if the UE is configured to override the NAS signalling low access priority indicator, then it has also to be configured to override Extended access barring (see 3GPP TS 23.401 [69] clause 4.3.17.4).

-  Override Extended access barring

Contents:

As described in TS 24.368 [65], used to determine whether the Extended access barring included in NAS messages can be overriden.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the Override_ ExtendedAccessBarring leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

The Override Extended access barring tag and the Override NAS signalling low priority tag shall be set to the same value, e.g., if the UE is configured to override Extended access barring, then it has also to be configured to override the NAS signalling low access priority indicator (see 3GPP TS 23.401 [69] clause 4.3.17.4).

-	Fast First Higher Priority PLMN Search

Contents:

Determine whether the UE can perform Fast First Higher Priority PLMN Search upon selecting a VPLMN as specified in 3GPP TS 23.122 [31].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the FastFirstHigherPriorityPLMNSearch leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	E-UTRA Disabling Allowed for EMM cause #15

Contents:

Determine whether the UE is allowed to disable the E-UTRA capability when it receives the Extended EMM IE with value cause "E-UTRAN not allowed" as specified in 3GPP TS 24.301 [51].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the EUTRADisablingAllowedforEMMcause15 leaf in 3GPP TS 24.368 [65] |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	SM_RetryWaitTime

Contents:

As described in TS 24.368 [65], provides a configured UE retry wait time value applicable when in HPLMN or EHPLMN (see 3GPP TS 23.122 [31]) for controlling the UE session management retry behaviour when prior session management request was rejected by the network with cause value #8, #27, #32, #33 as specified in 3GPP TS 24.008 [9] and 3GPP TS 24.301 [51], or when prior session management request was rejected by the network with cause value #8, #27, #32, #33, #70 as specified in 3GPP TS 24.501 [104].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | bits b1 to b8 to be interpreted as defined for the SM_RetryWaitTime leaf in 3GPP TS 24.368 [65] |

-	SM_RetryAtRATChange

Contents:

As described in TS 24.368 [65], indicates the UE's retry behaviour when in HPLMN or EHPLMN (see 3GPP TS 23.122 [31]) after inter-system change between S1 mode and A/Gb or Iu mode or N1 mode as specified in 3GPP TS 24.008 [9] and 3GPP TS 24.301 [51] and 3GPP TS 24.501 [104].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the SM_RetryAtRATChange leaf in 3GPP TS 24.368 [65] |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	Default_DCN_ID

Contents:

As described in 3GPP TS 24.368 [65], indicates the default DCN-ID which is provided by NAS to the lower layers at establishment of the NAS signalling connection as specified in 3GPP TS 24.301 [51].

Coding:

As the DCN-ID value inside the DCN-ID IEI defined in TS 24.008 [9] clause 10.5.5.35.

-	Exception Data Reporting Allowed

Contents:

As described in 3GPP TS 24.368 [65], for the UE in NB-S1 mode indicates whether the UE is allowed to use the RRC establishment cause mo-ExceptionData, as specified in 3GPP TS 24.301 [51].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for the ExceptionDataReportingAllowed leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	RLOSPreferredPLMNList

Contents:

As described is 3GPP TS 24.368 [65], it contains in descending priority order, a list of preferred RLOS PLMNs configured to the UE for selection of a PLMN offering access to RLOS as specified in 3GPP TS 23.122 [31].

Coding:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 to 3 | 1st PLMN (highest priority) | M | 3 bytes |
| 4 | 1st PLMN Config | M | 1 byte |
| 5 to 7 | 2nd PLMN | O | 3 bytes |
| 8 | 2nd PLMN Config | O | 1 byte |
| 9 to 11 | 3rd PLMN | O | 3 bytes |
| 12 | 3rd PLMN Config | O | 1 byte |
| : | : | : | : |
| (4n-3) to (4n-1) | nth PLMN | O | 3 bytes |
| 4n | nth PLMN Config | O | 1 byte |

Coding of PLMN Config:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1=0: This PLMN has higher priority than the next PLMN in the list b1=1: This PLMN has same priority than the next PLMN in the list |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	RLOSAllowedMCCList

Contents:

As described is 3GPP TS 24.368 [65], it contains a list of allowed MCC configured to the UE to allow the UE to check whether the MCC of the PLMN ID of the serving network that advertises RLOS service is present in the list and whether the MCC part of the IMSI configured in the USIM is present in the list of MCCs before initiating the RLOS connection as specified in 3GPP TS 23.122 [31].

Coding:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 to 2 | 1st MCC | M | 2 bytes |
| 3 to 4 | 2nd MCC | O | 2 bytes |
| 5 to 6 | 3rd MCC | O | 2 bytes |
| : | : | : | : |
| (2n-1) to (2n) | nth MCC | O | 2 bytes |

Coding of MCC: according to TS 24.008 [9].

-	A BCD value of 'D' in any of the MCC digits shall be used to indicate a "wild" value for that corresponding MCC digit. A value of 'DDD' represents "any MCC" value.

-	No E-UTRA Disabling In 5GS

Contents:

As described in TS 24.368 [65], used to indicate whether E-UTRA Disabling In 5GS is disabled or enabled.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for NoEUTRADisablingIn5GS leaf in TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	Additional NAS configuration parameters

Contents:

The additional NAS configuration parameters contains some of the NAS configuration parameters defined in TS 24.368 [65].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1 value to be interpreted as defined for DefaultNSSAIInclusionMode leaf in clause 5.10zz4 of 3GPP TS 24.368 [65]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

-	Satellite Disabling Allowed for 5GMM cause #15

Contents:

Determine whether the UE is allowed to disable the satellite access when it receives the Extended 5GMM IE with value cause "Satellite access not allowed" as specified in 3GPP TS 24.501 [104].

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | b1=0: "Satellite Access Disabling for 5GMM cause #15" is disabled, see 3GPP TS 24.501 [104]. b1=1: "Satellite Access Disabling for 5GMM cause #15" is enabled, see 3GPP TS 24.501 [104]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [11]) |

If any of these NAS configuration parameters is neither included in EFNASCONFIG nor stored in the ME's non-volatile memory, the default value as defined for the corresponding leaf in TS 24.368 [65] shall apply.

Unused bytes shall be set to 'FF'.

* * * Next Change * * * *

Annex D (informative):
Tags defined in 31.102

| Tag | Name of Data Element | Usage |
| --- | --- | --- |
| '43' | Full name for network IEI | PLMN Network Name (EFPNN) |
| '45' | Short name for network IEI | PLMN Network Name (EFPNN) |
| '53' | MBMS Data Object | AUTHENTICATE command parameter, in MBMS security context |
| '53' | MBMS operation response Data Object The following tags are encapsulated within '53' 'DB' successful MBMS operation tag | Response to AUTHENTICATE command, in MBMS security context |
| '73' | Key Derivation Data Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag '81' Counter limit tag '82' Request MAC tag '83' NAF_ID tag '84' Terminal_ID tag '85' Terminal_appli_ID_tag '86' UICC_appli_ID tag '87' RANDx tag 'A0' Key Identifier tag | AUTHENTICATE command parameter, in Local Key Establishment security context |
| '73' | Key Derivation Operation Response Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag '82' Request MAC tag | Response to AUTHENTICATE command, in Local Key Establishment security context |
| '73' | Key Availability Check Data Object The following tags are encapsulated within '73' '80' Local Key Establishment Control tag 'A0' Key Identifier tag | AUTHENTICATE command parameter in Local Key Establishment security context |
| '80' | NAF_ID tag | GBA NAF List (EFGBANL) |
| '80' | NAF Key Centre address tag | NAF Key Centre Address (EFNAFKCA) |
| '80' | Icon Tag (Icon link is URI) | Service Provider Name Icon (EFSPNI) |
| '80' | Reauthentication Identity tag | WLAN Reauthentication Identity (EFWRI) |
| '80' | NAS signalling priority Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '80' | MMS Implementation tag | MMS User Preference (EFMMSUP) Multimedia Messages List (EFMML) |
| '80' | IARI TLV TAG | UICC IARI (EFUICCIARI) |
| '80' | Graphics CSG Type tag (Icon link is URI) | CSG Type (EFCSGT) |
| '80' | HNB Name tag | Home NodeB Name (EFHNBN) |
| '80' | PLMN Additional information tag | PLMN Network Name (EFPNN) |
| '80' | ICE Free Format Label tag | In Case of Emergency – Free Format (EFICE-FF) |
| '80' | HPLMN ProSe Function tag | Address of the HPLMN ProSe Function (EFPROSEFUNC) |
| '80' | ProSe Group Counter | Counter for ProSe group (EFPROSE_GC) |
| '80' | ProSe ServerAddress tag | Server address for usage information reports (EFPROSE_UIRC) |
| '80' | Home ePDG Identifier TLV | Home ePDG Identifier (EFePDGId) |
| '80' | ePDG Selection Information TLV | ePDG Selection Information (EFePDGSelection) |
| '80' | Emergency ePDG Identifier TLV | Emergency ePDG Identifier (EFePDGIdEm) |
| '80' | ePDG Selection Information for Emergency Services TLV | ePDG Selection Information for Emergency Services (EFePDGSelectionEm) |
| '80' | XCAP_conn_params_policy TLV TAG | EFXCAPConfigData |
| '80' | Serving network name TLV tag | EFTN3GPPSNN |
| '80' | list of PLMN(s) to be used in disaster condition tag | Disaster roaming information (EFDRI) |
| '80' | IMS configuration data encoding | EFIMSConfigData |
| '81' | IMS configuration data | EFIMSConfigData |
| '80' | KAUSF tag | EF5GAUTHKEYS |
| '80' | Protection Scheme Identifier List data object tag | Protection Scheme Identifier List (EFSUCI_Calc_Info) |
| '80' | Network Specific Identifier TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |
| '80' | SOR-CMCI data object tag | EFSOR-CMCI |
| '80' | PLMN TLV tag | 5MBS UE pre-configuration for (EF5MBSUECONFIG) |
| '80' | USD tag | 5MBS User Service Description (EF5MBSUSD) |
| '80' | Operator controlled signal threshold per access technology tag | EFOCST |
| '80' | Applet NAF Access Control TLV object tag | EFAC_GBAUAPI |
| '80' | UE policy part contents Tag | A2X configuration data (EFA2X_CONFIG) |
| '80' | Served by NG-RAN Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| '80' | A2X service identifiers for unicast communication mode of DAA deconfliction Tag | A2X Direct Detect And Avoid policy data over PC5 (EFA2X_DDAAP_PC5) |
| '80' | Served by NG-RAN for DC2 Tag | A2X Direct C2 communication policy data over PC5 (EFA2X_DC2P_PC5) |
| '80' | A2X service identifier to PDU session parameters mapping rules Tag | A2X policy data over Uu (EFA2XP_Uu) |
| '81' | Global Line Identifier Tag TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |
| '82' | Global Cable Identifier TLV data object tag | SUPI as Network Access Identifier (EFSUPI_NAI) |
| '80' | MuD_and_MiD_configuration_data encoding | EFMuDMiDConfigData |
| '81' | MuD_and_MiD_configuration_data | EFMuDMiDConfigData |
| '81' | B-TID tag | GBA NAF List (EFGBANL) |
| '81' | Icon Tag (Icon link is record number) | Service Provider Name Icon (EFSPNI) |
| '81' | Master key tag | WLAN Reauthentication Identity (EFWRI) |
| '81' | Time Stamp counter tag | MBMS User Key (EFMUK) |
| '81' | MMS User preference profile name tag | MMS User Preference (EFMMSUP) |
| '81' | Login Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '81' | NMO I Behaviour Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '81' | Graphics CSG Type tag (Icon link is record number) | CSG Type (EFCSGT) |
| '81' | ICE Free Format Content tag | In Case of Emergency – Free Format (EFICE-FF) |
| '81' | MM File Identifier / SFI tag | Multimedia Messages List (EFMML) |
| '81' | ProSe CollectionPeriod tag | Collection Period Parameter (EFPROSE_UIRC) |
| '81' | TMGI List tag | 5MBS UE pre-configuration for (EF5MBSUECONFIG) |
| '81' | Not served by NG-RAN Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| '81' | A2X service identifiers for broadcast communication mode of DAA deconfliction Tag | A2X Direct Detect And Avoid policy data over PC5 (EFA2X_DDAAP_PC5) |
| '81' | Not served by NG-RAN for DC2 Tag | A2X Direct C2 communication policy data over PC5 (EFA2X_DC2P_PC5) |
| '81' | PLMN infos Tag | A2X policy data over Uu (EFA2XP_Uu) |
| '82' | Counter tag | WLAN Reauthentication Identity (EFWRI) |
| '82' | MMS User Preference information tag | MMS User Preference (EFMMSUP) |
| '82' | Password Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '82' | Attach with IMSI Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '82' | MM Content Data Object Tag | Multimedia Messages List (EFMML) |
| '82' | ProSe ReportingWindow tag | Reporting Window Parameter (EFPROSE_UIRC) |
| '82' | KSEAF for non-3GPP access tag | EF5GAUTHKEYS |
| '82' | NR-ARFCN List TLV tag | 5MBS UE pre-configuration for (EF5MBSUECONFIG) |
| '82' | A2X service identifier to PC5 RAT(s) and Tx profiles mapping rules Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| '80' | Home Network Public Key Identifier tag | Home Network Public Key Identifier (EFSUCI_Calc_Info) |
| '81' | KSEAF for 3GPP access tag | EF5GAUTHKEYS |
| '83' | Data Destination Address Range Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '83' | Minimum Periodic Search Timer Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '83' | MM Size tag | Multimedia Messages List (EFMML) |
| '83' | SOR counter tag | EF5GAUTHKEYS |
| '83' | Privacy config Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| '84' | UE parameter update counter tag | EF5GAUTHKEYS |
| '83' | ProSe ReportGroupParameters tag | Reporting Parameter for Goups (EFPROSE_UIRC) |
| '81' | Home Network Public Key tag | Home Network Public Key (EFSUCI_Calc_Info) |
| '84' | Bearer Description Tag | Network Connectivity Parameters for USIM IP connections (EFNCP-IP) |
| '84' | Extended access barring Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '84' | MM Status tag | Multimedia Messages List (EFMML) |
| '84' | A2X communication in E-UTRA-PC5 Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| '84' | ProSe ReportTimeStampsFirstTransmissionAndReception tag | Reporting Parameter (EFPROSE_UIRC) |
| '85' | Timer T3245 Behaviour Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '85' | MM Alpha Identifier tag | Multimedia Messages List (EFMML) |
| '85' | ProSe ReportDataTransmitted tag | Reporting Parameter for transmitted Data (EFPROSE_UIRC) |
| '85' | A2X communication in NR-PC5 Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| '86' | Override NAS signalling low priority Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '86' | ProSe ReportDataReceived tag | Reporting Parameter for received Data (EFPROSE_UIRC) |
| '86' | PLMN-ID tag | PLMN-ID (EF5GS 3GPP Access NAS Security Context) |
| '87' | Override Extended access barring Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '87' | ProSe ReportTimeStampsOutOfCoverage tag | Reporting Parameter (EFPROSE_UIRC) |
| '88' | Fast First Higher Priority PLMN Search Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '88' | ProSe ReportLocationInCoverage tag | Reporting Parameter (EFPROSE_UIRC) |
| '89' | Text CSG Type tag | CSG Type (EFCSGT) |
| '89' | E-UTRA Disabling Allowed for EMM cause #15 Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '89' | ProSe ReportRadioParameters tag | Reporting Parameter for Radio Parameters (EFPROSE_UIRC) |
| '8A' | SM RetryWaitTime Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '8B' | SM RetryAtRATChange Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '8C' | Default_DCN_ID Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '8D' | Exception Data Reporting Allowed Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '8E' | RLOSPreferredPLMNList Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '8F' | RLOSAllowedMCCList Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '90' | No E-UTRA Disabling In 5GS Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '91' | Additional NAS configuration parameters Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| '92' | Satellite Acces Disabling Allowed for 5GMM cause #15 Tag | Non Access Stratum Configuration (EFNASCONFIG) |
| 'A0' | MUK ID tag The following tags are encapsulated within 'A0' '80' MUk IDr tag '82' MUk IDi tag | MBMS User Key (EFMUK) |
| 'A0' | EPS NAS security Context tag The following tags are encapsulated within 'A0' '80' Key set identifier KSIASME Tag '81' ASME key (KASME) Tag '82' Uplink NAS count Tag '83' Downlink NAS count Tag '84' Identifiers of selected NAS integrity and encryption algorithms Tag | EPS NAS Security Context (EFEPSPSC) |
| 'A0' | CSG List TLV object tag The following tags are encapsulated within 'A0' '80' PLMN tag '81' CSG Information tag | Allowed CSG List (EFACSGL) |
| 'A0' | GSM cell information The following tags are encapsulated within 'A0': '80' GSM Camping Frequency Information data object '81' GSM Neighbour Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A0' | Operator CSG List TLV object Tag The following tags are encapsulated within 'A0' '80' PLMN Tag '81' CSG Information Tag '82' CSG Display indicator tag | Operator CSG Lists (EFOCSGL) |
| 'A0' | ProSe Discovery monitoring parameters The following tags are encapsulated within 'A0': '80' PLMN tag '81' RFU '82' Model tag | ProSe Monitoring Parameters (EFPROSE_MON) |
| 'A0' | ProSe Discovery announcing parameters The following tags are encapsulated within 'A0': '80' PLMN tag '81' Range tag '82' Model tag | ProSe Announcing Parameters (EFPROSE_ANN) |
| 'A0' | ProSe Policy parameters The following tags are encapsulated within 'A0': '80' ProSe Layer-2 Group ID tag '81' ProSe UE ID tag '82' ProSe Group IP multicast address tag '83' Address type tag '84' Ipv4 address as source tag '85' Group related security tag '86' Application Layer Group ID tag | ProSe Policy Parameters (EFPROSE_POLICY) |
| 'A0' | ProSe PLMN Parameters tag The following tags are encapsulated within 'A0' '80' PLMN tag '81' Direct communication authorisation tag | ProSe PLMN Parameters (EFPROSE_PRMN) |
| 'A0' | ProSe Direct Communication parameters tag The following tags are encapsulated within 'A0' '80' Geographical Area – Polygon tag '81' Radio parameters tag | ProSe Direct Communication Radio Parameters (EF PROSE_RADIO_COM) |
| 'A0' | ProSe Radio parameters tag The following tags are encapsulated within 'A0' '80' Geographical Area – Polygon tag '81' Radio parameters tag | ProSe Direct Discovery Monitoring Radio Parameters (EFPROSE_RADIO_MON) |
| 'A0' | ProSe Radio parameters tag The following tags are encapsulated within 'A0' '80' Geographical Area – Polygon tag '81' Radio parameters tag | ProSe Direct Discovery Announcing Radio Parameters (EFPROSE_RADIO_ANN) |
| 'A0' | ACDC OS tag | ACDC List (EFACDC_LIST) |
| 'A0' | ACDC App Id tag The following tags are encapsulated within 'A0' '80' ACDC category tag '81' OS App Id tag | ACDC OS Configuration (EFACDC_OS_CONFIG) |
| 'A0' | Group member discovery parameters tag The following tags are encapsulated within 'A0' '80' User Info ID tag '81' Discovery Group ID tag '82' Application Layer Group ID tag | ProSe Group Member Discovery Parameters (EFPROSE_GM_DISCOVERY) |
| 'A0' | ProSe Relay Parameters tag The following tags are encapsulated within 'A0' '80' PLMN tag '81' Relay type tag | ProSe Relay Parameters (EFPROSE_RELAY) |
| 'A0' | Remote UE parameters tag The following tags are encapsulated within 'A0' '80' Relay Service Code tag '81' User Info ID of Relay tag '82' IP Versions tag '83' Security content tag | ProSe Relay Discovery Parameters (EFPROSE_RELAY_DISCOVERY) |
| 'A0' | TMGI List Tag | TV Configuration (EFTVCONFIG) |
| 'A0' | USD Tag | TV User Service Description (EFTVUSD) |
| 'A0' | XCAP_conn_params_policy part tag The following tags are encapsulated within 'A0' '81' AccessForXCAP Tag '82' Number of XCAP connection parameters policy part TLV's Tag 'A1' XCAP connection parameters policy part tag | EFXCAPConfigData |
| 'A0' | EARFCN List tag The following tags are encapsulated within 'A0' '80' EARFCN tag '81' Geographical Area – Polygon tag | EARFCN list for MTC/NB-IOT UEs (EFEARFCNList) |
| 'A0' | 5GS 3GPP access NAS security Context tag or 5GS non-3GPP access NAS security Context tag The following tags are encapsulated within 'A0' '80' ngKSI Tag '81' KAMF Tag '82' Uplink NAS count Tag '83' Downlink NAS count Tag '84' Identifiers of selected NAS integrity and encryption algorithms Tag '85' Identifiers of selected EPS NAS integrity and encryption algorithms for use after mobility to EPS Tag | 5GS 3GPP Access NAS Security Context (EF5GS3GPPSNSC) or 5GS non-3GPP Access NAS Security Context (EF5GSN3GPPSNSC) |
| 'A0' | Protection Scheme Identifier List data object tag | Protection Scheme Identifier List (EFSUCI_Calc_Info) |
| 'A0' | V2X data policy over PC5 data object tag The following tags are encapsulated within 'A0' '80' Served by E-UTRA or served by NR Tag '81' Not Served by E-UTRA or not served by NR Tag '82' V2X service identifier to Tx profiles mapping rules Tag '83' Privacy config Tag '84' V2X communication over PC5 in E-UTRA Tag '85' V2X communication over PC5 in NR Tag | V2X data policy over PC5 (EFV2XP_PC5) |
| 'A0' | V2X data policy over Uu_data object tag The following tags are encapsulated within 'A0' '80' V2X service identifier to PDU session parameters mapping rules Tag '81' PLMN infos Tag | V2X data policy over PC5 (EFV2XP_Uu) |
| 'A0' | 5G ProSe configuration data for direct discovery Tag The following tags are encapsulated within 'A0' '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '82' ProSe identifiers Tag '83' ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules Tag '84' Group member discovery parameters Tag '85' Validity timer Tag '86' ProSe direct discovery UE ID Tag '98'….HPLMN 5G DDNMF address information Tag | 5G ProSe configuration data for direct discovery (EF5G_PROSE_DD) |
| 'A0' | 5G ProSe configuration data for direct communication Tag The following tags are encapsulated within 'A0' '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '87' Privacy config Tag '88' 5G ProSe direct communication in NR-PC5 Tag '89' ProSe application to path preference mapping rules Tag '85' Validity timer Tag '91'….ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules Tag | 5G ProSe configuration data for direct communication (EF5G_PROSE_DC) |
| 'A0' | 5G ProSe configuration data for UE-to-network relay UE Tag The following tags are encapsulated within 'A0' '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '8A' Default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation Tag '8B' RSC info list Tag '8C' 5QI to PC5 QoS parameters mapping rules Tag '8D' ProSe identifier to ProSe application server address mapping rules Tag '85' Validity timer Tag '8E' User info ID for discovery Tag '92' Privacy timer Tag '93' 5G PKMF address information Tag '9B' Warning message broadcast Tag | 5G ProSe configuration data for UE-to-network relay UE (EF5G_PROSE_U2NRU) |
| 'A0' | 5G ProSe configuration data for remote UE Tag The following tags are encapsulated within 'A0' '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '8F' Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information Tag '8B' RSC info list Tag '90' N3IWF selection information for 5G ProSe layer-3 remote UE Tag '85' Validity timer Tag '8E' User info ID for discovery Tag '93' 5G PKMF address information Tag '9B' Warning message broadcast Tag | 5G ProSe configuration data for remote UE (EF5G_PROSE_RU) |
| 'A0' | 5G ProSe configuration data for usage reporting information Tag The following tags are encapsulated within 'A0' '85' Validity timer Tag '94' Collection period Tag '95' Reporting window Tag '96' Reporting indicators Tag '97' 5G DDNMF CTF address for uploading the usage information reports Tag | 5G ProSe configuration data for usage reporting information (EF5G_PROSE_UIR) |
| 'A0' | PLMN 5MBS pre-configuration data object Tag The following tags are encapsulated within 'A0' '80' PLMN Tag '81' STMGI List Tag '82' NR-ARFCN List Tag 'A1' PLMN 5MBS pre-configuration data object Tag | 5MBS UE pre-configuration for (EF5MBSUECONFIG) |
| 'A0' | 5G ProSe configuration data for UE-to-UE relay UE Tag The following tags are encapsulated within 'A0' '85' Validity timer Tag '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '99' Default destination layer-2 IDs for sending the discovery signalling for announcement and for receiving the discovery signalling for solicitation Tag '8B' RSC info list Tag '8E' User info ID for discovery Tag '9A' Default destination layer-2 ID(s) for sending or receiving the direct communication for 5G ProSe UE-to-UE relay communication with integrated discovery Tag '93' 5G PKMF address information Tag | 5G ProSe configuration data for UE-to-UE relay UE (EF5G_PROSE_U2URU) |
| 'A0' | 5G ProSe configuration data for end UE Tag The following tags are encapsulated within 'A0' '85' Validity timer Tag '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '9B' Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement Tag '8E' User info ID for discovery Tag '8B' RSC info list Tag '9A' Default destination layer-2 ID(s) for sending or receiving the direct communication for 5G ProSe UE-to-UE relay communication with integrated discovery Tag '93' 5G PKMF address information Tag | 5G ProSe configuration data for end UE (EF5G_PROSE_EU) |
| ‘A0’ | A2X configuration data Tag The following tags are encapsulated within 'A0' '80' UE policy part contents Tag | A2X configuration data (EFA2X_CONFIG) |
| 'A0' | A2X data policy over PC5 Tag The following tags are encapsulated within 'A0' '80' Served by NG-RAN Tag '81' Not served by NG-RAN Tag '82' A2X service identifier to PC5 RAT(s) and Tx profiles mapping rules Tag '83' Privacy config Tag '84' A2X communication in E-UTRA-PC5 Tag '85' A2X communication in NR-PC5 Tag | A2X policy data over PC5 (EFA2XP_PC5) |
| 'A0' | A2X Direct Detect And Avoid policy data over PC5 Tag The following tags are encapsulated within 'A0' '80' A2X service identifiers for unicast communication mode of DAA deconfliction Tag '81' A2X service identifiers for broadcast communication mode of DAA deconfliction Tag | A2X Direct Detect And Avoid policy data over PC5 (EFA2X_DDAAP_PC5) |
| 'A0' | A2X Direct C2 communication policy data over PC5 Tag The following tags are encapsulated within ‘A0’ ‘80’ Served by NG-RAN Tag ‘81’ Not served by NG-RAN Tag | A2X Direct C2 communication policy data over PC5 (EFA2X_DC2P_PC5) |
| ‘A0’ | A2X data policy over Uu Tag The following tags are encapsulated within 'A0' '80' A2X service identifier to PDU session parameters mapping rules Tag '81' PLMN infos Tag | A2X policy data over Uu (EFA2XP_Uu) |
| 'A1' | XCAP connection parameters policy part tag The following tags are encapsulated within 'A0' '81' Access Tag '82' Application name Tag '83' Provider ID Tag '84' URI Tag '85' XCAP Aithentication User Name Tag '86' XCAP Authentication password Tag '87'…XCAP Authentication type Tag '88'…Address type Tag '89'…Address Tag '8A'…PDP Authentication type Tag '8B'…PDP Authentication Name Tag | EFXCAPConfigData |
| 'A1' | FDD cell information The following tags are encapsulated within 'A1': '80' FDD Intra Frequency Information data object '81' FDD Inter Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A1' | Relay parameters tag The following tags are encapsulated within 'A0' '80' Relay Service Code tag '81' PDN type tag '82' APN tag '83' ProSe Relay UE ID tag '84' Security content tag | ProSe Relay Discovery Parameters (EFPROSE_RELAY_DISCOVERY) |
| 'A1' | EARFCN List Tag | TV Configuration (EFTVCONFIG) |
| 'A1' | SUCI TLV data object tag | Response to GET IDENTITY |
| 'A1' | Home Network Public Key List data object The following tags are encapsulated under 'A1' '80' Home Network Public Key Identifier tag '81' Home Network Public Key tag | Home Network Public Key List (EFSUCI_Calc_Info) |
| 'A1' | PDU Info List data object Tag The following tags are encapsulated within 'A1': '83' DNN Tag '84' S-NSSAI Tag | 5MBS UE pre-configuration for (EF5MBSUECONFIG) |
| 'A2' | TDD frequency information The following tags are encapsulated within 'A2': '80' TDD Intra Frequency Information data object '81' TDD Inter Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A3' | Service provider display information The following tags are encapsulated within 'A3': '80' Service provider PLMN list | Service Provider Display Information (EFSPDI) |
| 'A8' | Indicator for type 1 EFs (amount of records equal to master EF) The following tags are encapsulated within 'A8': 'C0' EFADN data object 'C1' EFIAP data object 'C3' EFSNE data object 'C4' EFANR data object 'C5' EFPBC data object 'C6' EFGRP data object 'C9' EFUID data object 'CA' EFEMAIL data object 'CC' EFPURI data object | Phone Book Reference File (EFPBR) |
| 'A9' | Indicator for type 2 EFs (EFs linked via the index administration file) The following tags are encapsulated within 'A9': 'C3' EFSNE data object 'C4' EFANR data object 'CA' EFEMAIL data object 'CC' EFPURI data object | Phone Book Reference File (EFPBR) |
| 'AA' | Indicator for type 3 EFs (EFs addressed inside an object using a record identifier as a pointer) The following tags are encapsulated within 'AA': 'C2' EFEXT1 data object 'C7' EFAAS data object 'C8' EFGAS data object 'CB' EFCCP1 data object | Phone Book Reference File (EFPBR) |
| 'AB' | MMS Connectivity Parameters: The following are encapsulated under 'AB': '80' MMS Implementation Tag '81' MMS Relay/Server Tag '82' Interface to core network and bearer Tag '83' Gateway Tag '84' Reserved for 3GPP2: MMS Authentication Mechanism Tag '85' Reserved for 3GPP2: MMS Authentication User Name Tag | MMS Connectivity Parameters (EFMMSICP / EFMMSUCP) |
| 'DB' | Successful 3G authentication | Response to AUTHENTICATE |
| 'DB' | Successful VGCS/VBS operation authentication tag | Response to AUTHENTICATE |
| 'DB' | Successful GBA operation tag | Response to AUTHENTICATE |
| 'DC' | Synchronisation failure | Response to AUTHENTICATE |
| 'DD' | Access Point Name | APN Control List (EFACL) |
| 'DD' | GBA Security Context Bootstrapping Mode tag | AUTHENTICATE command parameter, in GBA security context |
| 'DE' | GBA Security Context NAF Derivation Mode tag | Response to AUTHENTICATE |

NOTE:	the value 'FF' is an invalid tag value. For ASN.1 tag assignment rules see ISO/IEC 8825-1 [35].

* * * End of Changes * * * *