3GPP TSG-CT6 Meeting #84	C6-1700XX

# Zhangjiajie, China, 16th May 2017 - 19th May 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 045X | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of E-UTRAN test sequences under cl. 27.22.7.4 and 27.22.7.17 to test NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT-CT |  |  |  |  |  | Date: |  |  | 2017-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Modification of some existing E-UTRAN test cases to verify NB-IoT implementations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases under 27.22.7.4 and 27.22.7.17 have been modified |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Features are not tested for NB-IoT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.7.4 and 27.22.7.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Table of optional features

Support of USIM Application Toolkit is optional for Mobile Equipment. However, if an ME states conformance with a specific 3GPP release, it is mandatory for the ME to support all functions of that release, as stated in table B.1, with the exception of the functions:

-	"Alpha identifier in REFRESH command supported by terminal";

-	"Event Language Selection";

-	"Proactive UICC: PROVIDE LOCAL INFORMATION (language)"; and

-	"Proactive UICC: LANGUAGE NOTIFICATION".

The support of letter classes, which specify mainly ME hardware dependent features, is optional for the ME and may supplement the USIM Application Toolkit functionality described in the present document. If an ME states conformance to a letter class, it is mandatory to support all functions within the respective letter class.

The supplier of the implementation shall state the support of possible options in table A.1.

Table A.1: Options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
|  | [….] |  |  |  |
| 18 | Mobile supporting TCP | O |  | O_TCP |
|  | […..] |  |  |  |
| 132 | Class E: Terminal does support eFDD | O |  | pc_BIP_eFDD |
| 133 | Class E: Terminal does support eTDD | O |  | pc_BIP_eTDD |
|  | [xxxx] |  |  |  |
| 173 | Terminal does support NB-IoT | O |  | pc_NB |
| 174 | Terminal supports EMM-REGISTERED with PDN | O |  | pc_AttachWithPDN |

## 3.4	Applicability table

Table B.1: Applicability of tests

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 27.22.7.4: location status event | R99 | 1.1 | M | M | M | M | M | M | M | M | M | M | M | E.1/37 AND E.1/33 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.4: location status event, E-UTRAN | Rel-8 | 1.2 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | E.1/37 AND E.1/33 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | [………] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27.22.7.12: Access Technology change event |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Single access technology | Rel-8 | 1.1 |  |  |  |  |  | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | E.1/45 AND E.1/33 | UMTS System Simulator and E-USS |  |  |
|  | 27.22.7.17: Network Rejection Event, ATTACH REJECT | Rel-8 | 1.1 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | E.1/33 AND E.197 | E-USS or NB-SS (See NOTE) |  |  |
|  | 27.22.7.17: Network Rejection Event, TRACKING AREA UPDATE REJECT | Rel-8 | 1.2 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | E.1/33 AND E.197 | E-USS or NB-SS (See NOTE) |  |  |
|  | [….] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| C182 | IF A.1/18 AND (A.1/132 OR A.1/133) THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD) |
| --- | --- | --- |
| C220 | IF ((A.1/139 OR A.1/140 OR A.1/173) AND A.1/153) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MO |
| C221 | IF ((A.1/139 OR A.1/140 OR A.1/173) AND A.1/152) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MT |
| C222 | IF (A.1/139 OR A.1/140 OR A.1/173) THEN M ELSE N/A | -- pc_eTDD OR pc_eFDD OR pc_NB |
| [….] |  |  |

#### 27.22.7.4	Location Status Event

##### 27.22.7.4.1	Location Status Event (normal)

27.22.7.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.4.1.2	Conformance requirement

The ME shall support the EVENT: Location Status event as defined in:

-	TS 31.111 [15] clause 5.2, 7.5 and clause 6.4.16

and

-	UTRAN/GERAN for sequence 1.1

-	E-UTRAN (WB-S1 mode and NB-S1 mode) for sequence 1.2.

27.22.7.4.1.3	Test purpose

To verify that the ME informs the UICC that an Event: MM_IDLE state has occurred using the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

To verify that the ME supporting E-UTRAN/EPC informs the UICC that an Event: EMM_IDLE state has occurred using the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

To verify that the ME supporting E-UTRAN/EPC correctly encodes the E-UTRAN Cell Id in the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

27.22.7.4.1.4	Method of test

27.22.7.4.1.4.1	Initial conditions

For sequence 1.1 the ME is connected to the USIM Simulator and the USS.

The elementary files are coded as the USIM Application Toolkit default.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01 ;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

Two cells are defined. Cell 1 has location area code 1 and cell 2 has location area code 2.

MS is in service on Cell 1.

For sequence 1.2 the ME is connected to the USIM Simulator and the E-USS/NB-SS.

The default E-UTRAN/EPC UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E-UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

For cell 1:

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Id  = 0001 (28 bits);

For cell 2:

-	Tracking Area Code (TAC) = 0002;

-	E-UTRAN Cell Id  = 0002 (28 bits).

The NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

For cell 1:

-	Tracking Area Code (TAC) = 0001;

-	NB-IoT Cell Id  = 0001 (28 bits);

For cell 2:

-	Tracking Area Code (TAC) = 0002;

-	NB-IoT Cell Id  = 0002 (28 bits).

27.22.7.4.1.4.2	Procedure

Expected Sequence 1.1(EVENT DOWNLOAD -LOCATION STATUS)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | IF A.1/171 THEN ME sends a ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.1A [apply for GERAN/UTRAN parameters] or ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.1B [apply for PCS1900 parameters]. |
| 5 | USS | Cell 1 is switched off |  |
| 6 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.1 |  |
| 7 | USS | Cell 2 is switched on after Location Status “No service” has been received in step 6 |  |
| 8 | ME | ME performs cell reselection to cell 2 |  |
| 9 | ME  USS | LOCATION UPDATING REQUEST or ROUTING AREA UPDATE REQUEST | The ME is CS and/or PS registered depending on its capabilities |
| 10 | USS  ME | LOCATION UPDATING ACCEPT or ROUTING AREA UPDATE ACCEPT |  |
| 11 | ME  USS | TMSI REALLOCATION COMPLETE or ROUTING AREA UPDATE COMPLETE |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.2A or ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.2B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] [Note: The inclusion of the location information is optional: (If location status indicates normal status) |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Location status

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 03 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.1.1

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	No service

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.1.1A

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID			RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |

Note 1: Depending on the presence of the Extended Cell Identity Value the length is '13' or '15'

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

EVENT DOWNLOAD - LOCATION STATUS 1.1.1B

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D6 | 13 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |  |

EVENT DOWNLOAD - LOCATION STATUS 1.1.2A

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0002)

Cell ID	Cell Identity Value (0002)

Extended Cell ID			RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 2 | 00 | F1 | 10 | 00 | 02 | 00 | 02 | Note 3 |  |  |

Note 1: Depending on the presence of the Extended Cell Identity Value the length is '13' or '15'

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

EVENT DOWNLOAD - LOCATION STATUS 1.1.2B

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0002)

Cell ID	Cell Identity Value (0002)

Coding:

| BER-TLV: | D6 | 13 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 02 | 00 | 02 |  |  |  |

Expected Sequence 1.2 (EVENT DOWNLOAD -LOCATION STATUS, E-UTRAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME is registered to cell one and in EMM_IDLE |  |
| 2 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 3 | ME  UICC | FETCH |  |
| 4 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 5 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | IF A.1/171 THEN ME sends a ENVELOPE: EVENT DOWNLOAD - Location Status 1.2.1A |
| 6 | E-USS/NB-SS | Cell 1 is switched off |  |
| 7 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.2.1 |  |
| 8 | E-USS(NB-SS | Cell 2 is switched on after Location Status “No service” has been received in step 7 |  |
| 9 | ME | ME performs cell reselection to cell 2 |  |
| 10 | ME  E-USS/NB-SS | ME performs EPS ATTACH or TRACKING AREA UPDATE procedure | [E-UTRAN/NB-IoT cell 2 accepts] |
| 11 | ME | ME reaches EMM_IDLE state |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.2.2 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in sequence 1.1

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Same as Terminal Response: SET UP EVENT LIST 1.1.1 in sequence 1.1

EVENT DOWNLOAD - LOCATION STATUS 1.2.1

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	No service

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.2.1A

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

TAC	0001

E-UTRAN cell id:	0001 (28bits)

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 00 | F1 | 10 | 00 | 01 | 00 | 00 | 00 | 1F |  |

EVENT DOWNLOAD - LOCATION STATUS 1.2.2

Logically:

Event list: 	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

TAC	0002

E-UTRAN cell id:	0002 (28bits)

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 00 | F1 | 10 | 00 | 02 | 00 | 00 | 00 | 2F |  |

27.22.7.4.1.5	Test requirement

The behaviour of the test shall be as defined in expected sequences 1.1 and 1.2.

##### [……..]

#### 27.22.7.17	Network Rejection Event

27.22.7.17.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.17.1.2	Conformance requirement

The ME shall support the EVENT: Network Rejection event E-UTRAN as defined in:

-	TS 31.111 [15] clause 4.7, 5.2, 7.5.2, 8.62 and clause 8.99.

27.22.7.17.1.3	Test purpose

To verify that the ME informs the UICC with the Event Network Rejection about the Network Rejection.

To verify that the Rejection Cause Code sent to the UICC is the value from the EMM cause information element received from the E-UTRAN.

To verify that the correct Access Technology is indicated ENVELOPE: EVENT DOWNLOAD – Network Rejection after the unsuccessful attempt to access the E-UTRAN.

To verify that the correct Update/Attach Type is indicated ENVELOPE: EVENT DOWNLOAD – Network Rejection.

27.22.7.17.1.4	Method of test

27.22.7.17.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS.

The default E-UTRAN/EPC UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E-UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

The NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

27.22.7.17.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD – Network Rejection, ATTACH REJECT)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | E-USS/NB-SS | No E-UTRAN/NB-IoT available |  |
| 2 | USER  ME | Switch on the terminal |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 7 | E-USS/NB-SS | The E-UTRAN/NB-IoT cell is switched on |  |
| 8 | USERME | The terminal is made to start a registration attempt to the E-USS/NB-SS |  |
| 9 | MEE-USS/NB-SS | The terminal requests RRC CONNECTION and therefore starts the EPS Attach procedure |  |
| 10 | E-USS/NB-SSME | The E-USS/NB-SS sends EMM ATTACH REJECT with cause "PLMN not allowed" |  |
| 11 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.1.1 or 1.1.2 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Network Rejection

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 12 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.1.1

Logically:

Event list: 	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	EPS Attach

Rejection Cause Code:	PLMN not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 09 | 75 | 01 |
|  | 0B |  |  |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.1.2

Logically:

Event list: 	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	Combined EPS/IMSI Attach

Rejection Cause Code:	PLMN not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 0A | 75 | 01 |
|  | 0B |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.2 (EVENT DOWNLOAD – Network Rejection, TRACKING AREA UPDATE REJECT)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME is registered to the E-USS/NB-SS and in EMM_IDLE | The E-USS/NB-SS transmits on cell 1 : MCC: 001 MNC: 01 TAC: 0003 |
| 2 | E-USS/NB-SS | Cell 1 is switched off |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 7 | E-USS/NB-SS | The E-UTRAN/NB-IoT cell 2 is switched on | The E-USS/NB-SS transmits on cell 2 : MCC: 001 MNC: 01 TAC: 0001 |
| 8 | ME | The terminal is made to start a re-registration attempt to the E-USS/NB-SS |  |
| 9 | MEE-USS/NB-SS | The terminal send TRACKING AREA UPDATE REQUEST |  |
| 10 | E-USS/NB-SS ME | The E-USS/NB-SS sends TRACKING AREA UPDATE REJECT with cause "TRACKING AREA not allowed" |  |
| 11 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.2.1 or 1.2.2 |  |

EVENT DOWNLOAD –Network Rejection 1.2.1

Logically:

Event list: 	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	TA Updating

Rejection Cause Code:	Tracking Area not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 0B | 75 | 01 |
|  | 0C |  |  |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD –Network Rejection 1.2.2

Logically:

Event list: 	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	Combined TA/LA updating

Rejection Cause Code:	Tracking Area not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 0C | 75 | 01 |
|  | 0C |  |  |  |  |  |  |  |  |  |  |  |

27.22.7.17.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 and 1.2.