### 3GPP TSG-CT6 Meeting #85							C6-170485

# Krakow , Poland, 22nd Aug 2017 - 25th Aug 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0476 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new test case for Call Control on PS PDP connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Gemalto, COMPRION |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  | Date: |  |  | 2017-08-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | There is currently no PDP connection test case for Call Control. Introduction of new test case for Call Control on PS PDP connection |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case 27.22.X for Call Control on PS PDP connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call Control on PS PDP connection features cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.4	Applicability table

Table B.1: Applicability of tests

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | [ …………..] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CALL CONTROL on EPS PDN - PDN connection activation from OPEN CHANNEL command | R99 | 1.7 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C182 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | E-USS only |  |  |
| X | Call Control on PS PDP connection 27.22.X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CALL CONTROL on PS PDP – default PDP connection activation, allowed without modification | R99 | 1.1 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PS PDP – default PDP connection activation, not allowed | R99 | 1.2 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PS PDP – default PDP connection activation, allowed with modification | R99 | 1.3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PS PDP – PDP connection triggered by user, UICC sends 90 00 | R99 | 1.4 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PS PDP – PDP connection triggered by user, UICC sends 93 00 | R99 | 1.5 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PS PDP – PDP connection triggered by user, allowed with modification | R99 | 1.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PS PDP - PDP connection activation from OPEN CHANNEL command | R99 | 1.7 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C191 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |

[…….]

### 27.22.X	CALL CONTROL on PS PDP Connection

#### 27.22.X.1	Procedure for Mobile Originated calls

##### 27.22.X.1.1	Definition and applicability

See clause 3.2.2.

##### 27.22.X.1.2	Conformance requirement

The ME shall support the CALL CONTROL for PDP Context Activation facility as defined in:

-	TS 31.111 [15] clause 7.3, clause 7.3.1.7, clause 8.72

-  	TS 24.008 [10], clause 6.1.3.3, 9.5.7 and 9.5.8

-  	TS 36.508 [33], clause 4.8.4.

##### 27.22.X.1.3	Test purpose

To verify that when the service "call control on GPRS by USIM" is available in the USIM Service Table, then for all PS PDP connection activation (including those resulting from an OPEN CHANNEL proactive UICC command where UTRAN is selected), the ME shall first pass the corresponding PDP Connectivity Request message to the UICC, using the ENVELOPE (CALL CONTROL) command. The ME shall also pass to the UICC in the ENVELOPE (CALL CONTROL) command the current serving cell.

To verify that the ME interpret the UICC returns response correctly.

##### 27.22.X.1.4	Method of tests

27.22.X.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS/SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default UTRAN/EPC UICC, the default UTRAN parameters and the following parameters are used:

Network access name: 		TestGp.rs

User login:			UserLog

User password:			UserPwd

UICC/ME interface transport level

Transport format:		TCP

Port number: 		44444

Data destination address:	01.01.01.01 (as an example)

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity Value = 0001;

-	The simulator must accept connections requests for APNs: TestGp.rs, Test12.rs and Test13.rs

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

- The call control on GPRS by USIM service is available in the USIM Service Table.

##### 27.22.X.4.2	Procedure

Expected Sequence 1.1 (CALL CONTROL on PS PDP – default PDP connection activation, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN “TestGp.rs” in the terminal configuration if required. | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure |
| 2 | UICC ME | CALL CONTROL RESULT 1.1.1 | [Call control result: "Allowed", no modification] |
| 3 | ME USS/SS | The PDP connection is established successfully without modification | Same PDP activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection |

ENVELOPE CALL CONTROL 1.1.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

PDP Context Activation parameters

Protocol Discriminator:                         		group call control

Procedure Transaction Identity:    		1

Request PDP context activation message identity: IMSI DETACH INDICATION

Requested NSAPI: 					NSAPI 5

Requested LLC SAPI: 				SAPI 3

Requested QoS: 					Subscribed QoS parameters

Offered PDP address:

PDP type organisation: 					as declared by the ME

PDP type: 					as declared by the ME

Address: 					as declared by the ME

Access point name: 					09 54 65 73 74 47 50 2e 72 73 (length + "TestGP.rs")

Protocol configuration options:

Protocol config. optional contents: 		content not checked

Location Information

MCC:	                      		001

MNC:	               		       	01

Location Area Code:           	       		0001

Cell Identity Value:               			0001

Extended Cell Identity Value: 			RNC-id value (for Rel-4 onwards), see also Note 6

Coding:

| BER-TLV | D4 | Note1 | 02 | 02 | 82 | 81 | 52 | Note2 | 0A | 41 | 53 | 0E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | 00 | Note3 | 28 | 09 | 08 | 69 | 6E | 74 | 65 | 72 | 6E |
|  | 65 | 74 | Note4 | 13 | Note 5 | 00 | F1 | 10 | 00 | 01 | 00 | 01 |
|  | Note 6 |  |

Note 1: Length of BER-TLV, dependant of optional fields.

Note 2: Length of PDP context activation parameters, dependant of optional fields.

Note 3: The PDP Type.

Note 4: Optional fields.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 1.1.1

Logically:

Call control result:	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

Expected Sequence 1.2 (CALL CONTROL on PS PDP – default PDP connection activation, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER  ME | Set and configure APN “Test.Gp.rs” in the terminal configuration if required | [see initial conditions] |
| 1 | ME  UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure |
| 2 | UICC  ME | CALL CONTROL RESULT 1.2.1 | [Call control result: " not Allowed", ] The ME may retry to send the command. |
| 3 | ME  USS/SS | The ME shall not send the PDN Connectivity Request message. |  |

CALL CONTROL RESULT 1.2.1

Logically:

Call control result:	'01' = not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

Expected Sequence 1.3 (CALL CONTROL on PS PDP – default PDP connection activation, allowed with modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure |
| 2 | UICC ME | CALL CONTROL RESULT 1.3.1 | [Call control result: "Allowed with modifications"] |
| 3 | ME USS/SS | The PDP connection is established successfully with modification | Same PDP activation parameters returned by the UICC within the CALL CONTROL RESULT 1.3.1 are used to establish the PDP connection |

CALL CONTROL RESULT 1.3.1

Logically:

Call control result:	'02' = Allowed with modifications

Address:

PDP Context Activation parameters

Protocol Discriminator:                         		group call control

Procedure Transaction Identity:    		2

Request PDP context activation message identity: IMSI DETACH INDICATION

Requested NSAPI: 					NSAPI 5

Requested LLC SAPI: 				SAPI 3

Requested QoS: 					Subscribed QoS parameters

Offered PDP address:

PDP type organisation: 			as declared by the ME

PDP type: 					as declared by the ME

Address: 					as declared by the ME

Access point name: 					09 54 65 73 74 31 32 2e 72 73 (length + "Test12.rs")

Coding:

| BER-TLV: | 02 | Note1 | 41 | 53 | 0E | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | Note2 |
|  | 28 | 0A | 09 | 54 | 65 | 73 | 74 | 31 | 32 | 2E |
|  | 72 | 73 | Note 3 |

Note 1: Length dependant of optional fields.

Note2: The PDP Type.

Note3: Optional fields.

Expected Sequence 1.4 (CALL CONTROL on PS PDP – PDP connection triggered by user, UICC sends 90 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN “Test.Gp.rs” in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure Same PDP activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection. |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to establish a PDP connection | [see initial conditions] |
| 4 | ME USS/SS | PDP CONNECTIVITY REQUEST |  |
| 5 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 6 | UICC ME | 90 00 |  |
| 7 | USS/SS ME | ACTIVATE DEFAULT PS BEARER CONTEXT REQUEST | [The UTRAN parameters are used] |
| 8 | ME USS/SS | ACTIVATE DEFAULT PS BEARER CONTEXT ACCEPT |  |
| 9 | ME USS/SS | The PDP connection is established successfully without modification | Same PDP activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.4.1 are used to establish the PDN connection |

ENVELOPE CALL CONTROL 1.4.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

PS PDP connection activation parameters

Protocol Discriminator:                         		group call control

Procedure Transaction Identity:                 		2

Request PDP context activation message identity: 	IMSI DETACH INDICATION

Requested NSAPI: 					NSAPI 5

Requested LLC SAPI: 					SAPI 3

Requested QoS: 					Subscribed QoS parameter

Offered PDP address:

PDP type organisation: 			as declared by the ME

PDP type: 					as declared by the ME

Address: 					as declared by the ME

Access Point Name:					09 54 65 73 73 31 32 2E 72 73 (length + "Test12.rs")

Other Protocol configuration options:

Protocol config. options contents:			not checked

Location Information

MCC:	                      		001

MNC:	               		       	01

Location Area Code:           	       		0001

Cell Identity Value:               			0001

Extended Cell Identity Value: 			RNC-id value (for Rel-4 onwards), see also Note 6

Coding:

| BER-TLV | D4 | Note 1 | 02 | 02 | 82 | 81 | 52 | Note 2 | 0A | 41 | 53 | 0E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | 00 | Note 3 | 28 | 0A | 09 | 54 | 65 | 73 | 74 | 31 | 32 |
|  | 2E | 72 | 73 | Note 4 | 13 | Note 5 | 00 | F1 | 10 | 00 | 01 | 00 |
|  | 01 | Note 6 |  |

Note 1: Length of BER-TLV, dependant of optional fields.

Note 2: Length of PDP context activation parameters, dependant of optional fields.

Note 3: The PDP Type.

Note 4: Optional fields.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

Expected Sequence 1.5 (CALL CONTROL on PS PDP – PDP connection triggered by user, UICC sends 93 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN “Test.Gp.rs” in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure. Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection. |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to establish a PDN connection | [see initial conditions] |
| 4 | ME USS/SS | PDP CONNECTIVITY REQUEST |  |
| 5 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 6 | UICC ME | 93 00 | The ME may retry to send the command. |
| 7 | ME USS/SS | The ME shall not send the PDN Connectivity Request message. |  |

Expected Sequence 1.6 (CALL CONTROL on PS PDP for UTRAN – PDP connection triggered by user, allowed with modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure Same PDP parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDP connection. |
| 2 | ME UICC | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to establish a PDP connection | [see initial conditions] |
| 4 | ME USS/SS | PDN CONNECTIVITY REQUEST |  |
| 5 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 6 | UICC ME | CALL CONTROL RESULT 1.6.1 | [Call control result: "Allowed with modifications", ] |
| 7 | USS/SS ME | ACTIVATE PS BEARER CONTEXT REQUEST | [The UTRAN parameters are used] |
| 8 | ME USS/SS | ACTIVATE PS BEARER CONTEXT ACCEPT |  |
| 9 | ME USS/SS | The PDP connection is established successfully with modification | Same PDP parameters returned by the UICC within the CALL CONTROL RESULT 1.6.1 are used to establish the PDP connection. |

CALL CONTROL RESULT 1.3.1

Logically:

Call control result:	'02' = Allowed with modifications

Address:

PDP Context Activation parameters

Protocol Discriminator:                         		group call control

Procedure Transaction Identity:    		2

Request PDP context activation message identity: IMSI DETACH INDICATION

Requested NSAPI: 					NSAPI 5

Requested LLC SAPI: 				SAPI 3

Requested QoS: 					Subscribed QoS parameters

Offered PDP address:

PDP type organisation: 			as declared by the ME

PDP type: 					as declared by the ME

Address: 					as declared by the ME

Access point name: 	09 54 65 73 74 31 33 2e 72 73 (length + "Test13.rs")

Coding:

| BER-TLV: | 02 | Note1 | 41 | 53 | 0E | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | Note 2 |
|  | 28 | 0A | 09 | 54 | 65 | 73 | 74 | 31 | 33 | 2E |
|  | 72 | 73 | Note 3 |

Note 1: Length dependant on optional fields.

Note1: The PDP Type.

Note2: Optional fields.

Expected Sequence 1.7 (CALL CONTROL on PS PDP - PDP connection activation from OPEN CHANNEL command)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure Same PDP parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDP connection. |
| 2 | UICC ME | 90 00 |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 1.1.1 |  |
| 6 | ME USER | The ME may display channel opening information |  |
| 7 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 8 | UICC ME | 90 00 |  |
| 9 | ME USS/SS | PDP CONNECTIVITY REQUEST |  |
| 10 | USS/SS ME | ACTIVATE DEFAULT PS BEARER CONTEXT REQUEST | [The UTRAN parameters are used] |
| 11 | ME USS/SS | ACTIVATE DEFAULT PS BEARER CONTEXT ACCEPT |  |
| 12 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 1.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 1.1.1B | [Command performed successfully OR Command performed with modifications] |
| 13 | ME USS/SS | The PDP connection is established successfully without modification | Same PDP parameters used by the ME within the ENVELOPE CALL CONTROL 1.4.1 are used to establish the PDP connection. |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:			OPEN CHANNEL

Command qualifier:			immediate link establishment

Device identities

Source device:			UICC

Destination device:			ME

Bearer

Bearer type:			GPRS / UTRAN packet service / E-UTRAN

Precedence Class:			03

Delay Class:			04

Reliability Class:			02

Peak throughput class: 	09

Mean throughput class:	31

Packet data protocol:			02 (IP)

Buffer

Buffer size:			1400

Network access name: 		TestGp.rs

Text String:				"UserLog" (User login)

Text String:				"UserPwd" (User password)

UICC/ME interface transport level

Transport format:			TCP

Port number: 			44444

Data destination address			01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:			1

Command type:			OPEN CHANNEL

Command qualifier:			immediate link establishment

Device identities

Source device:			ME

Destination device:			UICC

Result

General Result:			Command performed successfully

Channel status			Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:			GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:			IP (Internet Protocol, IETF STD 5)

Precedence Class:			03

Delay Class:			04

Reliability Class:			02

Peak throughput class: 	09

Mean throughput class:	31

Packet data protocol:			02 (IP)

Buffer

Buffer size:			1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:			1

Command type:			OPEN CHANNEL

Command qualifier:			immediate link establishment

Device identities

Source device:			ME

Destination device:			UICC

Result

General Result:			Command performed with modifications

Channel status			Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:			GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:			IP (Internet Protocol, IETF STD 5)

Precedence Class:			03

Delay Class:			04

Reliability Class:			02

Peak throughput class: 	09

Mean throughput class:	31

Packet data protocol:			02 (IP)

Buffer

Buffer size:			1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

#### 27.22.X.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.